3GPP TSG-RAN WG4 Meeting #112bis                                      R4-2416184

Hefei, Anhui, China, October 14 - 18, 2024

Source: 	Huawei, HiSilicon

Title: 	On RF requirements for Rel-19 NR-MIMO

Agenda Item:	6.19.2

Document for:	Approval

Introduction

In previous meeting, RAN4 initially discussed on this WI. The progress can be found in [1].

Issue 1-3: UE RF impact of asymmetric DL sTRP/UL mTRP - Proposals - Option 1: Identified (e.g., spherical coverage with a different TCI state for UL compared to DL) - Option 2: Not yet. Wait for further progress in RAN1 - Way forward - Companies are encouraged to provide clear view based on WID and RAN1 status - <Topic 2> 3Tx Issue 2-1: Applicable UE types - Proposals - Option 1: Handheld UE included - Option 2: Non-handheld UE only - Option 3: Depend on condition (e.g., ULFPTx) - Agreement - Both handheld and non-handheld UEs will be considered for UE RF requirements - FFS on whether a single set or two sets of UE RF requirements, e.g., MPR, will be specified. Issue 2-2: Applicable Frequency Ranges - Proposals - Option 1: FR1 only - Option 2: Others - Agreement - Both FR1 and FR2 will be considered for potential UL mTRP requirements - Only FR1 is considered for 3Tx objective Issue 2-3: Power classes and PA configuration for 3Tx UL MIMO - Proposals - Option 1: PC3 and PC2 - Option 2: PC1.5 only - Option 3: Others - Way forward - Companies are encouraged to provide clear view on 3Tx power class and its PA assumptions Issue 2-5: Non ULFPTx mode - Proposals - Option 1: 3-layer with precoding matrix of W=13100010001. - Option 2: Others - Agreement - 3-layer with precoding matrix of W=13100010001. Issue 2-6: ULFPTx mode 0 - Proposals - Option 1: Add both 1-layer and 2-layer for 3Tx with - Option 2: Add 1-layer for 3Tx with W=13100,13010,13001 - Tentative agreement - Take 1-layer as baseline - Further discuss whether 2-layer is needed

In RAN#105, the WI scope was updated as in [2]. In this contribution, we would like to provide our views on this topic.

Discussion

On FR1 3Tx operation

Based on the WF, following aspects on FR1 3Tx operation need further discussions:

- UE architecture – Power class and corresponding PA configuration

- MPR requirement for 3Tx

- ULFPTx (mode 0) requirement for 3Tx

- Power class, PA configuration and MPR

In terms of power class, it is notably that only PC1.5 associated requirements were specified for 4Tx towards non-handheld UE in Rel-18. Although in Rel-19 it was agreed that both handheld and non-handheld UEs would be considered for 3Tx, it can be expected that the commercialization of this feature especially for handheld UE are not coming too soon. Thus for the sake of balancing the standardization efforts, we prefer to preclude PC1.5 during the analysis under this RAN1 leading WI.

Proposal 1: For Rel-19 RAN4 3Tx discussion under Rel-19 NR_MIMO_Ph5 WI, suggest to preclude PC1.5 for both handheld and non-handheld UEs.

Technically, the respective achievable output power can be listed in the following table by taking advertised power class, whether ULFPTx mode 0 is configured and the non-zero port number into account. Note that the RAN1 “equal power split among non-zero ports” principle as defined in TS 38.213 is considered when ULFPTx mode 0 is not configured.

| Power class | Non-zero port number | Achievable output power w/o ULFPTx mode 0 | Achievable output power with ULFPTx mode 0 |
| --- | --- | --- | --- |
| PC3 | 1 | 18.3dBm | 23dBm |
|  | 2 | 21.3dBm |  |
|  | 3 | 23dBm |  |
| PC2 | 1 | 21.3dBm | 26dBm |
|  | 2 | 24.3dBm |  |
|  | 3 | 26dBm |  |

In this sense, possible PA configurations can be summarized as in below table.

| Power class | PA configuration (with Mode 0) | PA configuration (w/o Mode 0) |
| --- | --- | --- |
| PC3 | 3x23dBm | 3x23dBm |
| PC2 | 3x26dBm | 3x26dBm 23dBm+2x26dBm 2x23dBm+26dBm |

While the PA configuration that can be assumed for PC3 3Tx UE is straightforward, at least for handheld UE, we think 23dBm+2x26dBm or 2x23dBm+26dBm is more realistic to be considered for future study on PC2 3Tx UE.

As for 3Tx MPR requirements, the PC3 MPR requirement specified in TS 38.101-1 Table 6.2.2-1 and the PC2 dual Tx MPR requirement specified in TS 38.101-1 Table 6.2D.2-1 can be used as starting point, respectively. Necessary relaxation on PC2 can be further discussed when that is identified.

Proposal 2: The PA configuration assumption for 3Tx operation can be:

- 3x23dBm for PC3 handheld UE

- 23dBm+2x26dBm or 2x23dBm+26dBm for PC2 handheld UE

Proposal 3: For 3Tx MPR requirements, the PC3 MPR requirement specified in TS 38.101-1 Table 6.2.2-1 and the PC2 dual Tx MPR requirement specified in TS 38.101-1 Table 6.2D.2-1 can be used as starting point, respectively.

- ULFPTx (mode 0) requirement for 3Tx

In previous meeting, how to consider the ULFPTx mode 0 configuration to verify corresponding MOP requirement was discussed. From test burden perspective, we think Option 2 in the WF, which is to only test for single layer, is a preferable way since it can verify the advertised full power capability on each port.

Proposal 4: Suggest to only specify single layer precoders for 3Tx ULFPTx mode 0 verification.

On the asymmetric DL sTRP/UL mTRP

Another interesting issue is whether to additionally consider SSB transmission capability for the “UL-specific TRP” in RAN4. Before the proponent can further clarify potential impacts on FR2 RF requirement and test framework, we think from procedure point of view, RAN4 should seek RAN1 confirmation on whether the aforementioned scenario can be covered by the currently RAN1 discussion. If it can be obtained from RAN1, the follow-up work can be carried out in RAN4.

Proposal 5: On the asymmetric DL sTRP/UL mTRP, before the proponent can further clarify potential impacts on FR2 RF requirement and test framework, RAN4 should seek RAN1 confirmation on whether the aforementioned scenario can be covered by the currently RAN1 discussion.

Conclusion

In this contribution we discussed on the RF requirements for Rel-19 NR MIMO. We have the following proposals:

Proposal 1: For Rel-19 RAN4 3Tx discussion under Rel-19 NR_MIMO_Ph5 WI, suggest to preclude PC1.5 for both handheld and non-handheld UEs.

Proposal 2: The PA configuration assumption for 3Tx operation can be:

- 3x23dBm for PC3 handheld UE

- 23dBm+2x26dBm or 2x23dBm+26dBm for PC2 handheld UE

Proposal 3: For 3Tx MPR requirements, the PC3 MPR requirement specified in TS 38.101-1 Table 6.2.2-1 and the PC2 dual Tx MPR requirement specified in TS 38.101-1 Table 6.2D.2-1 can be used as starting point, respectively.

Proposal 4: Suggest to only specify single layer precoders for 3Tx ULFPTx mode 0 verification.

Proposal 5: On the asymmetric DL sTRP/UL mTRP, before the proponent can further clarify potential impacts on FR2 RF requirement and test framework, RAN4 should seek RAN1 confirmation on whether the aforementioned scenario can be covered by the currently RAN1 discussion.

- References

- R4-2414310, “WF on UE RF requirements for Rel-19 MIMO enhancement”, Samsung, 3GPP TSG RAN4 Meeting #112, Maastricht, Netherlands, August 19-23, 2024.

- RP-242394, “WID revision: NR MIMO Phase 5”, Samsung (Moderator), 3GPP TSG RAN Meeting #105, Melbourne, Australia, September 9-12, 2024.