3GPP TSG-RAN WG4 Meeting #112bis                                      R4-2416185

Hefei, Anhui, China, October 14 - 18, 2024

Source: 	Huawei, HiSilicon

Title: 	On general aspects for SBFD

Agenda Item:	6.20.1

Document for:	Approval

Introduction

During RAN4#112, general aspects (including RAN4 aspects for SBFD system parameters) for SBFD have been discussed. Following conclusions are reproduced from [1].

- Band/channel bandwidth support for SBFD - Agreement: - Declaration-based method, and SBFD is possible to be used on any TDD band, with the following principle: - The supported channel bandwidth(s) is declared by gNB vendors - The supported channel bandwidth(s) is from existing channel bandwidth defined in TS 38.104 - The supported channel bandwidth shall be no less than X MHz - X = [20] - For next meeting, we will further discuss the potential to increase the value of X beyond X = [20]. Companies are encouraged to bring t-docs to discuss the benefits or drawback to increase this value. - FFS for some specific band if this principle rules out this band, which is required for SBFD by operators. - FFS how to implement in the spec: - Option 1: To include the following text in Clause 5.2 in TS 38.104: subband full duplex can be applied to TDD bands with channel bandwidth >= X MHz, given in Table 5.2-1. - Option 2: Add SBFD entry in Table 5.2-1 in Clause 5.2 in TS 38.104 for the TDD bands with channel bandwidth >= X MHz. - Agreement: - BS channel bandwidth for SBFD-capable BS: - BS channel bandwidth for SBFD-capable BS: RF bandwidth supporting a single NR RF carrier with the transmission bandwidth configured in the uplink or downlink or SBFD operation. - The existing definition of BS channel bandwidth (for information only): - BS channel bandwidth: RF bandwidth supporting a single NR RF carrier with the transmission bandwidth configured in the uplink or downlink. - Transmission bandwidth configuration for SBFD - Agreement: - BS transmission bandwidth configuration for SBFD: - BS transmission bandwidth configuration for SBFD NRB,SBFD: set of resource blocks located within the BS channel bandwidth which may be used for transmitting or/and receiving by the BS. - The transmission bandwidth configuration for SBFD NRB,SBFD for each BS channel bandwidth and subcarrier spacing shall reuse the values for BS transmission bandwidth configuration in table 5.3.2-1 for FR1 and table 5.3.2-2 for FR2-1. - FFS a corresponding figure for SBFD is needed - The existing definition of BS transmission bandwidth configuration (for information only): - BS transmission bandwidth configuration: set of resource blocks located within the BS channel bandwidth which may be used for transmitting or receiving by the BS. - DL/UL subband configurations support for SBFD - Agreement: - Transmission bandwidth configuration for SBFD UL/DL subband: - NRB,SBFD,DL: Transmission bandwidth configuration for SBFD DL subband, expressed in resource blocks. - NRB,SBFD,UL: Transmission bandwidth configuration for SBFD UL subband, expressed in resource blocks. - Guardband between SBFD UL and DL subband: - NRB,SBFD,GB: The size of guardband between SBFD UL subband and DL subband, expressed in resource blocks. - Agreement: - To specify DL/UL subband configuration in BS RF requirements, the following 3 options for DL/UL subband configuration are identified: - Option-1 (with the least flexibility): - Only a number of selected DL/UL subband configurations are specified for RAN4 BS RF requirement. - Example (given in Nokia Tdoc R4-2413238): - For FR1 DUD configuration in 100 MHz channel and 30 kHz SCS: < NRB,SBFD,DL, NRB,SBFD,UL, NRB,SBFD,GB > = <106, 51, 5> - For FR1 DU or UD configuration in 100 MHz channel and 30 kHz SCS: < NRB,SBFD,DL, NRB,SBFD,UL, NRB,SBFD,GB > = <217, 51, 5> - Option-2 (with the middle flexibility): - Option-2a: - DL subband configuration shall be selected from the existing values of BS transmission bandwidth configuration, as provided in Table 5.3.2-1 for FR1 and Table 5.3.2-2 for FR2-1. - UL subband size and guardband size(s) depends on BS implementation - Example (FR1 DU or UD in 100MHz and 30kHz SCS, 80-20MHz case): As long as BS transmission configuration for SBFD shall be 273 (NRB,SBFD for 100MHz and 30kHz SCS) and NRB,SBFD,DL is selected from Table 5.3.2-1 (according to the 30kHz SCS), from BS RF requirement perspective, no limitation on the values NRB,SBFD,UL and NRB,SBFD,GB. - Option-2b: - UL subband configuration shall be selected from the existing values of BS transmission bandwidth configuration, as provided in Table 5.3.2-1 for FR1 and Table 5.3.2-2 for FR2-1. - DL subband size(s) and guardband size(s) depends on BS implementation - Example (FR1 DU or UD in 100MHz and 30kHz SCS, 80-20MHz case): As long as BS transmission configuration for SBFD shall be 273 (NRB,SBFD for 100MHz and 30kHz SCS) and the value of NRB,SBFD,UL is selected from Table 5.3.2-1 (according to the 30kHz SCS), from BS RF requirement perspective, no limitation on the values NRB,SBFD,DL and NRB,SBFD,GB. - Option-3 (with the highest flexibility): - No restriction on the subband sizes, i.e., RF requirement shall be specified to enable all DL and UL subband configurations (and implicitly guardband configurations). - Example (FR1 DU or UD in 100MHz and 30kHz SCS): From BS RF requirement perspective, no limitation on the values of NRB,SBFD,DL, NRB,SBFD,UL, NRB,SBFD,GB - FFS how to implement BS RF requirements based on the above options for different levels of flexibility. - BS transmission configuration in BS RF requirements (for information only): Table 5.3.2-1: Transmission bandwidth configuration NRB for FR1 SCS (kHz); 3 MHz; 5 MHz; 10 MHz; 15 MHz; 20 MHz; 25 MHz; 30 MHz; 35 MHz; 40 MHz; 45 MHz; 50 MHz; 60 MHz; 70 MHz; 80 MHz; 90 MHz; 100 MHz. NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB. 15; 15; 25; 52; 79; 106; 133; 160; 188; 216; 242; 270; N/A; N/A; N/A; N/A; N/A. 30; N/A; 11; 24; 38; 51; 65; 78; 92; 106; 119; 133; 162; 189; 217; 245; 273. 60; N/A; N/A; 11; 18; 24; 31; 38; 44; 51; 58; 65; 79; 93; 107; 121; 135. Table 5.3.2-2: Transmission bandwidth configuration NRB for FR2-1 SCS (kHz); 50 MHz; 100 MHz; 200 MHz; 400 MHz. NRB; NRB; NRB; NRB. 60; 66; 132; 264; N/A. 120; 32; 66; 132; 264. - Restriction on guardband size for SBFD - Agreement: - Restriction on guardband size for SBFD: - Option 1: Definition based on existing guardbands requirement in TS38.104 (i.e., minimum guardband) - Option 2: Typical values of guardband defined for requirement, but the guardband should not be restricted to those in RAN4 specifications. - Option 3: No restriction (manufacturer declaration-based), but with restriction on maximum allowed number of RBs for guardband: - Option 3a: No restriction (manufacturer declaration-based), and no restriction on maximum/minimum number of RBs for guandband: - SBFD time-domain terminology - Agreement: - When specifying RAN4 requirements for SBFD-capable BS, SBFD symbols (instead of SBFD slots/symbols) shall be used to designate the time domain duration in which SBFD related requirements is applied. - CLI handling impact on RF requirement - Agreement: - RAN4 shall define RF requirements for the frequency outside of wanted carrier based on no gNB-to-gNB CLI handling as baseline - FFS Revisit particular RF requirements pending on RAN1 progress on the gNB-to-gNB CLI handling work. - BS RF Specification structure for SBFD requirements - Way forward: - RAN4 shall study how to introduce BS RF new requirements for SBFD-capable BS: - Option 1: Embedding the corresponding SBFD RF requirements in the corresponding section of TS 38.104 (similar as NB-IoT RF requirement in TS36.104) - Option 2: Creating new sub-clauses in TS 38.104 for RF requirements that applies for SBFD - Option 2a: details (like new suffix or not) can be FFS when RF requirements are stable. - Option 2b: Set up new sub-clause with suffix-B for modified existing RF requirements and SBFD-dedicated new RF requirements - Option 3: A new specification for SBFD BS RF requirements

In this contribution, we would like to share our views regarding this topic.

Discussion

To be further discussed, one of the hot topics in last meeting is subband configuration for SBFD. In addition, we would like to share our preference on specification structure regarding how to capture the SBFD feature.

The guard band and subband configuration for SBFD

To specify DL/UL subband configuration in BS RF requirements, the Option 2b can achieve a good balance between flexibility and complexity in our view.

In the Rel-18 SI for duplex, extensive analysis and discussion regarding the interference caused by simultaneous Tx/Rx at gNB side have been conducted. To guarantee the feasibility of SBFD, multiple techniques in RF domain have been proposed. One straightforward solution is to have dedicated subband filter in order to provide sufficient interference suppression between subbands. As a result, the size of guard band should be carefully considered with expected interference suppression level, transition band of the subband filter and the RB overheads. Note that different vendor may have different kinds of trade-off, so we prefer to leave the size of guard band for manufacturer declaration but not to have limitation on minimum/maximum value.

Observation 1: The size of guard band for SBFD is suitable to be manufacturer declaration based as it can vary depending on implementation for different BS type or frequency band, thus having limitation on the minimum/maximum value of the size of guard band would be a great loss on implementation flexibility.

To reduce the specification complexity and guarantee the testability at the same time, we can only specify the UL subband configuration also due to following reason.

- From Demod test in the performance part, in current PUSCH-related test, all RBs in the whole channel BW should be filled to define FRC. If we go with Option 2a or 3, RAN4 must define FRC to cover different RB numbers with granularity of 1RB, which is a non-negligible workload.

Proposal 1: To specify DL/UL subband configuration in BS RF requirements:

- UL subband configuration shall be selected from the existing values of BS transmission bandwidth configuration, as provided in Table 5.3.2-1 for FR1 and Table 5.3.2-2 for FR2-1.

- DL subband size(s) and guardband size(s) depends on BS implementation.

The supported BS CBW for SBFD

For single carrier operation, as captured in the WF, RAN4 will check whether to preclude smaller CBWs higher than 20MHz. From our understanding, the difficulty on adding new modules or incorporating new architecture with legacy equipment especially operating in low bands cannot be ignored. So targeting U6G, 4.9GHz and 3.5GHz which acquire more frequency resources would better fulfil the advertised SBFD gain. On the other hand, SBFD itself could need additional frequency to accommodate the guard band between UL and DL subband. So we have the following proposal.

Proposal 2: For single carrier operation, the supported BS CBW for SBFD should be no less than 60MHz.

For CA operation, we prefer to limit the SBFD operation on only one carrier for all the listed example on CA configuration in our proposal.

Proposal 3: For multi-carriers operation, the SBFD can be supported on only one of the carriers, and the supported CBW for this carrier should be no less than 60MHz and applying 100MHz as maximum.

- For instance, 100MHz CC for SBFD + N*100MHz CC, where N could be e.g. 1, 2, and 3

As for the spec implementation aspect, we prefer Option 2 since it can be more intuitive and easier for adding the SBFD band considering there might be operator request in the future.

Proposal 4: Add SBFD entry in Table 5.2-1 in Clause 5.2 in TS 38.104 for the TDD bands with channel bandwidth >= 60MHz.

BS RF Specification structure for SBFD requirements

According to the WF, RAN4 will further study how to capture the SBFD feature in the TS 38.104. For the time being, we prefer to embed the corresponding SBFD RF requirements in the corresponding section of TS 38.104 as much as possible (e.g. RAN4 already agreed to embed the supporting band of SBFD in clause 5) and consider new clauses or new suffix for SBFD for the RF requirements that are necessary.

Proposal 5: Embed the corresponding SBFD RF requirements in the corresponding section of TS 38.104 as much as possible (e.g. RAN4 already agreed to embed the supporting band of SBFD in clause 5) and consider new clauses or new suffix for SBFD for the RF requirements that are necessary.

Conclusion

In this contribution we discussed on the general aspects for SBFD. We have the following observation and proposals:

Observation 1: The size of guard band for SBFD is suitable to be manufacturer declaration based as it can vary depending on implementation for different BS type or frequency band, thus having limitation on the minimum/maximum value of the size of guard band would be a great loss on implementation flexibility.

Proposal 1: To specify DL/UL subband configuration in BS RF requirements:

- UL subband configuration shall be selected from the existing values of BS transmission bandwidth configuration, as provided in Table 5.3.2-1 for FR1 and Table 5.3.2-2 for FR2-1.

- DL subband size(s) and guardband size(s) depends on BS implementation.

Proposal 2: For single carrier operation, the supported BS CBW for SBFD should be no less than 60MHz.

Proposal 3: For multi-carriers operation, the SBFD can be supported on only one of the carriers, and the supported CBW for this carrier should be no less than 60MHz and applying 100MHz as maximum.

- For instance, 100MHz CC for SBFD + N*100MHz CC, where N could be e.g. 1, 2, and 3

Proposal 4: Add SBFD entry in Table 5.2-1 in Clause 5.2 in TS 38.104 for the TDD bands with channel bandwidth >= 60MHz.

Proposal 5: Embed the corresponding SBFD RF requirements in the corresponding section of TS 38.104 as much as possible (e.g. RAN4 already agreed to embed the supporting band of SBFD in clause 5) and consider new clauses or new suffix for SBFD for the RF requirements that are necessary.

- References

- R4-2413514, “Way Forward for [112][307] NR_duplex_evo_General”, Samsung, 3GPP TSG-RAN4 Meeting #112, Maastricht, Netherlands, August 19-23, 2024.