3GPP TSG-RAN WG4 Meeting #112bis	R4-2416188

Hefei, China, Oct 14 – Oct 18, 2024

Source:	Nokia

Title:	On the reply LS to RAN1 on contention based Msg3 transmission.

Agenda item:	6.27.3

Document for:	Discussion

Introduction

In RAN2 #126, RAN2 has reached agreements in the Rel-19 WI related to NB-IoT over NTN (IoT_NTN_Ph3-Core) which might have impact/be impacted by RAN4 UE requirements. This was indicated by RAN2 in their LS to RAN4 [1].

RAN2 has asked RAN4 to analyze whether it is possible for a UE in IDLE mode to skip MSG1 in the RACH procedure and still obtain timing synchronization before MSG3. A reply was provided by RAN4 in R4-2414114 [2]. RAN1 was asked to assess the same subject, which triggered some questions on RAN1 about RAN4 assumptions.

Further inquiries regarding this subject were sent to RAN4 by RAN1 [3] as an outcome of their investigation .  In this document we provide our views on the further inquiries posed by RAN 1.

Discussion

In the analysis performed by RAN4 in the previous meeting, a contentious topic was the definition of  reply LS submitted by RAN4, to the initial LS posed by RAN2, it was indicated that the value of the network-controlled of timing advance was not initialized when analyzing the MSG 3 transmission without MSG1/2.

RAN4 has previously indicated that in our initial investigation, the value of the network controlled component of the timing advance was not initialized when MSG 3 was transmitted without MSG1/2.

One important aspect is that this means that, in our view, this represents that the UE can fulfill the Tx Timing Requirements from UE point of view, but that does not necessarily means that the UL transmission will be time aligned in the UL synchronization point.

In the absence of the network controlled component of the timing advance, the UE is capable of fulfilling the UE transmit timing requirements from its point of view, but because the reference point has changed, the UL transmission might not be aligned in time in the UL synchronization point.

Nonetheless, for the scope of this feature, when MSG3 is transmitted in the absence of MSG1/2, the NW did not have a chance to initialize the closed loop component of timing advance, therefore, only two values are acceptable:

- Option A: NTA = 0. This is the most straightforward approach, and it only indicates that in the absence of information the UE will consider the network controlled part to be equal to 0.

- Option B: NTA = K, where K is a constant value either pre-defined or set by network in broadcast channel. This solution would mean that a fixed offset is expected in timing for UEs adopting MSG3 transmission without MSG1/2, but such approach would depend on specificities of feature development, and they are out of scope of RAN4.

From RAN4 point of view, neither of the options above would mitigate the problem of the potential lack of synchronization from the UL signal in the reception at the NW side. Based on this, RAN 4 does not have strong reason to force a definition of  NTA, and as such it becomes a characteristic of this feature, left for RAN1/2 definition.

Therefore, we propose:

Reply to RAN1 that the exact definition of NTA  is up to RAN1, but RAN4 has assumed to this point that the non-initalized value of NTA is equal to 0.

In what regards the applicability of the requirements, RAN4 can confirm to RAN1 that the requirements in specified in Table 7.20A.2-1 and Table 7.24A.2-1 of TS 36.133 apply to the Msg3 transmission without Msg1/Msg2 in NB-IoT over NTN and eMTC over NTN, respectively.

Confirm to RAN1 that NOTE 2 in R1-2407548 is valid.

Conclusion

The paper has provided our views on the LS sent by RAN1 in R1-2407548 and the following observation and proposal were made

Observation 1: RAN4 has previously indicated that in our initial investigation, the value of the network controlled component of the timing advance was not initialized when MSG 3 was transmitted without MSG1/2.

Observation 2: In the absence of the network controlled component of the timing advance, the UE is capable of fulfilling the UE transmit timing requirements from its point of view, but because the reference point has changed, the UL transmission might not be aligned in time in the UL synchronization point.

Proposal 1: Reply to RAN1 that the exact definition of NTA  is up to RAN1, but RAN4 has assumed to this point that the non-initalized value of NTA is equal to 0.

Proposal 2: Confirm to RAN1 that NOTE 2 in R1-2407548 is valid.

- References

- R2-2405769, “LS on UL synchronization for contention based Msg3 transmission without Msg1/Msg2”, ZTE, RAN2#126, Fukuoka, Japan, May 20 – May 24, 2024.

- R4-2414114, “Reply LS to RAN2 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2”,

- R1-2407548 “Reply LS to RAN2 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2”, ZTE, RAN1 #118, Maastricht, NL, August 19th – 23rd, 2024

- TS 36.133 “Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management”, 3GPP, Release 18.6.0

- TS 36.211 “Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation”, 3GPP, Release 18.6.0