3GPP TSG-RAN WG4 Meeting #112bis	R4-2416189

Source:	Nokia

Title:	Adapting NGSO requirements for RedCap support in Rel-19

Agenda item:	6.26.4.1

Document for:	Discussion

Introduction

In [1], RAN plenary has approved a new WI for NTN, which will progress the effort of specification for NTN systems in 3GPP technologies in Rel-19 (NR_NTN_PH3). The most recent version of the WID can be found in [2]. It includes a handful of objectives such as:

- Downlink Coverage Enhancements

- Uplink Capacity/Coverage Enhancements

- Signaling of the intended service area of a broadcast service (MBS broadcast)

- Support of regenerative payload for NTN systems

- Support of Rel-17 and Rel-18 RedCap and (e)RedCap UEs in NTN systems

This document is related to the 5th topic on the list above: RedCap deployments over NTN. The discussions for this objective were initiated in the previous meeting RAN4 #112. And as the outcome of the discussion a way-forward (WF) document was approved which contains a set of agreements and other way-forward (for further discussion) guidance for currently identified open issues [3].

In this document we provide our views, related to the RAN4 impact of the introduction of RedCap over NTN considering the scope of this WI and/or the open issues already identified in the WF.

Discussion

On supported scenarios and parameters

A few issues in the WF were related to supported scenarios and parameters for the (e)RedCap support over NTN. In this subsection of the present document we will discuss them, in the WF they are presented in:

- Issue 6-1-2 Bandwidth of eRedCap UE

- Issue 6-1-5 The network scenario considered for (e)RedCap UE in FR1-NTN

- Issue 6-1-6 The SCS considered for (e)RedCap UE with FR1-NTN.

For issue 6-1-2, the discussion is whether the eRedCap UEs (Rel-18 RedCaps) in NTN can be supported with both configurations (reduced bandwidth and non-reduced bandwidth). In Rel-17, RedCaps UE can only support reduced bandwidth (up to 20 MHz) whereas in Rel-18 eRedCap UEs would be able to support regular bandwidth (non-supported) by declaring the support of such capability (eRedCapNotReducedBB-BW-r18). Analyzing the impact of supporting regular bandwidth, we see no showstopper for this operation in NTN.

Therefore, in what regards this issue we propose:

The eRedCap feature for support of non-Reduced bandwidth is supported in Rel-19 RedCap over NTN.

The next issues seem as straightforward as the issue above, by adopting the same principle as used for FR1-NTN. In FR1-NTN only NR SA operation is supported (there is no scenario where dual connectivity modes are useful for FR1-NTN, as there is no LTE support for NTN). The same principle shall be adopted for RedCap.

As for FR1-NTN, only NR SA operation mode is supported in eRedCap over NTN.

For the next issue we have a different view from what is proposed in the way forward. For FR1-NTN, only 15 kHz and 30 kHz SCS are supported. The 60 kHz was excluded from FR1-NTN support because due to the timing relaxations needed for a FR1-NTN to derive timing pre-compensations, the total transmit timing error could surpass half of the cyclic prefix and that could compromise the performance of the system.

Considering that a RedCap UE has reduced capabilities compared to a normal legacy NR UE, i.e., it is a simpler, cheaper device, it is not reasonable to expect that it would perform better pre-compensations than the regular NTN UE. As such, the same issues identified for FR1-NTN for legacy UEs are also applicable to RedCap and they are impeditive to the operation over 60 kHz

Do not support 60 kHz SCS in eRedCap over NTN.

General Principle for RRM requirements

The WF has captured in issue 6-2-1 two different approaches for the general working principle used for defining requirements in this WI, as showed in the box below:

Issue 6-2-1: The general principle for defining the RRM requirements for (e)RedCap UE with FR1-NTN <Way Forward> : Further discuss the following proposal - Proposal 1 (CATT, MTK, Xiaomi, Samsung, HW, vivo, ZTE): - A general principle is that to define the RRM requirements for (e)RedCap UE with FR1-NTN bands based on the existing requirements for FR1-NTN. - Proposal 2 (Ericsson): - RRM requirements for RedCap UE in NTN can be started by taking RedCap UE in TN as baseline. - The requirements and enhancements for NR NTN in Rel-17 and Rel-18 shall be introduced for RedCap UE in NTN.

Although, both proposals are very similar for the most part and will probably result in the same requirements for the most part, we would prefer to use FR1-NTN as baseline and introduce the changes needed as requested by RedCap operations (proposal 1). The main difference in our view is related to several relaxation of measurements in RedCap that may not translate well in FR1-NTN and only a subset of them should be picked based on their relevance to enhance this feature.

Define RRM requirements for (e)RedCap UE with FR1-NTN bands based on the existing requirements for FR1-NTN.

Transmit Timing Considerations

In issue 6-2-2 of the WF, it was identified  that timing considerations were among the RRM requirements that would require specification for the RedCap operation over NTN, more speficically the following timing requirements were listed:

  - UE transmit timing

  - UE timer accuracy

  - Timing Advance

It is important to notice that, even though there are separate subclauses for RedCap compared to legacy NR operation, there are no differences in the accuracy requirements nor in the total transmit timing error when comparing the two specification clauses. There are, though, differences related to the procedure to acquire the DL reference timing, as the RedCap operation introduced the concept of NCD-SSB, which is used to accommodate SSB transmissions in a BWP that allows the RedCap UE to function with reduced bandwidth in legacy scenarios.

So, it is reasonable to assume that a RedCap UE has the same capabilities in terms of timing tracking and fine tuning then a regular UE. Therefore, we believe that there should be no differentiation, between legacy Rel-18 FR1-NTN timing requirements (in absolute values) and RedCap over NTN requirements, except for the acquisition of NCD-SSB.

The absolute value for transmit timing error shall be the same for RedCap over NTN as defined for FR1-NTN.

Capture the same side condition used in TN Redcap related to the acquisition of NCD-SSB every 160 ms in RedCap over NTN specification

Reuse UE timer accuracy and timing advance requirements form FR1-NTN for RedCap over NTN.

Measurement Relaxation and eDRX in RedCap over NTN

There are one issue related to measurement Relaxation (6-3-2) and one issue related to eDRX in RedCap over NTN (6-5-1) in the previous WF.

In issue 6-3-2, there is a discussion whether there should be and what should be the relaxation for the requirements for RedCap UEs using 1RX compared to the baseline requirements from FR1-NTN (which assumes 2 Rx as minimum).

This exercise has already been done by RAN4 when specifying the RedCap requirements in both Rel-17 and Rel-18. And we think the assumptions performed in that investigation are not impacted by any of the NTN features nor the NTN specific functionalities. In this sense, we tend to agree that this relaxation should be introduced, but the values related to such relaxations shall be the same as defined in TN Redcap, unless a NTN related issue is identified.

In issue 6-3-2 on the previous WF, follows proposal 4 and adopt the same relaxations for 1Rx Redcap as previously adopted in TN Redcap.

For the eDRX applicability several options were also explrored in the previous meeting. As evidenced in the box below:

Issue 6-5-1: How to consider the impact of eDRX enhancement for (e)Redcap UE with FR1-NTN? <Way Forward> : Further discuss the following proposal - Proposal 1 (CATT, CMCC, Samsung): - The eDRX enhancement introduced for (e)RedCap UEs should be considered when defining RRC_IDLE or/and RRC_INACTIVE state mobility requirements for (e)RedCap UEs with FR1-NTN bands. - Proposal 1a (CATT): RAN4 need to discuss whether the eDRX enhancement for both Rel-17 RedCap and Rel-18 eRedCap UEs should be introduced for (e)RedCap UEs with FR1-NTN bands. - Option 1: Only introduce the eDRX enhancement for Rel-17 RedCap UEs. - Option 2: Introduce the eDRX enhancement for both Rel-17 RedCap and Rel-18 eRedCap UEs. - Proposal 1b (CMCC): Support eDRX configuration for Redcap over NTN, and define following applicability rule for each deployment scenario: - For GEO deployment, all DRX and eDRX cycle can be supported. - For earth-fixed LEO deployment, requirements are applicable for up to 10.24s eDRX cycle - For earth-moving LEO deployment, requirements are not applicable for eDRX cycle - Proposal 1c (Samsung): - RAN4 to discuss and specify the requirements related to eDRX for GSO and NGSO (LEO) separately. Not all eDRX cycle can be applicable for NGSO (LEO) scenario. - Proposal 1d (QC): For eRedCap UE, RAN4 can consider the following aspect for RRM requirement definition if necessary and applicable for NR NTN support. - Enhanced eDRX in RRC INACTIVE - Proposal 2 (LG): - For NTN RRM requirements for RedCap UE, - For IDLE/INACTIVE mode: Preclude eDRX_IDLE cycle configuration for RedCap NTN

It is important to stress that for FR1-NTN, there is some exclusion of supported eDRX values from the NTN operation in NGSO scenarios. Note 1 in Table 4.2C.2.2-1 indicates that: “Note 1:	The UE is not required to meet the requirements for 2.56s DRX cycle length for earth-moving LEO deployment.”. The reason for that is that such scenarios are considered so dynamic that there is not point in considering large DRX configurations, as the UE won’t be capable to finalize the measurements in time if adopting such configurations.

On the other hand, in our view, GSO scenarios are much alike terrestrial implementations (with much larger delays). We see no apparent reason for excluding eDRX settings (that can optimize battery savings at the UE) from GSO scenarios. Therefore, considering the two aspects above, we would support proposal 1B in the set of options above.

Support eDRX configuration for Redcap over NTN, and define following applicability rule for each deployment scenario:

  - For GEO deployment, all DRX and eDRX cycle can be supported.

  - For earth-fixed LEO deployment, requirements are applicable for up to 10.24s eDRX cycle

  - For earth-moving LEO deployment, requirements are not applicable for eDRX cycle

On Conditional HO and RACH-less HO support

It is still uncertain whether RAN4 will agree to specify RedCap requirements for Conditional Ho and RACH-less HO in NTN. As a matter of fact, it is not clear whether such features will actually be supported for this type of device. This has been captured in Issue 6-2-2 of the WF

<Way Forward> - FFS for the following requirements: - Minimization of Drive Tests (MDT) in RRC_IDLE state and RRC_INACTIVE state - NR Rel-17/18 Conditional Handover, and RACH-less Handover - Note: further check whether they are supported for RedCap NTN in RAN1/2 spec. - FFS: Send LS to RAN1/2 to ask the status. RAN4 does not trigger the RAN1/2 discussion to support these features.

The WF indicates that RAN4 will not trigger the discussion for the support of CHO and RACH-less HO for RedCap UEs. If that is the case, RAN4 must wait for RAN2 progress. But in case those features become supported for RedCap or eRedCap UEs, they become an an important feature to be tested, as mobility enhancements are a key aspect of the NTN ecosystem.

Wait for RAN2 progress on whether CHO and RACH-less HO are supported by (e)RedCap UEs. Specify requirements if the features are supported.

On the specification structure and RedCap applicability

Another topic was discussed in the WF is related to the specification structure. There are currently  discussions whether the RedCap over NTN shall be captured under a new suffix (e.g. E) or whether it should be new subclauses to the NTN clauses in TS 38.133. This was discussed in Issue 6-8-1 in [3].

Among the proposed options, in order to minimize the specification effort and the size of the specification, but still maintaining coherence and completeness of the specification, we would prefer that RedCap subclauses are inserted as subsections of NTN section (suffix C).

Adopt RedCap subclauses under NTN sections (under Suffix C).

But another thing that needs to be discussed is the applicability of the requirements for RedCap operation under NTN. For a UE to support this feature (RedCap over NTN), it has to support RedCap features and NTN features. There are therefore, two options to declare such capabilities:

- Option A: Declare support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) AND declare support of NTN (nonTerrestrialNetwork-r17)

- Option B: Declare explicitly – via a new capability – the support to RedCap over NTN.

Both options have their pros and cons (flexibility versus capability signalling overhead). Therefore, we would like to introduce this discussion in RAN4 and propose:

RAN4 to discuss whether a new capability is needed to declare RedCap support of NTN or whether this support is implicit from declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) AND declare support of NTN (nonTerrestrialNetwork-r17)

Conclusion

This paper has discussed the open issues on the specification of requirements for RedCap operations in FR1-NTN. Based on our investigations the following observations and proposals were made:

Proposal 1: The eRedCap feature for support of non-Reduced bandwidth is supported in Rel-19 RedCap over NTN.

Proposal 2: As for FR1-NTN, only NR SA operation mode is supported in eRedCap over NTN.

Proposal 3: Do not support 60 kHz SCS in eRedCap over NTN.

Proposal 4: Define RRM requirements for (e)RedCap UE with FR1-NTN bands based on the existing requirements for FR1-NTN.

Proposal 5: The absolute value for transmit timing error shall be the same for RedCap over NTN as defined for FR1-NTN.

Proposal 6: Capture the same side condition used in TN Redcap related to the acquisition of NCD-SSB every 160 ms in RedCap over NTN specification

Proposal 7: Reuse UE timer accuracy and timing advance requirements form FR1-NTN for RedCap over NTN.

Proposal 8: In issue 6-3-2 on the previous WF, follows proposal 4 and adopt the same relaxations for 1Rx Redcap as previously adopted in TN Redcap.

Proposal 9: Support eDRX configuration for Redcap over NTN, and define following applicability rule for each deployment scenario:

a.	For GEO deployment, all DRX and eDRX cycle can be supported.

b.	For earth-fixed LEO deployment, requirements are applicable for up to 10.24s eDRX cycle

c.	For earth-moving LEO deployment, requirements are not applicable for eDRX cycle

Proposal 10: Wait for RAN2 progress on whether CHO and RACH-less HO are supported by (e)RedCap UEs. Specify requirements if the features are supported.

Proposal 11: Adopt RedCap subclauses under NTN sections (under Suffix C).

Proposal 12: RAN4 to discuss whether a new capability is needed to declare RedCap support of NTN or whether this support is implicit from declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) AND declare support of NTN (nonTerrestrialNetwork-r17)

- References

- RP-234078, New WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Huawei

- RP-241789 Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT

- R4-2414037, "The WF on RRM requirements for Rel-19 NR NTN phase3", Moderator (CATT), RAN4 #112, Maastricht, Netherlands, Aug. 19-23, 2024.