3GPP TSG-RAN4 Meeting #112bis	R4-2416196

# Hefei, CN, 14th Oct 2024 – 18th Oct 2024

Source:	LG Electronics

Title:	Discussion on REFSENS and ASCS for the LP-WUS/WUR

Agenda item:	6.23.2.2

Document for:	Discussion

Introduction

In the RAN4#111 meeting,  the WF for LU-WUS_UERF was approved. In this discussion, we share our point of view for the LP-WUS/WUR based on the approved WF[1]

Discussion

Table 1 simulation assumption

LP-WUS Simulation Results

Filter order impact

Figure 1 LLS simulation results on OOK-1 to see the filter order impact

Figure 2 LLS simulation results on OOK-2 to see the filter order impact

Observation 1: The MDR performance difference among the 3rd, 5th and 8th order filters is below 0.3 dB

CFO impact

Figure 3 LLS simulation results on OOK-1 to see the CFO impact

Figure 4 LLS simulation results on OOK-4 to see the CFO impact

Observation 2:CFO does not affect OOK-1 detection performance.

Observation 3: The MDR performance of OOK-4 degrades significantly when the CFO value mored than 10 ppm.

ADC bits

Figure 5 LLS simulation results on OOK-1 to see the ADC bits impact

Figure 6 LLS simulation results on OOK-4 to see the ADC bits impact

Observation 4: There are no big MDR performance differences among 2, 4, 16 and 256 bits

ASCS guard RB

Figure 7 LLS simulation results on OOK-1 to see the ASCS guard RB impact

Figure 8 LLS simulation results on OOK-4 to see the ASCS guard RB impact

Observation 5:There is no performance improvement for the ASCS guard RB.

ACS guard RB

Figure 9 LLS simulation results on OOK-1 to see the ACS guard RB impact

Figure 10 LLS simulation results on OOK-4 to see the ACS guard RB impact

Observation 6: LLS simulation results to see the ACS guard RB impact

- For OOK-1

  - The performance degradation for OOK-1 transmission in BW5/10 MHz is ~6 dB, for 20MHz is ~7 dB

  - Guard RBs of 3 give a performance improvement of less than 1 dB

- For OOK-4

  - The performance degradation for OOK-4 transmission in BW5/10 MHz is ~7.5 dB, for 20MHz is ~8.3 dB

  - Guard RBs of 3 give a performance improvement of less than 0.5~1 dB

SNR evaluation

Figure 11 LLS simulation results on OOK-1 and OOK-4

Observation 7: The target SNR can differ by 4 dB between signal types OOK-1 and OOK-4

Simulation summary

-Filter order impact

Observation 1: The performance difference between the 3rd and 8th order filter is below 0.3 dB

-CFO impact

Observation 2:CFO does not affect OOK-1 detection performance.

Observation 3: The MDR performance degrades significantly when the CFO value mored than 10 ppm.

-ADC bits

Observation 4: There are no big performance differences among 2, 4, 16 and 256 bits

-ASCS guard RB

Observation 5:There is no performance improvement for the ASCS guard RB.

-ACS guard RB

Observation 6: LLS simulation results to see the ACS guard RB impact

-SNR evaluation

Observation 7: The target SNR can differ by 4 dB between signal types OOK-1 and OOK-4

Proposal 1: It is necessary to consider the CFO value below 10 ppm for LLS on OOK-4.

Proposal 2: The 4 bits ADC is enough for LP-WUS.

Proposal 3: Do not specify the ASCS guard RB.

Proposal 4: The target SNR for OOK modulation can be derived based on the signal type OOK-4.

LP-WUR REFSENS requirement

NF value for LP-WUR

Table 2 WF on NF and REFSENS requirements from RAN4#112 meeting[1]

Issue 2-2-2: Baseline architecture for OOK-based LP-WUS Agreement: - use zero-IF receiver as a baseline architecture for envelop based LP_WUR Issue 2-2-3: Baseline architecture for OFDM-based LP-WUS Agreements: - Wait for RAN1’s conclusion on the OFDM-based LP-WUR. Issue 2-2-4: whether different NF for OOK-based and OFDM-based Agreement: - Differentiate NF for OOK-based and OFDM-based receiver. Issue 2-2-9: MDR value for REFSENS test case Agreements: - FFS the MDR for conformance testing should be aligned with the value for RF requirement.

Table 3 Homodyne/zero-IF architecture with baseband envelope detection charcteristic [2]

In the laste meeting, using the zero-IF recever as the baseline architecture for OOK-based LP-WUS was agreed. In the previous LP-WUS/WUR study, the noise figure range was 10~16 dB with homodyne/zero-IF arechitecture as shown in table 3. Therefore, assuming worst case NF, a 16 dB NF value can be a starting point.

Proposal 5: Consider an NF value of 16 dB as a starting point for OOK-based LP-WUS.

Conclusion

Observation 1: The performance difference between the 3rd and 8th order filter is below 0.3 dB

Observation 2:CFO does not affect OOK-1 detection performance.

Observation 3: The MDR performance of OOK-4 degrades significantly when the CFO value mored than 10 ppm.

Observation 4: There are no big performance differences among 2, 4, 16 and 256 bits

Observation 5:There is no performance improvement for the ASCS guard RB.

Observation 6: LLS simulation results to see the ACS guard RB impact

- For OOK-1

  - The performance degradation for OOK-1 transmission in BW5/10 MHz is ~6 dB, for 20MHz is ~7 dB

  - Guard RBs of 3 give a performance improvement of less than 1 dB

- For OOK-4

  - The performance degradation for OOK-4 transmission in BW5/10 MHz is ~7.5 dB, for 20MHz is ~8.3 dB

  - Guard RBs of 3 give a performance improvement of less than 0.5~1 dB

Observation 7: The target SNR can differ by 4 dB between signal types OOK-1 and OOK-4

Proposal 1: It is necessary to consider the CFO value below 10 ppm for LLS on OOK-4

Proposal 2: The 4 bits ADC is enough for LP-WUS

Proposal 3: Do not specify the ASCS guard RB.

Proposal 4: The target SNR for OOK modulation can be derived based on the signal type OOK-4

Proposal 5: Consider an NF value of 16 dB as a starting point for OOK-based LP-WUS.

- References

# [1] R4-2414309, WF for [111][136] NR_LPWUS_UERF (VIVO)

# [2] TR38.869