3GPP TSG-RAN WG4 Meeting #112bis 	R4-2416199

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item:	4.2.2.2

Source: 	Ericsson

Title:	On remaining performance issues for SL positioning

Document for:	Discussion

# Introduction

The latest agreed WF on SL positioning and CPP is in [22].

In this contribution, we discuss some remaining issues related to SL positioning performance part.

# Discussion on SL positioning

## Test cases

### Signal levels in delay test cases

The most outstanding issue for SL test cases is the signal levels.

Given the agreed side conditions, it is proposed to configure SL-PRS in UEs so that SL-PRS of UE1 and UE2 collide, but not with UE3 SL-PRS which is time shifted. Es/Iot level of UE1 SL-PRS is close to 0 dB, while Es/Iot of UE2 SL-PRS and UE3 SL-PRS are close to -3 dB.

- Proposal 1: In SL RSTD test cases, reference UE1’s and anchor UE2’s SL-PRS transmissions (but not necessarily their PSSCH) overlap with each other, but not with anchor UE3’s SL-PRS.

- Proposal 2: SL-PRS Es/Iot for the measured anchor (not reference) UE2 and UE3 is close to side conditions in the requirements.

- Proposal 3: For SL RSTD delay test case, the following SL-PRS signal level configurations are proposed:

|  | UE1 | UE2 | UE3 |
| --- | --- | --- | --- |
| SL-PRS Es/Noc | 5 dB | 3.2 dB | -3 dB |
| SL-PRS Es/Iot | 0.1 dB | -2.99 dB | -3 dB |

- Proposal 4: For SL Rx-Tx delay test case, the following SL-PRS signal level configurations are proposed:

|  | UE1 | UE2 |
| --- | --- | --- |
| SL-PRS Es/Noc | 5 dB | 3.2 dB |
| SL-PRS Es/Iot | 0.1 dB | -2.99 dB |

- Proposal 5: In SL AoA delay test case, the SL-PRS signal level configurations can be the same as in SL Rx-Tx test case.

- Proposal 6: In SL RTOA delay test case, it is proposed to add one interfering UE, so the total number of UEs in the test case is 3, including the target UE. The SL-PRS signal configuration for the three UEs can then be the same as for SL Rx-Tx:

|  | Interfering UE | Target UE |
| --- | --- | --- |
| SL-PRS Es/Noc | 5 dB | 3.2 dB |
| SL-PRS Es/Iot | 0.1 dB | -2.99 dB |

# Summary

The following have been proposed in the current contribution.

- Proposal 1: In SL RSTD test cases, reference UE1’s and anchor UE2’s SL-PRS transmissions (but not necessarily their PSSCH) overlap with each other, but not with anchor UE3’s SL-PRS.

- Proposal 2: SL-PRS Es/Iot for the measured anchor (not reference) UE2 and UE3 is close to side conditions in the requirements.

- Proposal 3: For SL RSTD delay test case, the following SL-PRS signal level configurations are proposed:

|  | UE1 | UE2 | UE3 |
| --- | --- | --- | --- |
| SL-PRS Es/Noc | 5 dB | 3.2 dB | -3 dB |
| SL-PRS Es/Iot | 0.1 dB | -2.99 dB | -3 dB |

- Proposal 4: For SL Rx-Tx delay test case, the following SL-PRS signal level configurations are proposed:

|  | UE1 | UE2 |
| --- | --- | --- |
| SL-PRS Es/Noc | 5 dB | 3.2 dB |
| SL-PRS Es/Iot | 0.1 dB | -2.99 dB |

- Proposal 5: In SL AoA delay test case, the SL-PRS signal level configurations can be the same as in SL Rx-Tx test case.

- Proposal 6: In SL RTOA delay test case, it is proposed to add one interfering UE, so the total number of UEs in the test case is 3, including the target UE. The SL-PRS signal configuration for the three UEs can then be the same as for SL Rx-Tx:

|  | Interfering UE | Target UE |
| --- | --- | --- |
| SL-PRS Es/Noc | 5 dB | 3.2 dB |
| SL-PRS Es/Iot | 0.1 dB | -2.99 dB |

# References

[1] RP-230328, Revised WID on Expanded and Improved NR Positioning, Intel Corp, March, 2023.

[2] R4-2303190, WF on Expanded and Improved NR Positioning RRM requirements, Ericsson, March, 2023.

[3] R1-2301950, Summary #1 on Measurements and reporting for SL positioning, vivo, March 2023.

[4] R1-2301951, Summary #2 on Measurements and reporting for SL positioning, vivo, March 2023.

[5] R1-2301952	Summary #3 on Measurements and reporting for SL positioning, vivo, March 2023.

[6] R4-2306351, WF on RRM requirements for NR sidelink positioning and carrier phase positioning, CATT, April 2023.

[7] R4-2310163, WF on NR Positioning – SL positioning and CPP, CATT, May 2023.

[8] R4-2310077, Simulation assumptions for SL positioning, CATT, May 2023.

[9] R4-2314343, Ad-hoc minutes on R18 NR Positioning RRM requirements, Ericsson, Aug. 2023.

[10] R4-2314354, WF on R18 NR positioning – SL positioning and Carrier Phase Positioning, CATT, Aug. 2023.

[11] R4-2314358, LS on SL positioning and CPP measurements report mapping, Aug. 2023.

[12] R4-2312724, On SL positioning, Ericsson, Aug. 2023.

[13] RP-232670, Revised WID on Expanded and Improved NR Positioning, Intel Corporation, CATT, MediaTek, Sep. 2023.

[14] R4-2317380, WF on R18 NR positioning – SL positioning and Carrier Phase Positioning, CATT, Oct. 2023.

[15] R4-2321524, WF on R18 NR positioning – SL positioning and Carrier Phase Positioning, CATT, Nov. 2023.

[16] R4-2403368, WF on R18 NR positioning – SL positioning and Carrier Phase Positioning, CATT, March 2024.

[17] R4-2406384, WF on SL positioning and carrier phase positioning, CATT, Apr. 2024.

[18] R4-2410193, WF on SL positioning and carrier phase positioning, CATT, May 2024.

[19] R4-2411003, Reply LS on SL positioning measurement, RAN1, Aug. 2024.

[20] R4-2410191, Updated work split on test cases for positioning, Ericsson, May 2024.

[21] R4-2410160, endorsed draft big CR on performance, Ericsson, May 2024.

[22] R4-2414033, WF on SL positioning and carrier phase positioning, CATT, Aug. 2024.