3GPP TSG-RAN WG4 Meeting #112bis 	R4-2416202

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item:	6.17.1

Source: 	Ericsson

Title:	On general AI/ML aspects

Document for:	Discussion

# Introduction

The following was agreed in RAN4#112 on general issues for AI/ML:

Issue 1-1: Post deployment testing (tentative agreement) [1]

- Option 1: Conduct the conformance testing for new/updated AI model/functionality before its deployment in already deployed UEs (considering the UE hardware that it will be used in)

  - FFS on the feasibility

- Option 2: Design the test to verify the performance monitoring and proactive recovery from potential performance degradation

  - Depend on the other WG progress

  - Monitoring can be used for managing fallback, model update/model switching/model transfer, if applicable

- Option 3: Capture model input during conformance testing for later testing of new models.

- Other options are not precluded.

Issue 1-2: Channel modeling (agreement) [2]

- Use synthetic channels as the default assumption for conformance requirements/test.

- RAN4 may conduct an analysis for each use case to determine the reliability of using synthetic channels for test data in evaluating models trained on real data.

- The field data can be considered for the analysis.

Issue 1-3: Testing goals (agreement)

- The testing goal is to verify whether the minimum performance of AI/ML functionality/feature can be achieved

- FFS on whether and how many different test configurations/parameters are used for tests.

- LCM would be tested

# Discussion

## Post-deployment testing and verification

Below we discuss the three options for post-deployment testing listed by RAN4.

RAN4 earlier agreed that conformance testing of UEs that are already deployed is not feasible. Hence, three options are being discussed for post-deployment verification. Post-deployment verification refers to processes for building confidence in continuing adherence to minimum performance requirements (indirectly) for UEs deployed in the field. Post-deployment verification is needed due to the potential for model update, reinforcement learning and the general issue that compliance testing can never exhaustively test model performance in all circumstances.

Option 1 (Conducting the testing for new/updated AI model/functionality before its deployment in already deployed UEs, considering the UE hardware that it will be used in) refers to testing of new/updated models in a realistic way, considering the UE hardware that they will be deployed to, prior to use of the models in deployed UEs. For example, prior to sending a model to UEs, a UE vendor could verify that the model meets RAN4 requirements in representative hardware, which could be tested in a lab. The major disadvantage with this option is a mismatch between test environment and actual environment and that it is still challenging to guarantee the minimum UE performance in all possible scenarios in which the UEs may end up and which may not be known to the tester a priori. If tested in a lab the tested scenarios are limited to the ones defined for RAN4 requirements. So, option 1, if it works, can resolve the issue of model updates degrading performance compared to the original model, but it cannot solve the issue of verifying performance in other real-world scenarios than the ones defined for RAN4 requirements.

- Proposal 1: With Option 1, it is challenging to ensure performance in all scenarios not known a priori. The tested scenarios are likely to be limited to the ones defined for RAN4 requirements. This option also implies a greater amount of logistics for updating and deploying new models.

Option 2 (Design the test to verify the performance monitoring and proactive recovery from potential performance degradation) to some extent addresses the issue with Option 1 when it is not possible to explicitly test a priori all possible scenarios. This option suggests verifying the model’s ability to monitor the UE performance and take timely actions to prevent performance degradation, including falling back to the non-AI mode. It may be feasible to verify a priori the model’s ability to monitor the UE performance, based on measurements and a given limited set of KPIs, provided the KPIs reflect the actual performance. The important question will be whether a performance monitoring KPI can be identified can reliably separate the AI model behaviour from the complexity of other factors that could impact UE performance in the real world (e.g. changing channel, traffic/scheduling, interference, RF behaviours, other AI functionality in the UE or network that may interact etc.). In addition, it is important to verify not only the process but also the reliability of performance monitoring and the accuracy of reports used for KPIs, for which RAN4 and RAN1 may need to work together to derive KPIs, measurements, and corresponding requirements. To verify the model’s ability to pro-actively recover from the potential performance degradation is even more complex, since this depends on how well the model is able to predict the performance degradation, how fast it reacts, and which actions it takes. However, if it is feasible to define such KPIs and there are corresponding requirements for such model behaviours, it should be possible to verify them in a test environment where the KPIs to be monitored degrade in a certain way. Another challenge here is how to ensure a smooth fallback/transition between AI and non-AI modes.

- Proposal 2: Option 2 provides means to verify the model’s ability to perform KPI-based performance monitoring and its ability to act in a certain way upon KPI degradation. However, it is also important to ensure and verify the reliability of performance monitoring and the accuracy of reports used for KPIs, for which RAN4 and RAN1 may need to work together to derive KPIs, measurements, and corresponding requirements.

  - FFS: KPIs that isolate the model behaviour from other factors.

  - FFS: ensuring smooth fallback/transition between AI and non-AI model.

Option 3 is for the UE vendor to store internal data on the input to a model during conformance testing (prior to UE deployment), such that model updates can later be tested using the stored data prior to the models being activated in deployed UEs. This can be done, e.g., as described below:

- During compliance testing under controlled lab conditions, the UE records the inputs to the model under test. The inputs may be UE-specific as they may be after, e.g., analog processing.

- The data on model inputs may be stored in the UE or stored in another location.

- When a model is to be updated, the same data as used for the laboratory test is input to the model.

  - This could be done inside the UE in the field, but it could also be done offline in a test facility.

Depending on the model input, this option may require the capture and storage of UE internal data, which may need to be done by the UE vendor. Option 3 enables a fair comparison between different models for the same UE or between similar (e.g., from the architecture and RF/baseband perspective) UEs. This option is useful to ensure that the newly deployed model outperforms the current model. The principles of Option 3 can even be used together with Option 1 and Option 2, to enable a fair comparison.

- Proposal 3: Option 3 provides means for a fair comparison between different models in the same UE or similar (e.g., from the architecture and RF/baseband perspective) UEs. This option is advantageous when model comparison is needed, e.g., to ensure that a newly deployed model is not worse than the previous model. The principles of Option 3 can be used also together with Option 1 and Option 2.

For all three options, it is important that RAN4 discusses on their feasibility, complexity, and benefits for model testing and verification. It is also proposed that RAN4 informs other groups about its observations.

- Proposal 4: RAN4 sends an LS to inform other groups (e.g., RAN1 and RAN2) about its observations on feasibility, complexity, advantages, and disadvantages of Option 1, Option 2, and Option 3, to facilitate their discussions and to avoid standardizing solutions which are not feasible from RAN4 perspective.

## LCM RRM requirements for AI/ML

In RAN4#111, it was agreed that RAN4 will consider defining RRM requirements for selected LCM procedures, one of which is the fallback procedure to non-AI operation.

It is generally not expected that the performance will be worse with AI/ML than the legacy operation, since the UE will have to meet at least the legacy requirements even under AI/ML operation. However, a UE capable of AI/ML operation, may still need to go to non-AI/ML operation mode. Some of the critical reasons for a fallback to non-AI/ML operation can be, e.g.,

- potential performance degradation below the expected legacy performance,

- due to an error, from which it cannot reliably recover in AI/ML mode, etc.

Hence it is important to have a requirement on how efficient the UE can go back to the legacy non-AI/ML operation and what are the requirements for transitions between AI/ML and non-AI/ML modes.

- Proposal 5: RAN4 to study the performance expectations and potential requirements for the following transitions:

  - AI/ML mode → non-AI/ML mode,

  - Non-AI/ML mode → AI/ML mode.

- Proposal 6: RAN4 will study the desired performance gap between AI/ML and non-AI/ML operation modes and how it can be evaluated and achieved (which may require the same KPI(s) applicable for both AI/ML and non-AI/ML modes). This would be needed at least for UE-triggered fallback.

- Proposal 7: RAN4 will study a maximum delay requirement for switching between AI/ML and non-AI/ML operation modes, at least in the fallback direction (AI/ML →non-AI/ML).

- Proposal 8: The maximum fallback delay may include one or more components, e.g.,

  - obtaining and starting to use a fallback tx/rx configuration,

  - performing the necessary transmission and/or detection, identification, or measurement procedures,

  - informing the network about the fallback decision (e.g., if the UE decides this autonomously) or about completion of the fallback, etc.

# Summary

The following have been proposed in the current contribution:

- Proposal 1: With Option 1, it is challenging to ensure performance in all scenarios not known a priori. The tested scenarios are likely to be limited to the ones defined for RAN4 requirements. This option also implies a greater amount of logistics for updating and deploying new models.

- Proposal 2: Option 2 provides means to verify the model’s ability to perform KPI-based performance monitoring and its ability to act in a certain way upon KPI degradation. However, it is also important to ensure and verify the reliability of performance monitoring and the accuracy of reports used for KPIs, for which RAN4 and RAN1 may need to work together to derive KPIs, measurements, and corresponding requirements.

  - FFS: KPIs that isolate the model behaviour from other factors.

  - FFS: ensuring smooth fallback/transition between AI and non-AI model.

- Proposal 3: Option 3 provides means for a fair comparison between different models in the same UE or similar (e.g., from the architecture and RF/baseband perspective) UEs. This option is advantageous when model comparison is needed, e.g., to ensure that a newly deployed model is not worse than the previous model. The principles of Option 3 can be used also together with Option 1 and Option 2.

- Proposal 4: RAN4 sends an LS to inform other groups (e.g., RAN1 and RAN2) about its observations on feasibility, complexity, advantages, and disadvantages of Option 1, Option 2, and Option 3, to facilitate their discussions and to avoid standardizing solutions which are not feasible from RAN4 perspective.

- Proposal 5: RAN4 to study the performance expectations and potential requirements for the following transitions:

  - AI/ML mode → non-AI/ML mode,

  - Non-AI/ML mode → AI/ML mode.

- Proposal 6: RAN4 will study the desired performance gap between AI/ML and non-AI/ML operation modes and how it can be evaluated and achieved (which may require the same KPI(s) applicable for both AI/ML and non-AI/ML modes). This would be needed at least for UE-triggered fallback.

- Proposal 7: RAN4 will study a maximum delay requirement for switching between AI/ML and non-AI/ML operation modes, at least in the fallback direction (AI/ML →non-AI/ML).

- Proposal 8: The maximum fallback delay may include one or more components, e.g.,

  - obtaining and starting to use a fallback tx/rx configuration,

  - performing the necessary transmission and/or detection, identification, or measurement procedures,

  - informing the network about the fallback decision (e.g., if the UE decides this autonomously) or about completion of the fallback, etc.

# References

[1]	RAN4#112 Chairman’s notes, main session, Aug. 2024.

[2] R4-2414323, WF on AI/ML, Qualcomm Inc., Aug. 2024.