3GPP TSG-RAN WG4 Meeting #112bis 	R4-2416203

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item:	6.25.2

Source: 	Ericsson

Title:	On RRM core requirements for XR

Document for:	Discussion

# Introduction

The RAN4-related objectives of the Rel-19 XR Phase 3 WI are captured in [1].

RAN4 also received RAN1 LS [2], were RAN4 is requested to take the below information from [2] into account and decide whether or not to introduce any UE assistance information related to measurement occasions:

RAN1 Agreement

RAN1 continues to discuss and decide whether or not to introduce new UE assistance information for solution(s) to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements. At least the following UE assistance information is considered for further study:

- FFS: UE assistance information related to measurement occasions:

  - FFS: The number of needed measurement gaps/SMTC with restrictions within a time period;

  - FFS: The maximum number or ratio of MGs/SMTC with restrictions that can be skipped within a time period;

  - FFS: The number of required SSBs within a time period;

  - FFS: The number of consecutive RRM measurements that can be skipped;

  - FFS: The maximum interval between two consecutively reserved gap/restriction occasions for RRM measurements;

  - FFS: The patterns of gap(s)/restriction(s) where skipping is feasible or acceptable;

- FFS: UE assistance information related to channel conditions:

  - FFS: RSRP is below/above search threshold (s-MeasureConfig);

- FFS: UE assistance information related to traffic:

  - FFS: PSI (PDU set importance);

- FFS: UE assistance information related to UE mobility:

  - FFS: L3 parameters related to mobility, e.g., static or not

Companies are encouraged to provide additional details (e.g. how often the UE assistance info is provided, timing, applicable scenarios, performance gains, etc) on their preferred scheme.

Note: From specification point of view, there is no mandated gNB behavior in response to any of the UE assistance information.

RAN1 to make decision, from RAN1 perspective, in RAN1#117 on the support of UE assistance information.

Particularly, it was discussed whether impact on RRM performance from skipping measurement occasions may be reduced if UE sends additional information:

- Information about the maximum number of MGs/SMTC with restrictions that can be skipped within a time period.

- Information about the patterns of gap(s)/restriction(s) where skipping is feasible or acceptable.

Yet another RAN1 LS was received in [3], on UE’s gaps/restrictions that are caused by RRM measurements, where RAN1 is asking to review the working assumption, provide feedback if there are any concerns, and decide the value(s) for the minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of the corresponding skipped gap/restriction occasion indicated by the DCI:

Working Assumption

For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option:

- Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements.

  - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction;

    - Indication is included as part of scheduling DCI:

      - Bit-field size is one bit;

        - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI.

Send an LS to RAN4 to inform them of the above working assumption and ask them if there is any issue with it.

Agreement

If Alt. 1 from RAN1#117 agreement is supported, minimum time offset(s) X between indication to skip and skipped measurement occasion is up to RAN4 to discuss and decide on particular value(s).

The RAN4#112 agreements and WF are in [4]:

Issue 1-1: Work scope

Agreement:

- As starting point, RAN4 to discuss the Tx/Rx in occasions of L3 measurement.

  - The progress in RAN1/2 should be taken into account.

- FFS whether/when to start the discussion of Tx/Rx in occasions of scheduling restriction due to L1 operation.

- RAN4 to discuss

  - The potential impact on measurement requirements due to measurement cancellation

  - The corresponding solution to address the impact if needed.

Issue 1-2: Deployment scenarios

Way forward:

- Discuss if there are aspects related to deployment scenarios to be considered.

Issue 1-3: Types of measurement gaps to consider

In this issue we discuss the types of measurement gaps to be considered in this WID. In order to help the discussion, we reuse the definition of Type-1 and Type-2 measurement gaps as agreed in Rel-18 NR_MG-enh2:

- Type-1 MG: Gap(s) configured via GapConfig without suffix - Type-2 MG: Gap(s) configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17 - Rel-18 Concurrent Gap: gap with assigned priority which can be also NCSG or Pre-MG

Way forward: Please discuss the priorities for RAN4 to work based on the following table:

| Cap types | First prio | Second prio | Don’t include |
| --- | --- | --- | --- |
| Type 1 | Nokia, MTK, QC, Vivo, ZTE, Apple, CMCC, HW, Ericsson, OPPO, CATT, IDC |  |  |
| Type 2 | QC, vivo, Apple, CMCC, HW, OPPO, CATT | Nokia | ZTE |
| NCSG | Apple, CMCC, HW, OPPO, MTK | QC, Nokia, CATT, IDC | vivo |
| MUSIM |  | HW | Vivo, Apple, Nokia, Ericsson, CATT |
| Positioning |  | QC, vivo, HW | ZTE, Apple, Nokia, Ericsson, CATT, IDC |
| Rel 18 concurrent gap | QC, Apple, CMCC, HW, CATT, MTK |  | Vivo, ZTE, Nokia, Ericsson |
| PreConfigured | Apple, QC, CATT, CMCC, HW, OPPO, MTK |  |  |

Issue 1-4: Types of measurements

Way forward:

- Discuss types of measurements that may potentially be impacted by the XR operation.

  - FFS: measurements without gaps

Below we further discuss the RAN1 LSs and the impact of the WI objectives on RAN4 core requirements.

# Discussion

## RRM requirements enhancements for XR

This WI aims to realize system capacity gains by enabling transmission/reception in gaps/restrictions reserved for RRM measurements, while keeping impact to mobility performance limited. This was studied in TR 38.835 B.1.7.

The RRM measurement core and performance requirements are tightly linked to the number of measurement occasions. If some of the measurement occasions are not available for the measurements, the existing requirements may not be met, at least at the existing side conditions. Hence, to maintain the RRM measurement performance, two approaches could be envisioned:

- Approach 1: The simplest approach would be to limit the XR operation to unused measurement occasions, e.g., in good radio conditions, when RRM measurements could be completed by UE faster. No extension of the RRM measurement period would be needed in this case, but the availability of the unused measurement occasions may not be always guaranteed, e.g., at the cell edge, resulting in no time-frequency resources available for XR for UEs in poor radio conditions, e.g., when the interference is high and/or along cell edges.

- Approach 2: A more practical approach is to compensate for the configured measurement occasions reallocated to the XR operation by extending the RRM measurement period during the XR operation (the compensation means a longer measurement period compared to the UE operation without XR), to ensure the availability of the sufficient number of measurement occasions to complete the RRM measurements at the required accuracy level for any UE, regardless of the experienced radio conditions.

- Approach 3: Combination of Approach 1 and Approach 2, i.e., selectively (upon the need) extend the measurement period only for UEs in poor radio conditions.

With Approach 2 or Approach 3, the extension of the measurement period can be fixed or pre-defined or can be dynamically extendable based on the availability of the measurement occasions during the UE XR operation. The choice of how the extension is defined depends on the RAN1/RAN2 solution and the specified signalling, but the dynamically extendable RRM measurement period may be more suitable for XR due to the dynamic nature of the XR traffic.

- Proposal 1: To avoid limiting the XR operation to good network radio conditions only and hereby limiting the XR service coverage, RRM measurement requirements are enhanced for UEs under XR operation, at least for UEs not equipped with an additional rx chain for XR.

- Proposal 2: For UEs not equipped with an additional rx chain for XR, RAN4 to discuss possible enhancements to the legacy RRM requirements to account for the UE XR operation, e.g., discuss the options:

  - pre-defined fixed extended RRM measurement period for all UEs under XR operation,

  - dynamically extendable RRM measurement period for all UEs under XR operation,

  - pre-defined fixed extended RRM measurement period only for UEs under XR operation in poor radio conditions,

  - dynamically extendable RRM measurement period only for UEs under XR operation in poor radio conditions.

NOTE: 1) and 2) are based on Approach 2, while 3) and 4) are based on Approach 3.

As captured in the WF [4], prior to discussing the impact on RRM requirements, it is important to align the understanding on the deployment scenarios considered for Rel-19 XR enhancements. This includes frequency range (e.g., FR2-1 only or both FR1 and FR2-1), NR SA/EN-DC/NE-DC, carrier aggregation, etc.

- Proposal 3: RAN4 to discuss and agree on the applicable scenarios:

  - FR2-1,

    - FFS: FR1;

  - NR SA,

    - FFS: MR-DC, including NR-DC, EN-DC, and NE-DC;

  - FFS: Carrier aggregation.

In [4], RAN4 ad hoc discussion was captured on MG types to be considered for Rel-19 XR enhancements. RAN4 needs to conclude on this issue. Type 1 MGs can be taken as a baseline for MG cancelling. Relevance for cancellation of other MG types and MG purposes needs further confirmation in RAN4. For example, since the configured PRS (positioning) measurements may be intended for emergency positioning, it is proposed that MGs associated with PRS measurements shall not be cancelled. As of concurrent gaps, it is difficult to discuss MG cancellation for UE configured with concurrent MGs, until the single MG pattern framework is settled.

- Proposal 4.1: RAN4 considers Type 1 MGs for MG cancelling in Rel-19 XR enhancements, as a baseline.

- Proposal 4.2: MGs associated with PRS (positioning) measurements shall not be cancelled.

- Proposal 4.3: MUSIM MGs shall not be cancelled.

- Proposal 4.4: FFS: Network controlled pre-configured MGs, at least until the core requirements impacts are settled for Type 1 MGs.

- Proposal 4.5: FFS: Concurrent MGs, at least until the core requirements impacts are settled for Type 1 MGs.

Further, there also exist different types and purposes of RRM measurements, and to understand the RRM impact, RAN4 needs to understand which RRM measurements are impacted.

- Proposal 5: RAN4 to agree on the RRM measurements which may be impacted, e.g., intra/inter-frequency, inter-RAT, specific-purpose measurements, e.g., positioning, etc.

Even Type 1 MGs are configured by several configuration parameters, including MGL and MGRP and can be per-UE or per-FR. Not all MG configurations are equally important for XR, when MGs are cancelled to reallocate resource to XR.

- Proposal 6: RAN4 to discuss which conditions the measurement gap configurations need to meet to be relevant for the UE XR operation, e.g.,

  - minimum MGL,

  - maximum MGL,

  - maximum MGRP, etc.

At the same time, it is also noted that when some of the measurement occasions are used for the UE XR operation, there will also be impact on RRM measurement samples and how they are combined, since the samples may become non-equally spaced or disrupted, resulting in inaccurate or lost measurements, timing, etc., even if the total measurement period is met. This may result in the RRM measurement accuracy degradation.

- Proposal 7: RAN4 to discuss the impact on RRM samples and how they are combined during the UE XR operation, e.g., maximum acceptable time separation between two measurement occasions available for RRM measurements.

## On DRX impact on cancelled MGs

In addition to all the above considerations, we also note that a UE can be configured with DRX, which also has own periodicities (which can be longer or shorter than MGRP) and DRX ON duration, not to mention that also DRX and XR can be configured by different network nodes. When configured with DRX, the UE is not required to perform measurements more than once per DRX cycle, and the UE is not expected to monitor control channel outside DRX ON time period. Hence, not all MG occasions are relevant for cancelling and not any MG occasion is relevant XR.

- Proposal 8: RAN4 to discuss the impact of DRX on MG cancellation.

## On UE assistance information for XR

RAN1 has been discussing UE assistance data to suggest which and/or how RRM measurement occasions (e.g., in measurement gaps) are to be reallocated to XR. According to [2], RAN1 discussed whether the impact on RRM performance from skipping measurement occasions may be reduced if UE sends additional information:

- Information about the maximum number of MGs/SMTC with restrictions that can be skipped within a time period,

- Information about the patterns of gap(s)/restriction(s) where skipping is feasible or acceptable.

As correctly also pointed out in [2], from the specification point of view, there is no mandated gNB behavior in response to any of the UE assistance information. This means that there cannot be any explicit or implicit requirement on gNB behaviour upon receiving such assistance data.

- Proposal 9: RAN4 will not specify any explicit or implicit requirement on gNB behaviour in relation to the UE assistance data.

On the information about the maximum number of MGs/SMTC with restrictions that can be skipped within a time period: this is more related to RRM measurement performance and therefore can be pre-defined in the specification (TS 38.133). Furthermore, such number may depend on MGRP, so it can be more relevant to specify the maximum time between the remaining occasions available for RRM measurements.

On the information about the patterns of gap(s)/restriction(s) where skipping is feasible or acceptable: RAN4 can discuss whether this problem can be solved by the above, e.g., by defining in the specification the maximum time between the occasions available for RRM measurements. Also, using patterns may limit network scheduling flexibility and at the same time not all UEs will have the same preferred patterns, while the network will have to support all UEs, while will ultimately result in high complexity in the network. This is in addition to that gNB is not required to follow the UE patterns, anyway.

- Proposal 10: RAN4 to discuss whether the information related to the maximum number of measurement occasions allowed for reallocated to XR over a time can be defined in RAN4 specification.

- Proposal 11: UE assistance containing patterns still cannot guarantee that gNB configuration will follow the UE pattern, since the configuration is ultimately the network decision.

- Proposal 12: RAN4 considers the impact of DRX configuration on UAI, e.g., which measurement gaps are relevant for cancellation in favour of XR and how the measurement gaps occasions are counted at the UE and network side.

A draft LS is provided in a companion contribution.

## On the time offset

RAN1 LS [3] suggests that RAN4 discusses the minimum time offset between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI.

We note that both minimum offset (i.e., the DCI should not be received later than...) and the maximum offset (i.e., the DCI is not expected to be received earlier than...) values can be further discussed by RAN4.

- Proposal 13: RAN4 to discuss the range of acceptable time offset values between DCI reception and the intended MG, considered for XR operation, including:

  - the minimum time offset value, and

  - the maximum time offset value.

A draft LS is provided in a companion contribution.

# Summary

The following have been proposed in the current contribution:

- Proposal 1: To avoid limiting the XR operation to good network radio conditions only and hereby limiting the XR service coverage, RRM measurement requirements are enhanced for UEs under XR operation, at least for UEs not equipped with an additional rx chain for XR.

- Proposal 2: For UEs not equipped with an additional rx chain for XR, RAN4 to discuss possible enhancements to the legacy RRM requirements to account for the UE XR operation, e.g., discuss the options:

  - pre-defined fixed extended RRM measurement period for all UEs under XR operation,

  - dynamically extendable RRM measurement period for all UEs under XR operation,

  - pre-defined fixed extended RRM measurement period only for UEs under XR operation in poor radio conditions,

  - dynamically extendable RRM measurement period only for UEs under XR operation in poor radio conditions.

NOTE: 1) and 2) are based on Approach 2, while 3) and 4) are based on Approach 3.

- Proposal 3: RAN4 to discuss and agree on the applicable scenarios:

  - FR2-1,

    - FFS: FR1;

  - NR SA,

    - FFS: MR-DC, including NR-DC, EN-DC, and NE-DC;

  - FFS: Carrier aggregation.

- Proposal 4.1: RAN4 considers Type 1 MGs for MG cancelling in Rel-19 XR enhancements, as a baseline.

- Proposal 4.2: MGs associated with PRS (positioning) measurements shall not be cancelled.

- Proposal 4.3: MUSIM MGs shall not be cancelled.

- Proposal 4.4: FFS: Network controlled pre-configured MGs, at least until the core requirements impacts are settled for Type 1 MGs.

- Proposal 4.5: FFS: Concurrent MGs, at least until the core requirements impacts are settled for Type 1 MGs.

- Proposal 5: RAN4 to agree on the RRM measurements which may be impacted, e.g., intra/inter-frequency, inter-RAT, specific-purpose measurements, e.g., positioning, etc.

- Proposal 6: RAN4 to discuss which conditions the measurement gap configurations need to meet to be relevant for the UE XR operation, e.g.,

  - minimum MGL,

  - maximum MGL,

  - maximum MGRP, etc.

- Proposal 7: RAN4 to discuss the impact on RRM samples and how they are combined during the UE XR operation, e.g., maximum acceptable time separation between two measurement occasions available for RRM measurements.

- Proposal 8: RAN4 to discuss the impact of DRX on MG cancellation.

- Proposal 9 (UAI): RAN4 will not specify any explicit or implicit requirement on gNB behaviour in relation to the UE assistance data.

- Proposal 10 (UAI): RAN4 to discuss whether the information related to the maximum number of measurement occasions allowed for reallocated to XR over a time can be defined in RAN4 specification.

- Proposal 11 (UAI): UE assistance containing patterns still cannot guarantee that gNB configuration will follow the UE pattern, since the configuration is ultimately the network decision.

- Proposal 12 (UAI): RAN4 considers the impact of DRX configuration on UAI, e.g., which measurement gaps are relevant for cancellation in favour of XR and how the measurement gaps occasions are counted at the UE and network side.

- Proposal 13 (time offset): RAN4 to discuss the range of acceptable time offset values between DCI reception and the intended MG, considered for XR operation, including:

  - the minimum time offset value, and

  - the maximum time offset value.

# References

[1] RP-240791, WID on XR (eXtended Reality) for NR Phase 3, March 2024.

[2] R4-2411007/R1-2405736, LS on UE assistance information, RAN1, Aug./May 2024.

[3] R4-2414910/ R1-2407561, LS on gaps/restrictions that are caused by RRM measurements, RAN1 LS, Oct./Aug. 2024.

[4] R4-2414043, WF on RRM requirements for XR_Ph3, Nokia, Aug. 2024.