3GPP TSG-RAN WG4 Meeting #112bis 	R4-2416206

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item:	8.1.2.1

Source: 	Ericsson

Title:	On RRM specification quality improvement – clean up CRs

Document for:	Discussion

# Introduction

The agreed WF from RAN4#112 is in [6].

# Discussion

## Editor’s notes

The agreed WF [6] suggests removing redundant or unnecessary notes.

The editor’s note in clause A.3.15.3 of TS 38.133 seems to be not needed in this late release and it is more confusing than helping. It might be needed in the beginning of NR, but appears to be strange to have in late releases of NR and gives an impressions of uncertainty and incompleteness of the specification.

- Proposal 1: Agree on removing the editor’s note below in clause A.3.15.3 of TS 38.133:

A.3.15.3	Setup 3: 2 AoAs

There are 2 active probes in the test. The DL signals, and noise if applicable, transmitted from the two active probes, align to directions (AoAs) which are from the set of directions corresponding to the EIS spherical coverage percentile of the DUT as defined in clause 7.3.4 of TS 38.101-2 [19] for each UE power class. The relative angular offset between the directions (AoAs) of the 2 active probes, shall be changed for each test iteration. The applicable set of relative angular offsets between the 2 active probes is given in Table 3.15.3-1 for each UE power class.

## On RAN5 impact of the RRM specification improvement work

Since some parts of RAN5 specifications are based on RAN4 specifications, the RAN4 updates in test cases in the already complete WIs may potentially also impact also RAN5. It is therefore proposed to inform RAN5 about relevant updates (e.g., TBDs and FFSs) in RAN4 test cases of the complete WIs, carried out during the specification quality improvement framework. One way to inform RAN5 could be to send an LS to RAN5 listing the relevant RAN4 CRs agreed during this RAN4 framework.

- Proposal 2 (RAN5 LS): Inform RAN5 by sending an LS to RAN5 listing the relevant RAN4 CRs agreed during this RAN4 framework on RRM specification quality improvement.

## General RAN4 CR handling procedure

Since the discussion on general CR handling procedure did not conclude and did not even get any attention in RAN4#112, the improvements to ways of working with CRs in general are discussed below, repeating proposals from the last meeting RAN4#112.

### Handling big CRs after closing core WIs

Issue: Many maintenance core-part CRs after a core WI is closed.

After closing a WI, there can still be extensive maintenance CR work remaining. In this case, it may be more efficient to have a big maintenance CR running over a couple of meetings after closing the WI.

- Proposal 3: A big maintenance CR running over a couple of RAN4 meetings after closing the core WI.

### Handling big CRs before closing core WIs

Issue: Insufficient time to properly revise/review all CRs in the last meeting before the core WI is closed.

The feature CRs are often left to the last meetings before closing the feature, when the technical discussions are still on-going. This results in many misses in the agreed CRs in the last meeting of the core WI and lot of maintenance work afterwards, which is an indication of that there had been insufficient time for properly finalizing the CRs.

- Proposal 4: Allocate separate AI/TU for discussing and finalizing the feature CRs after the technical discussions are over.

  - Depending on the feature and the amount of specification impact, the time allocation can vary and can be up to the entire meeting week in the worst case.

### More detailed CR implementation plan by WI Rapporteurs

Issue: There is a need to discuss CR structures before submitting first CRs.

Currently, for all WIs a “work plan” is proposed by the Rapporteur detailing how the work is expected to be carried out. One or two meetings prior to creation of CRs, the Rapporteur could also be asked to create a more detailed “CR implementation plan”, capturing, e.g.:

- an overview of how the feature would be introduced (including specification section details, etc.),

- how requirements are expected to be stated,

- all relevant agreements during the WI, etc.

The above have been already practiced in RAN4 for some WIs (e.g., positioning, RedCap), but not all. In the same way as a work plan, the CR implementation plan can be debated and agreed. The CR implementation plan could be combined with the previous proposal of reserving a meeting for CR writing and reviewing.

- Proposal 5: WI Rapporteurs present a CR implementation plan (e.g., similar to the workplan, but focused on CR handling for the WI), discuss, and get it approved. This applies also when there is no work split.

- Proposal 6: For situations where similar text needs to be repeated across multiple sections (or specifications), the general text could firstly be agreed as a reference and then used across different sections/CRs/specifications to improve consistency.

### Assistance tools for CR editors

Issue: CR editors may need more assistance in the form of checkbox, templates, editable figures, etc.

#### CR editor checkbox list and extended Foreword section

For CR writing, it would be helpful to have an informal checkbox list listing the key specification editing aspects to go through and double check before submitting a CR, e.g., including:

- using consistently should/shall,

- CA configuration vocabulary, etc.

Completion of the checkbox list may enable a more consistent specification. Some of the aspects are already described in the Foreword section of the specification, but it could also be extended to cover more terms.

- Proposal 7: Creating a checkbox list of key specification editing aspects to remember and check while preparing CRs and/or extend the Forword section of the specification to ensure consistent usage of frequently used terms, notation, abbreviation, CA configuration vocabulary, etc.

#### 3GPP repository and templates with editable diagrams, formulas, figures

To assist CR editors, figure templates could be useful, to also ensure format/style consistency between different figures across the specification. Also, not all diagrams in the specifications are editable, and editable versions can be difficult to find.

- Proposal 8: Creating a 3GPP repository of figure templates, editable diagrams, and formulae. The link with templates could be included in the checkbox list described in Proposal 7.

# Summary

The following have been proposed in the current contribution:

- Proposal 1: Agree on removing the editor’s note below in clause A.3.15.3 of TS 38.133:

A.3.15.3	Setup 3: 2 AoAs

There are 2 active probes in the test. The DL signals, and noise if applicable, transmitted from the two active probes, align to directions (AoAs) which are from the set of directions corresponding to the EIS spherical coverage percentile of the DUT as defined in clause 7.3.4 of TS 38.101-2 [19] for each UE power class. The relative angular offset between the directions (AoAs) of the 2 active probes, shall be changed for each test iteration. The applicable set of relative angular offsets between the 2 active probes is given in Table 3.15.3-1 for each UE power class.

- Proposal 2 (RAN5 LS): Inform RAN5 by sending an LS to RAN5 listing the relevant RAN4 CRs agreed during this RAN4 framework on RRM specification quality improvement.

- Proposal 3: A big maintenance CR running over a couple of RAN4 meetings after closing the core WI.

- Proposal 4: Allocate separate AI/TU for discussing and finalizing the feature CRs after the technical discussions are over.

  - Depending on the feature and the amount of specification impact, the time allocation can vary and can be up to the entire meeting week in the worst case.

- Proposal 5: WI Rapporteurs present a CR implementation plan (e.g., similar to the workplan, but focused on CR handling for the WI), discuss, and get it approved. This applies also when there is no work split.

- Proposal 6: For situations where similar text needs to be repeated across multiple sections (or specifications), the general text could firstly be agreed as a reference and then used across different sections/CRs/specifications to improve consistency.

- Proposal 7: Creating a checkbox list of key specification editing aspects to remember and check while preparing CRs and/or extend the Forword section of the specification to ensure consistent usage of frequently used terms, notation, abbreviation, CA configuration vocabulary, etc.

- Proposal 8: Creating a 3GPP repository of figure templates, editable diagrams, and formulae. The link with templates could be included in the checkbox list described in Proposal 7.

# References

[1] RP-240782, Moderator's summary on RAN4 spec quality, Huawei, March, 2024.

[2] R4-2406710, WF on [110bis][142] RRM_Spec_Improvement, Apple, Apr. 2024.

[3] R4-2314393, CR 38.133 Correction of normative text in requirements, BeammWave, August, 2023

[4] R4-2403444, (TEI17) CR 36.133: Correction to normative text in requirements – Rel-17, Ericsson, February, 2024

[5] R4-2410715, WF on [111][140] RRM_Spec_Improvement, Apple Inc., May 2024.

[6] R4-2414103, WF on [112[229] RRM_Spec_Improvement, Apple Inc., Aug. 2024.