3GPP TSG-RAN WG4 Meeting #112-bis	R4-2416214

Hefei, China, 14th – 18th Oct. 2024

Source: 	Nokia

Title: 					n48 NS_27 A-MPR simulations beyond 40 MHz CBW

Agenda Item: 			5.12.2

Document for:	Information

# 1	Introduction

Currently, NS_27 defines emission criteria and A-MPR for selected channel bandwidths up to 40 MHz in n48. In this contribution, we present A-MPR simulation results for channel bandwidths 50…100 MHz.

# 2	Simulation assumptions and scenario

## 2.1	Simulation assumptions

- Operating band n48 (UL 3550 MHz – 3700 MHz)

- Channel bandwidths 50, 60, 70, 80, 90, 100 MHz

- Subcarrier spacing: 15 kHz for 50 MHz, 30 kHz for channel bandwiths > 50 MHz

- Power class PC3

- WOLA processing

- I/Q image -28 dBc

- Carrier leakage -28 dBc

- CIM3 -60 dBc, CIM5 -70 dBc

- NS_27 SEM from TS 38.101-1, Table 6.5.2.3.8-1, applied to all channel bandwidths:

Table 6.5.2.3.8-1: Additional requirements for "NS_27"

| ΔfOOB MHz | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  |  |  |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 20 | 30 | 40 |  |
| ± 0 - 1 | -13 |  |  |  |  |  | 1 % channel bandwidth |
| ± 1 - X | -13 |  |  |  |  |  | 1 MHz |
| < – X or > X | -25 |  |  |  |  |  |  |
| NOTE 1: X is occupied channel bandwidth as defined in Table 6.5.1-1. NOTE 2: The requirements apply only at the frequency range from 3540 MHz to 3710 MHz. |  |  |  |  |  |  |  |

- NS_27 additional spurious emission mask from TS 38.101-1, Table 6.5.3.3.14-1:

Table 6.5.3.3.14-1: Additional requirements for "NS_27"

| Frequency range (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5, 10, 15, 20, 30, 40 |  |
| 9 kHz – 3530 MHz | -40 | 1 MHz |
| 3530 MHz – 3540 MHz | -25 |  |
| 3710 MHz – 3720 MHz | -25 |  |
| 3720 MHz – 12.75 GHz | -40 |  |

- General criteria (ACLR, SEM, etc.) were excluded from the simulation as MPR covers them.

# 3	Simulation results and discussion

In this section we provide examples of the simulation results. The plots show required back-off where it exceeds the MPR. In an appendix, we provide complete sets of A-MPR triangles at lower edge and center of the n48 UL. Since the additional emission  requirements are symmetric w.r.t. n48 UL, the results at the upper edge of n48 UL are the mirror image of the respective results at the lower band edge.

Only the minimum SCS of each channel bandwidth was simulated since these results are applicable to all SCSs. The minimum SCS has the smallest guardband, and linear regrowth is no issue in this simulation.

The maximum back-off is 17 dB for OFDM and 8.5 dB for DFT-S-OFD, caused by CIM3 of narrow RB allocations close to channel edges. These values apply to  relatively few RB allocations. The majority of allocations requiring A-MPR do so due to spectral regrowth, with back-off £ 10 dB (OFDM) or £ 8 dB (DFT-S-OFDM).

When a 50 or 60 MHz channel is located at the center of n48, only very few allocations need a low A-MPR. In 70 MHz channel, more allocations start to require A-MPR, as depicted by Fig. 1.

Figures 2 and 3 present the simulated back-off for 64-QAM in 100 MHz channel. These example cases show clearly the A-MPR regions caused by different emissions.

Figure 1: Simulated back-off for QPSK in a 70 MHz channel at the center of n48 UL

Figure 2: Simulated back-off for 64-QAM in a 100 MHz channel at the lower edge of n48 UL

Figure 3: Simulated back-off for 64-QAM in a 100 MHz channel at the center of n48 UL; 
emissions reach the additional spurious mask on both sides of the band

# 4	Conclusions

We presented A-MPR simulation results for channel bandwidths 50…100 MHz in n48 / NS_27.

# 5	References

[1] R4-2411148, n48 100MHz UL, Apple, RAN4#112

[2] R4-2413200, On NS_27 A-MPR for larger than 40MHz UL CBW for n48, Skyworks Solutions, Inc., RAN4#112

# 6	Appendix

The documents embedded below contain A-MPR triangles at two channel positions within the operating band.

Channel at lower edge of operating band:

Channel at center of operating band: