# 3GPP TSG-RAN WG4 Meeting #112bis 　　　　　　　　　　　　　　　　　	R4-2416216

Hefei, Anhui, China, 14th  – 18th October, 2024

Source: 	NTT DOCOMO, INC.

Title:	Discussion on MPR improvement for FR2 CA

Agenda item:	6.1.1.2.3

Document for:	Discussion

- Introduction

In Rel-19, in the WI “UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”[1], MPR reduction is included as one of the objectives. RAN4#112 discussed MPR applicability for FR2 UL CA with DL intra band CA, and agreed to apply MPR based on UL BWchannel_CA in the WF[2].

#### Issue 3-2-2: Applicable MPR for FR2 UL CA with DL intra band CA - WF - In general, MPR based on UL BWchannel_CA applies instead that based on cumulative aggregated channel BW (CABW) - If only 1 UL CC is activated, the MPR requirements of single carrier could be reused - No need to specify new column for <400MHz CABW in the MPR table. FFS specific spec change to accommodate 200MHz BW granularity - FFS whether to take the minimum values of MPR between the intra-band CA case and the single carrier for FR2-2

However, the details of the MPR applicability are FFS. This paper provides our view on these issues.

- Discussion

the granularity of the 200MHz BW

For cumulative aggregated BW (CABW)  400MHz, the MPR for DFT-s-BPSK or DFT-s-QPSK is further divided at 200MHz granularity as described in the paper at previous meeting[3].

In section 6.2A.2.4.1 in TS 38.101-2[4] In case of a contiguous RB, DFT-s-BPSK or DFT-s-QPSK UL allocation in a single CC of a CA configuration with contiguous CCs, and whose cumulative aggregated BW 400 MHz, MPRC_CA shall be derived instead as MAX(MPR1, MPR2), where: MPR1 shall be determined from Table 6.2.2.3-1 if CABW 200 MHz, from Table 6.2.2.3-2 if CABW > 200 MHz. MPR2 shall be determined from Table 6.2.2.3-1 if UL BWchannel_CA 200 MHz, from Table 6.2.2.3-2 if UL BWchannel_CA > 200 MHz.

Under the current requirement, the worse MPR value is applied by comparing the cases where CABW and UL BWchannel_CA are applied respectively. This means that even if the UE supports the capability for MPR improvement, the MPR may still be dictated by CABW in some situations, which prevents the UE capability from being fully utilized. Therefore, it is preferable to apply only UL BWchannel_CA based on capability, even in such cases.

Proposal 1: For cumulative aggregated BW (CABW)  400MHz, the MPR for DFT-s-BPSK or DFT-s-QPSK is set to MPR2, which is determined based on CABW only, with UE capability for MPR improvement.

the values of MPR for FR2-2

According to the WF[2], MPR reduction applies to both FR2-1 and FR2-2.

#### Issue 3-1-4: sub-FR2 frequency ranges - Agreement in main session: - MPR reduction applies to both FR2-1 and FR2-2.

However, in some FR2-2 MPR, the MPR of single carrier case may be larger than that of intra-band contiguous CA. Therefore, it is necessary to take the minimum values of MPR between the intra-band CA case and the single carrier case.

For PC3 in FR2-2[4], the MPR is constant independent of CABW, which makes it ineffective to apply UL BWchannel_CA instead of CABW for intra band UL contiguous CA with DL intra band contiguous CA case.

Table 6.2A.2.4-2: Maximum power reduction (MPRWT_C_CA) for UE power class 3 in FR2-2 Waveform Type; Cumulative aggregated channel bandwidth. < 400 MHz; ≥ 400 MHz and < 800 MHz; ≥ 800 MHz and ≤ 1400 MHz; > 1400 MHz and ≤ 2000 MHz. Pi/2 BPSK; ≤ 1.0; ≤ 1.0; ≤ 1.0; ≤ 1.0. QPSK; ≤ 2.0; ≤ 2.0; ≤ 2.0; ≤ 2.0. 16 QAM; ≤ 4.0; ≤ 4.0; ≤ 4.0; ≤ 4.0. 64 QAM; ≤ 10.0; ≤ 10.0; ≤ 10.0; ≤ 10.0.

Proposal 2: For FR2-2, it is necessary to take the minimum values of MPR between the intra-band CA case and the single carrier case.

Observation 1: For PC3 in FR2-2, the MPR is constant independent of CABW, which makes it ineffective to apply UL BWchannel_CA instead of CABW for MPR applicability for intra band UL contiguous CA with DL intra band contiguous CA case.

UE capability for MPR improvement

In addition, as stated in our paper to last meeting[5], the possibility of implementations other than independent LO is not excluded. In order not to exclude future possibilities for LO implementations, capability should not be dependent on a specific implementation (e.g. MPRimprovementULdependent-R19).

Proposal 3: Capability should not be dependent on a specific implementation (e.g. MPRimprovementULdependent-R19).

While the decision of capability name should be up to RAN2, in our understanding, RAN4 should take it into account when drafting CR and LS.

CC activation based MPR improvement

At this meeting, we are submitting a draft CR (R4-2416215) as well, but this is for configuration based MPR improvements. For activation based MPR improvements, this CR will be updated at a future meeting if agreed.

Proposal 4: RAN4 to continue to discuss activation based MPR improvement whether there are any technical issues.

- Conclusion

Here we summarize our proposals:

Proposal 1: For cumulative aggregated BW (CABW)  400MHz, the MPR for DFT-s-BPSK or DFT-s-QPSK is set to MPR2, which is determined based on CABW only, with UE capability for MPR improvement.

Proposal 2: For FR2-2, it is necessary to take the minimum values of MPR between the intra-band CA case and the single carrier case.

Observation 1: For PC3 in FR2-2, the MPR is constant independent of CABW, which makes it ineffective to apply UL BWchannel_CA instead of CABW for MPR applicability for intra band UL contiguous CA with DL intra band contiguous CA case.

Proposal 3: Capability should not be dependent on a specific implementation (e.g. MPRimprovementULdependent-R19).

Proposal 4: RAN4 to continue to discuss activation based MPR improvement whether there are any technical issues.

4	References

[1] RP-241656, “Revised WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, RAN4 chair (Huawei)

[2] R4-2414441, “WF on power domain enhancements”, Huawei, HiSilicon

[3] R4-2411693, “Discussion on MPR reduction for FR2 CA”, Samsung

[4] TS 38.101-2 V18.7.0 (2024-09), User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone (Release 18)

[5] R4-2412356, “Discussion on FR2 CA MPR improvement”, NTT DOCOMO, INC.