3GPP TSG-RAN WG4 Meeting#112bis                              R4-2416229

Hefei , China, 14th – 18th Oct, 2024

Agenda item:	6.21.2.2

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on Co-existence evaluations

Document for:	Approval

Introduction

In the RAN#102 meeting, Study on solutions for Ambient IoT (Internet of Things) in NR has been approved and it’s expected to start the normative work from this April meeting. In the previous RAN4 meetings, we have reached lots of consensus on simulation assumptions for coexistence study. In this contribution, we would like to share some observations based on the initial coexistence simulation results.

Discussion

- 2.1. Simulation assumptions

In the previous RAN4 meeting, there are two options for NR in-band emission with some PRBs punctured for A-IoT service. As shown in the following figure with both 1 PRB punctured and 4 PRBs punctured, it seems that average leakage in PSD level is not as much as high as 30dBc, therefore we propose to use the 17dBc as assumption to draw the conclusion considering NR BS in-band emission towards A-IoT BS reception. This in-band emission is useless for A-IoT device reception since all signals within 10MHz should be received unless there are any 180kHz LBP filtering assumption.

Figure 1. PSD for 1 puncture PRB and 4 punctured PRBs

Proposal 1: use the in-band leakage as 17dBc for coexistence sharing study.

In the last RAN4 meeting, it was agreed to activate the A-IoT BS with following assumptions and leave its details up to the implementation. In our simulation platform, we use the activation scheme as shown in Figure 2 for the coexistence simulation. Namely for the row at the bottom, A-IoT BSs with the same color are activated at the same time and it also applies for A-IoT BS in other rows.

Agreement: - Minimum distance between active readers: 60m as baseline, other values can be reported by other companies. - 2 readers are activated in one drop as baseline

Figure 2. Activation scheme for A-IoT BS

Since the modulation/coding scheme for A-IoT system is still under the discussion in RAN1. Based on the previous discussion in RAN4, it was agreed to use the outage performance (e.g. > BLER 10%) if SINR degradation is larger than 1dB. Based on the initial simulation provided in Figure 3, SNR for OOK-1 with BW 180kHz is around 9dB@BLER 10%. This is used as basic criteria to distinguish whether device is outage or not.

Figure 3. BLER performance for OOK detection at device/tag side

- 2.2. Simulation results

According to the initial simulation results in the attached excel sheet, it seems that ACLR requirement around 45dBc for A-IoT BS for R2D transmission is sufficient to ensure the NR DL UE reception performance from coexistence sharing perspective.

Proposal 2: for A-IoT BS ACLR requirement, propose to define it as 45dBc;

According to the initial simulation results in the attached excel sheet, it need more discussions on A-IoT BS ACS requirement especially considering the NR UE within the indoor scenario with its maximum power transmission, this will cause significant negative impacts of A-IoT BS receiver performance. Currently, due to activated A-IoT device per indoor factory is only two, so co-channel interference from other device within the same factory is quite limited, however NR UE transmission leakage will have big impacts on A-IoT BS SINR degradation, this kind of degradation could be even observed when NR UE is located in the outdoor scenario with some penetration loss considered between aggressive NR UE and victim A-IoT BS receiver.

Proposal 3: need some further discussion on A-IoT BS ACS requirement.

Conclusions

In this contribution, we shared some further views on coexistence evaluations and proposals are made as following:

Proposal 1: use the in-band leakage as 17dBc for coexistence sharing study.

Proposal 2: for A-IoT BS ACLR requirement, propose to define it as 45dBc;

Proposal 3: need some further discussion on A-IoT BS ACS requirement.

References

- RP-240826,Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR (CMCC), Approved.

- R4-2410567,WF on co-existence study for ambient IoT and NR/LTE, Approved.

- R4-2410567,WF on co-existence study for ambient IoT and NR/LTE, Approved.