3GPP TSG-RAN WG4 Meeting#112bis                              R4-2416235

Hefei, China, 14th – 18th Oct, 2024

Agenda item:	6.10.2.1

Source: 	ZTE Corporation, Sanechips

Title: 	RF requirement for Intra-band contiguous CA of ATG UE in Rel-19

Document for:	Approval

Introduction

In the RAN#103 meeting, new WID on Enhancements for Air-to-ground network for NR has been approved and it’s expected to start the normative work from this April meeting. In last RAN4 meeting, we have some initial discussions for ATG UE supporting intra-band CA. In this contribution, we would like to share some further views on these remaining issues.

Issue 2-1-2: whether/how to reflect operation band information in spec Agreement: - BCS 4 and 5 also needs to be specified besides BCS0. Table 1: BCS for CA_n79C NR CA configuration / Bandwidth combination set. NR CA configuration; Uplink CA configurations or single uplink carrier5; Channel bandwidths for carrier (MHz); Channel bandwidths for carrier (MHz); Channel bandwidths for carrier (MHz); Channel bandwidths for carrier (MHz); Channel bandwidths for carrier (MHz); Maximum aggregated bandwidth (MHz); Bandwidth combination set. CA_n79C; CA_n79C; 50; 60, 80, 100; 200; 0. 60; 60, 80, 100. 80; 80, 100. 100; 100. See n79 channel bandwidths in Table 5.3.5-1 for each carrier2; 200; 4 and 5. Issue 2-2: ACS testing parameters for case 2 Agreement: - Further check the following table. - Table 3: Test parameters for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, case 2 Rx Parameter; Units; NR CA bandwidth class. C. Pw in Transmission Bandwidth Configuration, per CC; dBm; -73.54 -61.55. PInterferer; dBm; -424 -305. BWInterferer; MHz; BWchannel CA. FInterferer (offset); MHz; BWchannel CA / -BWchannel CA. NOTE 1: The transmitter shall be set to 24 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the carrier closest to the interferer in MHz. The interferer is an NR signal with an SCS equal to that of the closest carrier. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. NOTE 4: Pinterferer shall be set to -42dBm for omni-directional antenna. NOTE 5: Pinterferer shall be set to -30dBm for antenna array..

Discussion

- 2.1. RF requirements for Intra-band CA

In this section, we provide some initial analysis on the overall requirement for ATG UE supporting intra-band DL CA and also provided the summary of proposals for it.

Table 2. Analysis and summary of proposals for ATG UE supporting intra-band DL CA

| The existing RF requirement for intra-band contiguous CA | ATG UE supporting intra-band contiguous CA |
| --- | --- |
| 7.3A.2.1 Reference sensitivity power level for Intra-band contiguous CA | Reference sensitivity power level for Intra-band contiguous CA For intra-band contiguous carrier aggregation, the throughput of each component carrier shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3.2-1a, Table 7.3.2-1b, Table 7.3.2-2, and Table 7.3.2-3. For UE(s) supporting one uplink carrier, the uplink configuration of the PCC shall be in accordance with Table 7.3.2-3 and the downlink PCC carrier center frequency shall be configured closer to uplink operating band than any of the downlink SCC center frequency. |
| 7.4A.1 Maximum input level for Intra-band contiguous CA | Maximum input level for Intra-band contiguous CA For intra-band contiguous carrier aggregation maximum input level is defined as the maximum mean power received at the UE antenna port, over the Transmission bandwidth configuration of each CC. The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexs A.3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.4A.1-1 for each component carrier. Table 7.4A.1-1: Maximum input level for Intra-band contiguous CA Rx Parameter; Units; NR CA Bandwidth Class. B; C; D. Power in largest transmission bandwidth configuration CC, Plargest BW; dBm; [-42]2 for omini antenna [-30] for array antenna; [-42]2 for omini antenna [-30] for array antenna; N/A. [-44]3 for omini antenna [-32] for array antenna; [-44]3 for omini antenna [-32] for array antenna; N/A. Power in each other CC; dBm; Plargest BW +10*log{(NRB,c*SCSc)/(NRB,largest BW*SCSlargest BW)}. NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 and 7.3F.2-3 for shared spectrum channel access operation with PCMAX_L,f,c as defined in clause 6.2.4. NOTE 2: Reference measurement channel is A.3.2.3 or A.3.3.3 for 64 QAM. NOTE 3: Reference measurement channel is A.3.2.4 or A.3.3.4 for 256 QAM.. |
| 7.5A.1 Adjacent channel selectivity for Intra-band contiguous CA | To follow the existing requirement defined in clause 7.5A.1 with the following power to be updated for ATG UE with agreed maximum input power for Case 2 and also scale with wanted signal power level. - Pw in Transmission Bandwidth Configuration, per CC - PInterferer |
| 7.6A.2.1 In-band blocking for Intra-band contiguous CA | To follow the existing requirements defined in Clause 7.6A.2.1. |
| 7.6A.3.1 Out-of-band blocking for Intra-band contiguous CA | To follow the existing requirements defined in Clause 7.6A.3.1. |
| 7.6A.4.1 Narrow band blocking for Intra-band contiguous CA | N/A |
| 7.7A.1 Spurious response for Intra-band contiguous CA | To follow the existing requirements defined in Clause 7.7A.1. |
| 7.8A.2.1 Wide band intermodulation for Intra-band contiguous CA | To follow the existing requirements defined in Clause 7.8A.2.1. |

Conclusions

In this contribution, we shared some further views on the RF requirements for ATG UE supporting intra-band DL CA and proposals are made as following:

Proposal 1: for RF requirements for inter-band DL CA, refer to the proposals in table 2.

References

- RP-240839,New WID on Enhancements for Air-to-ground network for NR, Approved.

- R4-2321919,Big CR for TS 38.101-1 for NR ATG, Agreed.

- R4-2403864, CR for 38101-1 to update ATG related signaling name, Agreed

- R4-2406594, WF on Rel-19 ATG UE requirements, Approved.

- R4-2410687,WF on Rel-19 ATG UE RF requirements, Approved.

- R4-2414322, WF on Rel-19 ATG UE RF requirements, Approved.

Table 7.5A.1-3: Test parameters for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, case 2

| Rx Parameter | Units | NR CA bandwidth class |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | B | C | D |  |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | -49.5 + 10log(NRB,c/NRB_agg) | -56.5 | -48.7 + 10log(NRB,c/NRB_agg) |  |
| PInterferer | dBm | -25 | -25 | -25 |  |
| BWInterferer | MHz | 20 | BWchannel CA | 50 |  |
| FInterferer (offset) | MHz | 10 + Foffset / -10 -Foffset | BWchannel CA/2 + Foffset / -BWchannel CA/2 + Foffset | 25 + Foffset / -25 -Foffset |  |
| NOTE 1: The transmitter shall be set to 24 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the carrier closest to the interferer in MHz. The interferer is an NR signal with an SCS equal to that of the closest carrier. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |  |  |  |