3GPP TSG-RAN WG4 Meeting#112bis                              R4-2416240

Hefei, China, 14th – 18th Oct, 2024

Agenda item:	6.11.4

Source: 	ZTE Corporation, Sanechips

Title: 	Further discussion on Expected EIRP mask for upper 6GHz

Document for:	Approval

Introduction

In the RAN#103 meeting, new WID for Rel-19 BS RF evolution has been approved and it’s expected to start the normative work from this April meeting. In last RAN4 meeting, we reached lots of consensus for U6GHz EIRP mask as captured in [5], however there are still lots of open issues left for further discussions. In this contribution, we would like to share further understandings on these remaining issues.

Issue 2-8 Expected EIRP sampling grid for average EIRP - Recommended for further discussion: - Need further discussions Issue 2-9 Other related with conformance testing declaration and RF channels - Recommended for further discussion: - The following issues to be considered: - the number of mechanical down-tilt and its corresponding angular coverage range; - For each angular coverage range, the number of beams for conformance testing and its corresponding weighting factor; - To balance the number for beams for conformance testing/complexity and test accuracy; - The impacts of potential factors (measurement sampling grid for summation error etc) on EIRP accuracy. - in conformance testing, consideration of converting coordinate system to 3GPP coordinate system, to align with other test cases - Other issues are not precluded. Issue 2-10 Other related with RF channels - Recommended for further discussion: - Need further discussions

Discussion

  - .The number of beams for conformance testing and weighting factor

In the previous RAN4 meetings, there were some initial discussions on how to balance the number for test beams for conformance testing/complexity and test accuracy. Technically speaking, the number of serving beams for practical NR BSs could be up to 300-400 beams considering the finer steering granularity in the elevation domain and the relative coarse granularity in the azimuth domain since its steering range is supposed to range from -60 degree to +60degree. If all these practical serving beams to tested within the OTA chamber, then to be further combined according to the appropriate weighting factors, then the measurement time in total would be quite long compared with Rel-15 TRP or EIRP measurement. In addition, if considering repeating the measurement among different mechanical down-tilt if needed, then the measurement time for EEIRP mask would be further longer which is not quite efficient for the BS conformance testing. The detailed analysis for the measurement time would be discussed in section 2.4 accordingly.

In the contribution [6], some initial EEIRP mask evaluations has been conducted to identify the appropriate test number for conformance testing. In the following section, the similar EEIRP evaluation results are provided based on the following antenna geometry assumption in Table 2.2.-1.

Table 2.1-1: Array antenna geometry

| Parameter | Value |
| --- | --- |
| Frequency | 6775 MHz |
| Element model | Vertical HPBW: 65° |
|  | Horizontal HPBW: 90° |
|  | Sidelobe suppression: 30dB |
|  | Element directivity: 6.4 dBi |
| Array configuration | Number of rows: 4 |
|  | Number of columns: 8 |
|  | Sub-array: 3x1 |
|  | Sub-array tilt: 0° |
|  | Horizontal element separation: 0.5λ |
|  | Vertical element separation: 0.7λ |
| conducted power per sub-array | 28dBm per subarray |
| Peak EIRP target | 55.7 dBm/100 MHz |

According to the initial simulation results provided in Table 2.1.-2, in order to have more accurate EEIRP mask measurement, the EEIRP measurement on top of beam set J and K are provided with the following assumptions:

- Beam set J:

- θ° is ranging from 0° to 12° with step size as 1°

- φ° is ranging from -60° to 60° with step size as 5°

- Uniform weighing factor

- Beam set K:

- θ° is ranging from 0° to 12° with step size as 1°

- φ° is ranging from -60° to 60° with step size as 10°

- Uniform weighing factor

Table 2.1-2: updated simulation results for EEIRP mask

| Vertical angle range | Sufficient beams to be tested |  | Beams set to be tested [R4-2409118] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Beam weight factors for Beam set | J | K | A | B | C | D | E |
| 0° - 5° | 25.69 | 25.66 | 26.18 | 26.45 | 26.83 | 25.73 | 22.39 |
| 5° - 10° | 17.00 | 16.97 | 17.44 | 17.20 | 18.03 | 17.13 | 16.19 |
| 10° - 15° | 13.83 | 13.79 | 14.11 | 14.02 | 14.65 | 14.88 | 14.62 |
| 15° - 20° | 11.70 | 11.66 | 11.86 | 11.64 | 12.47 | 12.31 | 11.92 |
| 20° - 30° | 9.35 | 9.28 | 9.43 | 8.91 | 10.20 | 9.39 | 8.84 |
| 30° - 60° | 5.30 | 5.17 | 5.35 | 4.68 | 6.73 | 5.91 | 5.26 |
| 60° - 90° | -0.95 | -1.11 | -0.79 | -1.60 | 3.22 | 1.79 | -0.69 |
| Number of testing beams | 325 | 169 | 25 | 25 | 9 | 9 | 9 |
| Weighting factor | [uniform] | [uniform] | [non-uniform] | [uniform] | [non-uniform] | [uniform] | [non-uniform] |
| NOTE: the measurement step size in elevation domain is 1 degree and measurement step size in the azimuth domain is 1 degree. |  |  |  |  |  |  |  |

Table 2.1-3: the EIRP measurement difference between different beam sets

| Vertical angle range | Beams set to be tested [R4-2409118] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Beam weight factors for Beam set | A-J | B-J | C-J | D-J | E-J |
| 0° - 5° | 0.49 | 0.76 | 1.14 | 0.04 | -3.3 |
| 5° - 10° | 0.44 | 0.2 | 1.03 | 0.13 | -0.81 |
| 10° - 15° | 0.28 | 0.19 | 0.82 | 1.05 | 0.79 |
| 15° - 20° | 0.16 | -0.06 | 0.77 | 0.61 | 0.22 |
| 20° - 30° | 0.08 | -0.44 | 0.85 | 0.04 | -0.51 |
| 30° - 60° | 0.05 | -0.62 | 1.43 | 0.61 | -0.04 |
| 60° - 90° | 0.16 | -0.65 | 4.17 | 2.74 | 0.26 |

It could be observed that EEIRP measurement difference between 325 testing beams and 25 testing beams (e.g. beam set A and B) are quite limited to be lower than 0.8dB which could reduce the measurement time and complexity significantly. For the measurement difference between beam set J (325 testing beams) and beam set C (9 testing beams) could be maintained within 1.5dB except for 60° - 90°. For the measurement difference between beam set J (325 testing beams) and beam set D (9 testing beams) could be maintained within 1.05dB except for 60° - 90°. For EEIRP measurement for beam set E (9 testing beams), it seems to the best one in term of the interference towards FSS service due to the low weighting factors for the following beam directions with φ° as 0 degree. In other words, it also give the U6GHz BS one candidate optimization to comply with EEIRP mask requirement, however this is not the worst case for the conformance testing.

Table 2.1-4: beam weight factors for Beam set E

| Test Beam [k] | Direction θ° | Direction φ° | Portion weights [w_k] |
| --- | --- | --- | --- |
| 1 | 6 | 0 | 1/16 |
| 6 | 3 | 0 | 1/16 |
| 7 | 9 | 0 | 1/16 |
| 8 | 0 | 0 | 1/32 |
| 9 | 12 | 0 | 1/32 |

From our understanding, both 25 testing beams and 9 testing beams could be acceptable from the measurement error perspective compared with the sufficient testing beams. From the measurement time and complexity perspective, beams set C, D with 9 testing beams are more preferred.

Similar as NR or MSR test model design, if RAN4 could specify the testing beams with its corresponding weighting factors, this will be quite useful to have aligned EEIRP conformance testing among different vendors.  And these selected testing beams will not only provide the test coverage but also ensure the reasonable measurement uncertainty.

Proposal 1: propose to use the beam set D with 9 testing beams for EEIRP conformance testing.

  - .The declaration of mechanical downtilt and its corresponding coverage range

As mentioned in the ITU-R resolution requirements, the EEIRP mask requirement should be applicable for any mechanical down-tilt which is reasonable since NR BSs deployed in different targeted scenarios are supposed to be configured with different mechanical down-tilts to provide the corresponding coverage. If NR BS is capable to be deployed in lots of different scenarios, then NR BS should fulfill the EEIRP mask requirement at its supported mechanical down-tilt. In short, the concept proposed in [3] seems more comprehensive solutions to consist of both coverage angular range and steering ranges for each mechanical down-tilt.

Table 2.2-1: Manufacturer declarations

| CAR ID | Parameter | Description | Example values |
| --- | --- | --- | --- |
| 1 | min | Minimum angle along  axis | 0 degrees |
|  | max | Maximum angle along  axis | 6 degrees |
|  | min | Minimum angle along axis | -60 degrees |
|  | max | Maximum angle along  axis | 60 degrees |
|  | Δk | Test beam power back-off | 0, 0, 0, 0, 0, 0, 0, 0, 0 dB |
| 2 | min | Minimum angle along  axis | 0 degrees |
|  | max | Maximum angle along  axis | 12 degrees |
|  | min | Minimum angle along axis | -60 degrees |
|  | max | Maximum angle along  axis | 60 degrees |
|  | Δk | Test beam power back-off | 0, 0, 0, 0, 0, 0, 0, 0, 0 dB |
| ⋮ | ⋮ | ⋮ | ⋮ |
| Q | min | Minimum angle along  axis | 0 degrees |
|  | max | Maximum angle along  axis | 15 degrees |
|  | min | Minimum angle along axis | -60 degrees |
|  | max | Maximum angle along  axis | 60 degrees |
|  | Δk | Test beam power back-off | 2, 2, 2, 2, 2, 2, 2, 2, 2 dB |

Proposal 2: to consider the manufacture declarations in Table 2.2-1 for different mechanical down-tilt and its the corresponding coverage angular range for the declaration.

As mentioned before, to repeat the EEIRP measurement under the different mechanical downtilt, the measurement time would also increase significantly, therefore it is essential to consider the reduction of measurement time for different mechanical downtilt. In the following tables 2.2.2a/b/c/d, some initial EEIRP measurement comparison under the different mechanical downtilts are provided for the initial analysis. It should be noted that electrical testing beams are kept the same for different mechanical downtilt. It could be observed according to the following simulation results, the EEIRP measurement would decrease when the mechanical downtilt increase. It could be understandable since the lower sidebloes and grate lobes steering towards direction over the horizontal plane. In other words, EEIRP measurement under the mechanical downtilt should be the most stringent scenario expect for beam steering upwards the horizontal.

Table 2.2-2a: EEIRP measurement comparison for different mechanical down-tilt

| Vertical angle range | Beams set to be tested [R4-2409118] Beam set A |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Mechanical downtilt | 0 degree | 1 degree | 2 degree | 4 degree | 6 degree |
| 0° - 5° | 26.18 | 24.82 | 23.35 | 20.57 | 18.78 |
| 5° - 10° | 17.44 | 16.73 | 16.22 | 15.26 | 14.33 |
| 10° - 15° | 14.11 | 13.60 | 13.17 | 12.47 | 11.77 |
| 15° - 20° | 11.86 | 11.48 | 11.10 | 10.46 | 9.92 |
| 20° - 30° | 9.43 | 9.16 | 8.88 | 8.30 | 7.75 |
| 30° - 60° | 5.35 | 5.07 | 4.78 | 4.21 | 3.65 |
| 60° - 90° | -0.79 | -1.31 | -1.83 | -2.84 | -3.84 |
| NOTE: for the different mechanical down-tilt, the beam steering range doesn’t change in this simulation. |  |  |  |  |  |

Table 2.2-2b: EEIRP measurement comparison for different mechanical down-tilt

| Vertical angle range | Beams set to be tested [R4-2409118] Beam set B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Mechanical downtilt | 0 degree | 1 degree | 2 degree | 4 degree | 6 degree |
| 0° - 5° | 26.45 | 25.30 | 24.02 | 21.45 | 19.59 |
| 5° - 10° | 17.20 | 16.41 | 15.96 | 15.29 | 14.48 |
| 10° - 15° | 14.02 | 13.43 | 12.90 | 12.20 | 11.69 |
| 15° - 20° | 11.64 | 11.35 | 10.99 | 10.20 | 9.62 |
| 20° - 30° | 8.91 | 8.65 | 8.43 | 8.03 | 7.51 |
| 30° - 60° | 4.68 | 4.43 | 4.16 | 3.57 | 2.97 |
| 60° - 90° | -1.60 | -2.07 | -2.54 | -3.53 | -4.58 |
| NOTE: for the different mechanical down-tilt, the beam steering range doesn’t change in this simulation. |  |  |  |  |  |

Table 2.2-2c: EEIRP measurement comparison for different mechanical down-tilt

| Vertical angle range | Beams set to be tested [R4-2409118] Beam set C |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Mechanical downtilt | 0 degree | 1 degree | 2 degree | 4 degree | 6 degree |
| 0° - 5° | 26.83 | 25.12 | 23.20 | 19.63 | 17.71 |
| 5° - 10° | 18.03 | 17.24 | 16.60 | 15.25 | 14.32 |
| 10° - 15° | 14.65 | 14.14 | 13.75 | 12.84 | 11.75 |
| 15° - 20° | 12.47 | 11.96 | 11.57 | 11.14 | 10.40 |
| 20° - 30° | 10.20 | 9.93 | 9.59 | 8.82 | 8.32 |
| 30° - 60° | 6.73 | 6.42 | 6.13 | 5.61 | 5.05 |
| 60° - 90° | 3.22 | 2.65 | 2.12 | 1.17 | 0.33 |
| NOTE: for the different mechanical down-tilt, the beam steering range doesn’t change in this simulation. |  |  |  |  |  |

Table 2.2-2d: EEIRP measurement comparison for different mechanical down-tilt

| Vertical angle range | Beams set to be tested [R4-2409118] Beam set D |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Mechanical downtilt | 0 degree | 1 degree | 2 degree | 4 degree | 6 degree |
| 0° - 5° | 25.73 | 24.39 | 22.88 | 19.63 | 17.30 |
| 5° - 10° | 17.13 | 16.04 | 15.63 | 15.34 | 14.36 |
| 10° - 15° | 14.88 | 14.00 | 13.08 | 12.06 | 11.92 |
| 15° - 20° | 12.31 | 12.25 | 11.92 | 10.64 | 9.65 |
| 20° - 30° | 9.39 | 8.99 | 8.78 | 8.60 | 8.17 |
| 30° - 60° | 5.91 | 5.70 | 5.46 | 4.82 | 4.12 |
| 60° - 90° | 1.79 | 1.33 | 0.86 | -0.15 | -1.25 |
| NOTE: for the different mechanical down-tilt, the beam steering range doesn’t change in this simulation. |  |  |  |  |  |

In addition, during the previous RAN4 discussions, some companies clarified that EEIRP measurement under the different mechanical downtilt could be post-processed by one EIRP measurement over the testing beams since NR BS in the chamber would be also rotated for EIRP measurement. In other words, the EIRP measurement under the different mechanical downtilt could be indirectly measured by the spatial EIRP sampling in the OTA chamber. For example, in the following figure 2.2-1,

- for the mechanical downtilt 0 degree, its electrical test beam in the elevation domain is 0, 3, 6, 9, 12,

- for the mechanical downtilt 3 degree, its electrical test beam in the elevation domain is 0, 3, 6, 9,

- for the mechanical downtilt 6degree, its electrical test beam in the elevation domain is 0, 3, 6;

Indeed the test beams under the mechanical downtilt 3, 6 could be tested when the mechanical downtilt as 0 degree with BS rotate in 3 and 6 degree in the elevation angle for EIRP measurement in the OTA chamber.

Observation 1: if electrical testing beams under different mechanical tilt are the same or subset of full set beam, EEIRP measurement under different mechanical tilt could be post-processed to save the measurement time.

Figure 2.2-1. beam sets under the different mechanical down-tilt

However for different mechanical downtilt, the electrical beams might be different according to the practical deployment. For example, in the following figure 2.2-2,

- for the mechanical downtilt 0 degree, its electrical test beam in the elevation domain is 0, 3, 6, 9, 12,

- for the mechanical downtilt 3 degree, its electrical test beam in the elevation domain is -3, 0, 3, 6, 9,

- for the mechanical downtilt 6degree, its electrical test beam in the elevation domain is -6,-3, 0, 3, 6;

Figure 2.2-2. beam sets under the different mechanical down-tilt

Since the electrical testing beams are somehow different under the different mechanical downtilt and also not subset of any other mechanical downtilt, therefore it is not exchangeable or could be post-processed by full set testing beam measurement. Even though from our initial expectations, even though the number of testing beams under different mechanical downtilt are the same, the EEIRP at mechanical downtilt 6 degree still achieve the lowest EEIRP measurement since its sidelobe from antenna element is supposed to be lowest one.

In short, if the testing beams under the different mechanical tilt are somehow different, then EEIRP measurement under different mechanical tilt could not be post-processed by single EIRP measurement in the full sphere or upper sphere.

Observation 2: if electrical testing beams under different mechanical tilt are different, EEIRP measurement under different mechanical tilt could not be post-processed by single EIRP measurement in the full sphere or upper sphere.

In addition, as we know that, for the mechanical downtilt, this is quite dependent on the operators’ deployment, it is also difficult to specify the any conformance testing requirement related with mechanical tilt. What can we do in RAN4 is to try to save the measurement efforts under the different mechanical tilt. From our understanding, the following two clarification NOTE could be added in the conformance testing specification.

Proposal 3:

- if electrical testing beams under different mechanical tilt are the same or subset of others, EEIRP measurement under different mechanical tilt could be post-processed on top of full set of testing beams to save the measurement time.

- if electrical testing beams under different mechanical tilt are different, EEIRP measurement under different mechanical tilt should be tested separately.

  - .Sampling grid for EEIRP mask measurement

In the previous RAN4 meeting, there were some initial discussions on sampling grid as shown in Figure 2.3.2 for EEIRP measurement to reduce the testing time accordingly similar as TRP measurement grid shown in Figure 2.3.1 as defined in TS 38.141-2 Annex. In this section, some initial EEIRP measurement results for single beam with mechanical down-tilt as 0 degree are provided based on the antenna geometry assumption as defined in 2.2-1 similar as the contribution [7].

Figure 2.3.1. spatial sampling grid for full sphere

Figure 2.3.2. spatial sampling grid for upper semi-sphere

Table 2.3-1: Initial simulation results for EEIRP mask [updated]

| Vertical angle range | Full resolution | Vertical / Horizontal Angular Step Size |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Theta/phi | 0.1° /0. 1° | 5° / 5° | 2.5° / 5° | 2.5° / 2.5° | 1° / 15° | 1° / 10° | 1° / 5° | 1° / 1° |
| 0° - 5° | 25.52 | 26.02 | 25.76 | 25.76 | 26.04 | 25.73 | 25.73 | 25.73 |
| 5° - 10° | 16.91 | 19.36 | 18.37 | 18.37 | 17.37 | 17.13 | 17.13 | 17.13 |
| 10° - 15° | 13.70 | 14.37 | 14.81 | 14.81 | 15.06 | 14.88 | 14.88 | 14.88 |
| 15° - 20° | 11.69 | 11.90 | 12.13 | 12.13 | 12.45 | 12.31 | 12.31 | 12.31 |
| 20° - 30° | 9.35 | 10.46 | 10.15 | 10.15 | 9.48 | 9.39 | 9.39 | 9.39 |
| 30° - 60° | 5.27 | 6.02 | 5.89 | 5.89 | 5.92 | 5.91 | 5.91 | 5.91 |
| 60° - 90° | -1.02 | 1.37 | 1.18 | 1.18 | 1.79 | 1.79 | 1.79 | 1.79 |
| Number of measurement points |  | 1296 | 2592 | 5184 | 2160 | 3240 | 6480 | 32400 |

According to the initial measurement results provided in Table 2.3.1, when the EEIRP measurement results are almost the same when the spatial sampling grid of theta and phi is between 1° / 1° and 1° / 10°. When the sampling grid is up to 1° / 15°, then there will be some additional variations/errors compared with other dense sampling grid. In addition, when the sampling grid in the vertical domain is increased to 5° or 2.5°, then the measurement uncertainty will be further increased.

Observation 3: the EEIRP measurement results are almost the same when spatial sampling grid of theta and phi between 1° / 1° and 1° / 10°.

Proposal 4: for the elevation angle, propose the measurement step size as 1 degree.

Proposal 5: for the azimuth angle, propose the measurement step size as 5 degree.

  - .Measurement time

- Regarding the criteria to determine the number of test beams for EEIRP measurement, at least the two factors should be taken into account:

- Measurement uncertainty

- Testing time.

For the legacy far filed OTA testing with single testing probe within the chamber as shown in Figure 2.4.1, in order to get the upper semi-sphere EIRP measurement to calculate the EEIRP measurement, then it is necessary to rotate the BS or positioner in both azimuth and elevation angle for spatial sampling by testing antenna or probe. If considering the elevation angle step size as 1 degree and azimuth angle step size as 2 degree, then the measurement time in total should be roughly around 22.5 hour which is mainly contributed by the mechanical rotation of BS or positioner. As analyzed before, if to conduct the spatial sampling per beam basis, then total measurement time for all testing beams would be in the month level which is not reasonable and very expensive for the practical OTA testing. One potential way to reduce the OTA measurement time for testing beams efficiently is to measure all testing beams as shown in Figure 2.4-2a/2b in each rotation of BS or positioner instead of single test beam in each rotation of BS or positioner, however the testability or signal stability for the measurement is still under the evaluation.

Figure 2.4-1. baseline measurement setup for OTA testing

Figure 2.4-2a the timeline for EEIRP measurement for different testing beams

Figure 2.4-2b the timeline for EEIRP measurement for different testing beams

Another enhanced OTA chamber with multiple probes within the chamber in Figure 2.4.3 which is kind of similar as BS passive antenna pattern testing, in order to get the upper semi-sphere EIRP measurement to calculate the EEIRP measurement, then it is necessary to rotate the BS or positioner in the elevation angle only for spatial sampling by testing antenna or probe. If considering the elevation angle step size as 1degree, then the measurement time in total should be roughly around 15 minute which is also mainly contributed by the mechanical rotation of BS or positioner. However this enhanced OTA testing chamber is also very complicated and also expensive. Such kind of enhanced OTA testing chamber might be not available in different regions for the bidding testing. In other words, it’s preferred to use the baseline OTA chamber for EEIRP conformance testing.

Figure 2.4-3. enhanced measurement setup for OTA testing

Proposal 6: propose to use the baseline OTA chamber as shown in Figure 2.4-1 for further discussion on the measurement time if necessary.

  - .Method of test

During the last RAN4 meeting, there were some discussions on the RF channel for the EEIRP conformance testing, however no consensus has been reached and the following options have been captured in the WF.

Issue 4-4: RF channels for the conformance testing - Proposal 1: B, T [Nokia] - Proposal 2: M. [Huawei] - Proposal 3: T [Ericsson]

During the discussions, some companies argued that the typical BS antenna implementation will use the supported highest frequency as baseline for the antenna design, however some other companies argued that the typical BS antenna implementation might use the middle frequency as baseline for the antenna design. In fact, both might be correct which is total up to the vendors’ implementation. However one common observation on top of these two different antenna design is that NR BS in the supported highest frequency would achieve higher EIRP performance which should be most stringent scenario to be optimized if necessary to comply with EEIRP mask requirement. In general, for the same antenna aperture size, if the operating frequency is increasing, then the achieved EIRP performance will be also higher.

Proposal 7: propose to use the T as RF channels for the conformance testing.

The following procedure for measuring EEIRP is based on the directional power measurements as described in annex X. When calibrated and operated within the guidance of 3GPP TR 37.941 [29] the measurement methods are applicable and selected depending on availability at the test facility.

1)	Place the BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the BS with the test system.

3)	Configure the BS according to the test beams in clause X applied during the power measurement step 6 are consistent with the grid and measurement approach for the EEIRP test.

4)	Set the BS to transmit according to the applicable test configuration in clause 4.8 using the corresponding test model(s) in clause 4.9.2.

For a BS declared to be capable of multi-carrier and/or CA operation use the applicable test signal configuration and corresponding power setting specified in clauses 4.7.2 and 4.8 using the corresponding test model(s) in clause 4.9.2 on all carriers configured.

5)	Orient the positioner (and BS) in order that the direction to be tested aligns with the test antenna such that measurements to determine EIRP can be performed (see annex N).

6)	Measure the radiated power of each testing beam for any two orthogonal polarizations (denoted p1 and p2) and calculate total radiated transmit power for particular beam direction pair as EIRP = EIRPp1 + EIRPp2.

7)	Repeat step 6-7 for all directions in the appropriated EEIRP measurement grid needed for EEIRP estimation (see annex XX).

8)	Repeat step 3, 6,7 for all test beams needed for EEIRP estimation (see annex XX).

8)	Calculate EEIRP using the EIRP measurements of the test beams according to Annex XX

For multi-band RIBs and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carriers activated in the other band.

Proposal 8: propose to use the above test procedures for EEIRP measurement using TP measurement as basis.

  - . Measurement uncertainty

Regarding the confidence level of 95% is widely used in the RAN4 conformance testing specification as following,

the Test System shall enable the stimulus signals in the test case to be adjusted to within the specified tolerance and the equipment under test to be measured with an uncertainty not exceeding the specified values. All tolerances and uncertainties are absolute values, and are valid for a confidence level of 95 %, unless otherwise stated. A confidence level of 95 % is the measurement uncertainty tolerance interval for a specific measurement that contains 95 % of the performance of a population of test equipment.

From RAN4 RF requirement definition perspective, we need to define the measurement uncertainty and also test tolerance requirement on top of core requirement. From our understanding, we could still follow the legacy approach adopted for EIRP accuracy requirements as baseline, however considering the EEIRP mask requirements, some additional summation error due to the limited number of testing beams and spatial sampling grid for EEIRP mask measurement.

For the test tolerance requirement for U6GHz EEIRP mask, since this is the regulatory requirement. If we follow the legacy requirement defined for the transmitter spurious emission requirement, then test tolerance should be set as zero.

Proposal 9: follow the legacy approach adopted for TRP measurement uncertainty requirements as baseline, however considering the EEIRP mask requirements, some additional summation error due to the limited number of testing beams and spatial sampling grid for EEIRP mask measurement.

The detailed examples for CATR measurement could be found as following with exact value to be further discussed.

Taking into account the uncertainties highlighted we have for CATR:

| Table 11.2.3.3-1: CATR MU value derivation for OTA BS output power measurement, FR1 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UID | Uncertainty source | Uncertainty value (dB) |  |  | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui (dB) |  |  |
|  |  | f≤3 GHz | 3<f≤4.2 GHz | 4.2<f≤7.125 GHz |  |  |  | f≤3 GHz | 3<f≤4.2 GHz | 4.2<f≤7.125 GHz |
| Stage 2: BS measurement |  |  |  |  |  |  |  |  |  |  |
| A2-18a | Misalignment and pointing error of BS (for TRP) | 0.30 | 0.30 | 0.30 | Rectangular | 1.73 | 1 | 0.173 | 0.173 | 0.173 |
| C1-1 | Uncertainty of the RF power measurement equipment (e.g. spectrum analyzer, power meter) | 0.14 | 0.26 | 0.26 | Gaussian | 1.00 | 1 | 0.140 | 0.260 | 0.260 |
| A2-2a | Standing wave between BS and test range antenna | 0.21 | 0.21 | 0.21 | U-shaped | 1.41 | 1 | 0.148 | 0.148 | 0.148 |
| A2-3 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) | 0.00 | 0.00 | 0.00 | Gaussian | 1.00 | 1 | 0.001 | 0.001 | 0.001 |
| A2-4a | QZ ripple experienced by BS | 0.09 | 0.09 | 0.09 | Gaussian | 1.00 | 1 | 0.093 | 0.093 | 0.093 |
| A2-12 | Frequency flatness of test system | 0.25 | 0.25 | 0.25 | Gaussian | 1.00 | 1 | 0.250 | 0.250 | 0.250 |
| A2-17 | Measurement system dynamic range uncertainty | 0.51 | 0.51 | 0.51 | Gaussian | 1 | 1 | 0.51 | 0.51 | 0.51 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |
| C1-3 | Uncertainty of the network analyzer | 0.13 | 0.20 | 0.20 | Gaussian | 1.00 | 1 | 0.13 | 0.20 | 0.20 |
| A2-5a | Mismatch of receiver chain between receiving antenna and measurement receiver | 0.13 | 0.33 | 0.33 | U-shaped | 1.41 | 1 | 0.09 | 0.23 | 0.23 |
| A2-6 | Insertion loss of receiver chain | 0.18 | 0.18 | 0.18 | Rectangular | 1.73 | 1 | 0.10 | 0.10 | 0.10 |
| A2-3 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) | 0.00 | 0.00 | 0.00 | Gaussian | 1.00 | 1 | 0.00 | 0.00 | 0.00 |
| A2-7 | Influence of the calibration antenna feed cable | 0.02 | 0.02 | 0.02 | U-shaped | 1.41 | 1 | 0.02 | 0.02 | 0.02 |
| C1-4 | Uncertainty of the absolute gain of the reference antenna | 0.50 | 0.43 | 0.43 | Rectangular | 1.73 | 1 | 0.29 | 0.25 | 0.25 |
| A2-8 | Misalignment positioning system | 0.00 | 0.00 | 0.00 | Exp. normal | 2.00 | 1 | 0.00 | 0.00 | 0.00 |
| A2-18b | Misalignment and pointing error of calibration antenna (for TRP) | 0.50 | 0.50 | 0.50 | Exp. normal | 2.00 | 1 | 0.25 | 0.25 | 0.25 |
| A2-9 | Rotary joints | 0.05 | 0.05 | 0.05 | U-shaped | 1.41 | 1 | 0.03 | 0.03 | 0.03 |
| A2-2b | Standing wave between calibration antenna and test range antenna | 0.09 | 0.09 | 0.09 | U-shaped | 1.41 | 1 | 0.06 | 0.06 | 0.06 |
| A2-4b | QZ ripple experienced by calibration antenna (normal test conditions) | 0.01 | 0.01 | 0.01 | Gaussian | 1.00 | 1 | 0.01 | 0.01 | 0.01 |
| A2-11 | Switching uncertainty | 0.26 | 0.26 | 0.26 | Rectangular | 1.73 | 1 | 0.15 | 0.15 | 0.15 |
| Combined standard uncertainty (1σ) (dB) |  |  |  |  |  |  |  | 0.78 | 0.84 | 0.84 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) (dB) |  |  |  |  |  |  |  | 1.53 | 1.65 | 1.65 |
| EEIRP Summation error |  |  |  |  |  |  |  | NA | NA | [1.0] |
| Total MU |  |  |  |  |  |  |  | 1.71 | 1.81 | 2.10 |

* note there may be an error in A2-5a, the value in the SEM TRP budget is larger – check this value

Proposal 10: consider the additional EEIRP Summation error as 1dB to derive the total MU for EEIRP MU.

  - . Confidence interval

As we discussed in the previous RAN4 meeting, confidence interval is highly dependent on the number of testing beams and its average value from the measured value. Based on the initial analysis in section 2.1, 9 testing number with the equal weighting factor should be sufficient for both measurement uncertainty and measurement time perspective. In short, in the following formula N could be 9, for the probability distribution function, to follow the legacy RAN4 assumption, the normal distribution might be okay for it.

Proposal 11: for the confidence interval, propose to consider 9 testing beams and equal weighting factors for the CI calculation.

Conclusions

In this contribution, we shared some further views on the EIRP mask requirement for band n104 and observations / proposals are made as following:

Proposal 1: propose to use the beam set D with 9 testing beams for EEIRP conformance testing.

Proposal 2: to consider the manufacture declarations in Table 2.2-1 for different mechanical down-tilt and its the corresponding coverage angular range for the declaration.

Observation 1: if electrical testing beams under different mechanical tilt are the same or subset of full set beam, EEIRP measurement under different mechanical tilt could be post-processed to save the measurement time.

Observation 2: if electrical testing beams under different mechanical tilt are different, EEIRP measurement under different mechanical tilt could not be post-processed by single EIRP measurement in the full sphere or upper sphere.

Proposal 3:

- if electrical testing beams under different mechanical tilt are the same or subset of others, EEIRP measurement under different mechanical tilt could be post-processed on top of full set of testing beams to save the measurement time.

- if electrical testing beams under different mechanical tilt are different, EEIRP measurement under different mechanical tilt should be tested separately.

Observation 3: the EEIRP measurement results are almost the same when spatial sampling grid of theta and phi between 1° / 1° and 1° / 10°.

Proposal 4: for the elevation angle, propose the measurement step size as 1 degree.

Proposal 5: for the azimuth angle, propose the measurement step size as 5 degree.

Proposal 6: propose to use the baseline OTA chamber as shown in Figure 2.4-1 for further discussion on the measurement time if necessary.

Proposal 7: propose to use the T as RF channels for the conformance testing.

Proposal 8: propose to use the above test procedures for EEIRP measurement using TP measurement as basis.

Proposal 9: follow the legacy approach adopted for TRP measurement uncertainty requirements as baseline, however considering the EEIRP mask requirements, some additional summation error due to the limited number of testing beams and spatial sampling grid for EEIRP mask measurement.

Proposal 10: consider the additional EEIRP Summation error as 1dB to derive the total MU for EEIRP MU.

Proposal 11: for the confidence interval, propose to consider 9 testing beams and equal weighting factors for the CI calculation.

References

- RP-240829,New WID: WI resulting from discussion on BS RF requirement evolution, Approved.

- WRC-23 Provisional Final Acts, https://www.itu.int/dms_pub/itu-r/opb/act/R-ACT-WRC.15-2023-PDF-E.pdf

- R4-2406088,Way Forward for [110bis][311] NR_BS_RF, ZTE, approved.

- R4-2404757,On introduction of protection requirement for FSS UL service operating within 6425 to 7125 MHz

- R4-2409955, Way forward for [111][311] NR_BS_RF, Approved

- R4-2409118,On introduction of OTA spatial emission above the horizon requirement for band n104, Ericsson

- R4-2409073,Discussion on expected EIRP mask above horizon, Nokia

- R4-2409401,Expected EIRP requirements, Huawei, Hisilicon

- TS 37.941

TP

6.1	General

In this section, it mainly provides some technical background for EEIRP conformance testing from the following issues:

- The calculation for EEIRP mask

- Testing beams including both the number of test beams and also weighting factor

- Spatial sampling grid

- The method of test including both testing conditions and also test procedure

- Measurement uncertainty

- Confidence interval

- Other

6.2	The calculation for EEIRP mask

The EEIRP calculation methodology in the discrete form is defined as following.

EIRPθn,φm=k=1KwkEIRPkθn,φm,αk,βk

, where wk is a beam weighting and EIRPkθn,φm,αk,βk is the EIRP pattern per measured test beam.

EEIRPi=θHHi−θHLisinθHHi−sinθHLiMN∙n=nLinHim=1MEIRPθn,θmcosθn

Where

- M is the number of the equally divided intervals over the horizontal (azimuth) range of −180° to +180°;

- N is the number of the equally divided intervals over the specified vertical (elevation) range of θL to θH.

- nL	is the lowest elevation sampling angle within the θL , θH bounding range

- nH	is the highest elevation sampling angle within the θL , θH bounding range

- θn is the vertical angle measured within the n-th interval out of the vertical range of θL to θH, where the interval is from θL+n−1NθH−θL to θL+nNθH−θL,

- φm is the horizontal angle measured within the m-th interval out of the horizontal range of −180° to +180°, where the interval is from −π+2π∙m−1M to −π+2π∙mM.

- i means different bin index.

6.3	Test beams

According to the evaluation results provided in Table 6.3-1 an 6.3-2, it could be observed that EEIRP measurement difference between 325 testing beams and 25 testing beams (e.g. beam set A and B) are quite limited to be lower than 0.8dB which could reduce the measurement time and complexity significantly. For the measurement difference between beam set J (325 testing beams) and beam set C (9 testing beams) could be maintained within 1.5dB except for 60° - 90°. For the measurement difference between beam set J (325 testing beams) and beam set D (9 testing beams) could be maintained within 1.05dB except for 60° - 90°. For EEIRP measurement for beam set E (9 testing beams), it seems to the best one in term of the interference towards FSS service due to the low weighting factors for the beam directions with φ° as 0 degree.

Table 6.3-1: The simulation results for EEIRP mask

| Vertical angle range | Sufficient beams to be tested |  | Beams set to be tested [R4-2409118] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Beam weight factors for Beam set | J | K | A | B | C | D | E |
| 0° - 5° | 25.69 | 25.66 | 26.18 | 26.45 | 26.83 | 25.73 | 22.39 |
| 5° - 10° | 17.00 | 16.97 | 17.44 | 17.20 | 18.03 | 17.13 | 16.19 |
| 10° - 15° | 13.83 | 13.79 | 14.11 | 14.02 | 14.65 | 14.88 | 14.62 |
| 15° - 20° | 11.70 | 11.66 | 11.86 | 11.64 | 12.47 | 12.31 | 11.92 |
| 20° - 30° | 9.35 | 9.28 | 9.43 | 8.91 | 10.20 | 9.39 | 8.84 |
| 30° - 60° | 5.30 | 5.17 | 5.35 | 4.68 | 6.73 | 5.91 | 5.26 |
| 60° - 90° | -0.95 | -1.11 | -0.79 | -1.60 | 3.22 | 1.79 | -0.69 |
| Number of testing beams | 325 | 169 | 25 | 25 | 9 | 9 | 9 |
| Weighting factor | [uniform] | [uniform] | [non-uniform] | [uniform] | [non-uniform] | [uniform] | [non-uniform] |
| NOTE: the measurement step size in elevation domain is 1 degree and measurement step size in the azimuth domain is 1 degree. |  |  |  |  |  |  |  |

The EEIRP measurement on top of beam set J and K are provided with the following assumptions:

- Beam set J:

- θ° is ranging from 0° to 12° with step size as 1°

- φ° is ranging from -60° to 60° with step size as 5°

- Uniform weighing factor

- Beam set K:

- θ° is ranging from 0° to 12° with step size as 1°

- φ° is ranging from -60° to 60° with step size as 10°

- Uniform weighing factor

Table 6.3-2: The EIRP measurement difference between different beam sets

| Vertical angle range | Beams set to be tested [R4-2409118] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Beam weight factors for Beam set | A-J | B-J | C-J | D-J | E-J |
| 0° - 5° | 0.49 | 0.76 | 1.14 | 0.04 | -3.3 |
| 5° - 10° | 0.44 | 0.2 | 1.03 | 0.13 | -0.81 |
| 10° - 15° | 0.28 | 0.19 | 0.82 | 1.05 | 0.79 |
| 15° - 20° | 0.16 | -0.06 | 0.77 | 0.61 | 0.22 |
| 20° - 30° | 0.08 | -0.44 | 0.85 | 0.04 | -0.51 |
| 30° - 60° | 0.05 | -0.62 | 1.43 | 0.61 | -0.04 |
| 60° - 90° | 0.16 | -0.65 | 4.17 | 2.74 | 0.26 |

Based on the above analysis and also to balance the measurement time and measurement uncertainty, RAN4 agree to select the following testing beams and its corresponding weighting factor

Table 6.3-3:: Testing beams and weighing factor

| Test Beam [k] | Direction θ° | Direction φ° | Portion weights [w_k] |
| --- | --- | --- | --- |
| 1 | 6 | 0 | 1/9 |
| 2 | 6 | -60 | 1/9 |
| 3 | 6 | 60 | 1/9 |
| 4 | 6 | -30 | 1/9 |
| 5 | 6 | 30 | 1/9 |
| 6 | 3 | 0 | 1/9 |
| 7 | 9 | 0 | 1/9 |
| 8 | 0 | 0 | 1/9 |
| 9 | 12 | 0 | 1/9 |

6.4	Spatial sampling grid

According to the measurement results provided in Table 6.4-1, when the EEIRP measurement results are almost the same when the spatial sampling grid of theta and phi is between 1° / 1° and 1° / 10°. When the sampling grid is up to 1° / 15°, then there will be some additional variations/errors compared with other dense sampling grid. In addition, when the sampling grid in the vertical domain is increased to 5° or 2.5°, then the measurement uncertainty will be further increased. Based on the above observation and also to balance the measurement time and measurement uncertainty, RAN4 agree to select 1 degree as step size for elevation angle and 5 degree to azimuth angle.

Table 6.4-1: simulation results for EEIRP mask

| Vertical angle range | Full resolution | Vertical / Horizontal Angular Step Size |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Theta/phi | 0.1° /0. 1° | 5° / 5° | 2.5° / 5° | 2.5° / 2.5° | 1° / 15° | 1° / 10° | 1° / 5° | 1° / 1° |
| 0° - 5° | 25.52 | 26.02 | 25.76 | 25.76 | 26.04 | 25.73 | 25.73 | 25.73 |
| 5° - 10° | 16.91 | 19.36 | 18.37 | 18.37 | 17.37 | 17.13 | 17.13 | 17.13 |
| 10° - 15° | 13.70 | 14.37 | 14.81 | 14.81 | 15.06 | 14.88 | 14.88 | 14.88 |
| 15° - 20° | 11.69 | 11.90 | 12.13 | 12.13 | 12.45 | 12.31 | 12.31 | 12.31 |
| 20° - 30° | 9.35 | 10.46 | 10.15 | 10.15 | 9.48 | 9.39 | 9.39 | 9.39 |
| 30° - 60° | 5.27 | 6.02 | 5.89 | 5.89 | 5.92 | 5.91 | 5.91 | 5.91 |
| 60° - 90° | -1.02 | 1.37 | 1.18 | 1.18 | 1.79 | 1.79 | 1.79 | 1.79 |
| Number of measurement points |  | 1296 | 2592 | 5184 | 2160 | 3240 | 6480 | 32400 |

6.5	Method of test

### 6.5.1	Initial conditions

For the test environment, RAN4 agree to test the EEIRP mask under the normal condition instead of extreme conditions to keep alignment with TRP measurement.

For RF channel for the conformance testing, RAN4 agree to select the top channel for the conformance testing since the top channel is supposed to be most stringent case with smallest wavelength under the same aperture size.

### 6.5.2	Test procedure

The following procedure for measuring EEIRP is based on the directional power measurements as described in clause 6.2. When calibrated and operated within the guidance of 3GPP TR 37.941 [29] the measurement methods are applicable and selected depending on availability at the test facility.

1)	Place the BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the BS with the test system.

3)	Configure the BS according to the test beams in clause X applied during the power measurement step 6 are consistent with the grid and measurement approach for the EEIRP test.

4)	Set the BS to transmit according to the applicable test configuration in clause 4.8 using the corresponding test model(s) in clause 4.9.2.

For a BS declared to be capable of multi-carrier and/or CA operation use the applicable test signal configuration and corresponding power setting specified in clauses 4.7.2 and 4.8 using the corresponding test model(s) in clause 4.9.2 on all carriers configured.

5)	Orient the positioner (and BS) in order that the direction to be tested aligns with the test antenna such that measurements to determine EIRP can be performed (see clause 6.2).

6)	Measure the radiated power of each testing beam for any two orthogonal polarizations (denoted p1 and p2) and calculate total radiated transmit power for particular beam direction pair as EIRP = EIRPp1 + EIRPp2.

7)	Repeat step 6-7 for all directions in the appropriated EEIRP measurement grid needed for EEIRP estimation (see clause 6.2).

8)	Repeat step 3, 6,7 for all test beams needed for EEIRP estimation (see clause 6.2).

8)	Calculate EEIRP using the EIRP measurements of the test beams according to clause 6.2

For multi-band RIBs and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carriers activated in the other band.

6.6	Measurement uncertainty

For the measurement uncertainty of EEIRP mask requirement, considering the number of test beams and also spatial sampling grid for the EEIRP calculation, additional EEIRP summation error 1dB is assumed on top of the legacy TRP measurement uncertainty estimation. The detailed MU budget for EEIRP calculation could be found in the following table 6.6-1.

Table 6.6-1. MU budget for CATR MU value derivation

| Table 11.2.3.3-1: CATR MU value derivation for OTA BS output power measurement, FR1 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UID | Uncertainty source | Uncertainty value (dB) |  |  | Distribution of the probability | Divisor based on distribution shape | ci | Standard uncertainty ui (dB) |  |  |
|  |  | f≤3 GHz | 3<f≤4.2 GHz | 4.2<f≤7.125 GHz |  |  |  | f≤3 GHz | 3<f≤4.2 GHz | 4.2<f≤7.125 GHz |
| Stage 2: BS measurement |  |  |  |  |  |  |  |  |  |  |
| A2-18a | Misalignment and pointing error of BS (for TRP) | 0.30 | 0.30 | 0.30 | Rectangular | 1.73 | 1 | 0.173 | 0.173 | 0.173 |
| C1-1 | Uncertainty of the RF power measurement equipment (e.g. spectrum analyzer, power meter) | 0.14 | 0.26 | 0.26 | Gaussian | 1.00 | 1 | 0.140 | 0.260 | 0.260 |
| A2-2a | Standing wave between BS and test range antenna | 0.21 | 0.21 | 0.21 | U-shaped | 1.41 | 1 | 0.148 | 0.148 | 0.148 |
| A2-3 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) | 0.00 | 0.00 | 0.00 | Gaussian | 1.00 | 1 | 0.001 | 0.001 | 0.001 |
| A2-4a | QZ ripple experienced by BS | 0.09 | 0.09 | 0.09 | Gaussian | 1.00 | 1 | 0.093 | 0.093 | 0.093 |
| A2-12 | Frequency flatness of test system | 0.25 | 0.25 | 0.25 | Gaussian | 1.00 | 1 | 0.250 | 0.250 | 0.250 |
| A2-17 | Measurement system dynamic range uncertainty | 0.51 | 0.51 | 0.51 | Gaussian | 1 | 1 | 0.51 | 0.51 | 0.51 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |
| C1-3 | Uncertainty of the network analyzer | 0.13 | 0.20 | 0.20 | Gaussian | 1.00 | 1 | 0.13 | 0.20 | 0.20 |
| A2-5a | Mismatch of receiver chain between receiving antenna and measurement receiver | 0.13 | 0.33 | 0.33 | U-shaped | 1.41 | 1 | 0.09 | 0.23 | 0.23 |
| A2-6 | Insertion loss of receiver chain | 0.18 | 0.18 | 0.18 | Rectangular | 1.73 | 1 | 0.10 | 0.10 | 0.10 |
| A2-3 | RF leakage (SGH connector terminated & test range antenna connector cable terminated) | 0.00 | 0.00 | 0.00 | Gaussian | 1.00 | 1 | 0.00 | 0.00 | 0.00 |
| A2-7 | Influence of the calibration antenna feed cable | 0.02 | 0.02 | 0.02 | U-shaped | 1.41 | 1 | 0.02 | 0.02 | 0.02 |
| C1-4 | Uncertainty of the absolute gain of the reference antenna | 0.50 | 0.43 | 0.43 | Rectangular | 1.73 | 1 | 0.29 | 0.25 | 0.25 |
| A2-8 | Misalignment positioning system | 0.00 | 0.00 | 0.00 | Exp. normal | 2.00 | 1 | 0.00 | 0.00 | 0.00 |
| A2-18b | Misalignment and pointing error of calibration antenna (for TRP) | 0.50 | 0.50 | 0.50 | Exp. normal | 2.00 | 1 | 0.25 | 0.25 | 0.25 |
| A2-9 | Rotary joints | 0.05 | 0.05 | 0.05 | U-shaped | 1.41 | 1 | 0.03 | 0.03 | 0.03 |
| A2-2b | Standing wave between calibration antenna and test range antenna | 0.09 | 0.09 | 0.09 | U-shaped | 1.41 | 1 | 0.06 | 0.06 | 0.06 |
| A2-4b | QZ ripple experienced by calibration antenna (normal test conditions) | 0.01 | 0.01 | 0.01 | Gaussian | 1.00 | 1 | 0.01 | 0.01 | 0.01 |
| A2-11 | Switching uncertainty | 0.26 | 0.26 | 0.26 | Rectangular | 1.73 | 1 | 0.15 | 0.15 | 0.15 |
| Combined standard uncertainty (1σ) (dB) |  |  |  |  |  |  |  | 0.78 | 0.84 | 0.84 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) (dB) |  |  |  |  |  |  |  | 1.53 | 1.65 | 1.65 |
| EEIRP Summation error |  |  |  |  |  |  |  | NA | NA | [1.0] |
| Total MU |  |  |  |  |  |  |  | NA | NA | 2.10 |

* note there may be an error in A2-5a, the value in the SEM TRP budget is larger – check this value

6.7	Confidence intervals

TBD

6.8	Testing requirements

Since the EEIRP mask requirement defined in WRC-23 is the regulatory requirement from RAN4 standardization perspective, the test tolerance requirement should be zero without any further relaxation on top of regulatory limits.

6.9	Others