3GPP TSG-RAN WG4 Meeting#112bis                             R4-2416243

Hefei, China, 14th – 18th Oct, 2024

Agenda item:	6.2.4.2

Source: 	ZTE Corporation

Title: 	Discussion on radio and antenna parameters for 14800 to 15350 MHz

Document for:	Approval

Introduction

In the latest ITU-R meeting, WP5D has sent one LS to RAN/RAN4 to request the Parameters of terrestrial component of IMT in the following frequency ranges for sharing and compatibility studies. During the RAN#103 meeting, the corresponding SID for WP5D reply LS [3] is approved to facilitate the technical discussions in RAN4. In this contribution, we would like to share some views on IMT parameters for 14800-15350MHz.

Way forward - Further discussing the following issues Issue 3-1 Common understanding on UE antenna array options - Option 1: “UE beamforming” (FR2 like) - Assumed to be FR2 like UE, with two panels similar to Rel-14 NR study 38.803 - Analogue like beamforming - Option 1a: 2x2 antennas per panel - Option 2: FR1 like - Assumed to be FR1 like UE with more than one TX/RX antenna - Isotropic antennas - Option 2a: 2 TX - Option 2b: 4 TX - Option 2c: 4 RX - Option 2d: 6 RX - Option 2e: 8 RX Issue 3-2 Initial deployment scenarios - Option 1: Follow 38.921 - Indoor 20m ISD - [0.2 - 0.45]km Urban - [<=0.9]km sub-urban - Option 2: 38.803 FR2 assumptions - Indoor - Dense Urban - Urban macro - Other options such as smaller ISD not precluded depending on antenna array assumptions Issue 3-3 Initial coordinated operation - Option 1: - Co-ordinated for indoor - Both co-ordinated and un-co-ordinated for outdoor Issue 3-4 Initial views on BS array size - Option 1: 1024 elements - Option 2: 2048 elements - Option 3: Other sizes Issue 3-5 Initial views on BS sub-array architecture - Option 1: No sub-arrays - Option 2: Sub array size 2 - Option 3: Sub array size 3 - Option 4: Sub-array size 4 - Option 5: Sub-array size 6 - Option 6: Sub-array size 8 - Option 7: Sub-array size 12

Discussion

## 2.1. System parameters

Regarding the channel bandwidth and transmission bandwidth configuration, even though this frequency range might be targeted for the usage of both IMT-2020 and IMT-2030, this is still discussed under the 5G-A phase and it’s not clear on 6G system parameter assumption. Based on the commercial network deployed so far in FR1 high bands (e.g. higher than 2.5GHz), only 30kHz is used, therefore15KHz SCS is not relevant in this spectrum 14.8-15.35GHz from our understanding. For 15GHz, it might be mixture between FR1 and FR2-1 as following.

|  | Maximum channel bandwidth |  |  |
| --- | --- | --- | --- |
|  | 100MHz | 200MHz | 400MHz |
| 30KHz | [Yes] | N/A | N/A |
| 60kHz | [Yes] | [Yes] | N/A |
| 120KHz | [Yes] | [Yes] | [Yes] |

Regarding the applicable subcarrier spacing and maximum channel bandwidth for this frequency range, we need to consider the following factors at least:

- The targeted deployment scenario

- The achievable BS antenna/power assumptions and UE antenna/power assumptions

- Others, e.g. the potential operators sharing these spectrum

- Regarding its targeted deployment scenario, if this is targeted to provide the wide area coverage instead of hotpot coverage, then 30kHz and 60kHz should be treated with high priorities since its longer CP could provide the larger coverage. If the targeted scenario is still the hotpot coverage to boost the capacity in certain regions, then 60kHz and 120kHz could be treated as high priorities.

| Numerology | Subcarrier spacing | CP for long symbols | Distance | CP for other symbols | Distance |
| --- | --- | --- | --- | --- | --- |
| 0 | 15 | 5.2us | 1560m | 4.69us | 1407m |
| 1 | 30 | 2.86us | 858m | 2.34us | 703m |
| 2 | 60 | 1.69us | 507m | 1.17us | 351m |
| 3 | 120 | 1.11us | 333m | 0.59us | 175m |

- From our understanding, 100MHz with 30KHz could be provide better coverage from both CP and transmission power density given limited transmission power assumption. Other channels bandwidth are not precluded.

Issue 1-2: Channel Bandwidth Agreement: - For LS about 15GHz, choose the single value out of 100MHz, 200MHz or 400MHz as baseline with understanding that the other channel bandwidths are not precluded.

Proposal 1: choose the 100MHz channel bandwidth in reply LS and indicate other channel bandwidth are not precluded.

## 2.2. Antenna parameters

If the targeted deployment is wide area coverage, then to provide the comparable and competitive deployment with FR1 4GHz bands, at least 12dB higher EIRP value at BS side could be achieved to counteract the increasing pathloss at 15GHz as shown in the following figure.

Figure 1. FSPL comparison between 4GHz and 15GHz

In order to enable this larger BS EIRP performance at 15GHz, then larger BS antenna array size is expected to achieve better beamforming gain e.g. 4096 antenna elements (e.g. 33dBi beamforming gain) compared with192 BS antenna elements (e.g. 19.8dBi beamforming gain) at 4GHz. Another important factor should be gap of TRP or total conducted power between 4GHz and 15GHz. For 4GHz, its total transmission power per polarization could be up to 52dBm per 100MHz. In order to achieve similar conducted power as 4GHz, for 15GHz with 4096 antenna elements and also with FR2 BS hybrid beamforming architecture assumption, then its supposed transmission power per PA within beamforming chipset should be up to 18.9dBm which is much higher than the assumption used in Rel-15 (e.g. 7-10dBm per PA based CMOS assumption). Indeed its more practical conducted transmission power with 4096 antenna elements assumption should be 40-43dBm per polarization under the Rel-15 mature CMOS PA assumption. In other words, even though the beamforming gain at 15GHz with 4096 antenna elements could be achieve 12dBi beamforming gain to counteract the increasing pathloss, the maximum transmission power at 15GHz is still 10dB lower than that at 4GHz under the 100MHz assumption.

Table 2.2-1: Antenna array parameters

| Parameter | Indoor | Urban macro | Dense urban |
| --- | --- | --- | --- |
| Element gain (dBi) (Note 2) | 5 | 6.4 | 6.4 |
| Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 90º for V | 90º for H 65º for V | 90º for H 65º for V |
| Horizontal/vertical front to back ratio (dB) | 30 for both H/V | 30 for both H/V | 30 for both H/V |
| Antenna polarization | Linear ±45º | Linear ±45º | Linear ±45º |
| Antenna sub-array configuration (M × N) (Note 4) | 4 x 4 | 16 x 24 / 8 x 32 | 16 x 24 / 8 x 32 |
| Horizontal/Vertical radiating sub-array spacing | 0.5 of wavelength for H, 0.5 of wavelength for V | 0.5 of wavelength for H, 2.8/5.6 of wavelength for V | 0.5 of wavelength for H, 2.8/5.6 of wavelength for V |
| Number of element rows in sub-array | N/A | (4x1)/(8x1) | (4x1)/(8x1) |
| Vertical element separation in sub-array (dv,sub) | 0.5 of wavelength of V | 0.7 of wavelength of V | 0.7 of wavelength of V |
| Pre-set sub-array down-tilt (degrees) | N/A | 3 | 3 |
| Array Ohmic loss (dB) (Note 2) | 2 | 2 | 2 |
| Conducted power (before Ohmic loss) per sub-array/element (dBm) (Note 3) | 9 | 17.14/18.9 | 17.14/18.9 |
| Base station horizontal coverage range (degrees) | +/-90 | +/-60 | +/-60 |
| Base station vertical coverage range (degrees) (Note 1) | 0-180 | 90-100 | 90-100 |
| Mechanical down-tilt (degrees) | 90 | 6 | 6 |
| Note 1: The vertical coverage range is given for the elevation angle θ, defined between 0° and 180°. Note 2: The element gain includes the loss and is per polarization. Note 3: The conducted power per sub-array/element assumes 2xMxN subarrays/elements (i.e., power per H/V polarized element). Note 4: M and N represents rows and columns of dual polarized elements of the array or subarrays in the array Note 5: For the case of 4/8 elements per sub array, dv will be 2.8/5.6 wavelengths. |  |  |  |

## 2.3. BS RF requirements

### 2.3.1	ACLR

According to the simulation results in clause 4.3 in TR 39.921, it is agreed to specify 38 dB ACLR for 6.425 - 7.125 GHz and 37 dB ACLR for 10.0 - 10.5 GHz. In addition, according to the companion contribution [9] on coexistence evaluation results for 15GHz, we propose to define the BS ACLR requirement around 35dBc.

Table 2.3.1-1: Base station ACLR limit

| BS channel bandwidth of lowest/highest carrier transmitted BWChannel (MHz) | BS adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit |
| --- | --- | --- | --- | --- |
| 100 | BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 35dBc |
|  | 2 x BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 35dBc |
| NOTE 1: BWChannel and BWConfig are the BS channel bandwidth and transmission bandwidth configuration of the lowest/highest carrier transmitted on the assigned channel frequency. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). |  |  |  |  |

### 2.3.2	Spectral mask

According to the proposed ACLR requirement for 14.8-15.35GHz and also expected adjacent carrier bandwidth, we propose to reuse the spectral mask for 7 and 10GHz for 15GHz

Table 2.3.2-1: Wide Area BS operating band unwanted emission limits for 14.8-15.35 GHz for Category A

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 50 MHz | 0.05 MHz f_offset < 50.05 MHz | −7dBm−750foffsetMHz−0.05 | 100 kHz |
| 50 MHz f < min(100 MHz, fmax) | 50.05 MHz f_offset < min(100.05 MHz, f_offsetmax) | -14 dBm | 100 kHz |
| 100 MHz f fmax | 100.5 MHz f_offset < f_offsetmax | -13 dBm | 1MHz |

Table 2.3.2-2: Wide Area BS operating band unwanted emission limits for 14.8-15.35 GHz for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 50 MHz | 0.05 MHz f_offset < 50.05 MHz | −7dBm−750foffsetMHz−0.05 | 100 kHz |
| 50 MHz f < min(100 MHz, fmax) | 50.05 MHz f_offset < min(100.05 MHz, f_offsetmax) | -14 dBm | 100 kHz |
| 100 MHz f fmax | 100.5 MHz f_offset < f_offsetmax | -15 dBm | 1MHz |

### 2.3.3	Spurious emissions

According to ERC 74-01 report as following, the lower frequency limit could be defined as 30 MHz and higher frequency limit could be 2nd of harmonic. In addition, similar as 10-10.5GHz, without any specified spurious limit for AAS BS in between 6 and 24 GHz in ERC 74-01 [6], the spurious limits specified for above 24 GHz with upper frequency limited to 26 GHz will be used for 14.8-15.35 GHz as defined in Table 2.3.3-1 and Table 2.3.3-2.

Table 2.3.3-1: BS spurious emission limits for 14.8-15.35 GHz for Category B

| Spurious frequency range | Limit | Measurement bandwidth |
| --- | --- | --- |
| 30 MHz – 1 GHz | -36 dBm | 100 kHz |
| 1 GHz – 18 GHz | -30 dBm | 1 MHz |
| 18 GHz – 2nd harmonic of the upper frequency edge of the DL operating band | -20 dBm | 10 MHz |

Table 2.3.3-2: BS spurious emission limits for 14.8-15.35 GHz for Category A

| Frequency range | Limit | Measurement Bandwidth | Note |
| --- | --- | --- | --- |
| 30 MHz – 1 GHz | -13 dBm | 100 kHz | Note 1 |
| 1 GHz – 2nd harmonic of the upper frequency edge of the DL operating band |  | 1 MHz | Note 1, Note 2 |
| NOTE 1: Bandwidth as in ITU-R SM.329, s4.1 NOTE 2: Upper frequency as in ITU-R SM.329, s2.5 table 1. |  |  |  |

Table 1: Recommended measurement ranges

| Fundamental frequency range | Frequency range for measurements |  |
| --- | --- | --- |
|  | Lower frequency | Upper frequency* |
| 9 kHz - 100 MHz | 9 kHz | 1 GHz |
| 100-300 MHz | 9 kHz | 10th harmonic |
| 300-600 MHz | 30 MHz | 3 GHz |
| 600 MHz - 5.2 GHz | 30 MHz | 5th harmonic |
| 5.2-13 GHz | 30 MHz | 26 GHz |
| 13-150 GHz | 30 MHz | 2nd harmonic |
| 150-300 GHz | 30 MHz | 300 GHz |
| *The test should include the entire harmonic band and not be truncated at the precise upper frequency limit stated |  |  |

| 2.1.6 | Base Stations using AAS and beamforming with integrated antennas operating below 24.25 GHz | 9 kHz f 1 GHz | -36 dBm (Note 4) |
| --- | --- | --- | --- |
|  |  | 1 GHz < f Fupper (see recommend 3) | -30 dBm (Note 4) |
| 2.1.7 | Base Stations using AAS and beamforming with integrated antennas operating above 24.25 GHz | 9 kHz f 1 GHz | -36 dBm (Note 4) |
|  |  | 1 GHz < f 18 GHz | -30 dBm (Note 4) |
|  |  | 18 GHz < f Fupper (see recommend 3) | -20 dBm/10 MHz (other limits apply for specific frequency separations, see Figure 7) (Note 4) |

Regarding the offset ΔfOBUE for the boundary between OBUE and spurious emissions, this could be further discussed until the spurious emission requirement and also unwanted emission requirement are finalized.

### 2.3.4	Maximum output power

The maximum output power will be provided in the antenna parameter table. It was agreed to be aligned with antenna characteristics.

The Total Radiated Power for two polarizations was agreed as shown in table 2.3.4-1 below.

Table 2.3.4-1: The Total Radiated Power

| Parameter | Macro Sub-urban | Macro Urban | Micro Urban |
| --- | --- | --- | --- |
| Total Radiated Power for two polarizations (dBm) | 46 | 46 | 37 |

### 2.3.5	Noise figure

From the TR 38.820 for 7 - 24 GHz, the typical Noise Figure for a Wide Area BS operating at 15 GHz is 8 dB (13 dB for Medium Range BS and 16 dB for Local Area BS).

Table 6.2.1-1: Typical noise figure for 7 – 24 GHz example frequencies

| Example frequency (GHz) | Typical NF values for NR BS (dB) | Typical NF values for NR UE (dB) |
| --- | --- | --- |
| 10 | 7 | 9 |
| 15 | 8 | 11 |
| 20 | 9 | 10 |

### 2.3.6	Sensitivity

Similar as 6GHz and 10GHz discussion, we propose to define BS OTA sensitivity requirement based on the NF and the antenna gain:

EISREFSENS=PkT+10∗log10BW+NF+IM+SNR−G    (dBm)

Where:

-	BW is the configured bandwidth of the FRC,

-	NF is the noise figure,

-	IM is implementation margin not related to antenna array,

-	SNR is the required SNR to reach 95% throughput, and

-	G is the antenna gain including RF losses and 3dB off peak margin.

### 2.3.7	Blocking response

The in-band blocking levels are reused from existing FR1 requirements.

Table 6.2.3-1: Base station general blocking requirement

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Interfering signal centre frequency minimum offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 100 | PREFSENS + 6 dB | Wide Area BS: -43 Medium Range BS: -38 Local Area BS: -35 | ±30 | 20 MHz DFT-s-OFDM NR signal 15 kHz SCS, 100 RBs |
| NOTE: PREFSENS depends on the RAT. |  |  |  |  |

Regarding the offset ΔfOOB for the boundary between out of band blocking and in-band blocking requirement, this could be further discussed until the OOBB and IBB power level are confirmed.

### 2.3.8	ACS

According to the simulation results in clause 4.3 in TR 38.921, it is agreed to specify 42 dB ACS for 6.425 - 7.125 GHz and 40 dB ACLR for 10.0 - 10.5 GHz. In addition, According to the companion contribution [9], we propose to define the BS ACS requirement around 38dBc for 14.8-15.35GHz.

## 2.3. UE RF requirements

Regarding the UE RF requirement for 14800-15350MHz, according to the simulation results so far in the companion contribution, whether it is possible to relax ACLR requirement similar as BS side, this is highly dependent on the UE architecture design and also target deployment scenario.

If FR1 like UE implementation assumption with the indoor ratio as 20%, then it will be very challenging to meet the 5% performance degradation criteria, however to reduce the indoor deployment ratio, then it is possible to relax ACLR requirement (e.g. 10dBc for ACLR R relaxation with 0% indoor deployment). If FR2 like UE, then it is acceptable to relax the ACLR requirement regardless of its targeted deployment due to its better coverage perspective.

- Proposal 9: postpone the ACLR requirement until the UE implementation assumptions are confirmed.

- For the UE ACS requirement, it seems that DL ACIR requirement could be relaxed around 8-10dBc, in other words, it is acceptable to have further relaxation on top of the legacy 33dBc ACS requirement. In addition, based on the agreed adjacent channel selectivity (ACS) is agreed as 32 dBc for 6425 – 7125 MHz and 31 dBc for 10 - 10.5 GHz, we propose to define the ACS 30dBc for 15GHz..

- Proposal 10: define UE ACS requirement around 30dBc for 15GHz.

Conclusions

In this contribution, we want to share some initial views on IMT parameters for 14800-15350MHz and proposals/observations are made as following:

Proposal 1: choose the 100MHz channel bandwidth in reply LS and indicate other channel bandwidth are not precluded.

Proposal 3: define ACLR requirement as 35dBc;

Proposal 4: propose to reuse the spectral mask for 7 and 10GHz for 15GHz;

Proposal 5: propose the typical Noise Figure for a Wide Area BS operating at 15 GHz is 8 dB (13 dB for Medium Range BS and 16 dB for Local Area BS).

Proposal 6:propose to define BS OTA sensitivity requirement based on the NF and the antenna gain similar 7 and 10GHz;

Proposal 7:propose to reuse FR1 licensed band IBB requirement for it.

Proposal 8:propose to define BS ACS requirement as 38dBc.

- Proposal 9: postpone the ACLR requirement until the UE implementation assumptions are confirmed.

- Proposal 10: define UE ACS requirement around 30dBc for 15GHz.

References

- R4-2400333 Parameters of terrestrial component of IMT for sharing and compatibility studies in the frequency bands 4 400-4 800 MHz, 7 125-8 400 MHz and 14.8-15.35 GHz.

- RP-210037, Reply LS to WP5D, Approved.

- RP-240787,New SI proposal: Study on IMT parameters for 4400-4800 MHz, 7125-8400 MHz and 14.8-15.35 GHz, Approved.

- TR 38.921

- https://www.itu.int/en/ITU-D/Regional-Presence/AsiaPacific/Documents/Events/2017/Aug-ISS2017/PAPER_Workshop_S3_Timur.pdf

- R4-2211224, Introduction of NR licensed band 6425 -7125 MHz, Approved.

- R4-2414283	WF on 15GHz BS parameters, ZTE, Approved.

- R4-2414284	WF on 15GHz UE parameters,Qualcomm, Approved.

- R4-24xxxx Discussion on co-existence evaluation for 14800 to 15350 MHz, ZTE Corporation

TP

8.1	BS antenna characteristics

8.1.1		Array antenna model

In Table 8.1.1-1, the parameters used by the parameterized array antenna model are described.

Table 8.1.1-1: Parameters of the parameterized array antenna model

| Parameter | Symbol | Unit |
| --- | --- | --- |
| Front to back ratio | Am | dB |
| Side lobe suppression | SLAv | dB |
| Horizontal HPBW | 3dB | Degrees |
| Vertical HPBW | 3dB | Degrees |
| Array element peak gain | GE,max | dBi |
| Number of radiating elements rows and columns | (M, N) | Integer |
| Horizontal element separation | dh | m |
| Vertical element separation | dv | m |
| Electrical down-tilt angle | etilt | Degrees |
| Electrical scan angle | escan | Degrees |

The parameterized antenna model is built around array antenna model where the element factor, array factor and linear phase progressing is characterized as described by equations in Table 8.1.1-2.

Table 8.1.1-2: Array antenna model details

| Description | Equation | Unit |
| --- | --- | --- |
| Peak normalized element radiation pattern | Aθ,φ=−min−−min12φφ3dB2,Am−min12θ−90θ3dB2,SLAv ,Am | dB |
| Peak gain normalized element radiation pattern | AEθ,φ=GE,max+Aθ,φ | dBi |
| Composite array radiation pattern | AAθ,φ=AEθ,φ+10log10m=1Mn=1Nwm,nvm,n2 , where vm,n=expj2πm−1dvλcosθ+n−1dℎλsinθsinφ wm,n=1MNexpj2πm−1dvλsinθetilt−n−1dℎλcosθetiltsinφescan | dBi |

8.1.2	Array antenna parameters

In Table 8.1.2-1, base station array antenna parameters for different deployment scenarios is listed.

Table 8.1.2-1: BS array antenna parameters

| Parameter | Indoor | Urban macro | Dense urban |
| --- | --- | --- | --- |
| Element gain (dBi) (Note 2) | 5 | 6.4 | 6.4 |
| Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 90º for V | 90º for H 65º for V | 90º for H 65º for V |
| Horizontal/vertical front to back ratio (dB) | 30 for both H/V | 30 for both H/V | 30 for both H/V |
| Antenna polarization | Linear ±45º | Linear ±45º | Linear ±45º |
| Antenna sub-array configuration (M × N) (Note 4) | 4 x 4 | 16 x 24 / 8 x 32 | 16 x 24 / 8 x 32 |
| Horizontal/Vertical radiating sub-array spacing | 0.5 of wavelength for H, 0.5 of wavelength for V | 0.5 of wavelength for H, 2.8/5.6 of wavelength for V | 0.5 of wavelength for H, 2.8/5.6 of wavelength for V |
| Number of element rows in sub-array | N/A | (4x1)/(8x1) | (4x1)/(8x1) |
| Vertical element separation in sub-array (dv,sub) | 0.5 of wavelength of V | 0.7 of wavelength of V | 0.7 of wavelength of V |
| Pre-set sub-array down-tilt (degrees) | N/A | 3 | 3 |
| Array Ohmic loss (dB) (Note 2) | 2 | 2 | 2 |
| Conducted power (before Ohmic loss) per sub-array/element (dBm) (Note 3) | 9 | 17.14/18.9 | 17.14/18.9 |
| Base station horizontal coverage range (degrees) | +/-90 | +/-60 | +/-60 |
| Base station vertical coverage range (degrees) (Note 1) | 0-180 | 90-100 | 90-100 |
| Mechanical down-tilt (degrees) | 90 | 6 | 6 |
| Note 1: The vertical coverage range is given for the elevation angle θ, defined between 0° and 180°. Note 2: The element gain includes the loss and is per polarization. Note 3: The conducted power per sub-array/element assumes 2xMxN subarrays/elements (i.e., power per H/V polarized element). Note 4: M and N represents rows and columns of dual polarized elements of the array or subarrays in the array Note 5: For the case of 4/8 elements per sub array, dv will be 2.8/5.6 wavelengths. |  |  |  |