3GPP TSG-RAN WG4 Meeting#112bis                              R4-2416245

Hefei, China, 14th – 18th Oct, 2024

Agenda item:	6.8.2.1

Source: 	ZTE Corporation

Title: 	Discussion on coexistence study for NTN HPUE

Document for:	Discussion

Introduction

In RAN#103 meeting, the work item [RP-240857] on Enhanced requirements and test methodology for NR and IoT NTN was approved as one of Rel-19 RAN4 package. The NTN HPUE is one of objective for Rel-19 NTN evolution. According to the legacy evaluation approach, RAN4 need to conduct the coexistence study to figure out the appropriate ACLR requirement for NTN HPUE similar as TN HPUE or NTN PC3 in Rel-17 and Rel-18. In this contribution, we want to share some initial understanding on coexistence cases and simulation assumptions for coexistence study and detailed evaluation results would be provided later on.

High power UE (HPUE) for NTN

- Conduct the coexistence analysis based on TR 38.863 for the above example bands and corresponding power classes

  - Evaluate the impact on the uplink performance of TN (Terrestrial network)

    - Example bands include bands n256 and 256, n255 and 255

    - PC2, PC1.5 (for NR-NTN only) and PC1 will be evaluated for ACLR and ACS

    - For NR-NTN and IoT-NTN: coexistence studies shall be carried out under the same conditions, including geographical separation between NTN UE and Terrestrial Networks (TN), as the co-existence studies in Rel-17 and Rel-18

- Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario

  - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study.

  - Support other power classes based on the co-existence and feasibility study

    - Non-hand held UE is prioritized

  - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands

    - Necessary Tx requirements, at least including

      - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE

      - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary

    - Necessary Rx requirements, at least including

      - Specify the RSD requirements on the example bands, if necessary

- Specify the necessary signaling to support the above objectives

Discussion on coexistence study for NTN HPUE

## 2.1. Coexistence cases

First of all, in Rel-17 NR NTN coexistence study and Rel-18 IoT NTN coexistence study, the following coexistence scenarios as listed in Table 2.1-1 have been evaluated to specify the appropriate the ACLR/ACS requirements. The coexistence evaluations results for NR NTN are captured in TR 38.863 and however for coexistence evaluation results for IoT NTN are not captured in any TR.

For the coexistence evaluation for Rel-19 NTN HPUE, coexistence evaluation should focus on the case 4 and case 5 since these two cases will determine NTN UE ACLR requirements.

Table 2.1-1. the coexistence evaluation cases for NTN PC3 UE

| No. | Combination | Aggressor | Victim | Notes | Study Phase |
| --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | TN DL | NTN DL | Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD. | Phase 1 |
| 2 | TN with NTN | TN UL | NTN UL | Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD. | Phase 1 |
| 3 | TN with NTN | NTN DL | TN DL | Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD. | Phase 1 |
| 4 | TN with NTN | NTN UL | TN UL | Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD. | Phase 1 |
| 5 | TN with NTN | NTN UL | TN DL | Applicable for satellite operating in S-band, for e.g. coexistence with n34 TDD. | Phase 1 |
| 6 | TN with NTN | TN DL | NTN UL | Applicable for satellite operating in S-band, for e.g. coexistence with n34 TDD. | Phase 1 |

## 2.2. Simulation assumptions

Regarding the simulation assumption for NTN NR HPUE, the simulation assumptions for FR1 NTN PC3 in TR 38.863 could be reused. However for NTN IoT HPUE, the simulation assumptions as agreed in R4-2217473 during Rel-18 phase could be reused.

Initial simulation results for IoT over NTN coexistence study

  - .Simulation results of Case 4

In case 4, IoT over NTN UL may interfere TN BS when TN BS receives UL transmission from TN UE. The effective ACLR of IoT over NTN UE should be provided according to the WF [1]. The simulation result is shown as the following table.

| Case 4 : Multiple tone | ACLR [dB] | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GEO_Urban_macro TN_AAS_UL PC2 | Average throughput loss | 23.91 | 13.15 | 7.15 | 3.86 | 2.03 | 1.17 | 0.56 | 0.30 |
|  | Cell edge through loss | 80.88 | 56.15 | 33.56 | 21.17 | 12.14 | 8.49 | 4.38 | 1.01 |
| GEO_Urban_macro TN_non_AAS_UL PC2 | Average throughput loss | 16.31 | 8.97 | 4.92 | 3.07 | 1.78 | 0.95 | 0.31 | 0.20 |
|  | Cell edge through loss | 50.95 | 27.64 | 10.78 | 2.55 | 0.62 | 0.14 | 0.01 | 0.00 |

| Case 4 : Multiple tone | ACLR [dB] | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GEO_Urban_macro TN_AAS_UL PC1 | Average throughput loss | 38.55 | 22.75 | 12.34 | 6.51 | 3.34 | 1.95 | 1.07 | 0.67 |
|  | Cell edge through loss | 91.13 | 76.00 | 48.91 | 30.10 | 18.52 | 10.17 | 7.16 | 1.24 |
| GEO_Urban_macro TN_non_AAS_UL PC1 | Average throughput loss | 30.20 | 16.32 | 8.27 | 4.34 | 2.53 | 1.68 | 0.76 | 0.40 |
|  | Cell edge through loss | 80.85 | 64.11 | 36.16 | 13.77 | 3.75 | 1.67 | 0.20 | 0.18 |

| Case 4: Single tone 15kHz | ACLR [dB] | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GEO_Urban_macro TN_AAS_UL PC2 | Average throughput loss | 26.16 | 14.52 | 8.14 | 4.28 | 2.24 | 1.14 | 0.62 | 0.39 |
|  | Cell edge through loss | 75.83 | 51.35 | 31.74 | 16.94 | 9.66 | 5.26 | 2.60 | 1.49 |
| GEO_Urban_macro TN_non_AAS_UL PC2 | Average throughput loss | 19.09 | 9.98 | 5.82 | 3.38 | 1.89 | 0.97 | 0.41 | 0.14 |
|  | Cell edge through loss | 63.78 | 35.54 | 17.95 | 5.93 | 1.04 | 0.20 | 0.07 | 0.00 |

| Case 4: Single tone 15kHz | ACLR [dB] | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GEO_Urban_macro TN_AAS_UL PC1 | Average throughput loss | 35.13 | 20.58 | 11.43 | 6.04 | 3.20 | 1.72 | 0.93 | 0.57 |
|  | Cell edge through loss | 86.12 | 65.88 | 43.41 | 25.01 | 11.31 | 5.01 | 1.75 | 1.20 |
| GEO_Urban_macro TN_non_AAS_UL PC1 | Average throughput loss | 29.18 | 15.96 | 8.16 | 4.32 | 2.54 | 1.40 | 0.83 | 0.61 |
|  | Cell edge through loss | 72.57 | 45.01 | 21.27 | 8.88 | 3.11 | 1.19 | 0.30 | 0.21 |

| Case 4: Single tone 3.75kHz | ACLR [dB] | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GEO_Urban_macro TN_AAS_UL PC2 | Average throughput loss | 27.35 | 15.54 | 8.50 | 4.58 | 2.45 | 1.23 | 0.66 | 0.39 |
|  | Cell edge through loss | 77.72 | 52.12 | 31.87 | 17.95 | 11.07 | 5.36 | 2.91 | 1.59 |
| GEO_Urban_macro TN_non_AAS_UL PC2 | Average throughput loss | 25.41 | 14.74 | 8.26 | 4.81 | 2.63 | 1.15 | 0.43 | 0.18 |
|  | Cell edge through loss | 70.13 | 42.75 | 18.98 | 5.59 | 1.53 | 0.37 | 0.17 | 0.11 |

| Case 4: Single tone 3.75kHz | ACLR [dB] | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GEO_Urban_macro TN_AAS_UL PC1 | Average throughput loss | 27.35 | 15.54 | 8.50 | 4.58 | 2.45 | 1.23 | 0.66 | 0.39 |
|  | Cell edge through loss | 77.72 | 52.12 | 31.87 | 17.95 | 11.07 | 5.36 | 2.91 | 1.59 |
| GEO_Urban_macro TN_non_AAS_UL PC1 | Average throughput loss | 25.41 | 14.74 | 8.26 | 4.81 | 2.63 | 1.15 | 0.43 | 0.18 |
|  | Cell edge through loss | 70.13 | 42.75 | 18.98 | 5.59 | 1.53 | 0.37 | 0.17 | 0.11 |

| Case 4: Single tone 3.75kHz | ACLR [dB] | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GEO_Urban_macro TN_AAS_UL PC1 | Average throughput loss | 31.40 | 18.04 | 9.89 | 5.48 | 2.99 | 1.63 | 0.92 | 0.56 |
|  | Cell edge through loss | 85.07 | 63.78 | 40.87 | 22.32 | 12.90 | 6.84 | 3.20 | 1.56 |
| GEO_Urban_macro TN_non_AAS_UL PC1 | Average throughput loss | 29.20 | 16.91 | 9.65 | 5.53 | 2.95 | 1.40 | 0.64 | 0.32 |
|  | Cell edge through loss | 74.23 | 46.43 | 20.92 | 7.83 | 2.92 | 0.85 | 0.20 | 0.06 |

Initial simulation results for NR over NTN coexistence study

  - .Simulation results of Case 4

In case 4, IoT over NTN UL may interfere TN BS when TN BS receives UL transmission from TN UE. The effective ACLR of IoT over NTN UE should be provided according to the WF [1]. The simulation result is shown as the following table.

| Case 4 : | ACLR [dB] | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GEO_Urban_macro TN_AAS_UL PC2 | Average throughput loss | 13.42 | 7.30 | 3.85 | 2.03 | 1.02 | 0.51 | 0.22 | 0.10 |
|  | Cell edge through loss | 50.00 | 31.07 | 16.47 | 6.90 | 5.25 | 2.99 | 0.73 | 0.00 |
| GEO_Urban_macro TN_non_AAS_UL PC2 | Average throughput loss | 9.58 | 5.34 | 3.10 | 1.76 | 0.91 | 0.38 | 0.13 | 0.04 |
|  | Cell edge through loss | 38.67 | 18.57 | 5.59 | 1.65 | 0.55 | 0.31 | 0.00 | 0.00 |

| Case 4 : | ACLR [dB] | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GEO_Urban_macro TN_AAS_UL PC1.5 | Average throughput loss | 19.31 | 10.62 | 5.79 | 3.04 | 1.62 | 0.81 | 0.36 | 0.16 |
|  | Cell edge through loss | 66.91 | 42.78 | 24.79 | 13.14 | 5.84 | 2.90 | 2.00 | 0.56 |
| GEO_Urban_macro TN_non_AAS_UL PC1.5 | Average throughput loss | 14.25 | 7.29 | 4.16 | 2.33 | 1.38 | 0.65 | 0.23 | 0.10 |
|  | Cell edge through loss | 56.91 | 31.16 | 10.89 | 3.09 | 0.93 | 0.34 | 0.12 | 0.00 |

| Case 4 : | ACLR [dB] | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GEO_Urban_macro TN_AAS_UL PC1 | Average throughput loss | 24.29 | 13.52 | 7.40 | 3.93 | 2.07 | 1.10 | 0.51 | 0.23 |
|  | Cell edge through loss | 76.49 | 51.60 | 30.94 | 18.30 | 7.89 | 3.94 | 2.08 | 0.63 |
| GEO_Urban_macro TN_non_AAS_UL PC1 | Average throughput loss | 18.46 | 9.54 | 5.35 | 3.09 | 1.72 | 0.87 | 0.35 | 0.11 |
|  | Cell edge through loss | 67.24 | 38.69 | 18.57 | 5.59 | 1.65 | 0.55 | 0.31 | 0.00 |

Conclusions

In this contribution, we want to share some initial understandings on coexistence study on Rel-19 NTN HPUE and proposals are made as following:

Proposal 1:for IoT NTN(NB1, NB2), propose to reuse the TN UE ACLR requirement 37dBc for single tone and FFS for multiple tone case.

Proposal 2: for NR NTN, propose to reuse the TN UE ACLR requirement.

References

- R4-2217473 WF on co-existence assumption for IoT over NTN, MTK, approved.

- RP-240857,New WID: Enhanced requirements and test methodology for NR and IoT NTN

- TR 38.863