3GPP TSG-RAN WG4 Meeting#112bis                             R4-2416248

Hefei, China, 14th – 18th Oct, 2024

Agenda item:	6.20.1

Source: 	ZTE Corporation

Title: 	Discussion on system parameters for SBFD BS

Document for:	Approval

Introduction

In the RAN#102 meeting, new WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD) has been approved. It’s expected to start the RAN4 meeting from April meeting. Since we have already reached good progress on the SBFD BS RF impacts in SI phase and outcome was captured in TR 38.858. During the last RAN4 meeting, we have some initial discussions on work plan and general aspects for SBFD BS, however there are still lots of open issues not addressed yet. In this contribution, we would like to share further views on these remaining issues.

Topic #1: General aspects (including RAN4 aspects for SBFD system parameters) - Issue 1-2-1: Band/channel bandwidth support for SBFD - Agreement: - Declaration-based method, and SBFD is possible to be used on any TDD band, with the following principle: - The supported channel bandwidth(s) is declared by gNB vendors - The supported channel bandwidth(s) is from existing channel bandwidth defined in TS 38.104 - The supported channel bandwidth shall be no less than X MHz - X = [20] - For next meeting, we will further discuss the potential to increase the value of X beyond X = [20]. Companies are encouraged to bring t-docs to discuss the benefits or drawback to increase this value. - FFS for some specific band if this principle rules out this band, which is required for SBFD by operators. - FFS how to implement in the spec: - Option 1: To include the following text in Clause 5.2 in TS 38.104: subband full duplex can be applied to TDD bands with channel bandwidth >= X MHz, given in Table 5.2-1. - Option 2: Add SBFD entry in Table 5.2-1 in Clause 5.2 in TS 38.104 for the TDD bands with channel bandwidth >= X MHz. - Agreement: - BS channel bandwidth for SBFD-capable BS: - BS channel bandwidth for SBFD-capable BS: RF bandwidth supporting a single NR RF carrier with the transmission bandwidth configured in the uplink or downlink or SBFD operation. - The existing definition of BS channel bandwidth (for information only): - BS channel bandwidth: RF bandwidth supporting a single NR RF carrier with the transmission bandwidth configured in the uplink or downlink. - Issue 1-2-3: Transmission bandwidth configuration for SBFD - Agreement: - BS transmission bandwidth configuration for SBFD: - BS transmission bandwidth configuration for SBFD NRB,SBFD: set of resource blocks located within the BS channel bandwidth which may be used for transmitting or/and receiving by the BS. - The transmission bandwidth configuration for SBFD NRB,SBFD for each BS channel bandwidth and subcarrier spacing shall reuse the values for BS transmission bandwidth configuration in table 5.3.2-1 for FR1 and table 5.3.2-2 for FR2-1. - FFS a corresponding figure for SBFD is needed - The existing definition of BS transmission bandwidth configuration (for information only): - BS transmission bandwidth configuration: set of resource blocks located within the BS channel bandwidth which may be used for transmitting or receiving by the BS. - Issue 1-2-4: DL/UL subband configurations support for SBFD - Agreement: - Transmission bandwidth configuration for SBFD UL/DL subband: - NRB,SBFD,DL: Transmission bandwidth configuration for SBFD DL subband, expressed in resource blocks. - NRB,SBFD,UL: Transmission bandwidth configuration for SBFD UL subband, expressed in resource blocks. - Guardband between SBFD UL and DL subband: - NRB,SBFD,GB: The size of guardband between SBFD UL subband and DL subband, expressed in resource blocks. - Agreement: - To specify DL/UL subband configuration in BS RF requirements, the following 3 options for DL/UL subband configuration are identified: - Option-1 (with the least flexibility): - Only a number of selected DL/UL subband configurations are specified for RAN4 BS RF requirement. - Example (given in Nokia Tdoc R4-2413238): - For FR1 DUD configuration in 100 MHz channel and 30 kHz SCS: < NRB,SBFD,DL, NRB,SBFD,UL, NRB,SBFD,GB > = <106, 51, 5> - For FR1 DU or UD configuration in 100 MHz channel and 30 kHz SCS: < NRB,SBFD,DL, NRB,SBFD,UL, NRB,SBFD,GB > = <217, 51, 5> - Option-2 (with the middle flexibility): - Option-2a: - DL subband configuration shall be selected from the existing values of BS transmission bandwidth configuration, as provided in Table 5.3.2-1 for FR1 and Table 5.3.2-2 for FR2-1. - UL subband size and guardband size(s) depends on BS implementation - Example (FR1 DU or UD in 100MHz and 30kHz SCS, 80-20MHz case): As long as BS transmission configuration for SBFD shall be 273 (NRB,SBFD for 100MHz and 30kHz SCS) and NRB,SBFD,DL is selected from Table 5.3.2-1 (according to the 30kHz SCS), from BS RF requirement perspective, no limitation on the values NRB,SBFD,UL and NRB,SBFD,GB. - Option-2b: - UL subband configuration shall be selected from the existing values of BS transmission bandwidth configuration, as provided in Table 5.3.2-1 for FR1 and Table 5.3.2-2 for FR2-1. - DL subband size(s) and guardband size(s) depends on BS implementation - Example (FR1 DU or UD in 100MHz and 30kHz SCS, 80-20MHz case): As long as BS transmission configuration for SBFD shall be 273 (NRB,SBFD for 100MHz and 30kHz SCS) and the value of NRB,SBFD,UL is selected from Table 5.3.2-1 (according to the 30kHz SCS), from BS RF requirement perspective, no limitation on the values NRB,SBFD,DL and NRB,SBFD,GB. - Option-3 (with the highest flexibility): - No restriction on the subband sizes, i.e., RF requirement shall be specified to enable all DL and UL subband configurations (and implicitly guardband configurations). - Example (FR1 DU or UD in 100MHz and 30kHz SCS): From BS RF requirement perspective, no limitation on the values of NRB,SBFD,DL, NRB,SBFD,UL, NRB,SBFD,GB - FFS how to implement BS RF requirements based on the above options for different levels of flexibility. - BS transmission configuration in BS RF requirements (for information only): Table 5.3.2-1: Transmission bandwidth configuration NRB for FR1 SCS (kHz); 3 MHz; 5 MHz; 10 MHz; 15 MHz; 20 MHz; 25 MHz; 30 MHz; 35 MHz; 40 MHz; 45 MHz; 50 MHz; 60 MHz; 70 MHz; 80 MHz; 90 MHz; 100 MHz. NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB. 15; 15; 25; 52; 79; 106; 133; 160; 188; 216; 242; 270; N/A; N/A; N/A; N/A; N/A. 30; N/A; 11; 24; 38; 51; 65; 78; 92; 106; 119; 133; 162; 189; 217; 245; 273. 60; N/A; N/A; 11; 18; 24; 31; 38; 44; 51; 58; 65; 79; 93; 107; 121; 135. Table 5.3.2-2: Transmission bandwidth configuration NRB for FR2-1 SCS (kHz); 50 MHz; 100 MHz; 200 MHz; 400 MHz. NRB; NRB; NRB; NRB. 60; 66; 132; 264; N/A. 120; 32; 66; 132; 264. - Issue 1-2-5: Restriction on guardband size for SBFD - Agreement: - Restriction on guardband size for SBFD: - Option 1: Definition based on existing guardbands requirement in TS38.104 (i.e., minimum guardband) - Option 2: Typical values of guardband defined for requirement, but the guardband should not be restricted to those in RAN4 specifications. - Option 3: No restriction (manufacturer declaration-based), but with restriction on maximum allowed number of RBs for guardband: - Option 3a: No restriction (manufacturer declaration-based), and no restriction on maximum/minimum number of RBs for guandband: - Issue 1-3-1: SBFD time-domain terminology - Agreement: - When specifying RAN4 requirements for SBFD-capable BS, SBFD symbols (instead of SBFD slots/symbols) shall be used to designate the time domain duration in which SBFD related requirements is applied. - Issue 1-4-1: CLI handling impact on RF requirement - Agreement: - RAN4 shall define RF requirements for the frequency outside of wanted carrier based on no gNB-to-gNB CLI handling as baseline - FFS Revisit particular RF requirements pending on RAN1 progress on the gNB-to-gNB CLI handling work. Topic #2: Potentially new requirements for SBFD - Issue 2-1-1: Requirement for transient period between SBFD and non-SBFD - Agreement: - RAN4 shall use existing transient period requirement as baseline for the following four cases identified for SBFD: Frequency regions for Transition; BS transmitter behavior during transition. Case-A (SBFD to DL); SBFD UL subband and guardband(s); BS transmitter OFF-to-ON. Case-B (SBFD to UL); SBFD DL subband; BS transmitter ON-to-OFF. Case-C (DL to SBFD); SBFD UL subband and guardband(s); BS transmitter ON-to-OFF. Case-D (UL to SBFD); SBFD DL subband; BS transmitter OFF-to-ON. - Issue 2-1-2: Transient period length between SBFD and non-SBFD - Agreement: - Transient period length between SBFD and non-SBFD: - The existing TDD BS transmitter transient period reused, i.e., 10us for FR1 and 3us for FR2-1. - Issue 2-1-3: Location of transient period between SBFD and non-SBFD - Agreement: - Location of transient period between SBFD and non-SBFD in RAN4 RF requirement: - FFS the location of transient period between SBFD and non-SBFD shall be located within the SBFD slot/symbol if it needs to be decided. - Issue 2-2-1: Requirement for transient period between different SBFD configurations - Agreement: - No need to specify transmitter transient period requirement between different SBFD configurations.

Discussion

- Issue 1-2-1: Band/channel bandwidth support for SBFD

During the last RAN4 meeting, we have some initial discussions on the specified minimum channel bandwidth for SBFD operation. As shown in the following table, if the specified minimum channel bandwidth is 20MHz, then the following band highlighted in yellow (e.g. band n34, n51, n53, n54 and n101) will be ruled out for SBFD operation in Rel-19. If to further increase the specified minimum channel bandwidth e.g. 50MHz or 60MHz, then more operation bands would be impacted which will have the direct impacts on the practical deployment for this feature.

Table 5.2-1: NR operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high | Duplex mode | Available spectrum | Maximum carrier bandwidth |
| --- | --- | --- | --- | --- | --- |
| n34 | 2010 MHz – 2025 MHz | 2010 MHz – 2025 MHz | TDD | 15 | 15 |
| n38 | 2570 MHz – 2620 MHz | 2570 MHz – 2620 MHz | TDD | 50 | 40 |
| n39 | 1880 MHz – 1920 MHz | 1880 MHz – 1920 MHz | TDD | 40 | 40 |
| n40 | 2300 MHz – 2400 MHz | 2300 MHz – 2400 MHz | TDD | 100 | 100 |
| n41 | 2496 MHz – 2690 MHz | 2496 MHz – 2690 MHz | TDD | 194 | 100 |
| n46 | 5150 MHz – 5925 MHz | 5150 MHz – 5925 MHz | TDD | 775 | 100 |
| n48 | 3550 MHz – 3700 MHz | 3550 MHz – 3700 MHz | TDD | 150 | 40 NOTE: 50-100MHz for Scell operation |
| n50 | 1432 MHz – 1517 MHz | 1432 MHz – 1517 MHz | TDD | 85 | 80 |
| n51 | 1427 MHz – 1432 MHz | 1427 MHz – 1432 MHz | TDD | 5 | 5 |
| n53 | 2483.5 MHz – 2495 MHz | 2483.5 MHz – 2495 MHz | TDD | 11.5 | 10 |
| n54 | 1670 MHz – 1675 MHz | 1670 MHz – 1675 MHz | TDD | 5 | 5 |
| n77 | 3300 MHz – 4200 MHz | 3300 MHz – 4200 MHz | TDD | 900 | 100 |
| n78 | 3300 MHz – 3800 MHz | 3300 MHz – 3800 MHz | TDD | 500 | 100 |
| n79 | 4400 MHz – 5000 MHz | 4400 MHz – 5000 MHz | TDD | 600 | 100 |
| n90 | 2496 MHz – 2690 MHz | 2496 MHz – 2690 MHz | TDD | 194 | 100 |
| n96 | 5925 MHz – 7125 MHz | 5925 MHz – 7125 MHz | TDD | 1200 | 100 |
| n101 | 1900 MHz – 1910 MHz | 1900 MHz – 1910 MHz | TDD | 10 | 10 |
| n1024 | 5925 MHz – 6425 MHz | 5925 MHz – 6425 MHz | TDD | 500 | 100 |
| n1048 | 6425 MHz – 7125 MHz | 6425 MHz – 7125 MHz | TDD | 700 | 100 |

Based on the above consideration, we are fine to adopt the 20MHz carrier bandwidth as low boundary for SBFD operation in FR1 to minimize the impacts on its operation bands. In addition, whether to support 20MHz for SBFD operation in certain carrier is also up to the vendor’s declaration and there are no mandate in the specification to support the SBFD operation above 20MHz carrier bandwidth.

Proposal 1: propose to adopt the 20MHz as the minimum carrier bandwidth for SBFD operation in FR1 TDD bands and also 50MHz as minimum carrier bandwidth for SBFD operation in FR2-1 TDD bands.

- Issue 1-2-3: Transmission bandwidth configuration for SBFD

Based on the discussion in the previous RAN4 meeting, it was discussed whether to have informative figure for SBFD transmission bandwidth configuration. As shown in the following Figure 1.2.3-1, similar as the legacy BS PRB utilization, this informative figure might be useful for the clarification in the specification.

Figure 1.2.3-1: SBFD BS PRB utilization

In addition, according to the SCS restriction in TS 38.331 specification, the SCS for active DL BWP and active UL BWP should be the same, we also highlighted in the following diagram to align the numerology in DL sub-band and UL sub-band.

Proposal 2: propose to add the clarification figure for the SBFD BS PRB utilization in the specification

- Issue 1-2-4: DL/UL subband configurations support for SBFD

- In the previous RAN4 meeting, we have some initial discussions on how to specify the DL/UL subband configuration for SBFD. Option 2b should be best option to move forward with the following reasons:

- DL sub-band: if DL sub-band is specified explicitly and leave the GB and UL sub-band configuration up to the manufacture declaration, then it is not possible to specify the SBFD BS receiver requirement properly. In addition, DL sub-band configuration is mainly used for transmitter requirement definition or conformance testing which is somehow similar as test model specified in TS38.104. Whether to specify the in-channel emission requirement or in-channel leakage requirement is also ongoing. In other words, the receiver requirement cannot be addressed for DL sub-band configuration only.

- UL sub-band: the main reasons to specify the UL sub-band explicitly is due to the BS receiver requirement. It’s well known that lots of BS receiver requirement (e.g. REFSENS FRC) is highly dependent on the UL PRB configurations, corresponding MCS and reference signal configuration. If UL sub-band is missing and leave it up to the manufacture declaration, then it is not possible to define the receiver RF requirement appropriately. The details could be found as following especially for FRC design for UL sub-band.

Proposal 3: propose to adopt the option -2b to specify the DL/UL subband configuration.

In addition,regarding the specific UL sub-band configuration for certain carrier bandwidth also need more discussions if option 2b is agreeable. For example, for 100MHz carrier bandwidth, whether all sub-band configuration e.g.20, 25, 30, 35, 40, 45,50, 60, 70,80,90MHz should be specified or only part of carrier bandwidth should be specified to simplify the specification.

From our understanding, it might be better to have all sub-band configuration to be specified with both FRC and power levels defined, otherwise some of RAN4 specification might be not aligned with the practical deployment especially considering the analog filtering design which is tightly related with certain DL-UL sub-band configuration, therefore in order to have more complete specification and allow for the alignment between specification and the practical deployment, all sub-band configuration should be specified.

Proposal 4:all UL sub-band configuration (e..g.reusing from the existing carrier bandwidth, 20~90MHz) should be specified for SBFD receiver requirements together with FRC and power levels. The detailed MCS, reference signal pattern for licensed band could align with WF R4-1801031. For unlicensed bands, the interleaved PRB pattern should be also configured.

- Issue 1-2-5: Restriction on guardband size for SBFD

In the previous RAN4 meeting, there were some discussions on the guard band size for SBFD. From our understanding, there are two kinds of guard band concept as following at least:

- Guard band at the carrier edge (e.g. in red color) which is supposed to the same as legacy GB design;

- Guard band between DL sub-band and UL sub-band (e..g in orange color);

- For the guard band between DL sub-band and UL sub-band, this is highly dependent on BS class, DL/UL sub-band configuration etc. For example, for FR1 wide area SBFD BS, due to the potential analog RF filtering between DL and UL sub-band, then guard band size would be expected to be larger, however for other BS class, the guard band might be quite limited e.g. limited to 1 PRB or even less than 1 PRBs. Whether to define maximum guard band size based on the worst assumption is also questionable since the analog RF filtering is still debatable so far for FR1 WA SBFD BS. In addition, guard band between different spectrum block is not something new in TS 38.104 specification. As extracted from the existing TS 38.104 specification, the guard band between different numerologies is up to the implementation dependent. This kind of design principle could be also reused for guard band between DL-UL sub-band configuration of SBFD BS.

Figure 1.2.5-1. DL/UL sub-band configuration

Figure 5.3.3-2: Guard band definition when transmitting multiple numerologies

NOTE:	Figure 5.3.3-2 is not intended to imply the size of any guard between the two numerologies. Inter-numerology guard band within the carrier is implementation dependent.

Proposal 5: propose to guard band between DL and UL sub-band is up to the declaration from the implementation perspective.

- Issue 2-1-3: Location of transient period between SBFD and non-SBFD

Based on the discussions in the previous meetings, the location of transient period between SFBD and non-SBFD symbols is not determined yet and the necessity of this requirement is also debatable so far. At least from our understanding, the transition period should be located within SBFD symbols instead of non-SBFD symbols to avoid the negative impacts on the legacy UE reception. It is well known that, during the BS Tx OFF-ON or ON-OFF, the transmitter signal quality within the transition period cannot be guaranteed properly. Usually from UE receiver demodulation perspective, the corrupted soft information should be dropped before feeding into the LPDC/Polar decoder. For the SBFD aware UE, UE could know the location of the transition period by the TDD DL-UL configuration and additional SBFD configuration in RRC inf, then the corrupted soft information could be dropped within the estimated window length and baseband performance could be ensured somehow. If the transition period is located within the non-SBFD symbols or across the non-SBFD symbols and SBFD symbols, then the performance degradation for non-SBFD symbols will be expected. In short, we propose to have the transition period between non-SFBD symbols and SBFD symbols located within SBFD symbols.

Figure 2.1.3-1. the location of transition period between SBFD and non-SBFD symbols

Proposal 6: propose to have the transition period between non-SFBD symbols and SBFD symbols located within SBFD symbols.

Conclusions

In this contribution, we want to share some initial views on general aspects for SBFD BS and proposals/observations are made as following:

Proposal 1: propose to adopt the 20MHz as the minimum carrier bandwidth for SBFD operation in FR1 TDD bands and also 50MHz as minimum carrier bandwidth for SBFD operation in FR2-1 TDD bands.

Proposal 2: propose to add the clarification figure for the SBFD BS PRB utilization in the specification

Proposal 3: propose to adopt the option -2b to specify the DL/UL subband configuration.

Proposal 4:all UL sub-band configuration (e..g.reusing from the existing carrier bandwidth, 20~90MHz) should be specified for SBFD receiver requirements together with FRC and power levels. The detailed MCS, reference signal pattern for licensed band could align with WF R4-1801031. For unlicensed bands, the interleaved PRB pattern should be also configured.

Proposal 5: propose to guard band between DL and UL sub-band is up to the declaration from the implementation perspective.

Proposal 6: propose to have the transition period between non-SFBD symbols and SBFD symbols located within SBFD symbols.

References

- RP-234035,New WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei, Samsung, Approved.

- TR 38.858

- R4-2321091, TP to TR 38.858 Impact on BS RF requirements, ZTE, Agreed

- R4-2406107, Way Forward for [110bis][313] NR_duplex_evo

- R4-2409958, Way forward for [110bis][313] NR_duplex_evo, Approved

- R4-2413514	Way Forward for [112][307] NR_duplex_evo_General, Approved.

- R4-2413514	Way Forward for [112][307] NR_duplex_evo_General

- R4-1801031, WF on NR BS RF FRCs, approved.