# 3GPP TSG-RAN WG4 Meeting # 112-bis                                                         R4-2416252

# Hefei, China, 14 – 18 October 2024

# Title:	HPUE CA/DC MSD Simplification

# Agenda Item:	6.1.1.1.1

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

# Introduction

This document presents our views on the options for MSD rules captured in the agreed Way Forward (WF) [1].

# Discussion

We summarize below the options and the proponents on simplifying the MSD rules for HPUE carrier-aggregation (CA)/dual-connectivity (DC) that were captured at the last meeting in [1]:

- Option 1: PC2/PC1.5 MSD requirements are not specified. The UE shall meet the Rx requirements of default Power Class (PC), with default power class transmission [2,3].

- Option 2: Using some equations or look-up tables or other means to derive PC2/PC1.5 MSD from respective PC3 MSD [4,5].

- Option 3:  Other solutions are not precluded. The proposals of [6] may be considered as an example of option 3 where all MSD tables, for all power classes, are to be removed for new CA requests and to be replaced with a reference to the single band’ REFSENS requirements.

Table 1 compares each option and presents our views on simplification benefits and challenges.

Table 1: HPUE CA/DC MSD simplification options

| MSD simplification | Default PC | HPUE PC | Simplification | Challenges |
| --- | --- | --- | --- | --- |
| Option 1 [2,3] | - Keep MSD requirements | - Remove the HPUE CA MSD requirements & tables. | RAN4: - Reduced TS complexity and maintenance, - Reduced TP for TR MSD workload RAN5: - Reduced conformance test complexity, workload, test-time. | - Impact on the lower MSD capability, - Handling of cases where no MSD occurs for the default power class but does occur for HPUE |
| Option 2 [4,5] | - Keep MSD requirements | - Remove the HPUE CA MSD tables, - Keep the MSD requirements by replacing the MSD tables either by equations, look-up tables or even a single default PC MSD de-rating value. | RAN4: - Reduced TS complexity and maintenance, - Reduced TP for TR MSD workload RAN5: It is up to RAN5 to decide how to simplify HPUE CA MSD conformance testing. | - Handling of cases where no MSD occurs for the default power class but does occur for HPUE |
| Option 3 [6] | - Remove MSD requirements and tables | - Remove MSD requirements and tables | RAN4 & RAN5: - Maximum simplification as the single band REFSENS remain the only RF requirements for conformance testing/verification. | - Impact on the lower MSD capability, - Major RAN4 and RAN5 change for which the acceptance by stakeholders is uncertain. |

Regarding simplifications, we observe that Option 1 and Option 2 bring near-identical benefits in terms of RAN4 TS simplification, TS maintenance, and a near-complete reduction of the workload on TP for TR HPUE MSD analyses since all HPUE MSD tables are removed in both options. The only difference is that Option 2 maintains the HPUE MSD RF requirements by either using a set of equations or using look-up tables. We have shown that equations or simple look-up tables can be used to replace the HPUE MSD tables due to Uplink harmonic interference and Rx harmonic mixing [7,8]. We believe this approach can also be used to remove HPUE MSD tables due to cross-band isolation, and the HPUE MSD tables due to dual-UL IMD interference. Option 2 does not preclude RAN5 from simplifying the UE Conformance TS, but we believe that such simplification is a RAN5’s responsibility. Further simplification may also be achieved by regional certification bodies such as PTRCB or GCF, where conformance could be restricted to verifying only the default power class CA/DC MSD performance.

The benefits of Option 1/Option 2 on reducing the number of NR-CA MSD tables are captured in Table 2. The number of MSD tables may be reduced by a factor 2, where 12 tables are needed to capture the default power class NR-CA MSD requirements vs a total of 24 tables currently.

Table 2: Option 1/2 NR-CA HPUE MSD table simplification. HPUE MSD tables are highlighted in yellow.

| CA type | Source of interference | Power Class | NR-CA MSD Tables 38.101-1 (18.7.0) | Kept NR-CA MSD tables after option 1 and option 2 simplification |
| --- | --- | --- | --- | --- |
| Intra-band | 1UL IMD | PC3 | Contiguous: Table 7.3A.2.1-1a Non-contiguous: Table 7.3A.2.2-1 | Contiguous: Table 7.3A.2.1-1a Non-contiguous: Table 7.3A.2.2-1 |
|  |  | PC2 | Contiguous: Table 7.3A.2.1-3 Non-contiguous: Table 7.3A.2.2-1a |  |
|  | 2UL IMD | PC3 | Contiguous: Table 7.3A.2.1-1 Non-contiguous: Table 7.3A.2.2-2 | Contiguous: Table 7.3A.2.1-1 Non-contiguous: Table 7.3A.2.2-2 |
| Inter-band | UL harmonic | PC3 | Table 7.3A.4-1 | Table 7.3A.4-1 |
|  |  | PC2 | Table 7.3A.4-2a |  |
|  | Rx harmonic mixing | PC5 | Table 7.3A.4-4d | Table 7.3A.4-4d |
|  |  | PC3 | Table 7.3A.4-4 | Table 7.3A.4-4 |
|  |  | PC2 | 1Tx: Table 7.3A.4-4a-1 2Tx: Table 7.3A.4-4a-2 |  |
|  |  | PC1.5 | Table 7.3A.4-4b |  |
|  | Cross-band isolation | PC5 | Table 7.3A.6-1c | Table 7.3A.6-1c |
|  |  | PC3 | Table 7.3A.6-1 2xUL PC3 Table 7.3A.6-3 | Table 7.3A.6-1 2xUL PC3 Table 7.3A.6-3 |
|  |  | PC2 | 1Tx: Table 7.3A.6-1a-1 2Tx: Table 7.3A.6-1a-2 |  |
|  |  | PC1.5 | Table 7.3A.6-1b |  |
|  | Dual-UL IMD & triple beat | PC3 | 2DL/2UL: Table 7.3A.5-1 3DL/2UL: Table 7.3A.5-2 | 2DL/2UL: Table 7.3A.5-1 3DL/2UL: Table 7.3A.5-2 |
|  |  | PC2 | 2DL/2UL: Table 7.3A.5-1a 3DL/2UL: Table 7.3A.5-2a |  |
|  |  | PC1.5 | 2DL/2UL: Table 7.3A.5-1b |  |
| Total number of NR-CA MSD tables |  |  | 24 | 12 |

Observation 1: By removing all HPUE CA/DC MSD tables, both Option 1 and Option 2 deliver near-identical benefits in terms of RAN4 TS simplification, TS maintenance, and a near-complete reduction of the workload on the TP for TR for HPUE MSD analyses. These options may divide by 2 the number of NR-CA MSD tables. By removing all HPUE CA MSD requirements, Option 1 de facto also reduces the RAN5 UE conformance complexity/test time. We believe Option 2 does not preclude similar savings, but we think it is up to RAN5 to implement any simplification. Regional certification bodies may also consider verifying only the default power class CA/DC MSD performance.

Regarding implementation challenges, Option 1 impacts the newly specified “lower MSD capability” since this capability is defined relative to the highest supported power class. One common challenge to both Option 1 and Option 2 are cases of band combinations where no MSD occurs for the default power class operation; however, for HPUE operation, some MSD may occur, and RF requirements are needed.

Observation 2: Removing the HPUE MSD requirements (Option 1) impacts the lower MSD reporting capability which is currently specified relative to the highest supported power class. For Option 1 and Option 2, RAN4 also needs to address MSD requirements for CA/DC combinations which are not affected by MSD at the default power class, but for which HPUE operation may introduce MSD requirements.

Proposal: Consider Option 2 to simplify the Rel-19 HPUE CA/DC MSD requirements. Interested companies are invited to propose either equations, or lookup tables or any other means (for example, a single derating value of the default power class MSD) to simplify the HPUE CA/DC MSD specifications.

# Conclusion

This paper captures our views on simplifying MSD requirements for HPUE CA/DC band combinations:

Observation 1: By removing all HPUE CA/DC MSD tables, both Option 1 and Option 2 deliver near-identical benefits in terms of RAN4 TS simplification, TS maintenance, and a near-complete reduction of the workload on the TP for TR for HPUE MSD analyses. These options may divide by 2 the number of NR-CA MSD tables. By removing all HPUE CA MSD requirements, Option 1 de facto also reduces the RAN5 UE conformance complexity/test time. We believe Option 2 does not preclude similar savings, but we think it is up to RAN5 to implement any simplification. Regional certification bodies may also consider verifying only the default power class CA/DC MSD performance.

Observation 2: Removing the HPUE MSD requirements (Option 1) impacts the lower MSD reporting capability which is currently specified relative to the highest supported power class. For Option 1 and Option 2, RAN4 also needs to address MSD requirements for CA/DC combinations which are not affected by MSD at the default power class, but for which HPUE operation may introduce MSD requirements.

Proposal: Consider Option 2 to simplify the Rel-19 HPUE CA/DC MSD requirements. Interested companies are invited to propose either equations, or lookup tables or any other means (for example, a single derating value of the default power class MSD) to simplify the HPUE CA/DC MSD specifications.

# References

[1]	R4-2414277 WF on HPUE for CA in TN, 3GPP TSG-RAN WG4 Meeting #112, Maastricht, Netherlands, Samsung.

[2]	R4-2412007 UE RF Enh 4: On necessity of additional MSD requirement for HPUE, 3GPP TSG-RAN WG4 Meeting #112, Maastricht, Netherlands, Nokia.

[3]	R4-2411316 Views on HPUE inter-band CA with 2Tx or 3Tx, 3GPP TSG-RAN WG4 Meeting #112, Maastricht, Netherlands, Samsung.

[4] 	R4-2412619 MSD for HPUE, 3GPP TSG-RAN WG4 Meeting #112, Maastricht, Netherlands, Qualcomm France.

[5]	R4-2413066 Further MSD simplifications, 3GPP TSG-RAN WG4 Meeting #112, Maastricht, Netherlands, Skyworks Solutions, Inc.

[6]	R4-2411147 Simplifying or removing MSD tables, 3GPP TSG-RAN WG4 Meeting #112, Maastricht, Netherlands, Apple.

[7]	R4-2407156 Guidelines for FDD PC2 Harmonic MSD Analysis, 3GPP TSG-RAN WG4 Meeting #111, Fukuoaka, Japan, Skyworks Solutions, Inc.

[8]	R4-2413068 Update on Harmonic MSD simplification, 3GPP TSG-RAN WG4 Meeting #112, Maastricht, Netherlands, Skyworks Solutions, Inc.