# 3GPP TSG-RAN WG4 Meeting # 112-bis                                                         R4-2416254

# Hefei, China, 14 – 18 October 2024

# Title:	RAN4 PRD input on Guidelines for harmonic MSD test points

# Agenda Item:	8.1.1.1

# Source:	Skyworks Solutions, Inc., Qualcomm France, Nokia.

# Document for:	Approval

# Introduction

This document provides the guidelines that were used to clean up the Rel-18 harmonic MSD test points [1,2,3,4,5,6,7] as an initial input to the RAN4 Permanent Reference Document (PRD).

# Discussion

For this meeting, the guidelines used to clean up the Rel-18 MSD test points are summarized with four proposals to help proponents write Rel-19 TP for TRs. These proposals will later be captured into a formal “TP for PRD” once the first PRD draft is made available to RAN4.

Proposal 1: Guidelines for the uplink (UL) band configuration

For MSD test points due to UL harmonic interference and MSD due to receiver (Rx) harmonic mixing:

  - UL band channel bandwidth (CBW) and SCS:

The UL band should be configured with the smallest UL CBW and the smallest SCS supported by the UL band for both the 1st and 2nd MSD test points whenever a 2nd test point is specified.

  - UL band RB allocation

    - For direct-hit MSD test points, the RB allocation length “LCRB” scales with the harmonic order according to the following rule for SCS 15kHz / SCS 30 kHz, respectively:

- For the UL1/DLx test point, Lcrb=25RB / 12RB,

- For the UL2/DLx test point, Lcrb=12RB / 6RB,

- For the UL3/DLx test point, Lcrb=8RB / 4RB,

- For the UL4/DLx test point, Lcrb=6RB / 3RB,

- For the UL5/DLx test point, Lcrb=5RB / 2RB.

    - For near-miss MSD test points, the UL band should be configured with the full RB allocation that can be supported by DFT-s-OFDM waveforms (e.g. Lcrb=50RB for CBW 10MHz, SCS 15kHz).

    - The RB allocation “RBstart” is removed from each MSD test point and is specified in the form of an equation which is captured in the first applicable generic footnote as:

“The requirements should be verified using RBstart = floor((NRB-LCRB)/2), where floor(x) is the greatest integer less than or equal to x, and where the UL parameters NRB and LCRB are respectively, the transmission bandwidth configuration and the number of RB’s for the specified UL band channel bandwidth and the UL band subcarrier spacing.”

This equation ensures the configured UL RBs are centered in the middle of the uplink channel.

Proposal 2: Guidelines for the downlink (DL) band configuration

For MSD test points due to UL harmonic interference and MSD due to receiver (Rx) harmonic mixing:

  - DL band CBW and number of MSD test points:

For direct-hit MSD test points, a maximum of two test points can be specified:

- A first “mandatory” test point to capture the MSD of the lowest DL band CBW, this test point is mandatory.

- A second “optional” test point to capture the MSD of the highest DL band CBW. Other DL band CBW configurations are not precluded.

For near-miss MSD test points:

- When the DL band may be affected by an UL2/DL1 direct-hit collision, the near-miss MSD test point is not needed. The footnote associated to the UL2/DL1 source of interference is modified to inform RAN5 that the DL band may be affected by near-miss interference for which the MSD is not specified.

- When the DL band may not be affected by a direct-hit collision, one near-miss MSD test point may be considered whenever applicable, and the MSD should be specified for the lowest DL band CBW.

  - DL band “lowest/highest” CBW:

- The DL band “lowest” and the “highest” CBW shall not exceed the CBW specified in the BCS configuration tables of clause 5.5A.

- The guidelines of WF [8] on handling of requests which introduce a new lowest or a new highest DL band CBW are the most up-to-date applicable guidelines.

Proposal 3: Guidelines on UL/DL fc condition

- Direct-hit UL/DL fc conditions remain unchanged and are captured with equations in dedicated footnotes.

- Near-miss UL/DL fc conditions for UL harmonic interference are different than the conditions for Rx harmonic mixing:

  - The UL harmonic interference near-miss MSD test points should be evaluated according to the updated UL/DL fc conditions below:

“The near-miss requirements are only applicable when direct-hit requirements do not apply. These requirements should be verified for downlink channel bandwidths no larger than 10 MHz and with a carrier frequency at ±10+BWChannelHB/2 MHz offset from  in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively”.

  - The Rx harmonic mixing near-miss MSD test points should be evaluated with the following UL/DL fc conditions:

“The requirements should be verified for the lowest NR ARFCN of the affected DL (lower) band and for the

highest NR ARFCN of the UL (higher) band.”

Proposal 4: Guidelines on UL/DL harmonic order

- The Rel-17 conventions remain unchanged, ie. the UL/DL harmonic order is captured using the notation “ULn/DLm” and “direct-hit” or “near-miss”, where “n” and “m” denote respectively the UL and the DL harmonic order.

# Conclusion

This contribution captures the revised guidelines for MSD test points due to uplink harmonic interference and MSD due to receiver harmonic mixing. The intent of this contribution is:

- To ensure that RAN4 proponents evaluate MSDs in accordance to rules used to cleanup the the Rel-18 MSD test points [1..7];

- To provide a preliminary text proposal to the RAN4 draft PRD; and

- To capture these proposals as formal "TP for PRD," once the first draft PRD is made available.

# References

[1]	R4-2414437 CR for EN-DC Harmonic Mixing clean-up PC3, 3GPP TSG-RAN WG4 Meeting #112, Maastricht, Qualcomm France, Skyworks Inc, Nokia.

[2]	R4-2414438 CR for EN-DC Harmonic Mixing clean-up PC2, 3GPP TSG-RAN WG4 Meeting #112, Maastricht, Qualcomm France, Skyworks Inc, Nokia.

[3]	R4-2414439 CR for EN-DC Harmonic Mixing clean-up PC3 PC-5, 3GPP TSG-RAN WG4 Meeting #112, Maastricht, Qualcomm France, Skyworks Inc, Nokia.

[4]	R4-2414440 CR for NR CA Harmonic Mixing clean-up PC2 PC1.5, 3GPP TSG-RAN WG4 Meeting #112, Maastricht, Qualcomm France, Skyworks Inc, Nokia.

[5] 	R4-2414384 CR to TS 38.101-1 Rel-18 NR CA Uplink Harmonic clean-up PC3, 3GPP TSG-RAN WG4 Meeting #112, Maastricht, Skyworks Solutions, Inc., Qualcomm France, Nokia.

[6]	R4-2414385 CR to TS 38.101-1 Rel-18 NR CA Uplink Harmonic clean-up PC2, 3GPP TSG-RAN WG4 Meeting #112, Maastricht, Skyworks Solutions, Inc., Qualcomm France, Nokia.

[7]	R4-2414386 CR to TS 38.101-3 Rel-18 EN-DC Uplink Harmonic clean-up, 3GPP TSG-RAN WG4 Meeting #112, Maastricht, Skyworks Solutions, Inc., Qualcomm France, Nokia.

[8]	R4-2310321 WF on guidelines on checks/specification updates required when BCS4/5 introduces a new smallest or largest CBW, 3GPP TSG-RAN WG4 Meeting #107, Incheon, Skyworks Solutions, Inc.