# 3GPP TSG-RAN WG4 Meeting # 112bis					R4-2416256

# Hefei, China, 14th – 18th  October, 2024

Source:		Ericsson

Title:			General issue for NTN RedCap

Agenda item:		6.26.1

Document for:		Approval

# Introduction

In this paper, we discuss the release independent for RedCap feature to support NTN.

# Discussion

In WID[1], there is no 38.307 as one of the impacted specification so the release independent aspect for RedCap needs to be discussed.

In 38.307, the additional operating band for RedCap is added as release independent feature for TN. As the NTN feature is separate clause in 38.307, the additional operating band for RedCap for NTN should be added under the NTN feature. For example:

# Release independent features for NR UE supporting satellite access operation

## 9.1	Additional NR operating bands for NR NTN in frequency range 1

Requirements for NR UE supporting satellite access operation, for additional NTN operating bands of TS 38.101-5 in Rel-P [8] are introduced via this clause.

Table 9.1-1: Additional NR operating bands for NR NTN in FR1-NTN

| Feature | Duplex-mode | Release independent from | Requirements to be fulfilled (see TS 38.307 of the release in which the band was introduced) |
| --- | --- | --- | --- |
| NTN operating bands | FDD | Rel-17 | Table F.1.1-1, Table F.1.2-1, Table F.1.3-1 |

Table 9.1-2: NR NTN UE power class in FR1-NTN

| Feature | Duplex-mode | Release independent from | Requirements to be fulfilled (see TS 38.307 of the release in which the power class was introduced) |
| --- | --- | --- | --- |
| Power Class 3 | FDD | Rel-17 | Table F.1.1-1 |

Table 9.1-3: NR NTN UE channel bandwidth in FR1-NTN

| Feature | Duplex-mode | Release independent from | Requirements to be fulfilled (see TS 38.307 of the release in which the UE channel bandwidth was introduced) |
| --- | --- | --- | --- |
| UE channel bandwidth | FDD | Rel-17 | Table F.1.1-1 |

RAN1 agrees to introduce specfication change on the RedCap on HD-FDD operation, such agreement are listed below:

Agreement The collision case 3 (Semi-statically configured DL reception collides with semi-statically configured UL transmission) can’t be assumed as error case. When the collision happens at UE side for collision case 3, one of the following options on priority rules is supported. - Option A: the priority of keeping one direction and overriding another direction is based on different use cases (e.g. the collision happening in different channel types or different scheduling cases) without network signalling - It is not precluded that some or all use cases are left to UE implementation regarding whether DL reception or UL transmission is dropped - Note: it is not precluded that the same direction is prioritized for all different use cases - Option B: network indicates the priority configuration to the UE - It is not precluded that some use cases are left to UE implementation regarding whether DL reception or UL transmission is dropped - It is not precluded to define default rules, if necessary

For any possible enhancement to the TA report, it is for further study, and it was captured as follows:

Agreement To mitigate the collisions of case3 and case4, the following TA reporting enhancements for Rel-19 NTN HD-FDD (e)Redcap UE can be further studied: - Finer TA report granularity - Smaller TA offset threshold - Trigger a TA report when the collision is detected at the UE - TA reporting with a new triggering mechanism from gNB - TA drifting rate reporting Note: The benefit, complexity, power consumption and signaling overhead of each scheme is to be further investigated. Note: other solutions can also be studied

As the scope of the RAN1 scope is HD-FDD, in the release independent discussion, it can be discussed whether the HD-FDD should be release independent from Rel-19 and other duplex mode should be release independent from the first release, e.g FDD and TDD from Rel-17, HD-FDD from Rel-19.

## 9.2 Additional operating bands for Redcap and eRedCap for NR frequency range 1

Requirements for a Redcap and eRedcap UE for additional operating bands within FR1 of TS 38.101-5 in Rel-P [2] are introduced via this clause.

Table 9.2 -1: NR NTN RedCap operating band

| Feature | Duplex-mode | Release independent from |  | Requirements to be fulfilled (see TS 38.307 of the release in which the band was introduced) |
| --- | --- | --- | --- | --- |
| Redcap operating band | FDD | Rel-17 |  |  |
|  | TDD | Rel-17 |  |  |
|  | HD-FDD | Rel-19 |  |  |

Table 9.2 -2: NR NTN eRedCap operating band

| Feature | Duplex-mode | Release independent from |  | Requirements to be fulfilled (see TS 38.307 of the release in which the band was introduced) |
| --- | --- | --- | --- | --- |
| eRedcap operating band | FDD | Rel-18 |  |  |
|  | TDD | Rel-18 |  |  |
|  | HD-FDD | Rel-19 |  |  |

Though it could be argued if the 38.307 should be updated when additional new band is added, this indeed cause confusion for RedCap release independent feature update in 38.307 in later release. We suggest RAN4 discuss this and see if the 38.307 should be added in a updated WID.

38.307 should be updated for RedCap NTN as release independent feature (operating band).

# Conclusions

In this contribution, we present our view on release independent RedCap feature with below proposal:

Proposal-1: 38.307 should be updated for RedCap NTN as release independent feature (operating band).

# References

- RP-234078, New WID: Non-Terrestrial Networks (NTN) for NR Phase 3 , Huawei