3GPP TSG-RAN WG4#112-bis	R4-2416264

Hefei, China, October 14th – 18th, 2024

Agenda item:	6.1.1.3.3

Source: 	Qualcomm France

Title: 	6Rx SRS antenna switching requirements

Document for:	Approval

- Introduction

Analysis and considerations 6Rx SRS AS requirements are provided in this contribution.

- WID description and WF in RAN4#111

WID [1] includes the following content for 6Rx:

6Rx for handheld and FWA UE [RAN4, RAN1]

- Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario

  - Example bands: n41, n77/n78, n79, n104

  - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers

  - Specify the Rx requirements including reference sensitivity requirements for support 6Rx

    - Note: the specified requirements can be applicable to both handheld UE and FWA devices

  - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, depending on UE capability

    - RAN1 to study and specify, if necessary, SRS antenna switching to support 3T6R depending on UE capability.

      - Note: RAN1 discussion starts from RAN1#118bis (Oct. 2024)

  - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

General for all the areas

- Specify release independence requirements for the core part in TS 38.307/36.307, if needed and feasible.

We note that for 3T6R is part of the WID, and RAN1 work begins from October 2024. We provide initial analysis already for this meeting even we are not fully sure RAN4 can agree anything yet.

WF [2] agreed in RAN4#111 says the following for SRS antenna switching:

# Topic 2:	SRS antenna switching and ΔTRxSRS

## Sub-topic 2-1:	General considerations for SRS antenna switching and ΔTRxSRS

Issue 2-1-1: Whether to consider an additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104

Way forward: Further discuss the following options.

Option 1: Do not consider an additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104

Option 2: Consider additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104

Issue 2-1-2: Whether to consider separate ∆TRxSRS values when the device is power class 2 in the band and ΔPPowerClass = 0dB and not indicating Tx diversity capability

Agreement: Have additional 3dB for power class 2 when ΔPPowerClass = 0dB and not indicating Tx diversity capability.

## Sub-topic 2-2:	ΔTRxSRS values

Issue 2-2-1: Proposed ∆TRxSRS values

Way Forward: RAN4 agrees to adopt the average values for Bands n41, n77, and n78. For Band n79 and n104, further analysis is needed. Take Option 10 and option 11 as the starting point, not precluding any other numbers.

-	Adopt the average values from companies (updated to include company proposals from RAN4#111) as summarized below and specify the values for ΔTRxSRS for bands whose FUL_high is lower than the FUL_low of n79.

Table: Averaged values based on proposals

|  | Operating Bands | ΔTRxSRS t1r6 (dB) | ΔTRxSRS t2r6 (dB) | ΔTRxSRS t1r6-t2r6 (dB) |
| --- | --- | --- | --- | --- |
| Average | Band n41, n77, n78 | 4.0 | 3.5 | 4.5 |

-	Option 10: Adopt the average values from companies (updated to include company proposals from RAN4#111) as summarized below and specify the values for ΔTRxSRS without considering an additional breakpoint for bands whose FUL_high is lower than the FUL_low of n104.

Table: Averaged values based on proposals

|  | Operating Bands | ΔTRxSRS t1r6 (dB) | ΔTRxSRS t2r6 (dB) | ΔTRxSRS t1r6-t2r6 (dB) |
| --- | --- | --- | --- | --- |
| Average | Band n79, n104 | 5.4 | 4.7 | 5.9 |

-	Option 11: Adopt the average values from companies (updated to include company proposals from RAN4#111) as summarized below and specify the values for ΔTRxSRS considering an additional breakpoint for bands whose FUL_high is lower than the FUL_low of n104.

Table: Averaged values based on proposals

|  | Operating Bands | ΔTRxSRS t1r6 (dB) | ΔTRxSRS t2r6 (dB) | ΔTRxSRS t1r6-t2r6 (dB) |
| --- | --- | --- | --- | --- |
| Average | Band n79 | 5.4 | 4.7 | 5.9 |
|  | Band n104 | 5.7 | 5.0 | 6.3 |

- Discussion

Issue 2-1-1: Whether to consider an additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104

Practically speaking the essential question with regards to this issue is if there is a number which can be used as a compromise for both n79 and n104, or if the discrepancy between the two numbers becomes so large that compromise is not achievable.

We compared and made a rough assessment of the worst case IL of the components in RFFE at n79 and at n104 frequencies. It’s well known that the IL of passives and PCB trace is higher at 7GHz than at 5GHz. Because of that, there is a need to account the fact that ∆TRxSRS should be larger for n104 compared to n79.

Figure 1 Worst case IL estimates for components in SRS path

We investigated different RFFE alternatives for SRS using the numbers shown in figure 1 and concluded that the difference between n79 and n104 IL is at both sides of 1dB, sometimes few tenths smaller sometimes few tenths larger. Looking at the big discrepancy in numbers proposed for n79 and n104 in previous meeting, we believe it is going to be extremely difficult to find a common number for n79 and n104.

Proposal 1: Introduce an additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104 unless a common number can be found for n79 and n104

Issue 2-2-1: Proposed ∆TRxSRS values

If common number is chosen, it must obviously cater both bands. We propose the following numbers. We note that especially with very high frequencies e.g. n104 it becomes more and more challenging to achieve low additional SRS IL for all antennas.

Proposal 2: The following numbers are used in case no additional breakpoint is introduced for 1T6R, 2T6R, and 1T6R-2T6R when the device is power class 3 or power class 5 or power class 1.5 in the band, or when the device is power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating Tx diversity capability.

| Operating Bands | ΔTRxSRS t1r6 (dB) | ΔTRxSRS t2r6 (dB) | ΔTRxSRS t1r6-t2r6 (dB) |
| --- | --- | --- | --- |
| Band n79, n104 | 6.5 | 6 | 7 |

Proposal 3: The following numbers are used in case no additional breakpoint is introduced for 1T6R, 2T6R, and 1T6R-2T6R when the device is power class 2 in the band and ΔPPowerClass = 0 dB and not indicating Tx diversity capability

| Operating Bands | ΔTRxSRS t1r6 (dB) | ΔTRxSRS t2r6 (dB) | ΔTRxSRS t1r6-t2r6 (dB) |
| --- | --- | --- | --- |
| Band n79, n104 | 9.5 | 9 | 10 |

Proposal 4: The following numbers are used in case additional breakpoint is introduced for 1T6R, 2T6R, and 1T6R-2T6R when the device is power class 3 or power class 5 or power class 1.5 in the band, or when the device is power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating Tx diversity capability.

| Operating Bands | ΔTRxSRS t1r6 (dB) | ΔTRxSRS t2r6 (dB) | ΔTRxSRS t1r6-t2r6 (dB) |
| --- | --- | --- | --- |
| Band n79 | 5.5 | 5 | 6 |
| Band n104 | 6.5 | 6 | 7 |

Proposal 5: The following numbers are used in case additional breakpoint is introduced for 1T6R, 2T6R, and 1T6R-2T6R when the device is power class 2 in the band and ΔPPowerClass = 0 dB and not indicating Tx diversity capability

| Operating Bands | ΔTRxSRS t1r6 (dB) | ΔTRxSRS t2r6 (dB) | ΔTRxSRS t1r6-t2r6 (dB) |
| --- | --- | --- | --- |
| Band n79 | 8.5 | 8 | 9 |
| Band n104 | 9.5 | 9 | 10 |

We also evaluated 3T6R, even RAN1 has just started working on these. From consistency point of view, RAN4 should in our view wait RAN1 conclusion until making official agreements on 3T6R even evaluated numbers can be used as a reference.

Proposal 6: Use the following 3T6R numbers as a reference when the device is power class 3 or power class 5 or power class 1.5 in the band, or when the device is power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating Tx diversity capability.

| Operating Bands | ΔTRxSRS t3r6 (dB) | ΔTRxSRS t1r6-t3r6 (dB) | ΔTRxSRS t2r6-t3r6 (dB) | ΔTRxSRS t1r6-t2r6-t3r6 (dB) |
| --- | --- | --- | --- | --- |
| Band n41, n77, n78 | 3 | 4.5 | 4.5 | 6 |
| Band n79 | 4.5 | 6 | 6 | 7.5 |
| Band n104 | 5.5 | 7 | 7 | 8.5 |

Proposal 7: Use the following 3T6R numbers as a reference when the device is power class 2 in the band and ΔPPowerClass = 0 dB and not indicating Tx diversity capability

| Operating Bands | ΔTRxSRS t3r6 (dB) | ΔTRxSRS t1r6-t3r6 (dB) | ΔTRxSRS t2r6-t3r6 (dB) | ΔTRxSRS t1r6-t2r6-t3r6 (dB) |
| --- | --- | --- | --- | --- |
| Band n41, n77, n78 | 6 | 7.5 | 7.5 | 9 |
| Band n79 | 7.5 | 9 | 9 | 10.5 |
| Band n104 | 8.5 | 10 | 10 | 11.5 |

Please note that these values are for the said “PC3” case in the specification, and 3dB is added to said “PC2” case as specified for 2Rx, 4Rx, 8Rx in current specification, according to agreement in ussie 2-1-2 in RAN4#112.

- Conclusion

# 6Rx SRS antenna switching was considered in this contribution with the following proposals:

Proposal 1: Introduce an additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104 unless a common number can be found for n79 and n104

Proposal 2: The following numbers are used in case no additional breakpoint is introduced for 1T6R, 2T6R, and 1T6R-2T6R when the device is power class 3 or power class 5 or power class 1.5 in the band, or when the device is power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating Tx diversity capability.

| Operating Bands | ΔTRxSRS t1r6 (dB) | ΔTRxSRS t2r6 (dB) | ΔTRxSRS t1r6-t2r6 (dB) |
| --- | --- | --- | --- |
| Band n79, n104 | 6.5 | 6 | 7 |

Proposal 3: The following numbers are used in case no additional breakpoint is introduced for 1T6R, 2T6R, and 1T6R-2T6R when the device is power class 2 in the band and ΔPPowerClass = 0 dB and not indicating Tx diversity capability

| Operating Bands | ΔTRxSRS t1r6 (dB) | ΔTRxSRS t2r6 (dB) | ΔTRxSRS t1r6-t2r6 (dB) |
| --- | --- | --- | --- |
| Band n79, n104 | 9.5 | 9 | 10 |

Proposal 4: The following numbers are used in case additional breakpoint is introduced for 1T6R, 2T6R, and 1T6R-2T6R when the device is power class 3 or power class 5 or power class 1.5 in the band, or when the device is power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating Tx diversity capability.

| Operating Bands | ΔTRxSRS t1r6 (dB) | ΔTRxSRS t2r6 (dB) | ΔTRxSRS t1r6-t2r6 (dB) |
| --- | --- | --- | --- |
| Band n79 | 5.5 | 5 | 6 |
| Band n104 | 6.5 | 6 | 7 |

Proposal 5: The following numbers are used in case additional breakpoint is introduced for 1T6R, 2T6R, and 1T6R-2T6R when the device is power class 2 in the band and ΔPPowerClass = 0 dB and not indicating Tx diversity capability

| Operating Bands | ΔTRxSRS t1r6 (dB) | ΔTRxSRS t2r6 (dB) | ΔTRxSRS t1r6-t2r6 (dB) |
| --- | --- | --- | --- |
| Band n79 | 8.5 | 8 | 9 |
| Band n104 | 9.5 | 9 | 10 |

Proposal 6: Use the following 3T6R numbers as a reference when the device is power class 3 or power class 5 or power class 1.5 in the band, or when the device is power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating Tx diversity capability.

| Operating Bands | ΔTRxSRS t3r6 (dB) | ΔTRxSRS t1r6-t3r6 (dB) | ΔTRxSRS t2r6-t3r6 (dB) | ΔTRxSRS t1r6-t2r6-t3r6 (dB) |
| --- | --- | --- | --- | --- |
| Band n41, n77, n78 | 3 | 4.5 | 4.5 | 6 |
| Band n79 | 4.5 | 6 | 6 | 7.5 |
| Band n104 | 5.5 | 7 | 7 | 8.5 |

Proposal 7: Use the following 3T6R numbers as a reference when the device is power class 2 in the band and ΔPPowerClass = 0 dB and not indicating Tx diversity capability

| Operating Bands | ΔTRxSRS t3r6 (dB) | ΔTRxSRS t1r6-t3r6 (dB) | ΔTRxSRS t2r6-t3r6 (dB) | ΔTRxSRS t1r6-t2r6-t3r6 (dB) |
| --- | --- | --- | --- | --- |
| Band n41, n77, n78 | 6 | 7.5 | 7.5 | 9 |
| Band n79 | 7.5 | 9 | 9 | 10.5 |
| Band n104 | 8.5 | 10 | 10 | 11.5 |

- Reference

[1] RP-241656, “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, Huawei

[2] R4-241482, “WF on requirements for 6Rx”, AT&T