TSG-RAN Working Group 4 (Radio) meeting #112-bis	R4-2416266

Hefei, China, October 14-18, 2024

Source:	Ericsson

Title:	Core aspects for OTA spatial emission above the horizon requirement for BS operating in band n104

Agenda item:	6.11.2

Document for:	Approval

# Introduction

One of the objectives of the NR_BS_RF_req_evo WI [1] is introduction of a new requirement for AAS BS operating in band n104 to protect FSS UL service. At the last RAN4 meeting (RAN4#112 in Maastricht) we captured all agreements in a way forward contribution [2] for guidance for this meeting and future meeting.

The proposal to develop a new requirement was part of the BS RF Rel-19 package [3]. The background for this new requirement comes from the WRC-23 final act [4] where a new condition was adopted for IMT base stations to protect FSS UL sharing spectrum in a co-channel situation.

At previous meetings we have provided contributions [5, 6] with information with the intention to stimulate the discussion to further progress the work to define BS RF core requirement and corresponding BS RF conformance test requirement and test method.  In this contribution we present new information on topics to be resolved to be able to capture the technical condition from ITU-R as a new requirement in the BS RF core specification TS 38.104. Companion contributions are also provided: a discussion paper on conformance testing aspects in [7],  draft CRs with specification text for TS 38.104 and TS 38.141-2 in [8] and [9].

In this contribution we propose a requirement definition to be captured in TS 38.104 following the agreements reached in [2].

# Discussion

  - Requirement definition

With relation to core requirements, in the way forward in [2] we have agreed on following:

- Without the extension of EERIP definition for generic use case;

- Expected EIRP (EEIRP) is defined as the average value of the EIRP, with the averaging being performed over supported weighted beamforming directions over time within the BS horizontal and vertical steering range and the averaging being performed over horizontal angles from −180° to +180° and the specified elevation angle range θHL ≤ θ < θHH in Table 9.9.2-1.

- FFS on the definition of beam steering range and further clarify the relationship between beam steering range and beam peak direction sets.

- Keep the above definition in the clause 9.9.

- θHL ≤ θ < θHH and capture this in the abbreviation section.

- To declare the supported mechanical tilt for CAR/beam steering direction sets for BS conformance testing.

- To focus on the single user beamforming for EEIRP measurement.

In ITU-R a new condition for the frequency range 6425 to 7075 MHz was adopted at WRC-23 for protection of FSS UL during co-channel spectrum sharing situations. In Table 2.1-1 the new condition from ITU-R which indicates that EEIRP limit is defined as the average of the average EIRP radiated above the horizon over horizontal angles between -180o to 180o, within specified vertical elevation ranges is captured.

Table 2.1-1: EEIRP limits as function of elevation angular range

| Elevation angular ranges θLi≤θ<θHi (Degrees) | EEIRP (dBm/MHz) |
| --- | --- |
| 0<q<5 | 27 |
| 5<q<10 | 23 |
| 10<q<15 | 19 |
| 15<q<20 | 18 |
| 20<q<30 | 16 |
| 30<q<60 | 15 |
| 60<q<90 | 15 |

Table 2.1-1 describes the limit, constituted by EEIRPi values creating a profile covering i elevation angular regions.

The EEIRP limits are defined with respect to the elevation angle, where θ equal to 0 degrees refers to the angle towards the horizon and 90 degrees refers to an angle straight up into the sky. These limits will directly constrain the power generated above the horizon, towards the sky, and will restrict the AAS BS implementation with respect to maximum power (EIRP) towards UEs within the angular coverage range, antenna array geometry and coverage range capability.

For each range of elevation angles EEIRP is defined as upper hemi-sphere two dimensional average EIRP:

EEIRPi=12πsinθHi-sinθLiθLiθHi-ππEIRPθ,φcosθdθdφ

, where EIRPθ,φ  denotes the average 3-D EIRP spectral density pattern, averaged over K EIRP patterns, and θLi≤θ<θHi  is the elevation angular range, i is the index of the vertical bins θ is the elevation angle and φ is the horizontal angle. Angles in the equation, are expressed in radians.

NOTE: 	Relation to TRP is visible in the case where only one vertical range is considered (where the elevation angle range is defined as θL1 is equal to -π2 and θH1 is equal to π2). In this case EEIRP is equal to TRP.

Figure 2.1-1: Definitions of θL and θH angles relative to the horizontal plane, assumed perpendicular to the antenna array surface (BS placed in the origin).

# Conclusion

Based on the information in this contribution we propose the following:

Proposal 1: Agreed EEIRP definition is tailored to this requirement, and it needs to contain complete information about the conditions under this parameter is calculated using averaging over beams, azimuth and elevation, as described in the formula above.

Note: We should not forget that EEIRP, EIRP and also TRP are ways to express radiated power and can be used for both wanted and unwanted signals, in different BS RF requirements.

# References

[1]	RP-240829, “New WID: NR base station (BS) RF requirement evolution for FR1/FR2 and testing”, ZTE

[2]	R4-2413509, “WF for [112][305] NR_BS_RF”, RAN4#112 , ZTE

[3]	R4-2402475, “NR BS RF enhancements in Rel-19”, Ericsson

[4] 	“WRC-23 Final Acts”, ITU-R

[5]	R4-2404757, “On introduction of protection requirement for FSS UL service operating within 6425 to 7125 MHz”, Ericsson

[6]	R4-2409118, “On introduction of OTA spatial emission above the horizon requirement for band n104”, Ericsson

[7]	R4-2416267, “Conformance test aspects for the requirement on OTA spatial emission above the horizon for BS operating in band n104”, Ericsson

[8]	R4-2416268, “Draft CR to TS 38.104: Addition of spatial emission requirement to protect FSS UL within band n104”, Ericsson

[9]	R4-2416269 “ Draft CR to TS 38.141-2: Addition of spatial emission requirement to protect FSS UL within band n104”, Ericsson