3GPP TSG-RAN WG4 Meeting #112bis	R4-2416272

Hefei, China, 14th – 18th October, 2024

Agenda Item:	6.16.1

Source:	Ericsson

Title:	On general issues for MMSE-IRC receiver under interference

Document for:	Discussion

# 1	Introduction

In RAN#104 meeting, the revised WID on NR demodulation performance: Phase 5 was approved [1]. According to the WID, the following are objectives for UE and BS parts:

- Define the following UE performance requirements for 8Rx CPE/FWA/vehicle/industrial devices: - PDSCH demodulation and CQI reporting requirements with inter-cell interference with MMSE-IRC receiver - Reuse the Rel-17 interference model for 2/4Rx UE as a baseline - PDSCH demodulation requirements with intra-cell inter-user interference with MMSE-IRC receiver - Reuse the existing interference model for 2/4Rx UE as a baseline - Define FR1 PUSCH performance requirements with inter-cell interference in homogeneous and heterogeneous deployment based on the following assumptions: - Receiver type: MMSE-IRC receiver - Scenarios: - FDD synchronous deployments - TDD synchronous deployments with aligned UL:DL configurations among cells - Slot-based transmission and aligned SCS among cells - Reuse LTE based interference profile in TR 36.884 as a starting point. Other interference profiles are not precluded.

In this contribution, we provided our view on general issues for UE and BS demodulation requirement with interference.

# 2	Discussion

In this section, we discuss the general issues related to the UE performance requirements for 8Rx with interference and BS performance requirements with IRC.

## 2.1	UE performance requirements for 8Rx

During the last meeting of RAN4 #112 in Maastricht, companies have reached agreements on the following aspects related to the test scope[2]:

Test scope - RAN4 to introduce UE demodulation requirements for both TDD and FDD - Not to cover PDSCH absolute physical layer throughput requirements for 8Rx - Candidate options on whether to cover CQI requirements for ICI scenario: - Option 1: CQI reporting requirements under ICI should be cover for 8Rx - Option 2: No need to cover such requirements

It’s undetermined whether to introduce CQI tests for ICI scenario. As indicated in the objective of WID, we think it’s also necessary to define CQI reporting requirement for inter-cell interfering cancellation using 8Rx MMSE-IRC receiver. With this new requirement, the UE’s ability of tracking the channel variations and selecting the largest transport format possible using 8Rx reception under the inter-cell interference condition can be tested.

In Rel-17, RAN4 defined CQI reporting test in fading condition for inter-cell interference scenario for 2Rx/4Rx UEs with SINR=-2dB. We therefore propose to define CQI reporting test in fading condition for 8Rx UE as well.

Proposal 1: Define CQI reporting requirement under ICI scenarios with 8Rx reception

Regarding the receiver assumption, we propose to first evaluate the potential performance gain between MMSE and MMSE-IRC with 8Rx reception, if there is only negligible gap exist then there is no need to define two different sets of requirements.

Proposal 2: RAN4 first evaluate the performance using both baseline receiver and the simplified receiver.

As for the PDSCH mapping type, there are two options shown below[2]:

PDSCH mapping type - Candidate options: - Option 1: Include both PDSCH type A and type B requirements. - Option 2: Reuse the same configuration for Rel-17 2/4Rx, i.e., PDSCH mapping type A only

We propose to reuse the same configuration for Rel-17 Performance enhancement (inter-cell interference for 2/4Rx) to consider PDSCH mapping type A only.

Proposal 3: Reuse the same configuration for Rel-17 Performance enhancement (inter-cell interference for 2/4Rx) to consider PDSCH mapping type A only

## 2.2	BS performance requirements

Test scenarios

- Proposals:

  - Proposal 1: Cover HomNet and HetNet scenarios, i.e., DIP set1 and set2 in TS36.104

  - Proposal 2: Consider the scenario with different TDD patterns between BS and discuss how to model the corresponding interference profile

Network type

- Only synchronous scenario

There is an open issue on test scenario[2] for NR MMSE-IRC BS requirements. In corresponding LTE BS requirements, both HomNet and HetNet are considered and also both the synchronization and asynchronization networks are considered. But in Rel-19 WID, it clearly limit the scope to synchronization network deployment only. It is strange to discuss asynchronization network scenario in RAN4 meeting. Furthermore, there is agreement in interference model discussion that only synchronous scenario is considered. In that case, it is not proper to consider network deployment with different TDD pattern in Rel-19 discussion.

Proposal 4: Don’t consider network deployment with different TDD patterns for Rel-19 NR MMSE-IRC BS demodulation requirements.

As for HomNet and HetNet, the interference profile could be different and HetNet could have higher level for the strongest interferer but lower level for the 2nd strongest interferer. Considering both deployment scenarios can keep a good requirement coverage, and extra test effort can be avoided by introducing proper applicability rules. In LTE requirements, the HomNet requirements won’t be applied to Local Area BS and Home BS by adding a note in the requirement table. In NR requirements, the similar approach could be followed but the applicability rule could depend on manufactory declaration on BS class (i.e. D.2 in 38.141-1 Table 4.6).

Proposal 5: Consider both HomNet and HetNet deployment requirements and introduce applicability rules based on BS class (i.e. D.2 in 38.141-1 Table 4.6).

# 4	Summary

In this contribution, we shared our views on how to introduce PDSCH and CQI reporting requirement for two scenarios. Our proposals are summarized as follows:

Proposal 1: Define CQI reporting requirement under ICI scenarios with 8Rx reception

Proposal 2: RAN4 first evaluate the performance using both baseline receiver and the simplified receiver.

Proposal 3: Reuse the same configuration for Rel-17 Performance enhancement (inter-cell interference for 2/4Rx) to consider PDSCH mapping type A only

Proposal 4: Don’t consider network deployment with different TDD patterns for Rel-19 NR MMSE-IRC BS demodulation requirements.

Proposal 5: Consider both HomNet and HetNet deployment requirements and introduce applicability rules based on BS class (i.e. D.2 in 38.141-1 Table 4.6).

# References

[1]	RP-241297: Revised WID on NR demodulation performance: Phase 5, China Telecom, NTT DOCOMO, RAN#104, Shanghai, China

[2]	RP-241297: WF on NR demodulation performance: Phase 5, China Telecom, RAN4 #112, Maastricht, Netherlands