3GPP TSG-RAN WG4 Meeting #112bis	R4-2416274

Hefei, China, 14th – 18th October, 2024

Agenda Item:	6.16.2

Source:	Ericsson

Title:	Simulation results for MMSE-IRC UE demodulation

Document for:	Information

# 1	Introduction

In RAN#104 meeting, the revised WID on NR demodulation performance: Phase 5 was approved [1]. According to the WID, the following are objectives for UE and BS parts:

- Define the following UE performance requirements for 8Rx CPE/FWA/vehicle/industrial devices: - PDSCH demodulation and CQI reporting requirements with inter-cell interference with MMSE-IRC receiver - Reuse the Rel-17 interference model for 2/4Rx UE as a baseline - PDSCH demodulation requirements with intra-cell inter-user interference with MMSE-IRC receiver - Reuse the existing interference model for 2/4Rx UE as a baseline - Define FR1 PUSCH performance requirements with inter-cell interference in homogeneous and heterogeneous deployment based on the following assumptions: - Receiver type: MMSE-IRC receiver - Scenarios: - FDD synchronous deployments - TDD synchronous deployments with aligned UL:DL configurations among cells - Slot-based transmission and aligned SCS among cells - Reuse LTE based interference profile in TR 36.884 as a starting point. Other interference profiles are not precluded.

In this contribution, we shared our simulation assumptions and results for both inter-cell and intra-cell interference scenarios.

# 2	PDSCH requirements with Inter-cell interference

In this section, we first expose the simulation assumption that we used for evaluation, then we shared our simulation results comparing the performance with baseline receiver and MMSE-IRC receiver.

## 2.1	Simulation assumption

In our initial evaluations, we mostly reuse the parameter assumption listed in the table 5.2.2.1.15-2 in TS38.101-4 with slightly extension for 8Rx reception:

Table 2.1-1: Test parameters

| Parameter |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 3 |
|  |  |  | Enabled | Enabled | Enabled for test 1-1~1-4, Disable for test 2-1, 2-2 |
| Duplex mode |  |  | FDD |  |  |
| Active DL BWP index |  |  | 1 |  |  |
| Physical cell ID |  |  | 0 | 1 | 2 |
| Transmission rank |  |  | 1 | Random rank with 70% and 30% probability for rank 1 and rank 2 | Random rank with 70% and 30% probability for rank 1 and rank 2 for Test 1-1~1-4 N/A for Test 2-1, 2-2 |
| Time offset to Cell1 |  | us | N/A | 3 | -1 |
| Frequency offset to Cell 1 |  | Hz | N/A | 300 | -100 |
| Interference Model |  |  | N/A | As specified in B.6.2 |  |
| INR (Note 2) |  | dB | N/A | 7.77 for Test 1-1~1-4 7.58 for Test 2-1, 2-2 | 2.29 for Test 1-1~1-4 N/A for Test 2-1, 2-2 |
| SSB configuration | SSB position in burst |  | First SSB in Slot #0 | 1st SSB in Slot#0 for Test 1-1~1-4 2nd SSB in Slot #0 for Test 2-1, 2-2 | 1st SSB in Slot#0 for Test 1-1~1-4 N/A for Test 2-1, 2-2 |
|  | SSB periodicity | ms | 20 | 20 | 20 |
| PDSCH configuration | Mapping type |  | Type A |  |  |
|  | k0 |  | 0 |  |  |
|  | Starting symbol (S) |  | 2 |  |  |
|  | Length (L) |  | 12 |  |  |
|  | PDSCH aggregation factor |  | 1 |  |  |
|  | PRB bundling type |  | Static |  |  |
|  | PRB bundling size |  | 2 |  |  |
|  | Resource allocation type |  | Type 0 |  |  |
|  | RBG size |  | Config2 |  |  |
|  | VRB-to-PRB mapping type |  | Non-interleaved |  |  |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |  |  |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |  |  |
|  | Number of additional DMRS |  | 1 |  |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |  |  |
| Number of HARQ Processes |  |  | 4 |  |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 |  |  |
| Note1: Cell 1 is the serving cell; Cells 2, 3 are interfering cells Note 2: INR is defined in Annex B.6.1 |  |  |  |  |  |

## 2.2	Simulation results

For the simulation results below, we only used the baseline receiver.

Table 2.2-1 PDSCH simulation results for 2 and 1 interference cell

| Test num | Rank | Bandwidth (MHz) / Subcarrier spacing (kHz) | MCS (Table 1) | Propagation condition | Correlation matrix and antenna configuration | Reference value 70% max Tput |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | MMSE-IRC SNR(dB) | MMSE SNR(dB) | Gain |
| 2 interference cells |  |  |  |  |  |  |  |  |
| 1 | 2 | 10 / 15 | MCS13, 16QAM, 0.48 | TDA30-10 | 2x8, ULA Low | 3.62 | 10.29 | 6.67 |
| 2 | 4 | 10 / 15 | MCS13, 16QAM, 0.48 | TDA30-10 | 4x8, ULA Low | 9.22 | 14.43 | 5.21 |
| 3 | 8 | 10 / 15 | MCS13, 16QAM, 0.48 | TDA30-10 | 8x8, ULA Low | 22.35 | 24.75 | 2.4 |
| 4 | 2 | 10 / 15 | MCS13, 16QAM, 0.48 | TDLC300-100 | 2x8, ULA Low | 5.78 | 11.13 | 5.35 |
| 5 | 4 | 10 / 15 | MCS13, 16QAM, 0.48 | TDLC300-100 | 4x8, ULA Low | 11.84 | 16.02 | 4.18 |
| 6 | 8 | 10 / 15 | MCS13, 16QAM, 0.48 | TDLC300-100 | 8x8, ULA Low | 24.68 | 25.92 | 1.24 |
| 7 | 2 | 10 / 15 | MCS19 | TDLC300-100 | 2x8, ULA medium B | 28.77 | N/A | INF |
| 8 | 4 | 10 / 15 | MCS17 | TDA30-10 | 4x8, ULA low | 12.79 | 18.07 | 5.28 |
| 9 | 8 | 10 / 15 | MCS17 | TDA30-10 | 8x8, ULA low | N/A | N/A | N/A |
| 1 interference cell |  |  |  |  |  |  |  |  |
| 10 | 2 | 10 / 15 | MCS13 | TDLA30-10 | 2x8, ULA Low | 2.79 | 7.84 | 5.05 |
| 11 | 4 | 10 / 15 | MCS13 | TDLA30-10 | 4x8, ULA Low | 7.3 | 12.79 | 5.48 |
| 12 | 8 | 10 / 15 | MCS13 | TDLA30-10 | 8x8, ULA Low | 19.6 | 23.36 | 3.73 |
| 13 | 2 | 10 / 15 | MCS19 | TDLA30-10 | 2x8, ULA Low | 7.5 | 12.35 | 4.83 |
| 14 | 4 | 10 / 15 | MCS17 | TDLA30-10 | 4x8, ULA Low | 11.0 | 16.57 | 5.54 |
| 15 | 8 | 10 / 15 | MCS17 | TDLA30-10 | 8x8, ULA Low | 27.2 | 29.19 | 2.04 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: Bandwidth/ Sub carrier spacing, Propagation Condition, Correlation matrix and antenna configuration parameters apply for each of Cell 1, Cell 2 and Cell 3. |  |  |  |  |  |  |  |  |

# 3	PDSCH requirements with Intra-cell inter-user interference

In this section, we first expose the simulation assumption that we used for evaluation, then we shared our simulation results comparing the performance with baseline receiver and MMSE-IRC receiver.

## 3.1	Simulation assumption

Table 3.1-1: Test parameters

| Parameter |  | Unit | Target UE | Co-scheduled UE |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  | FDD |  |
| Active DL BWP index |  |  | 1 |  |
| PDSCH configuration | Mapping type |  | Type A |  |
|  | k0 |  | 0 |  |
|  | Starting symbol (S) |  | 2 |  |
|  | Length (L) |  | 12 |  |
|  | PDSCH aggregation factor |  | 1 |  |
|  | PRB bundling type |  | Static |  |
|  | PRB bundling size |  | 2 |  |
|  | Resource allocation type |  | Type 0 |  |
|  | RBG size |  | Config2 |  |
|  | VRB-to-PRB mapping type |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |  |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |  |
|  | Number of additional DMRS |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |  |
|  | Antenna ports indexes |  | 1000 | 1001 |
|  | Number of PDSCH DMRS CDM group(s) without data |  | 1 | 1 |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Single Panel Type I, Randomized precoder selection for every PRB bundle and updated per slot, with equal probability of each applicable i1/i2 combination or codebook Index, chosen from section 5.2.2.2.1 of TS 38.214 [12]. | Select the precoder to ensure any column of precoder is orthogonal to any column of precoder for the target PDSCH |
| MU-MIMO Beamforming Model |  |  | As specified in B.4.2 |  |
| Number of HARQ Processes |  |  | 4 | N/A |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 | N/A |
| Note 1: The DMRS scrambling ID is same for both target UE and Co-scheduled UE. |  |  |  |  |

## 3.2	Simulation results

For the simulation results below, we only used the baseline receiver.

Table 3.2-1 PDSCH simulation results

| Test num | Rank combination (target+ co-scheduled) | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate |  | Propagation condition | Correlation matrix and antenna configuration | Reference value (70% max Tput) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Target UE | Co-scheduled UE |  |  | MMSE-IRC SNR(dB) | MMSE SNR(dB) | Gain |
| 1 | 1+1 | 10 / 15 | 16QAM, 0.48 | Random 16QAM symbols | TDLC300-100 | 2x8, ULA Low | 2.61 | 3.69 | 1.08 |
| 2 | 1+1 | 10 / 15 | 16QAM, 0.48 | Random 16QAM symbols | TDLA30-10 | 2x8, ULA Low | 2.13 | 3.08 | 0.95 |
| 3 | 2+2 | 10 / 15 | 16QAM, 0.48 | Random 16QAM symbols | TDLC300-100 | 4x8, ULA Low | 5.64 | 13.03 | 7.39 |
| 4 | 4+4 | 10 / 15 | 16QAM, 0.48 | Random 16QAM symbols | TDLC300-100 | 8x8, ULA Low | 13.9 | N/A | INF |
| 5 | 1+1 | 10 / 15 | MCS19 | Random 16QAM symbols | TDLC300-100 | 2x8, ULA Medium B | 13.41 | NA | INF |
| 6 | 1+1 | 10 / 15 | MCS19 | Random 16QAM symbols | TDLC300-100 | 2x8, ULA Low | 7.0 | 12.1 | 5.11 |
| 7 | 1+1 | 10 / 15 | MCS17 | Random 16QAM symbols | TDLA30-10 | 2x8, ULA Low | 5.3 | 8.6 | 3.29 |
| 8 | 2+2 | 10 / 15 | MCS17 | Random 16QAM symbols | TDLA30-10 | 4x8, ULA Low | 9.2 | NA | INF |
| 9 | 4+4 | 10 / 15 | MCS17 | Random 16QAM symbols | TDLA30-10 | 8x8, ULA Low | 19.2 | NA | INF |

# 4	Summary

In this contribution, we shared the PDSCH simulation assumptions and results for both inter-cell and intra-cell interference scenarios.

# References

[1]	RP-241297: Revised WID on NR demodulation performance: Phase 5, China Telecom, NTT DOCOMO, RAN#104, Shanghai, China

[2]	RP-241297: WF on NR demodulation performance: Phase 5, China Telecom, RAN4 #112, Maastricht, Netherlands