3GPP TSG-RAN WG4 Meeting #112-bis 	R4-2416277

Hefei, China, Oct 14 – 18, 2024

Source: 	Huawei, HiSilicon

Title: 	Further discussion on Expected EIRP test

Agenda Item:	6.11.4

Document for:	Approval

1 Introduction

In RAN4#112 the WF [1] was approved which had a number of open issues regarding the conformance testing of the EEIRP requirement.

Issue 4-2: Test vectors for EEIRP mask

Issue 4-3: Measurement grid over elevation angle and azimuth angle

Issue 4-4: RF channels for the conformance testing

Issue 4-6: Test procedures

Issue 4-7: Measurement uncertainty and CI

In addition there are a number of other open issues which require disunion an are covered in this paper:

- Worst case steering range and mechanical tilt

- Test signal generation

- Test beam directions

2 Discussion

2.1	Worst case electrical steering range and mechanical tilt

For EEIRP it is necessary to specify the Mechanical tilt in addition to the electrical steering range (peak directions sets). As the EEIRP requirement is related to the horizon it is of course the case that the mechanical tilt will affect it directly.

It is also the case that as the electrical steering range is variable it may be adjusted depending on the selected mechanical tilt.

For test purposes this has the potential to generate a very large number of cases, as we do for other requirements it is necessary to identify the worst case scenarios and describe these as the minimum compliance test cases.

For this discussion we have used ANT4, it uses sub-arrays and is typical of the antennas which will be used in this band.

Table: 2.1-1 Test antennas description

|  |  | Antenna | Unit |
| --- | --- | --- | --- |
|  |  | 4 |  |
| elevation | No elements/sub-arrays | 8 |  |
|  | elements per sub-array | 3 |  |
|  | element separation | 0.7 | λ |
| Azimuth | No elements | 16 |  |
|  | element separation | 0.5 | λ |
| Directivity (zero steer) |  | 32.4 | dBi |
| vertical BW |  | 3 | deg |
| horizontal BW |  | 6.4 | deg |

Figure 2.1-1 ANT4 vertical steering extreme

The useful elevation steering range of this antenna is approximately +/-10°, at this point the gain has dropped by just over 1dB and the main grating lobe is within 5dB of the wanted lobe (what the max steering is, is dependent on vendor declarations of course but this seems like a reasonable maximum).

With a maximum electrical steering range of +/-10° a minimum mechanical down tilt where the EEIRP requirement is met must also be declared.

For this example the minimum mechanical down tilt is 13°.

With the above settings an output power of 20dBm/MHz is required to pass the EEIRP requirement. Azimuth steering range is +/-60°.

Figure 2.1-2 ANT4 EEIRP, 13° Mechanical tilt, +/-10° Electrical steering

In this scenario the many of the angular bins pass the specification with large margins. The minimum margin is in the 10°-15° bin where there is only 1.5dB margin over the specification.

If we consider this the maximum steering range for the minimum mechanical down tilt it is important we confirm that other conditions that the ANT could conceivably be used for have a greater margin over the specification than this on.

Figure 2.1-3 ANT4 EEIRP, minimum margin vs steering range (fixed 13° mechanical tilt)

It can be seen that the minimum margin decreases as the steering range increases, as such testing only the maximum steering range is a valid worst case.

Observation 1: The minimum margin over the specification decreases as the steering range increases.

Proposal 1: The maximum steering range is a valid worst case.

It is also true that if a smaller mechanical tilt is required then this may result in a worse result and a failure of the specification.  If the steering range is help constant and the mechanical tilt is varied we see the results in figure 2.1-4.

Figure 2.1-4 ANT4 EEIRP, minimum margin vs mechanical tilt (+/-10° electrical steering)

Once again perhaps as expected we see that the margin reduces as the mechanical tilt deceases (i.e. closer to the horizon).

Observation 2: As mechanical tilt decreases the minimum EEIRP margin reduces.

Proposal 2: Minimum mechanical tilt represents a worst case.

Combining P1 & P2 the worst case can be identified as:

Proposal 3: The worst case EEIRP condition is the minimum mechanical down tilt and the maximum electrical steering range.

The optimum steering range and mechanical tilt for any antenna is declared by the vendor. However it might be the case that an antenna which has for example; a declared 13° mechanical tilt with +/-10° electrical steering range may be required to operate with a smaller mechanical tilt, in such as case restricting the electrical steering range may be required so that the EEIRP requirement is met.

This is a similar situation as the relationship between supported number of carriers and Rated maximum output power, for example:

4.8.3	TC3: UTRA and E-UTRA multi RAT operation

The purpose of TC3 is to test UTRA and E-UTRA multi-RAT aspects.

If the rated total output power and total number of supported carriers are not simultaneously supported in Multi-RAT operations, two instances of TC3 shall be generated using the following values for rated total output power and the total number of supported carriers:

1)	The rated total output power and the reduced number of supported carriers at the rated total output power in Multi-RAT operations

2)	The reduced total output power at the total number of supported carriers in Multi-RAT operations and the total number of supported carriers.

Tests that use TC3 shall be performed using both instances 1) and 2) of TC3.

Applying a similar approach to the mechanical tilt and the electrical steering range the worst case(s) can be declared as follows:

The minimum mechanical down tilt and the maximum electrical steering range shall be declared.

If the maximum steering range and the minimum mechanical down tilt are not simultaneously supported, the manufacturer shall declare and test the following two instances:

  - Minimum mechanical down tilt and the reduced electrical steering range

  - Maximum electrical steering range and increased mechanical down tilt

Figure 2-3 shows that reducing electrical steering range increases the margin over the specification, Figure 2-4 shows that increasing mechanical tilt also increases the margin as such these 2 corner cases should be sufficient to identify the worst cases. In Figure 2-5 we can see that if the electrical steering range is reduced the same relationship holds between mechanical tilt and minimum margin. The 2 identified worst cases are therefore valid.

Figure 2.1-5 ANT4 EEIRP, minimum margin vs mechanical tilt (+/-5° electrical steering)

Proposal 4: If the maximum steering range and the minimum mechanical down tilt are not simultaneously supported, the manufacturer shall declare and test the following two instances:

  - Minimum mechanical down tilt and the reduced electrical steering range

  - Maximum electrical steering range and increased mechanical down tilt

2.2	Test signal

The EEIRP requirement is expressed as a power spectral density (PSD) figure of dBm/MHz. As the output signal of the BS is a much greater bandwidth than 1 MHz some discussion is needed to identify the best output signal and method of doing any band width conversions.

The EEIRP requirement is somewhat unique in that it appears like a spurious emissions requirement however it is measuring the wanted signal albeit in specific directions away from the intended direction of transmission.

Wanted signal requirements are generally expressed as total power and spurious emissions requirements are generally expressed as PSD.

For spurious emissions where the interference to other systems is critical and we don’t know the BW of those other systems it makes sense to specify a PSD figure. Also as the source of spurious signals within the BS is unknown (harmonics of clocks, LO’s, non-linearities etc) their BW is unknown and hence converting between the emission BW and another is not straight forward.

Observation 1: PSD makes sense as a metric for spurious signals as we don’t know the victim channel BW

Observation 2: Spurious emissions from the BS are from spurious effects and have unknown BW

For EEIRP we consider interference to anther system and hence PSD makes sense, however we know exactly the source of the interfering signal as it is the wanted signal, as such converting between PSD and total power is straight forward.

Observation 3: EEIRP emissions are based on the wanted signal as such BW conversion is straight forward.

The primary requirement therefore for any test signal used to test EEIRP is that it is the highest PSD the BS is capable of. Adjacent to that is that the PSD would be flat over the test signal BW.

Proposal 1: The Test signal must be at the highest supported PSD with the PSD flat over the test signal BW.

Currently the total carrier power and the total operating power are declared independent of the PSD.

| D.37 | The rated carrier OTA BS power, Prated,c,TRP | Prated,c,TRP is declared as TRP OTA power per carrier, declared per supported operating band. (Note 12, 14, 18, 20) |
| --- | --- | --- |

If it is necessary to impose PSD limitation on the BS in order to meet the EEIRP requirement then an additional requirement for the frequencies where EEIRP applies must be made. An additional declaration for EEIRP PSD as follows:

| D.xx | EEIRP rated carrier OTA BS PSD, Prated,c,PSD | Prated,c,PSD is declared as TRP OTA PSD per carrier, declared for frequencies where the EEIRP requirement applies. |
| --- | --- | --- |

Clearly if this value is greater (or equal to) the PSD when Prated,c,TRP is applied to the lowest CBW then the declaration is not required.

Proposal 2: An addition declaration for EEIRP PSD is made.

To test The EEIRP at the declared PSD level we suggest that the 20MHz channel currently defined is suitable.

38.141-2, Table 4.7.2-1: Signal to be used to build NR TCs

| Operating Band characteristics |  | FDL_high – FDL_low <100 MHz | FDL_high – FDL_low ≥ 100 MHz |
| --- | --- | --- | --- |
| TC signal | BWchannel | 5 MHz (Note 1) | 20 MHz (Note 1) |
| characteristics | Subcarrier spacing | Smallest supported subcarrier spacing |  |
| NOTE 1: If this channel bandwidth is not supported, the narrowest supported channel bandwidth shall be used. |  |  |  |

This seems an appropriate starting point for the EEIRP test signal and it is already defined and widely used.

If the declared PSD cannot be achieved the 20MHz channel for some reason there is already an exception in Note 1 which already covers the case where 20MHz is not supported.

Proposal 3: The 20MHz channels defined already in 38.141-2 table 4.7.2-1 can be used for testing the declared PSD.

Lastly the issue of multi-carrier, if the BS output power is fixed then adding more carriers will of course reduce the PSD of each of the carriers, as such multi-carrier is not a worst case.

Observation 4: Addition of carriers will reduce the PSD of each.

If each carrier has equal power (so PSD is not affected) as the measurement is made on the wanted channel frequency the other channel will have no effect on the result. Multicarrier requirements are needed when potential non-linearities may affect the result. In normal spurious emissions where unwanted products of the 2 channels may be generated this is important. However the addition of another channel and any potential non-linearities will not significantly affect the wanted signal or the wanted signal beam shape. As such multi-carrier serves no purpose for the EEIRP text.

Observation 5: Unwanted emissions at mixing frequencies of multicarrier signals will have no significant effect the beam shape of the wanted signal and hence have no impact on EEIRP.

As such it is not necessary to test a multi-carrier case, single carrier is sufficient.

Proposal 4: Single carrier test only is required.

2.3	Beam directions set

In the last meeting a number of test beam directions sets were proposed and compared to reference power distributions, it was agreed further study was needed.

We have previously stated that the test beam direction set should be a compromise between accuracy (i.e. how well the EEIRP power distribution matches the reference case) and test time. However we should also perhaps consider the applicability of the test beam directions sets to the declared peak directions set

In most cases the peak directions sets (steering ranges) used were rectangular, we raised the point that it was more likely in reality that the peak direction sets would be elliptical in nature. This makes more sense than a rectangular peak direction set as the declaration would be of a contour of equal EIRP, in addition in system analysis it seems the distribution of beam directions tends to be naturally more elliptical in nature.

Figure 2.3-1: Simulated beam directions in a cell

Whilst rectangular declarations cannot be excluded it makes sense to identify test beam direction sets which are suitable for elliptical steering ranges.

Observation 8: Elliptical steering ranges are more likely than rectangular steering ranges.

Looking at some of the test beam direction sets proposed so far:

Figure 2.3-2: Proposed test beam direction sets

The 3x3 and 3x4 patterns are within the elliptic peak direction set whereas the 5x5 variants have points outside, it would be expected that these test beam directions set would be less accurate for the elliptical case.

Table 2.3-1. Comparison of EEIRP accuracy for rectangular and elliptical peak directions sets

| Antenna | 3x3 |  | 3x4 |  | 5x5(EB) |  | 5x5 type A |  | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Rect | Ellipse | Rect | Ellipse | Rect | Ellipse | Rect | Ellipse |  |
| ANT1 | 0.70 | 0.49 | 0.36 | 0.76 | 0.63 | 2.15 | 0.21 | 0.82 | dB |
| ANT2 | 0.54 | 0.45 | 0.79 | 0.37 | 0.63 | 2.11 | 0.10 | 0.80 | dB |
| ANT4 | 1.48 | 0.78 | 1.29 | 1.15 | 0.89 | 2.85 | 0.46 | 1.77 | dB |
| ANT5 | 1.32 | 0.64 | 0.64 | 1.04 | 0.63 | 3.04 | 1.65 | 0.83 | dB |
| RMS | 1.09 | 0.60 | 0.84 | 0.89 | 0.70 | 2.57 | 0.86 | 1.13 | dB |

It can be seen that the test beam direction sets with the inset directions perform better with the elliptical assumption that they do with the rectangular.

From this analysis it seems 5x5(EB) can be removed from consideration. It is not clear that of the remaining sets that any is significantly better than the other. As such we prefer to keep the number of directions to a minimum so that test tie is kept to a minimum.

Proposal 5: the 3x3 test beam direction set is used

2.4	Other open issues

We have provided analysis and our views on the other open issues in previous papers [2], our views on the other open issue from the WF are as follows:

Issue 4-3: Measurement grid over elevation angle and azimuth angle

Step size vs EEIRP accuracy sweeps were shown in [2], the results clearly howed:

Proposal 6: Elevation accuracy is not related to main beam width, results show that an elevation step size of 1° is optimum

Proposal 7: Azimuth accuracy is related to main bema width (similarly to TRP), azimuth step size should be at least half the main beam azimuth beam with

Issue 4-4: RF channels for the conformance testing

When the EEIRP for an array designed for M is calculated at frequency offsets of +/-5% (the EEIRP band has a 10% band width) the differences between the results are spurious in nature , it is not clear that the resulsts are better or worse at B or T.

Looking at the same case where the array is designed for T or B and examining at the other 2 frequency points has a similar result. It does not seem that any frequency is significantly worse than any other.

As the requirement has many averaging steps and as there is no clear worst case frequency the middle is the most obvious choice.

Proposal 8: Use RF channel M for test procedure.

Issue 4-7: Measurement uncertainty and CI

In RAN4#112 in paper [2] we gave a preliminary MU budget and introduced a EEIRP summation term analogous to TRP summation error with the same value (0.75dB).

This should be considered when the MU value is decided.

On the issue of the relationship between MU and CI, our understanding is that measurement uncertainty values are provided with a confidence interval (95%) and a probability distribution (e.g. Gaussian, U shaped, rectangular). The uncertainty is converted to an equivalent gaussian figure so that all the uncertainties can be added and then an overall MU is given with a 95% confidence.

3	Summary

3.1	Worst case electrical steering range and mechanical tilt

In this paper we have looked at the worst case conditions for the EEIRP test. The relationship between the minimum margin over the specification and the electrical steering range and the mechanical down tilt have been examined and the following observations have been made:

Observation 1: The minimum margin over the specification decreases as the steering range increases.

Proposal 1: The maximum steering range is a valid worst case.

Observation 2: As mechanical tilt decreases the minimum EEIRP margin reduces.

Proposal 2: Minimum mechanical tilt represents a worst case.

Proposal 3: The worst case EEIRP condition is the minimum mechanical down tilt and the maximum electrical steering range.

Proposal 4: If the maximum steering range and the minimum mechanical down tilt are not simultaneously supported, the manufacturer shall declare and test the following two instances:

  - Minimum mechanical down tilt and the reduced electrical steering range

  - Maximum electrical steering range and increased mechanical down tilt

3.2	Test signal

In this paper we have looked at the test signal required to test the EEIRP requirement. We have made the following observations and proposals.

Observation 3: PSD makes sense as a metric for spurious signals as we don’t know the victim channel BW

Observation 4: Spurious emissions from the BS are from spurious effects and have unknown BW

Observation 5: EEIRP emissions are based on the wanted signal as such BW conversion is straight forward.

Proposal 5: The Test signal must be at the highest supported PSD (declared) with the PSD flat over the test signal BW.

Proposal 6: An addition declaration for EEIRP PSD is made.

Proposal 7: The 20MH channels defined already in 38.141-2 table 4.7.2-1 can be used for testing the declared PSD.

Observation 6: Addition of carriers will reduce the PSD of each.

Observation 7: Unwanted emissions at mixing frequencies of multicarrier signals will have no significant effect the beam shape of the wanted signal and hence have no impact on EEIRP.

Proposal 8: Single carrier test only is required.

3.3	Beam directions sets

The likely steering range (OTA peak directions set has been discussed with the following observation:

Observation 8: Elliptical steering ranges are more likely than rectangular steering ranges.

The test beam direction sets have been examined again looking at both a rectangular and an elliptical steering range. The result indicate that for 3x3, 3x4 nd 5x5 type A have similarly acceptable results for both steering range types. As such the set with the fewest points and hence shortest test time is optimum.

Proposal 5: the 3x3 test beam direction set is used

3.4	Other open issues

The other open issues from the WF have been discussed with the following proposals:

Issue 4-3: Measurement grid over elevation angle and azimuth angle

Proposal 6: Elevation accuracy is not related to main beam width, results show that an elevation step size of 1° is optimum

Proposal 7: Azimuth accuracy is related to main bema width (similarly to TRP), azimuth step size should be at least half the main beam azimuth beam with

Issue 4-4: RF channels for the conformance testing

Proposal 8: Use RF channel M for test procedure.

References

[1]	R4-2413509 WF for [112][305] NR_BS_RF – ZTE

[2] R4-2413276 Expected EIRP conformance – Test Vectors, MU budget and Conformance test method - Huawei