3GPP RAN WG4 Meeting #112bis		R4-2416282

# Hefei, Anhui, China, 14th – 18th October, 2024

Source: 	Huawei, HiSilicon

Title: 	RF requirements for Ambient IoT BS

Agenda Item:	6.21.3.1

Document for:	Approval

1 Introduction

The new SID on solutions for Ambient IoT was approved at the RAN#102 meeting [1] and later revised [2] at the RAN#103 meeting. The RAN4-led objectives are copied as below:

- RAN4-led:

  - Coexistence study of Ambient IoT and NR/LTE.

  - RF requirements study for Ambient IoT:

    - Ambient IoT BS transmission and reception

    - Ambient IoT Device, as per the General Scope, transmission and reception

    - Intermediate node (UE), as per the General Scope, transmission and reception

In this contribution, we provide discussion on Ambient IOT BS RF requirements.

2 Discussion

2.1 General

For the ambient IOT BS RF requirement, since ambient IOT is operating within an LTE/NR channel, for the requirements related to out of LTE/NR channel, the existing BS RF requirements can be used as starting point. Within the LTE/NR channel, we need further consider the ACLR/ACS requirement based on the co-existence study. In the following we look at each TX and RX requirement for ambient IOT BS.

2.2 BS type

Issue 2-2: A-IoT BS class and BS type

Agreement in RAN4#112:

  - BS class: Use Micro-BS as baseline in SI stage (reference to SID RP-240826).

  - BS type: Priority A-IoT BS type 1-C, FFS for 1-H.

It is agreed to Micro BS as baseline in the study item stage.

For D1T1 deployment is indoor scenarios. An indoor BS is typically deployed by ceiling installation, especially for the scenarios of factory or warehouse. The main requirement is wide beam width on the hemispherical surface, so beam forming does not provide benefits for indoor deployment. And the target IMT bands are low frequency band, e.g. 800/900 MHz. It is proposed to define BS type 1-C for Ambient-IoT indoor scenarios.

Proposal 1: It is proposed to define BS type 1-C for Ambient-IoT indoor scenarios.

2.3 BS TX

Agreement in RAN4#112:

- Requirements potentially needed:

  - Base station output power: use current requirements of Micro NR BS type 1-C as the starting point.

  - Transmitted signal quality

    - Frequency error:

    - Transmit signal quality

  - Unwanted emissions

    - Occupied bandwidth

    - ACLR

    - Operating band unwanted emissions

    - Transmitter spurious emissions

- FFS whether requirements are needed or not:

  - Output power dynamic (RE power control dynamic range, Total power dynamic range)

  - Some requirements like Transmit ON/OFF power

  - Transmitter transient period

  - IBE

  - Transmitter intermodulation

- Requirements not needed:

  - Transmission times (requirements from RF ID reader)

  - TAE

Base station output power

BS output power is a key parameter for coverage. In existing NR specification, the rated carrier output power of the BS type 1-C is 38 dBm for MR BS, which is characterised by requirements derived from Micro Cell scenarios.

Table 6.2.1-1: BS type 1-C rated output power limits for BS classes

| BS class | Prated,c,AC |
| --- | --- |
| Wide Area BS | (Note) |
| Medium Range BS | ≤ 38 dBm |
| Local Area BS | ≤ 24 dBm |
| NOTE: There is no upper limit for the Prated,c,AC rated output power of the Wide Area Base Station. |  |

The above rated output power limits for Medium range BS is applicable to Ambient IOT BS for D1T1 deployment scenarios.

As a reference, the UHF RFID reader can transmit with 35 dBm EIRP, according to the ISM band regulation. From the view of better coverage than UHF RFID, it is reasonable to assume the transmit power of Ambient IoT BS to be no lower than that of the RFID reader, e.g. 33 dBm transmit power and 2 dBi antenna gain.

As agreed at RAN4#112, for base station output power, current requirements of Micro NR BS type 1-C is used as the starting point.

Output power dynamic

In current BS specification, the general output power dynamic range requirements include RE power control dynamic range and total power dynamic range. These two requirements are applied to legacy constellation modulation which are not applicable to OOK signal for Ambient IOT BS.

Proposal 2: The general output power dynamic (RE power control dynamic range, Total power dynamic range) is not needed.

Deployment scenarios for D1T1 was discussed in previous meetings. Option 2-1 scenario is AIOT and NR BS share the hardware. From the study in [R4-2414963], it is shown that power boosting need be larger than [20] dB for device 1 and 2a to decode the R2D signal within certain conditions. But we know current specification define a 6 dB power boosting for NB-IOT in-band operation. It is a huge gap for BS to support that A-IOT and NR BS share the hardware to generate the two RATs with 20 dB power boosting. Hence the power boosting for BS support both Ambient IOT and NR can be discussed in future release.

Proposal 3: The power boosting for BS support both Ambient IOT and NR can be discussed in future release.

Transmit ON/OFF power

In current BS specification, transmit OFF power and transmitter transient period are defined in the clause of transmit ON/OFF power. It is a TDD requirements and not applicable to Ambient IOT BS. For the case BS generates R2D and CW signal in the sequential way in D1T1-A1 and D1T1-A2, the switch is an implementation issue and no need to define it as a minimal RF requirement.

Proposal 4: Transmit ON/OFF power (transmit OFF power and transmitter transient period) is not needed for Ambient IOT BS.

Transmission times

The on-duration and the off-duration requirement is defined for RFID. More discussion is covered below for transmit signal quality.

As agreed at RAN4#112, it is not needed for Ambient IOT BS.

Transmit signal quality

As agreed at RAN4#112, frequency error is potentially needed. For NR BS, frequency error for both MR BS and LA BS is ±0.1 ppm. It is proposed to adopt ±0.1 ppm frequency error for Ambient IOT BS.

Proposal 5: It is proposed to adopt ±0.1 ppm frequency error for Ambient IOT BS.

For R2D link, OOK waveform generated by OFDM modulator. Pulse interval encoding (PIE) and Manchester encoding have been agreed as the study baseline line code candidates for the R2D transmission. Device 1 and Device 2a use RF envelope detection to decode the R2D signal. The RF envelope requirement might be needed to guarantee the decoding performance.

Figure 2-1. Manchester encoding and decoding

Figure 2-2. PIE encoding and decoding

As a reference, RFID uses ASK modulation with PIE data encoding. The ASK modulation mask was defined as shown in Figure 2-3 below. The pulsewidth (PW), Modulation Depth ((A–B)/A), RF Envelope Ripple (M), RF envelope Rise time (tr,10–90), and RF envelope fall time (tf,10–90) are the major requirements.

Figure 2-3: Interrogator-to-Tag RF envelope

Proposal 6: it is proposed to define RF envelope mask as the transmit signal quality requirement.

Adjacent Channel Leakage power Ratio

Proposal 7: ACLR can be defined based on the co-existence study

Operating band unwanted emissions

IBE is a requirement for partial transmission which is not case for Ambient IOT BS.

Proposal 8: IBE is not needed for Ambient IOT BS.

Proposal 9: Need more study for OBUE limits after ACLR is defined.

Spurious emission

Proposal 10: Existing spurious emission requirement is applicable

Transmitter intermodulation

Traditionally the transmitter intermodulation requirement was created to guarantee emission levels in a co-location scenario. The requirement is based on robustness against injection of a reverse interference signal from a co-located BS. for indoor deployment. For the same band it is not possible to have co-location transmitter since it will have interference to the device on the RF-ED. Hence no need to define transmitter intermodulation for A-IOT BS.

Proposal 11: no need to define transmitter intermodulation for A-IOT BS

Based on the above discussion, a summary is captured in the following Table

| RF Requirement for A-IoT BS- TX part |  |  |  |  |
| --- | --- | --- | --- | --- |
| TX requirement | Base station output power |  |  | Agreement in RAN4#112: use current requirements of Micro NR BS type 1-C as the starting point |
|  | Output power dynamic |  | RE power control dynamic range | Proposal 2-1: RE power control dynamic range is not needed |
|  |  |  | Total power dynamic range | Proposal 2-2: Total power dynamic range is not needed |
|  |  |  | Power boosting | Proposal 3: The power boosting for BS support both Ambient IOT and NR can be discussed in future release. |
|  | Transmitter OFF power |  |  | Proposal 4-1: Transmitter OFF power is not needed for Ambient IOT BS. |
|  | Transmitter transient period |  |  | Proposal 4-2: Transmitter transient period is not needed for Ambient IOT BS. |
|  | Transmission times |  |  | Agreement in RAN4#112: It is not needed for Ambient IOT BS. |
|  | Transmit signal quality | Frequency error |  | Proposal 5: It is proposed to adopt ±0.1 ppm frequency error for Ambient IOT BS. |
|  |  | Modulation quality |  | Proposal 6: it is proposed to define RF envelope mask as the transmit signal quality requirement. The pulsewidth (PW), Modulation Depth ((A–B)/A), RF Envelope Ripple (M), RF envelope Rise time (tr,10–90), and RF envelope fall time (tf,10–90) are the major requirements. |
|  |  | TAE |  | Agreement in RAN4#112: It is not needed for Ambient IOT BS. |
|  | Unwanted emissions | Occupied bandwidth |  | Agreement in RAN4#112: It is potentially needed for Ambient IOT BS. |
|  |  | ACLR |  | Agreement in RAN4#112: It is potentially needed for Ambient IOT BS. Proposal 7: ACLR can be defined based on the co-existence study |
|  |  | IBE |  | It is a requirement for partial transmission which is not case for Ambient IOT BS. Proposal 8: IBE is not needed for Ambient IOT BS. |
|  |  | Operating band unwanted emissions |  | Agreement in RAN4#112: It is potentially needed for Ambient IOT BS. Proposal 9: Need more study for OBUE limits after ACLR is defined. |
|  |  | Transmitter spurious emissions |  | Agreement in RAN4#112: It is potentially needed for Ambient IOT BS. Proposal 10: Existing spurious emission requirement is applicable |
|  | Transmitter intermodulation |  |  | Proposal 11: no need to define transmitter intermodulation for A-IOT BS |

2.4 BS RX

Agreement in RAN4#112:

- Requirements potentially needed:

  - Reference sensitivity level

  - Adjacent Channel Selectivity

  - Blocking requirement

  - Rx spurious emission

- FFS whether requirements are needed or not:

  - Dynamic range

  - In-channel selectivity

  - In-band blocking

  - Narrow-band blocking

  - Out-of-band blocking

  - Receiver intermodulation

  - Narrowband intermodulation

  - Receiver intermodulation

Reference sensitivity level

The residual noise from CW interference is higher than the noise floor. So it should be considered when defining receiver sensitivity. Reference sensitivity shall be specified according following formula:

PREFSENS(dBm)=-174dBm+10×log10BW+NF+IM+Icw+SNR

Where:

-	BW is the maximum transmission bandwidth

-	NF is BS noise figure, equal to 10 dB for Medium Range BS and 13 dB for Local Area BS.

-	IM is the implementation margin, equal to 2dB.

-	Icw is IoT due the residual CW interference, value will be defined in the WI.

-	SNR is the SNR value for which we reach 10% BLER target. Each company provided simulation results, and average will be done for each BW.

Proposal 12: IoT due the residual CW interference should be considered when define RX reference sensitivity

Dynamic range

For NR and LTE, receiver dynamic range requirement is to ensure that the base station can receive high throughput also in the presence of increased interference levels. The interference level is derived from simulation of co-channel interference from the same operator. It measures the effects of base station receiver impairments. For A-IOT indoor deployment, the co-channel interference comes from co-channel device transmission and CW interference. The necessity and the value can be studied at WI phase.

Proposal 13: The necessity and the value can be studied at WI phase

In-channel selectivity

For NR and LTE, in-channel selectivity (ICS) is a measure of the receiver ability to receive a wanted signal at its assigned resource block locations in the presence of another in-channel wanted signal received at a much larger power spectral density. For A-IOT, it depends on whether FDM operation is supported. It may be needed for FDM operation.

Proposal 14: It may be needed for FDM operation. The necessity and the value can be studied at WI phase

In-band selectivity and blocking

ACS can be defined based on the co-existence study. In-band blocking need more study after ACS is defined

As agreed at RAN4#112, it is potentially needed for Ambient IOT BS.

Out-of-band blocking

Out of band blocking is a measure of the receiver ability to receive a wanted signal at its assigned channel in the presence of an unwanted interferer out of the operating band, which is a CW signal for out-of-band blocking. Existing out-of-band blocking requirement is applicable.

Proposal 15: existing out-of-band blocking requirement is applicable

Receiver spurious emissions

Existing RX spurious emission requirement is applicable

Proposal 16: Existing RX spurious emission requirement is applicable

Receiver intermodulation

Intermodulation response rejection is a measure of the capability of the receiver to receive a wanted signal on its assigned channel frequency in the presence of two interfering signals which have a specific frequency relationship to the wanted signal. For A-IOT deployment, the typical case is that there in only one operator. Hence there is may no such an interference scenario. It can be further discussed at WI phase.

Proposal 17:  FFS whether requirement necessary based on the interference scenario

Based on the above discussion, a summary for RX part is captured in the following Table

|  | RF Requirement for A-IoT BS- RX part |  |
| --- | --- | --- |
| RX requirement | Reference sensitivity level | Agreement in RAN4#112: It is potentially needed for Ambient IOT BS. Proposal 12: IoT due the residual CW interference should be considered when define RX reference sensitivity |
|  | Dynamic range | Proposal 13: The necessity and the value can be studied at WI phase |
|  | In-channel selectivity | Proposal 14: It may be needed for FDM operation. The necessity and the value can be studied at WI phase |
|  | Adjacent Channel Selectivity | Agreement in RAN4#112: It is potentially needed for Ambient IOT BS. ACS can be defined based on the co-existence study. |
|  | Blocking requirement | Agreement in RAN4#112: It is potentially needed for Ambient IOT BS. |
|  | In-band blocking | need more study after ACS is defined |
|  | Narrow-band blocking | need more study after ACS is defined |
|  | Out-of-band blocking | Proposal 15: existing out-of-band blocking requirement is applicable |
|  | Receiver intermodulation | Proposal 17-1: FFS whether requirement necessary based on the interference scenario |
|  | Narrowband intermodulation | Proposal 17-2: FFS whether requirement necessary based on the interference scenario |
|  | Rx spurious emission | Agreement in RAN4#112: It is potentially needed for Ambient IOT BS. Proposal 16: Existing RX spurious emission requirement is applicable |

3	Conclusion

This contribution discusses some proposals on Ambient IOT BS RF requirements.

Proposal 1: It is proposed to define BS type 1-C for Ambient-IoT indoor scenarios.

A summary for TX part is captured in below Table

| RF Requirement for A-IoT BS- TX part |  |  |  |  |
| --- | --- | --- | --- | --- |
| TX requirement | Base station output power |  |  | Agreement in RAN4#112: use current requirements of Micro NR BS type 1-C as the starting point |
|  | Output power dynamic |  | RE power control dynamic range | Proposal 2-1: RE power control dynamic range is not needed |
|  |  |  | Total power dynamic range | Proposal 2-2: Total power dynamic range is not needed |
|  |  |  | Power boosting | Proposal 3: The power boosting for BS support both Ambient IOT and NR can be discussed in future release. |
|  | Transmitter OFF power |  |  | Proposal 4-1: Transmitter OFF power is not needed for Ambient IOT BS. |
|  | Transmitter transient period |  |  | Proposal 4-2: Transmitter transient period is not needed for Ambient IOT BS. |
|  | Transmission times |  |  | Agreement in RAN4#112: It is not needed for Ambient IOT BS. |
|  | Transmit signal quality | Frequency error |  | Proposal 5: It is proposed to adopt ±0.1 ppm frequency error for Ambient IOT BS. |
|  |  | Modulation quality |  | Proposal 6: it is proposed to define RF envelope mask as the transmit signal quality requirement. The pulsewidth (PW), Modulation Depth ((A–B)/A), RF Envelope Ripple (M), RF envelope Rise time (tr,10–90), and RF envelope fall time (tf,10–90) are the major requirements. |
|  |  | TAE |  | Agreement in RAN4#112: It is not needed for Ambient IOT BS. |
|  | Unwanted emissions | Occupied bandwidth |  | Agreement in RAN4#112: It is potentially needed for Ambient IOT BS. |
|  |  | ACLR |  | Agreement in RAN4#112: It is potentially needed for Ambient IOT BS. Proposal 7: ACLR can be defined based on the co-existence study |
|  |  | IBE |  | It is a requirement for partial transmission which is not case for Ambient IOT BS. Proposal 8: IBE is not needed for Ambient IOT BS. |
|  |  | Operating band unwanted emissions |  | Agreement in RAN4#112: It is potentially needed for Ambient IOT BS. Proposal 9: Need more study for OBUE limits after ACLR is defined. |
|  |  | Transmitter spurious emissions |  | Agreement in RAN4#112: It is potentially needed for Ambient IOT BS. Proposal 10: Existing spurious emission requirement is applicable |
|  | Transmitter intermodulation |  |  | Proposal 11: no need to define transmitter intermodulation for A-IOT BS |

A summary for RX part is captured in the following Table

|  | RF Requirement for A-IoT BS- RX part |  |
| --- | --- | --- |
| RX requirement | Reference sensitivity level | Agreement in RAN4#112: It is potentially needed for Ambient IOT BS. Proposal 12: IoT due the residual CW interference should be considered when define RX reference sensitivity |
|  | Dynamic range | Proposal 13: The necessity and the value can be studied at WI phase |
|  | In-channel selectivity | Proposal 14: It may be needed for FDM operation. The necessity and the value can be studied at WI phase |
|  | Adjacent Channel Selectivity | Agreement in RAN4#112: It is potentially needed for Ambient IOT BS. ACS can be defined based on the co-existence study. |
|  | Blocking requirement | Agreement in RAN4#112: It is potentially needed for Ambient IOT BS. |
|  | In-band blocking | need more study after ACS is defined |
|  | Narrow-band blocking | need more study after ACS is defined |
|  | Out-of-band blocking | Proposal 15: existing out-of-band blocking requirement is applicable |
|  | Receiver intermodulation | Proposal 17-1: FFS whether requirement necessary based on the interference scenario |
|  | Narrowband intermodulation | Proposal 17-2: FFS whether requirement necessary based on the interference scenario |
|  | Rx spurious emission | Agreement in RAN4#112: It is potentially needed for Ambient IOT BS. Proposal 16: Existing RX spurious emission requirement is applicable |

References

[1] RP-240829, New WID: NR base station (BS) RF requirement evolution for FR1/FR2 and testing, RAN4 Chair (Huawei)

[2] R4-2410597, WF on impacts of A-IoT on RF requirements, Huawei