3GPP RAN WG4 Meeting #112bis		R4-2416284

# Hefei, Anhui, China, 14th – 18th October, 2024

Source: 	Huawei, HiSilicon

Title: 	On modification of existing BS TX requirements for SBFD

Agenda Item:	6.20.2.2

Document for:	Approval

Introduction

During RAN4#112, BS RF requirements for SBFD have been discussed. Following conclusions for existing TX requirements are reproduced from [3].

- 1.1 Total dynamic range Agreement: - Formulate the total power dynamic range requirement for SBFD slots using equation based on declaration (of rated output power with all DL sub-band RBs active for SBFD), instead of table used for existing total power dynamic range requirement. - 1.2. Transmit ON/OFF power Agreement: - Transmitter ON/OFF power should apply to normal symbol and it is not applicable within SBFD symbol. - 1.3. ACLR and OBUE Agreement: - When coexisting with legacy TDD system in adjacent channel, RAN4 shall apply the legacy ACLR and OBUE requirement for SBFD-capable BS. WF: - FFS on the followings: - For FR1 macro deployments, when coexisting with new SBFD system in adjacent channel, RAN4 to define additional ACLR and/or OBUE requirement to ensure feasible coexistence with SBFD-capable gNB. The assumption for the victim BS DESENS and CL of BS2BS is FFS. - 1.4. Joint measurement for normal DL symbols/slots and SBFD symbols/slots WF: - FFS on the followings: - Averaged EVMs for SBFD slots and non-SBFD slots shall be measured separately. - Averaged EVM for SBFD slots shall be calculated only based on the data samples where DLs are allocated within the SBFD slots. The data samples where ULs are allocated within the SBFD slots shall be removed from the calculation of averaged EVM.

Good progress was made in last meeting. In this contribution, we would like to share our views regarding the open issues.

Discussion

General

For TX and RX requirements discussion, good progress were made in last meeting. Meanwhile there is some divergence on the concerned deployment scenario for FR1 Macro BS when defining RF requirements. For the already deployed band, co-existence with legacy TDD system in adjacent channel should be considered. For a new wide TDD band without deployment yet, it is promising to deploy SBFD BS to increase UL coverage and reduce latency. As such additional requirements should be defined for co-existence performance between different SBFD BSs operating in adjacent frequency. From the previous discussion and discussion in [R4-2416285], additional blocking requirements for RX and additional OBUE requirement are needed for SBFD BSs.

Proposal 1: Additional TX OBUE and RX blocking requirements should be defined for co-existence performance between different SBFD BSs operating in adjacent frequency.

ACLR and OBUE

Per WF in [3], the following agreement was reached.

Agreement:

- When coexisting with legacy TDD system in adjacent channel, RAN4 shall apply the legacy ACLR and OBUE requirement for SBFD-capable BS.

WF:

- FFS on the followings:

  - For FR1 macro deployments, when coexisting with new SBFD system in adjacent channel, RAN4 to define additional ACLR and/or OBUE requirement to ensure feasible coexistence with SBFD-capable gNB. The assumption for the victim BS DESENS and CL of BS2BS is FFS.

For coexistence between different SBFD BSs operating in adjacent frequency, it is BS to BS cross link interference. On the level of the requirements, one reference could be additional spurious emissions which is applied as optional protection of the systems that are deployed in the same geographical area as the BS. It can be found that -52 dBm/MHz is defined for protection of DL UE RX and -49 dBm/MHz for protection of UL BS.

Table 2.2-1: Table 6.6.5.2.3-1 in TS 38.104: BS spurious emissions basic limits for BS for co-existence with systems operating in other frequency bands

| System type for NR to co-exist with | Frequency range for co-existence requirement | Basic limits | Measurement bandwidth | Note |
| --- | --- | --- | --- | --- |
| UTRA FDD Band I or | 2110 – 2170 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n1 or n65 |
| E-UTRA Band 1 or NR Band n1 | 1920 – 1980 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n1 or n65, since it is already covered by the requirement in clause 6.6.5.2.2. |
| UTRA FDD Band II or | 1930 – 1990 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n2 or n70. |
| E-UTRA Band 2 or NR Band n2 | 1850 – 1910 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n2, since it is already covered by the requirement in clause 6.6.5.2.2. |
| UTRA FDD Band III or | 1805 – 1880 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n3. |
| E-UTRA Band 3 or NR Band n3 | 1710 – 1785 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n3, since it is already covered by the requirement in clause 6.6.5.2.2. |
| … | … |  |  |  |
| NR Band n77 | 3.3 – 4.2 GHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n48, n77 or n78 |
| NR Band n78 | 3.3 – 3.8 GHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n48, n77 or n78 |
| NR Band n79 | 4.4 – 5.0 GHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n79 |
| … | … |  |  |  |
| NR Band n104 | 6425 – 7125 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in Band n96, n102 or n104 |
| … | … |  |  |  |

The -49 dBm/MHz assumes 67 dB MCL and 7dB lower than noise floor, which is derived for 3G system at an intermediate case scenario (577/2 m base station shifting, 2GHz UMTS band). These requirements are defined as the same for all existing FR1 bands. It can be anticipated that at higher frequency band, e.g. n104 upper 6GHz, similar MCL assumption cover more scenarios. Table 2.2-2 shows an example link budget for adjacent channel co-existence if 67 dB MCL is assumed.

Table 2.2-2: Adjacent channel co-existence

| Aggressor | Victim | Aggressor (Operator A) Victim (Operator B) | Link budget |
| --- | --- | --- | --- |
| SBFD | SBFD | D D U U |  |
| TX |  | Guard band -15 dBm/MHz | -15 dBm |
| TX Filter | RX Filter |  | Required attenuation from TX filter: -15+49=34 dB |
| Pathloss/coupling loss |  | e.g. co-existence spurious emissions assume 67 dB MCL | 67 dB |
| Emission level |  | -49 dBm/1MHz in UL, assuming 67 dB MCL +7dB lower than noise floor. | -49 dBm/MHz |

The assumption for the victim BS DESENS and CL of BS2BS

Proposal 2: It is proposed to define the additional OBUE requirement based on MCL assumption and BS DESENS level.

As shown in Table 2.2-2, for protection of SBFD UL of operator B, the SBFD DL of operator B can be adopted as guard band for filter implementation of adjacent operator A. In existing FR1 networks, usually operators hold less than or equal to 400 MHz. Using n104 as an example, the available guard band for TX filter is analysed in table 2.2-3. For the band n104 which has 700 MHz band width, considering it is allocated to 2 ~ 3 operators, each may hold 200 MHz, 300 MHz or 400 MHz. Table 2.2-3 shows the available guard band examples for different operator’s spectrum scenarios.

Table 2.2-3: available guard band examples for different operator’s spectrum scenarios

| Scenario No. | Operator's spectrum | Achieved guard band (MHz) |  | min Ratio of transition guard/sub frequency band | Comments |
| --- | --- | --- | --- | --- | --- |
|  |  | 2 operator case | 3 operator case (in middle) |  |  |
| 1 | 100 MHz | 80 | 40 | 40% | Configuration 1: 40-20-40 Configuration 2: 80-20 |
| 2 | 200 MHz | 180 | 80 | 30% | Configuration 1: SBFD 20-80+ TDD 100 |
| 3 | 300 MHz | 260 | 100 | 33.30% | Configuration 1: SBFD 40-60+ TDD 100+ TDD 100 |
| 4 | 400 MHz | 360 | N/A | 90% | Configuration 1: SBFD 40-60+ TDD 100+ TDD 100+ TDD 100 |

Observation 1: available guard band for different operators’ spectrum cases are different, which depends on the spectrum allocation: number of operations in one band and bandwidth for each operator.

As a reference, in existing TX requirement, ΔfOBUE is defined based on filter implementation. For FR1 operating band less than 100 MHz, ΔfOBUE is 10 MHz, and for band wider than 100 MHz except n104, ΔfOBUE is extended to 40 MHz. ΔfOBUE is further extended to 100 MHz for band n104. It can be found min ratio of transition guard/sub frequency band is much higher than that of ΔfOBUE, which means it would not impose additional implementation difficulty for taking the advantage of per operator filter.

Table 2.2-4: Maximum offset ΔfOBUE outside the downlink operating band

| BS type | Operating band characteristics (MHz) | ΔfOBUE (MHz) | minimum Ratio of ΔfOBUE /bandwidth |
| --- | --- | --- | --- |
| BS type 1-O | FDL,high – FDL,low < 100 | 10 | 10% |
|  | 100 MHz ≤ FDL,high – FDL,low ≤ 900 (Note: operators hold less than or equal to 400 MHz ) | 40 | 10% |
|  | Band n104 | 100 | 14.3% |

Observation 2: The ratio of transition guard/sub frequency band is higher than that of ΔfOBUE, which means it would not impose additional implementation difficulty for taking the advantage of per operator filter.

There could be two ways to handle the frequency offset. The first one is to define the offsets in specification based on typical scenarios. Table 2.2-5 shows one example to define the additional OBUE requirements with different offsets considering different operators’ spectrum cases. It may define stringent requirement than necessary, e.g. for scenario 3, two operator case can adopt 260 MHz guard band but 3 operator case only 100 MHz can be used as guard band.

Table 2.2-5: additional OBUE requirement for protection of SBFD UL

| SBFD sub-band characteristics | Frequency offset of measurement filter -3dB point, f | Basic limits | Measurement bandwidth |
| --- | --- | --- | --- |
| 100 MHz ≤Band width < 200 | [40 MHz] f fmax | [-xx dBm] | 1MHz |
| 200 MHz ≤Band width < 300 | [80 MHz] f fmax | [-xx dBm] | 1MHz |
| 300 MHz ≤Band width ≤ 400 | [100 MHz] f fmax | [-xx dBm] | 1MHz |

The other one is to leave it to manufacture declaration, then it can be defined in regional requirement more specifically and more precisely.

Proposal 3: It is proposed to declare the frequency offset for the additional OBUE requirement.

Transmitter signal quality

Per WF in [R4-2413515], we approved the following WF on Joint measurement for normal DL symbols/slots and SBFD symbols/slots

WF:

- FFS on the followings:

  - Averaged EVMs for SBFD slots and non-SBFD slots shall be measured separately.

  - Averaged EVM for SBFD slots shall be calculated only based on the data samples where DLs are allocated within the SBFD slots. The data samples where ULs are allocated within the SBFD slots shall be removed from the calculation of averaged EVM.

We agree with the statement that Averaged EVMs for SBFD slots and non-SBFD slots shall be measured separately.

Conclusion

In this contribution we discussed on the BS TX requirement for SBFD. We have the following observations and proposals:

Proposal 1: Additional TX OBUE and RX blocking requirements should be defined for co-existence performance between different SBFD BSs operating in adjacent frequency.

Proposal 2: It is proposed to define the additional OBUE requirement based on MCL assumption and BS DESENS level.

Observation 1: available guard band for different operators’ spectrum cases are different, which depends on the spectrum allocation: number of operations in one band and bandwidth for each operator.

Observation 2: The ratio of transition guard/sub frequency band is higher than that of ΔfOBUE, which means it would not impose additional implementation difficulty for taking the advantage of per operator filter.

Proposal 3: It is proposed to declare the frequency offset for the additional OBUE requirement.

- References

- R4-2406107, “Way Forward for [110bis][313] NR_duplex_evo”, Samsung, 3GPP TSG-RAN4 Meeting #110bis, Changsha, China, April 15-19, 2024.

- R4-2409958	Way Forward for [111][313] NR_duplex_evo	Samsung

- R4-2413515, Way Forward for [112][308] NR_duplex_evo_BSRF, Huawei, HiSilicon