3GPP TSG-RAN WG4 Meeting #112bis	R4-2416288

Hefei, China, Oct 14 – Oct 18, 2024

Source:	Nokia

Title:	Discussion on RRM requirements for NR less than 5MHz phase 2

Agenda item:	6.4.3

Document for:	Discussion

Introduction

In the last meeting, the following agreements were made.

Agreement - Specify new SCell activation delay requirements to support less than 5MHz bandwidth in the target SCell in FR1. Agreement - Start from single SCell activation delay requirements. Agreement - Specify SSB index identification delay requirements for UE with deactivated SCell measurements on a target cell with less than 5MHz SSB BW in FR1, taking into consideration the impacts from PBCH puncturing. Agreement: - Specify new PSCell addition delay requirements for PSCell configured with less than 5MHz channel bandwidth. Agreement: - FFS, if target PSCell is using 3MHz, the SSB index acquisition of less than 5MHz cell may extend the SCG activation requirement requirement. Agreement: - not consider the CGI reading related requirement in Rel-19. Agreement: - the features/scenarios not considered in R18 less than 5MHz RRM shall not be discussed in R19 CA/DC with less than 5MHz band, e.g., CSI-RS based L1/L3/RLM/BFD/CBD measurement.

This contribution discusses about the open issues related to RRM requirements on supporting CA for < 5MHz FR1 bands.

Discussion

The main changes in the rel-18 were related to SSB index reading for L3 measurements due to PBCH puncturing and with PDCCH RLM and BFD requirements.

In the rel-18 WI, the following agreement was made on which component the extra delay is added.

Agreement: - For HO requirements, include the 2 additional SSB samples for SBI and MIB reading due to PBCH puncturing as part of T∆ instead of Tsearch. - For less than 5MHz requirement, include the additional time for T∆ in the spec.

In rel-19, follow rel-18 baseline and add 2 additional SSB samples for SBI and MIB reading due to PBCH puncturing as part of T∆, where applicable.

In rel-19, follow rel-18 baseline and add 2 additional SSB samples where SSB index reading is required.

Issue 2-1-3: Whether intra-band CA is in the scope: - Proposals - Option 1: Consider intra-band CA (both contiguous and non-contiguous) within the scope of the RRM requirements in Rel-19. - Option 2: Wait until RF room has concrete conclusion on intra-band CA combinations.

In the last meeting discussion on intra-band support was introduced. Currently only inter-band CA is considered in RAN4 RF for both contiguous non-contiguous case. Therefore, we don’t see the need for defining intra-band requirements.

Consider only inter-band CA for contiguous and non-contiguous case.

Issue 2-1-4: IDLE mode CA measurement reporting: - Proposals - Option 1: For 3MHz channel bandwidth, N2 = [3+3] if the NR inter-frequency carrier for idle mode CA/DC measurement reporting is in FR1.

Our understanding is that the WID is about CA, and considering EMR reporting within the WID seems to be out of scope.

No changes on top of rel-18 EMR measurement reporting are needed.

Issue 2-2-1: PSCell addition and release, conditional PSCell addition delay requirements: - Proposals - Option 1: Specify new PSCell addition delay requirements for PSCell configured with less than 5MHz channel bandwidth. Issue 2-2-2: PSCell change, conditional PSCell change delay requirements: - Proposals Option 1: RAN4 to discuss the impacts of less than 5MHz, in particular PBCH puncturing, to PSCell change delay requirements.

Legacy requirements can be used as a baseline for PSCell addition.

Legacy PSCell requirements are used as a baseline, T∆ may be updated, if needed.

Issue 2-3-1: NW indication on PBCH BW in MO configuration and/or HO command: - Proposals - Option 1: RAN4 to discuss whether NW indication on PBCH BW in MO configuration is needed considering a cell with less than 5MHz can be SCell. - Option 2: If SCell can use 12PRB SSB bandwidth in R19, it’s necessary to provide assistance information to UE regarding whether the PBCH is 12 or 20 PRBs in either MO or HO command.

This issue was discussed in rel-18, and it is not clear what part of the rel-19 CA support is requiring this change.

No changes needed, follow the agreement in 18.

Issue 2-2-4: SCG activation delay requirements: - Proposals - Option 1: Need FFS, if target PSCell is using 3MHz, the SSB index acquisition of less than 5MHz cell may extend the PSCell change requirement. Agreement: - FFS, if target PSCell is using 3MHz, the SSB index acquisition of less than 5MHz cell may extend the SCG activation requirement requirement.

Legacy PSCell requirements are used as a baseline, T∆ may be updated, if needed.

Issue 2-3-1: NW indication on PBCH BW in MO configuration and/or HO command: - Proposals - Option 1: RAN4 to discuss whether NW indication on PBCH BW in MO configuration is needed considering a cell with less than 5MHz can be SCell. - Option 2: If SCell can use 12PRB SSB bandwidth in R19, it’s necessary to provide assistance information to UE regarding whether the PBCH is 12 or 20 PRBs in either MO or HO command.

This issue seems to be related to HO and the WID is focusing on CA, and it is not clear when CD-SSB of SCell can be off sync raster.

Discuss what information is needed and when CD-SSB of SCell can be off sync raster

Conclusion

The following Observations and Proposals were made:

Proposal 1: In rel-19, follow rel-18 baseline and add 2 additional SSB samples for SBI and MIB reading due to PBCH puncturing as part of T∆, where applicable.

Proposal 2: In rel-19, follow rel-18 baseline and add 2 additional SSB samples where SSB index reading is required.

Proposal 3: Consider only inter-band CA for contiguous and non-contiguous case.

Proposal 4: No changes on top of rel-18 EMR measurement reporting are needed.

Proposal 5: Legacy PSCell requirements are used as a baseline, T∆ may be updated, if needed.

Proposal 6: No changes needed, follow the agreement in 18.

Proposal 7: Legacy PSCell requirements are used as a baseline, T∆ may be updated, if needed.

Proposal 8: Discuss what information is needed and when CD-SSB of SCell can be off sync raster