3GPP TSG-RAN WG4 Meeting #112bis	R4-2416289

Hefei, China, Oct 14 – Oct 18, 2024

Source:	Nokia

Title:	Discussion on Fast SCell activation for UE supporting Rel-18 EMR

Agenda item:	6.15.3

Document for:	Discussion

Introduction

RAN4 Rel-19 work item on fast SCell activation for UE supporting Rel-18 EMR officially trigered in RAN4#112. As per the outcomes of this meeting ‎[1], following points are open further discussion:

- Applicability of fast SCell activation delay requirements,

- Scope of fast SCell activation for UE supporting Rel-18 eEMR,

- Target scenarios of fast SCell activation,

- How to define the fast SCell activation delay requirements with valid eEMR reporting, and how to update the known condition with consideration of valid eEMR reporting,

In this paper, we address the above open points and provide our views on reduce the SCell activation delay with valid Rel-18 EMR reporting.

Discussion

Applicability of fast SCell activation delay requirements

Regarding applicability of fast SCell activation, following agreement is made during the previous discussion

#### Issue 1-1-2: Applicability of fast SCell activation delay requirements Agreement: - The fast SCell activation delay requirements are defined for the case when - the UE supports Rel-18 eEMR: - Including both EMR and cell reselection measurement, - Including the case when measIdleValidityDuration-r18 and / or measReselectionValidityDuration-r18 are configured - FFS Including the case when measIdleValidityDuration-r18 and / or measReselectionValidityDuration-r18 are not configured - and the UE has reported valid results on the SCell to be activated before SCell activation command.

The following text is the latest 38.133 specification from rel-18:

3GPP TS 38.133 V18.7.0 (2024-09) ## 4.7 Measurement report for fast CA/DC setup ### 4.7.1 Introduction The requirements of measurement report for fast CA/DC setup apply in this clause for: - UE supporting measValidationReportEMR-r18 and configured with measIdleCarrierListNR-r16 and/or measIdleCarrierListEUTRA-r16 by higher layers. - UE supporting measValidationReportReselectionMeasurements-r18 and configured with measReselectionCarrierListNR-r18 by higher layers. A UE supporting measValidationReportEMR-r18 and idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16 shall report the results of the idle mode measurement which is specified in clause 4.4, and according to the measurement reporting requirements specified in clause 4.7.3. A UE supporting measValidationReportReselectionMeasurements-r18 shall report the results of the idle mode measurement specified in the clause 4.2.2, and according to the measurement reporting requirements specified in the clause 4.7.3. UE is not required to report for fast CA/DC setup from any measurements during IDLE mode for carriers which are not configured by higher layers. ### 4.7.2 Void ### 4.7.3 Measurement Report Requirements A UE shall perform validity check and report valid measurement results: - if the UE supports measValidationReportEMR-r18 and measIdleValidityDuration-r18 is configured for carriers in measIdleCarrierListNR-r16 or measIdleCarrierListEUTRA-r16, - if the UEsupports measValidationReportReselectionMeasurements-r18 and measReselectionValidityDuration-r18 is configured for carriers in measReselectionCarrierListNR-r18 The measurement results are considered valid if the following conditions are met for the validity check: - the measurements are performed before msg1 transmission for RRC resume/setup request within the last: - measIdleValidityDuration-r18 seconds for carriers configured in measIdleCarrierListNR-r16 or measIdleCarrierListEUTRA-r16, and/or - measReselectionValidityDuration-r18 seconds for carriers configured in measReselectionCarrierListNR-r18, - the measurement results satisfy measurement accuracy requirement at the measurement instance. Otherwise, the measurement results are considered invalid. The UE shall not report invalid measurement results when measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18 is configured. If the measIdleValidityDuration-r18 is not configured, the UE is not required to perform validity check for carriers in measIdleCarrierListNR-r16 and measIdleCarrierListEUTRA-r16, and the UE may report measurement results given the measurement results satisfy measurement accuracy requirement at the measurement instance. If the measReselectionValidityDuration-r18 is not configured, the UE is not required to perform validity check for carriers configured in measReselectionCarrierListNR-r18, and the UE may report measurement results given the measurement results satisfy measurement accuracy requirement at the measurement instance. RSRP, RSRQ measurements contained in the measurement reports shall meet the corresponding accuracy requirements at the measurement instance specified in the clauses 10.1.2B, 10.1.3B, 10.1.4B, 10.1.5B, 10.1.7B, 10.1.8B, 10.1.9B, 10.1.10B.

As agreed in Rel-18 eEMR framework, when measReselectionValidityDuration-r18 or measIdleValidityDuration-r18 is not configured the UE may report the measurement report given the measurement results satisfy specified accuracy requirements at the measurement instance. In this case, the measurement validity is left up to UE implementation.

When measReselectionValidityDuration-r18 or measIdleValidityDuration-r18 is not configured, validity check is left up to UE implementation.

Even when measReselectionValidityDuration-r18 or measIdleValidityDuration-r18 is configured, the condition is only time based, and it does not guarantee that SCell activation will be success.

Configuring measReselectionValidityDuration-r18 or measIdleValidityDuration-r18 does not guarantee the success of SCell activation.

From the UE perspective, it is beneficial to perform a meaningful validity check, and report measurement results that can be used for CA/DC setup, resume. Otherwise, in addition to unnecessary overhead of measurement report transmission, the UE will not be able to perform SCell setup in the real world, which leads to additional delay and waste of resources for both the UE and the network. Therefore, from SCell setup point of view, when the validityDuration is not configured may also lead to successful SCell setup/resume and they should be considered in the scope of this WI.

- Based on Rel-18 eEMR framework, if the UE has not been configured with measReselectionValidityDuration-r18 or measIdleValidityDuration-r18, the UE may report measurement results given the measurement results satisfy measurement accuracy requirement at the measurement instance.

- It is up to the UE implementation to check the validity of measurement when measReselectionValidityDuration-r18 or measIdleValidityDuration-r18 has not been configured.

- When the UE has not been configured with measReselectionValidityDuration-r18 or measIdleValidityDuration-r18, the UE is expected to check and report measurement results when they can be used for CA/DC setup/resume.

About the last meeting’s agreement.

Agreement: - The fast SCell activation delay requirements are defined for the case when - the UE supports Rel-18 eEMR: - Including both EMR and cell reselection measurement, - Including the case when measIdleValidityDuration-r18 and / or measReselectionValidityDuration-r18 are configured

In the previous meeting, it was agreed that the fast SCell activation delay requirements apply when either measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18 is configured. This agreement also accommodates scenarios where one of the validity durations is configured while the other is not. In such cases, the UE can still provide measurement reports for the configuration without the validity timer, allowing SCell addition/activation to be based on those measurements. Since CA/DC and cell reselection measurements are performed and reported independently, configuring a validity timer for one does not provide information about the other. Therefore, it doesn't matter whether one or both validity durations are not configured, and the scenario where neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured can still be supported.

Previous RAN4 agreements support fast SCell activation for a case when measIdleValidityDuration-r18 and / or measReselectionValidityDuration-r18 are not configured

To make scenarios consistent, RAN4 to define requirements for the case when measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18 are not configured in the work item.

Scope of fast SCell activation for UE supporting Rel-18 eEMR

Following issue still needs further discussion from the previous meeting regarding the scope of fast Scell activation.

#### Issue 1-1-3: Scope of fast SCell activation for UE supporting Rel-18 eEMR Agreement: - Start from the following scope of fast SCell activation for UE supporting Rel-18 EMR: - Direct SCell Activation at SCell addition - FFS: Single MAC CE based DL SCell Activation - FFS: Single MAC CE based PUCCH SCell Activation - RAN4 to discuss whether and how to apply the solutions to other SCell activation procedures after RAN4 has conclusions for the above cases.

In 5G NR, SCell activation via MAC-CE is a critical mechanism for dynamically activating and deactivating of SCells previously configured through RRC signaling. This allows for fast, dynamically and optimized use of radio resources, improved data throughput when needed, and efficient power and resource management for both the UE and the network. For the UE in the inactive mode, the network may either provide the UE with Scell configuration before the UE switching to the inactive mode, then once the UE switch back to the connected mode, it can then activate the scell using CA/DC measurement report performed during inactive mode. Also, applicable for both idle/inactive mode, the UE can be provided with the Scell configuration (in deactivated-mode) during/after RRC setup/resume procedure and activate later activate the scell through MAC-CE.

RAN4 to support Single MAC CE based DL SCell Activation for UE supporting Rel-18 eEMR.

RAN4 to support Single MAC CE based PUCCH SCell Activation.

Target scenarios of fast SCell activation

#### Issue 1-1-4: Target scenarios of fast SCell activation Proposals - Proposal 1: - RAN4 shall specify scenarios where the delay is less than 100 ms, even close to 20 ms (RRC setup/resume delay). - Fast SCell activation for UE supporting Rel-18 EMR should aim to achieve less than 100ms activation delay. RAN4 to discuss the options how this is achieved. - Proposal 2: - Suggest to focus on the following scenario for fast SCell activation for UE supporting Rel-18 EMR: The SCell being activated belongs to FR2 and there is no active serving cell on that FR2 band provided that PCell or PSCell is FR1. - Proposal 3: - it is proposed to reduce SCell activation delay for unknown case in FR1 and FR2-1.

Both FR1 and FR2 needs to be considered to determine the targeting scenarios. First, in Rel-18 EMR, both FR1 and FR2 are mainly considered to specify the related RAN4 requirements, e.g., SCell delay enhancements. The delay reduction in SCell activation is also required to both FR1 and FR2 cases. Activating SCell enables the UE to increase a throughput by adding a transmission stream using SCell. When transmitting a given size of data, the UE can reduce the transmission delay by activating and using SCell. Thus, RAN4 should study and analyze the potential activation delay reduction in FR1 and FR2, respectively.

The gain on the total transmission delay reduction can highly depend on the initial throughput of the UE associated with PCell. If the transmission over PCell has a low throughput, the additional throughput provided by SCell can further reduce the total transmission delay. Comparing PCells in FR1 and FR2, the UE associated with PCell in FR1 has a low throughput. In that case, activating SCell in FR2 can effectively increase the performance gain due to the wider bandwidth part in FR2 than that of FR1. Hence, RAN4 can prioritize the scenario of FR1 Pcell FR2 Pscell while studying other combinations of FR1 and FR2 scenarios.

RAN4 to consider all possible FR1 and FR2 scenarios for SCell activation, and the work can be initiated for FR1 Pcell FR2 Pscell/SCell.

Table 1: Gain in throughput from achieving shorter SCell setup delay when offered load is 40 Mbps

With a faster SCell/PScell setup in both FR1 and FR2, the usage of DC/CA enables the UE to increase a throughput and achieve a lower latency. Also, a short setup delay can reduce the energy consumption of the UE due to a faster traffic offloading and release of the connection. Therefore, fast DC/CA resume/setup is essential to optimize the load balancing between PCell and SCell in the network. Based on the system-level simulation results, we observe a significant gain on the throughput increment. For instance, the 95-percentile users’ throughput can be increased by up to 467% and 375% when the setup delay is 20 ms and 100 ms, respectively. Thus, RAN4 should specify the scenario to achieve the targeting RRC setup/resume delay less than 100 ms or even close to 20 ms.

RAN4 should specify scenarios where the delay is less than 100 ms, even close to 20 ms (RRC setup/resume delay).

Known and unknown conditions are used to define the SCell activation delay Tactivation_time. The SCell activation delay in an unknown-cell case is defined with a longer value than the known cell’s activation delay. For instance, the SCell activation is TFirstATRS+ 5ms if the measurement period of the SCell being activated is equal to or smaller than 2400 ms. Also, when the SCell in FR1 is an unknown cell, the SCell activation is increased to TFirstATRS + Tgap + TATRS + 5ms. Therefore, while minimizing the SCell activation delay of a known cell, the activation delay of an unknown cell also needs to be reduced to minimize the total transmission delay and energy consumption of the UE in the network.

RAN4 to study both known and unknown conditions for fast SCell activation with Rel-18 EMR.

How to define the fast SCell activation delay requirements with valid eEMR reporting

- RAN4 to update the known condition for SCell activation to include the case when UE has valid EMR reporting before SCell activation command. - FFS how to update the known condition (issue 1-2-1a). - FFS the applicability of valid EMR reporting for the Rel-19 fast SCell activation

The known/unknown condition of an SCell significantly impacts the scell activation time/delay. When an SCell is in a known condition, activation is faster because the UE has valid measurement data about the cell. In contrast, for an unknown SCell, the UE must perform measurements before activation, resulting in longer activation times. When the SCell is in a known condition, it is assumed that the UE already has recent and accurate information about the SCell radio conditions which allows the UE to quickly activate the SCell and report CSI after receiving the activation command. The activation process takes much longer in unknown scenario because the UE has to measure SCell evaluate it before completing the activation.

Requirements for SCell activation delay for deactivated SCell have been specified in 38.133 Section 8.3.2 and 8.3.7 for single and multiple DL Scells respectively. To make the best use of this feature using Rel-eEMR measurements, RAN4 may need to first, define known/unknown conditions/definitions of Rel-18 eEMR measurement report. In this regards, known/unknown conditions are specified as:

SCell in FR1 is known if it has been meeting the following conditions: - During the period equal to max(5*measCycleSCell, 5*DRX cycles) for FR1 before the reception of the SCell activation command: - the UE has sent a valid measurement report for the SCell being activated and - the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3. - the SSB measured during the period equal to max(5*measCycleSCell, 5*DRX cycles) also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3. Otherwise SCell in FR1 is unknown. For the first SCell activation in FR2 bands, the SCell is known if it has been meeting the following conditions: - During the period equal to 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable): - the UE has sent a valid L3-RSRP measurement report with SSB index, and - SCell activation command is received after L3-RSRP reporting and no later than the time when UE receives MAC-CE command for TCI activation - During the period from L3-RSRP reporting to the valid CQI reporting, the reported SSBs with indexes remain detectable according to the cell identification conditions specified in clauses 9.2 and 9.3, and the TCI state is selected based on one of the latest reported SSB indexes. Otherwise, the first SCell in FR2 band is unknown. The requirement for unknown SCell applies provided that the activation commands for PDCCH TCI, PDSCH TCI (when applicable), semi-persistent CSI-RS for CQI reporting (when applicable), and configuration message for TCI of periodic CSI-RS for CQI reporting (when applicable) are based on the latest valid L1-RSRP reporting.

Second, for the case that the Scell is considered as known the activation time is equal:

If the SCell is known and belongs to FR1, Tactivation_time is: - TFirstSSB+ 5ms, if the measurement period of the SCell being activated is equal to or smaller than 2400ms. - TFirstSSB_MAX + Trs + 5ms, if the measurement period of the SCell being activated is larger than 2400ms.

For SCell addition/activation, it is essential that measurement reports are meaningful and reflect the radio conditions. Meaningful measurements ensure that RRM decisions, such as SCell activation, are based on the most accurate and up-to-date information, minimizing the risk of dropped connections or degraded service quality. Consequently, in system design, the transmission of measurement reports is prioritized, and once the network receives an L3 measurement report, it expects the measurements to be relatively recent and accurate. However, for Rel-18 EMR, as described in Figure 1, due to the configuration of measReselectionValidityDuration-r18 or measIdleValidityDuration-r18 (which can range from 5 to 100 seconds), the measurement results may already be outdated, even when reported in connected mode. Therefore, the same assumptions regarding known and unknown conditions used for legacy L3 measurement reports cannot be applied to Rel-18 eEMR measurement reports.

Known and unknown conditions defined for legacy L3 measurement report, may not be applicable for Rel-18 eEMR

RAN4 to define new known and unknown conditions for Rel-18 eEMR.

Figure 1: Age of Rel-18 eEMR measurement.

Conclusion

In this paper, the following Observations and Proposals were made:

Observation 1: When measReselectionValidityDuration-r18 or measIdleValidityDuration-r18 is not configured, validity check is left up to UE implementation.

Observation 2: Configuring measReselectionValidityDuration-r18 or measIdleValidityDuration-r18 does not guarantee the success of SCell activation.

Observation 4: Previous RAN4 agreements support fast SCell activation for a case when measIdleValidityDuration-r18 and / or measReselectionValidityDuration-r18 are not configured

Proposal 1: To make scenarios consistent, RAN4 to define requirements for the case when measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18 are not configured in the work item.

Proposal 2: RAN4 to support Single MAC CE based DL SCell Activation for UE supporting Rel-18 eEMR.

Proposal 3: RAN4 to support Single MAC CE based PUCCH SCell Activation.

Proposal 4: RAN4 to consider all possible FR1 and FR2 scenarios for SCell activation, and the work can be initiated for FR1 Pcell FR2 Pscell/SCell.

Proposal 5: RAN4 should specify scenarios where the delay is less than 100 ms, even close to 20 ms (RRC setup/resume delay).

Proposal 6: RAN4 to study both known and unknown conditions for fast SCell activation with Rel-18 EMR.

Observation 5: Known and unknown conditions defined for legacy L3 measurement report, may not be applicable for Rel-18 eEMR

Proposal 7: RAN4 to define new known and unknown conditions for Rel-18 eEMR.