3GPP TSG-RAN WG4 Meeting #112bis	R4-2416290

Hefei, China, Oct 14 – Oct 18, 2024

Source:	Nokia

Title:	Discussion on LP-WUS Simulation assumptions and results

Agenda item:	6.23.4.1

Document for:	Discussion

Introduction

In this paper we discuss simulation assumptions and provide evaluation of number of LP-SS samples needed.

Discussion

SINR setting

Issue 2-1-1: SINR setting Agreement: - As starting point for RAN4 RRM simulation purpose only, -3dB, 0.5dB, 2dB dB Ês/Iot value is used for serving cell for both OOK based and OFDM based LP-WUR. - The SINR value can be updated if any update on the noise figure value in RF session. - FFS the side condition for RAN4 requirement.

Use -3dB, 0.5dB, 2dB as simulation assumptions

Agreement To determine the binary sequences for LP-SS, the evaluation assumes the following: - Sync accuracy: timing estimation error smaller than T us for P=90 % of the time for at least SNR=-3dB and SNR=-6dB (lower priority), - T=2us(optional), 5us for OOK-1 - T=1us(optional), 2us for OOK-4 with M=2 - T=0.5us(optional), 1us for OOK-4 with M=4 - Other values of SNR, T, M are up to company report. - Additionally, companies can submit results on SINR with details on how the interference was modelled. - Assume one-shot estimation - The time error is based on 20ppm maximum frequency error for the detection of the first LP-SS - Note: Companies can assume other values inside of 20ppm maximum as long as the values are justified - RRM measurement accuracy: measurement accuracy within range ± XdB for Q=90% measurements based on Y LP-SS samples within a period comparable to Z=the length of I-DRX cycle that is larger or equal to 1.28s for at least SNR=-3dB and SNR=-6dB (lower priority), X, Y, and Z is up to company report, other values of SNR are up to company report. - As a starting point, the time error is based on 5-10ppm residual frequency error. - Note: 5-10ppm assumes frequency error correction is performed, e.g., RTC calibration and/or MR assistance - Note: Companies can assume other values outside of 5-10ppm as long as the values are justified - The frequency error: up to company report - Sampling rate: 3.84MHz (optional) or 7.68MHz assumed for 30kHz SCS. - Channel: AWGN and TDL-C 300ns is assumed.

RAN1 assumptions have been using 5-10ppm

RAN4 parameters:

| Frequency offset relative to UE frequency reference | Hz | OFDM based receiver [5] ppm OOK based receiver [ 10 20] ppm | N/A |
| --- | --- | --- | --- |

Update Frequency offset relative to UE frequency reference [5, 10] ppm

RAN1 agreement: Agreement [H][FL1] Proposal 3.1-1: Confirm the Working Assumption that OOK-4 with M=4 for 15KHz SCS is supported for LP-WUS. Agreement For OOK-4 with M >1, support M=2 & M=4 (working assumption) for LP-WUS. - M=4 for 15KHz SCS - M=4 for 30KHz SCS (working assumption) - FFS M=1 for OOK-4.

M=4 and 30Khz is the RAN1 working assumption but also M=4 and 15KHz SCS is supported.

RAN4 simulation assumption:

| Subcarrier spacing | 30KHz initially TBD for 15 KHz |
| --- | --- |

Assume 30KHz in simulations but companies are may bring 15KHz SCS results, in addition to 30KHz SCS if they wish to do so.

Accuracy baseline for simulation

Accuracy of the LP-WUR measurements is one of the issues that has been discussed in the previous meetings. When the UE enters the LP-WUS mode, meaning that LP-WUS is being monitored instead of paging, and MR serving cell measurements are offloaded to LP-WUR, the decision to exit the LP-WUS mode is done based on LP-WUR. This means that if the accuracy of the LR is not well enough determined from the network point of view, the exit thresholds are set very conservatively by the networks. This means that the benefits of the feature will be diminishing for the UE also. Hence, defining realistic accuracy requirements for LP-WUR for both PSS/SSS based signal and LP-SS based signal is essential.

It is essential to define realistic accuracy requirements for LP-WUR to avoid network being required to configure conservative thresholds

The assumption of defining requirements is that LP-SS is based on OOK receiver and PSS/SSS is OFDM capable receiver. Both LP-SS and PSS/SSS should have their own accuracy requirements defined.

Table 10.1A.2.1.1-1: SS-RSRP defines accuracy requirements for 1Rx redcap where Ês/Iot is -6dB.

Discuss / evaluate if 1Rx (10.1A.2.1.1-1) Redcap accuracy requirements can be used as baseline

LP-RSRP measurement metric for LP-WUR

Traditionally, RSRP is used as the measurement metric to determine the cell coverage/range. In the LP-WUS context, it is one of the possible reliable metrics to determine the RRM relaxation thresholds. To study the LP-RSRP estimation accuracy, a time correlated fading of 1.28s is used with 1e6 samples for the LP-RSRP evaluation under TDL-C 300ns channel. In this evaluation, we aimed to determine the value of X, which represents the number of measurements needed within 1.28s that are spaced at equal interval. The following table shows the percentile distribution of LP-RSRP averaging under different SNR operating point. We have determined LP-RSRP based on the signal energy received during the ON duration corrected with the (noise and interference) energy received in OFF duration of Manchester encoded LP-SS, i.e., EON-EOFF.  Table 1 outlines the LP-RSRP error distribution with different operating SNR condition. The “Delta” column highlights half the deviation between 90%ile and 10%ile value. As can be seen that the LP-RSRP deviation is approximately within ±2.5dB error margin for values with X having 320ms or less.

From the table we can notice that the impact of fading on the LP-RSRP estimation accuracy improves as the number of samples used for averaging, i.e., X, increases from 1→16. Furthermore, the LP-RSRP estimation is robust against the SNR as there is only a little variation across when looking at each row. The impact of M on the LP-RSRP estimation accuracy is also not that drastic to gain any obvious attention. However, as the value of X increase from 1→4, the LP-RSRP estimation accuracy improves noticeably, whereas increasing from 4→16, the gain is only marginal. This ensures that the effect of fading can be normalized by taking at least X≥4 samples that are spread across 1.28sec interval.

Table 1. Measured LP-RSRP percentile values for Manchester encoded OOK with M∈{2,4}  and X∈1,2,4,8,16.

| M | X | interval |  | SNR 0dB |  |  | Delta |  | SNR 6dB |  |  | Delta |  | SNR -3dB |  |  |  | Delta |  |  |  | SNR -9dB |  |  |  | Delta |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (ms) |  | 10%ile | 50%ile | 90%ile |  |  | 10%ile | 50%ile | 90%ile |  |  | 10%ile | 50%ile | 90%ile |  |  |  |  | 10%ile |  | 50%ile | 90%ile |  |  |
| 2 | 16 | 80 |  | -2,15 | -0,39 | 1,28 | 1,715 |  | -2,1 | -0,36 | 1,29 | 1,695 |  | -2,22 | -0,45 | 1,24 | 1,73 |  |  |  | -2,41 |  | -0,6 | 1,1 | 1,755 |  |
| 2 | 8 | 160 |  | -2,31 | -0,44 | 1,36 | 1,835 |  | -2,25 | -0,4 | 1,37 | 1,81 |  | -2,38 | -0,48 | 1,33 | 1,855 |  |  |  | -2,59 |  | -0,65 | 1,18 | 1,885 |  |
| 2 | 4 | 320 |  | -3,26 | -0,58 | 1,72 | 2,49 |  | -3,17 | -0,54 | 1,75 | 2,46 |  | -3,32 | -0,64 | 1,71 | 2,515 |  |  |  | -3,56 |  | -0,82 | 1,6 | 2,58 |  |
| 2 | 2 | 640 |  | -4,81 | -0,9 | 2,35 | 3,58 |  | -4,7 | -0,87 | 2,36 | 3,53 |  | -4,93 | -0,96 | 2,32 | 3,625 |  |  |  | -5,18 |  | -1,14 | 2,18 | 3,68 |  |
| 2 | 1 | 1280 |  | -7,16 | -1,58 | 3,03 | 5,095 |  | -7,02 | -1,53 | 3,05 | 5,035 |  | -7,24 | -1,65 | 3,02 | 5,13 |  |  |  | -7,73 |  | -1,88 | 2,89 | 5,31 |  |
| 4 | 16 | 80 |  | -2,27 | -0,47 | 1,22 | 1,745 |  | -2,17 | -0,41 | 1,27 | 1,72 |  | -2,38 | -0,56 | 1,17 | 1,775 |  |  |  | -2,63 |  | -0,78 | 0,98 | 1,805 |  |
| 4 | 8 | 160 |  | -2,42 | -0,51 | 1,3 | 1,86 |  | -2,31 | -0,44 | 1,34 | 1,825 |  | -2,53 | -0,58 | 1,25 | 1,89 |  |  |  | -2,83 |  | -0,8 | 1,08 | 1,955 |  |
| 4 | 4 | 320 |  | -3,38 | -0,66 | 1,69 | 2,535 |  | -3,26 | -0,59 | 1,72 | 2,49 |  | -3,53 | -0,75 | 1,63 | 2,58 |  |  |  | -3,8 |  | -0,98 | 1,46 | 2,63 |  |
| 4 | 2 | 640 |  | -5,01 | -0,99 | 2,3 | 3,655 |  | -4,81 | -0,91 | 2,33 | 3,57 |  | -5,2 | -1,07 | 2,25 | 3,725 |  |  |  | -5,57 |  | -1,33 | 2,12 | 3,845 |  |
| 4 | 1 | 1280 |  | -7,5 | -1,69 | 2,97 | 5,235 |  | -7,22 | -1,61 | 3,02 | 5,12 |  | -7,67 | -1,81 | 2,97 | 5,32 |  |  |  | -8,64 |  | -2,04 | 2,79 | 5,715 |  |

The objective is to determine the number of distinct LP-SS based measurement samples that are spaced at equal interval within a period of 1.28s, to normalize the effect of channel fading. As the fading varies slowly in case of static UEs, it is evaluated over a period of 1.28s, which is several times greater than the coherence time of 3kmph UE speed. Thus, the estimated LP-RSRP is a true estimate of path loss, and thus normalizing the effect of fading channel. The following figure illustrates the LP-RSRP estimation error with ideal path loss. It can be seen that LR may need more than four measurements using LP-SS that are spread across 1.28s to obtain a reliable LP-RSRP estimate that characterizes the actual path loss.

Fig. 1 LP-RSRP evaluation over 1.28s using X sample averages on LP-SS in TDL-C 300ns.

The effect of OOK modulation order M is not visible in the LP-RSRP estimation accuracy. Thus, using the above evaluations, we may need four measurements that are equally spaced between 1.28s to obtain a reliable LP-RSRP estimation that limits the error ≤3dB in fading conditions.

Depending on the operating SNR, a LP-RSRP presentative of the pathloss can be obtained using X ≥4 LP-SS samples that are spread over 1.28s at equal interval.

A minimum of X≥4 LP-SS samples are required to estimate LP-RSRP reliably irrespective of the operating SNR.

Our simulations indicate that X≥4 LP-SS samples are required to estimate LP-RSRP reliably irrespective of the operating SNR.

Conclusion

The following Observations and Proposals were made:

Proposal 1: Use -3dB, 0.5dB, 2dB as simulation assumptions

Proposal 2: Update Frequency offset relative to UE frequency reference [5, 10] ppm

Observation 1: M=4 and 30Khz is the RAN1 working assumption but also M=4 and 15KHz SCS is supported.

Proposal 3: Assume 30KHz in simulations but companies are may bring 15KHz SCS results, in addition to 30KHz SCS if they wish to do so.

Observation 2: It is essential to define realistic accuracy requirements for LP-WUR to avoid network being required to configure conservative thresholds

Proposal 4: Discuss / evaluate if 1Rx (10.1A.2.1.1-1) Redcap accuracy requirements can be used as baseline

Proposal 5: Our simulations indicate that X≥4 LP-SS samples are required to estimate LP-RSRP reliably irrespective of the operating SNR.