3GPP TSG-RAN WG4 Meeting #112	R4-2416292

Hefei, China, 14-18 October 2024

Source:	Nokia

Title:	Discussion on Measurements related enhancements for LTM

Agenda item:	6.24.2.1

Document for:	Discussion

Introduction

NR mobility enhancements Phase 4 work in RAN4 is set to start in the RAN4#112 meeting. The scope of the work item is to specify RRM requirements for the WI objectives.

In the plenary meeting #105, the scope for the earlier set WID has been further extended to support CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch and dual connectivity.

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure. - Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Configuration of conditional LTM is limited to the below cases: - UE is in non-DC case. - In case UE is configured with DC, configuration of conditional LTM is only in the SCG. - Specify RRM requirements related to the above objectives as necessary [RAN4]

In this contribution we discuss our views of the RRM impact of these objectives.

Measurement enhancements for LTM

The objectives of measurement enhancements for LTM are:

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs. Review progress at RAN#105 to see if any modification of objectives is required to avoid/manage any overlap in the work - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1]

Event-triggered L1-reporting

In Rel-18, RAN4 defined measurement requirements for SSB-based LTM L1 measurements in sections 9.14 and 9.15. Related reporting requirements were defined according to RAN1/RAN2 support for periodic, semi-persistent and aperiodic reporting. In Rel-19, RAN4 needs to define reporting requirements also for event-triggered L1 reporting for LTM candidate cells.

The discussion about event-triggered L1 reporting requirements started in the last RAN4 meeting, but no agreements were reached due to lack of relevant RAN2/RAN1 agreements at that time. As a summary of the latest status of the RAN2 and RAN1 agreements that are relevant to the RAN4 work, the framework of L1 event-triggered reporting after the August meeting is:

- Supported events:

- Event LTM2: Beam of serving cell becomes worse than absolute threshold;

- Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

- Event LTM4: Beam of candidate cell becomes better than absolute threshold;

- Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

- Beam level measurement result is used for LTM event evaluation

  - Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell. RAN1 is still discussing the option of using RS derived from one of the active TCI states.

  - Any beam in candidate RS configuration can be used for LTM event evaluation.

  - (Working assumption) Same RS type should be used for serving and candidate cell for event evaluation in events LTM3 and LTM5.

- SSB and CSI-RS based L1-RSRP measurements are supported for event-triggered L1 reporting. FFS CSI-RS based L1-SINR

- TTT/hysteresis based filtering of L1 measurements is assumed. FFS the details, and the additional filtering from RAN1.

According to RAN2 agreements, event-triggered L1 reporting covers both SSB and CSI-RS based reporting. Event triggered reporting delay requirements would comprise a new section under the existing sections 9.14.3 and 9.15.3, and new sections under CSI-RS based LTM L1 measurement requirements, which are to be defined.

RAN4 to define event-triggered L1 reporting delay requirement for SSB based and CSI-RS based measurements.

When looking at the existing L3 event triggered reporting delay principles, this comprises the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. For example, for intra-frequency L3 measurements, event triggered measurement reporting delay measured without L3 filtering shall be less than Tidentify intra with index = TPSS/SSS_sync_intra + T SSB_measurement_period_intra  ms or T identify intra without index = TPSS/SSS_sync_ntra + T SSB_measurement_period_intra + TSSB_time_index_intra ms. Additional delays may be expected due to L3 filtering, and the delay does not take into account uncertainties related to for example other RRC signalling on the DCCH or availability of UL resources. The type of event that is evaluated and triggers the report is not taken into account in the reporting delay requirement.

For event-triggered L1 reporting, the principle of L3 event triggered reporting may be used as the starting point. However, as the reporting conditions are evaluated based on L1 measurements, the reporting delay would obviously comprise L1 measurement period instead of L3 cell identification period.

It is reasonable that RAN4 starts the discussion from the SSB-based reporting as measurement requirements already exist for this case from Rel-18. CSI-RS based measurement requirements are to be defined in Rel-19. Event-triggered reporting delay for CSI-RS based measurements depends on the measurement delay, and it is difficult to discuss the delay with details before the CSI-RS based measurement delay requirements for LTM candidate cells have been defined.

RAN4 to start the discussion about event-triggered L1 reporting delay requirements from SSB-based reporting. CSI-RS based reporting delay discussion to follow based on the Rel-19 work on measurement requirements for CSI-RS based LTM measurements.

Following L3 event-triggered reporting definition, the delay is defined as the time from the moment the event condition becomes fulfilled until the UE starts sending the report over the air interface. This principle can likely be reused. The time it takes for the UE to realize the reporting condition is met is to be covered through L1 measurement period. Rel-18 LTM L1 measurement delay TL1-RSRP_Measurement_Period_SSB_intra, TL1-RSRP_Measurement_Period_SSB_inter or TL1-RSRP_Measurement_Period_SSB_inter_withoutGap can be used as the starting point for the reporting delay for SSB-based event-triggered reporting. Similar to L3 reporting delay, with the current RAN1/RAN2 agreements we do not expect that the event type has an impact on the reporting delay. This view may of course change in case there are any further RAN1/RAN2 agreements that would imply to that direction.

The L1 event-triggered reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface.

Starting point for the SSB-based LTM L1 event triggered reporting delay is TL1-RSRP_Measurement_Period_SSB_intra, TL1-RSRP_Measurement_Period_SSB_inter or TL1-RSRP_Measurement_Period_SSB_inter_withoutGap.

Any uncertainties in the reporting delay need to be discussed based on RAN1/RAN2 reporting design. For example, RAN1/RAN2 are currently discussing that some type of L1 filtering (e.g., TTT based) is to be considered for event-evaluation. The details of such L1 filtering are not clear yet, but this is expected to have an impact on the RAN4 definition of the reporting delay. Furthermore, RAN4 should follow up on other details of the event evaluation and report triggering, such as details of time to trigger (TTT). At the moment we do not see that the RAN1/RAN2 agreements on these details are sufficient to progress the RAN4 work.

The impact of RAN1/RAN2 event evaluation and reporting design (such as L1 filtering based on TTT etc.) to the event-triggered reporting delay is up to further RAN1/RAN2 agreements.

Regarding the accuracy of the report, which is part of the reporting requirements, RAN4 needs to wait for more details about the contents of the report. If L1-RSRP (or L1-SINR) value is reported, the accuracy should follow and can be verified similar to legacy. However, an option not to report the measurement result is also discussed in RAN1, in which case this may not be the case.

RAN4 to wait for RAN1 agreements about the reporting details, such as whether a measurement result (L1-RSRP or L1-SINR) is included in the report.

In addition to the event-triggered reporting delay requirements, RAN4 eventually needs to discuss the impact of event-triggered reporting to other LTM requirements. Rel-18 LTM discussion focused on the assumption of periodic and network-controlled reporting, and the requirements were written accordingly. For example, cell switch delay requirements assume a recent L1 or L3 report to be present for the requirements to apply. With event-triggered reporting there may not be reports available regularly, and the network is not aware when the report will be triggered and sent. It is also still unclear whether event becoming fulfilled may lead to one or more (periodic) event-triggered reports. In any case, we think RAN4 will need to discuss whether the introduction of event-triggered L1 reporting has an impact at least on known TCI state condition for early TCI state activation and cell switch, and the applicability of cell switch delay requirements

RAN4 to discuss potential impact of event-triggered L1 reporting to other LTM requirements such as:
- Known TCI state condition for early TCI state activation and cell switch
- Applicability of cell switch delay requirements

Even further, one benefit of event-triggered reporting is that it gives a direct indication to the network about a candidate cell being a potential target for cell switch. Hence, RAN4 may discuss whether event-triggered reporting condition becoming fulfilled may also be considered as a trigger for some of the preparation steps for LTM, for example early DL sync, if not valid at the time of reporting condition becoming fulfilled, and early ASN.1 decoding and validity check. RAN4 should also discuss whether LTM L1 measurement delay requirements are impacted by event triggered reporting. For example, are the requirements the same before and after a report is sent or should similar relaxation as in Rel-18 requirements due to TCI state activation be considered after the reporting is triggered.

RAN4 to consider the impact of L1 event triggered reporting to:
- early DL sync (Tfirst-RS = 0 conditions)
- early ASN.1 decoding and validity check
- LTM L1 measurement requirements

CSI-RS measurements for LTM

RAN1 has made the following agreements:

Agreement - Support L1-RSRP measurement based on CSI-RS - FFS: Support L1-SINR measurement based on CSI-RS Agreement - CSI-RS based L1-RSRP report is supported for gNB scheduled measurement reporting - FFS: CSI-RS based L1-SINR report is supported for gNB scheduled measurement reporting - Rel-18 LTM CSI reporting framework is the baseline for CSI-RS based L1-measurement report by gNB scheduled measurement reporting Agreement For gNB scheduled reporting and event triggered reporting - At least periodic CSI-RS is supported for L1-RSRP measurement for candidate cell - FFS: aperiodic and semi-persistent CSI-RS - At least CSI-RS for beam management is supported for L1-RSRP measurement for candidate cell - FFS: CSI-RS for mobility

One of the targets of CSI-RS based LTM cell switch is to enable UE being scheduled with higher throughput right after the cell switch, hence the CSI-RS measured should be the same as is used for beam management after the cell switch. Based on the current RAN1 agreements, L1-RSRP based CSI-RS measurements are supported, and at least periodical measurements are supported. RAN4 can start to discuss requirements for periodic measurements already for CSI-RS for L1-RSRP (beam management measurements) in the scope of LTM.

RAN4 to define requirements at least for periodic L1-RSRP CSI-RS measurements before and during the cell switch.

In the last meeting, companies were discussing which measurement types to support. From our at least intra-frequency measurements, and inter-frequency measurements without gap shall be supported. Whether to support inter-frequency with gap measurements, can be discussed once the measurement procedure for at least in intra-frequency scenario is clear.

RAN4 to define intra-frequency CSI-RS measurement requirements first.

Support inter-frequency L1-RSRP CSI-RS measurements without gap. FFS for inter-frequency with gap.

As discussed in the last meeting, there is a need to define CSI-RS measurement period requirements. As a starting point, RAN4 may discuss how many intra-frequency candidate cells UE is expected to measure before the cell switch.

Discuss the number of candidate cells and beams UE is expected to measure for intra-frequency L1-RSRP CSI-RS measurements.

Regarding the definition of intra-frequency L1-RSRP CSI-RS, at least the following conditions may be reused.

For intra-frequency the following conditions apply:

  - the centre frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of the CSI-RS resource of the serving cell indicated for L1 measurement, and

  - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of the CSI-RS resource on the serving cell indicated for L1 measurement,

Regarding the frequency ranges, we can follow the rel-18 baseline and not to define requirements for FR2-2.

Introduce support for L1-RSRP CSI-RS measurements for FR1 and FR2-1

Issue 2-3-5: RTD assumption - Candidate solutions: - Option 1: RTC<CP is taken as baseline. For the case of RTD>CP between serving cell and neighbor cell on the same carrier, UE capability should be introduced. (Apple) - Option 2: Further study whether to support RTD>CP case for CSI-RS based L1-RSRP measurement. (MTK) - Recommended WF - Continue discussion.

In rel-18 separate requirements were defined for RTD>CP and RTD < CP cases. The requirements for RTD>CP apply on for UEs that support the capability of L1 measurements with RTD>CP. The problem with these scenarios and requirements is that the network (still) does not know the RTD conditions the UE is observing. Making guarantees on RTD by deployment, for example only supporting intra-DU or collocated scenarios limits severely where LTM can be deployed. Therefore, from the network point of view, it is challenging to configure and use LTM for UEs that support only RTD < CP as there are no requirements for UEs when the actual RTD conditions are above CP.  A natural, but not particularly desirable, option for the network is to only configure LTM for UEs that support RTD > CP capabilities, and to use legacy handover for UEs not supporting the capability.

Network does not know UE RTD conditions, hence the RTD > CP capability is a prerequisite for LTM support in some networks

One good option is to only define requirements for UEs supporting RTD > CP in rel-19.

As the network does not know UE RTD conditions, requirements for RTD > CP shall be defined for rel-19 LTM. Whether the support of these requirements is up to UE capability or not can be FFS.

Issue 2-3-10: other applicability of RRM requirements for CSI-RS based L1 RSRP measurement - Candidate solutions: - P1: Consider SSB based L3 measurement as the pre-requisite condition to determine the target cells for CSI-RS based L1-RSRP measurement. (Apple) - P1a: In FR1, UE shall first perform SSB based L3 measurement on candidate neighbour cells. Then UE performs configured CSI-RS L1 measurement on these candidate neighbour cells. (HW) - P2: For L1 CSI-RS measurement on candidate neighbour cells in FR2, RAN4 needs to further discuss two possible options: (HW) - Option 1: UE performs SSB based L3 measurements and acquires SSB index information of the candidate cells. And then UE receives the configured CSI-RS resources and perform L1 CSI-RS measurement with Rx beam sweeping on candidate cells. - Option 2: UE performs SSB based L3 measurements and acquires SSB index information of the candidate cells. And then UE performs SSB based L1 measurement to refine the RX beam (with RX beam sweeping) which is the same as R18 LTM. Afterwards UE perform measurements without RX beam sweeping on the configured CSI-RS resources of candidate cells where each CSI-RS resource is QCL-typeD with SSB for L1-RSRP measurement. - P3: For CSI-RS resources with repetition OFF, L1-RSRP measurement is performed only after UE has performed L1-RSRP measurement on the associated SSB. (MTK) - Recommended WF - Continue discussion.

In rel-18 FR1, L1 SSB measurement is not a prerequisite for gNB to trigger the cell switch as the gNB can trigger cell switch based on L3 measurement. Only in FR2, L1 SSB measurement is a prerequisite for cell switch. Therefore, both cases should be supported from requirements point of view.

L3 SSB measurements can be considered to be performed before CSI-RS measurements can be performed.

L3 measurements are performed before CSI-RS measurements on candidate cells.

Support scenario where L3 SSB measurements are performed first, and L1 CSI-RS measurements are performed subsequently without Rel-18 LTM SSB-based L1-RSRP measurements. FFS if the scenario applies for both FR1 and FR2.

For FR1 and FR2, support scenario where L1 CSI-RS measurements are performed after Rel-18 LTM L1 SSB measurements

Conclusion

The following Observations and Proposals were made:

Proposal 1: RAN4 to define event-triggered L1 reporting delay requirement for SSB based and CSI-RS based measurements.

Proposal 2: RAN4 to start the discussion about event-triggered L1 reporting delay requirements from SSB-based reporting. CSI-RS based reporting delay discussion to follow based on the Rel-19 work on measurement requirements for CSI-RS based LTM measurements.

Proposal 3: The L1 event-triggered reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface.

Proposal 4: Starting point for the SSB-based LTM L1 event triggered reporting delay is TL1-RSRP_Measurement_Period_SSB_intra, TL1-RSRP_Measurement_Period_SSB_inter or TL1-RSRP_Measurement_Period_SSB_inter_withoutGap.

Proposal 5: The impact of RAN1/RAN2 event evaluation and reporting design (such as L1 filtering based on TTT etc.) to the event-triggered reporting delay is up to further RAN1/RAN2 agreements.

Proposal 6: RAN4 to wait for RAN1 agreements about the reporting details, such as whether a measurement result (L1-RSRP or L1-SINR) is included in the report.

Proposal 7: RAN4 to discuss potential impact of event-triggered L1 reporting to other LTM requirements such as: - Known TCI state condition for early TCI state activation and cell switch - Applicability of cell switch delay requirements

Proposal 8: RAN4 to consider the impact of L1 event triggered reporting to: - early DL sync (Tfirst-RS = 0 conditions) - early ASN.1 decoding and validity check - LTM L1 measurement requirements

Proposal 9: RAN4 to define requirements at least for periodic L1-RSRP CSI-RS measurements before and during the cell switch.

Proposal 10: RAN4 to define intra-frequency CSI-RS measurement requirements first.

Proposal 11: Support inter-frequency L1-RSRP CSI-RS measurements without gap. FFS for inter-frequency with gap.

Proposal 12: Discuss the number of candidate cells and beams UE is expected to measure for intra-frequency L1-RSRP CSI-RS measurements.

Proposal 13: For intra-frequency the following conditions apply:

	the centre frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of the CSI-RS resource of the serving cell indicated for L1 measurement, and

	the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of the CSI-RS resource on the serving cell indicated for L1 measurement,

Proposal 14: Introduce support for L1-RSRP CSI-RS measurements for FR1 and FR2-1

Observation 1: Network does not know UE RTD conditions, hence the RTD > CP capability is a prerequisite for LTM support in some networks

Proposal 15: As the network does not know UE RTD conditions, requirements for RTD > CP shall be defined for rel-19 LTM. Whether the support of these requirements is up to UE capability or not can be FFS.

Proposal 16: L3 measurements are performed before CSI-RS measurements on candidate cells.

Proposal 17: Support scenario where L3 SSB measurements are performed first, and L1 CSI-RS measurements are performed subsequently without Rel-18 LTM SSB-based L1-RSRP measurements. FFS if the scenario applies for both FR1 and FR2.

Proposal 18: For FR1 and FR2, support scenario where L1 CSI-RS measurements are performed after Rel-18 LTM L1 SSB measurements