3GPP TSG-RAN4 Meeting #112bis	R4-2416293

# Hefei, Anhui, China, 14th – 18th October, 2024

Title:	Discussion on coexistence studies and preliminary results

Source:	Samsung

Agenda item:	6.8.2.1

Document for:	Approval

1	Introduction

In the RAN4#112 meeting, the WF R4-2414452 [1] had achieved the baseline co-existence assumptions on HPUE. In this paper, we will provide the preliminary results derived from the baseline assumptions and continue discussing the left open options for some assumptions.

The observations based on these results are proposed in this paper for further discussions.

2	NR-NTN HPUE

Based on the assumptions agreed  in [1] and the reference TR 38.863, we have produced the following results for scenario 4.

Table 1-1. Throughput loss w.r.t. different ACLR for PC 3/2/1.5/1 and Scenario 4

|  | Average Throughput Loss (%) |  |  |  |  | Cell-edge Throughput Loss (%) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ACLR | 20 | 25 | 30 | 35 | 40 | 20 | 25 | 30 | 35 | 40 |
| PC3 | 1.91 | 0.97 | 0.49 | 0.25 | 1.31 | 9.31 | 4.84 | 2.53 | 1.23 | 0.67 |
| PC2 | 2.79 | 1.44 | 0.73 | 0.37 | 0.20 | 13.49 | 6.67 | 3.48 | 1.75 | 1.00 |
| PC1.5 | 4.07 | 2.16 | 1.11 | 0.57 | 0.30 | 20.88 | 10.22 | 5.24 | 2.71 | 1.49 |
| PC1 | 5.23 | 2.83 | 1.48 | 0.77 | 0.42 | 26.86 | 13.81 | 7.11 | 3.80 | 2.18 |

From above results, the suggested ACLR from Scenario 4 for each power class can be summarized in the table below.

Table 1-2. Suggested NR-NTN HPUE ACLR for PC 2/1.5/1 from Scenario 4

| Power Class | Suggested ACLR (dBc) |
| --- | --- |
| 3 | 24.8 |
| 2 | 27.6 |
| 1.5 | 30.5 |
| 1 | 33.2 |

Observation 1: The results from scenario 4 suggest the ACLR for PC3 can be 24.8 dBc; PC2 can be 27.6 dBc; and for PC1.5 can be 30.5 dBc, and 33.2 dBc for PC1 for NR-NTN HPUE.

Proposal 1: For NR-NTN power class 2, our results from scenario 4 suggest ACLR as 27.6 dBc.

Proposal 2: For NR-NTN power class 1.5, our results from scenario 4 suggest ACLR as 30.5 dBc.

Proposal 3: For NR-NTN power class 1, our reuslts from scenario 4 suggest ACLR as 33.2 dBc.

3	IoT-NTN HPUE

Based on the assumptions agreed  in [1] and the reference WF 	R4-2217473, we have produced the following results for scenario 4.

Table 2-1. Throughput loss w.r.t. different ACLR for PC 3/2/1 and Scenario 4

| 36 x 3.75kHz | Average Throughput Loss (%) |  |  |  |  | Cell-edge Throughput Loss (%) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ACLR | 20 | 25 | 30 | 35 | 40 | 20 | 25 | 30 | 35 | 40 |
| PC3 | 1.21 | 0.63 | 0.36 | 0.24 | 0.19 | 6.10 | 3.13 | 1.47 | 0.72 | 0.44 |
| PC2 | 1.73 | 0.91 | 0.52 | 0.35 | 0.29 | 8.33 | 4.60 | 2.48 | 1.44 | 1.02 |
| PC1.5 | 2.78 | 1.49 | 0.87 | 0.59 | 0.49 | 13.54 | 7.51 | 4.32 | 2.86 | 2.25 |

Table 2-2. Throughput loss w.r.t. different ACLR for PC 3/2/1 and Scenario 4

| 9 x 15kHz | Average Throughput Loss (%) |  |  |  |  | Cell-edge Throughput Loss (%) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ACLR | 20 | 25 | 30 | 35 | 40 | 20 | 25 | 30 | 35 | 40 |
| PC3 | 0.32 | 0.17 | 0.09 | 0.06 | 0.05 | 1.55 | 0.83 | 0.37 | 0.18 | 0.11 |
| PC2 | 0.50 | 0.26 | 0.15 | 0.10 | 0.08 | 2.34 | 1.25 | 0.71 | 0.40 | 0.28 |
| PC1.5 | 0.93 | 0.49 | 0.28 | 0.19 | 0.16 | 4.58 | 2.42 | 1.47 | 1.03 | 0.84 |

From above results, the suggested ACLR from Scenario 4 for each power class can be summarized in the table below.

Table 2-3. Suggested IoT-NTN HPUE ACLR for PC 2/1 from Scenario 4

| Power Class | Suggested ACLR (dBc) |
| --- | --- |
| 3 | 21.9 |
| 2 | 24.5 |
| 1 | 28.9 |

Observation 2: The results from scenario 4 suggest the ACLR for PC3 can be 21.9 dBc; PC2 can be 24.5 dBc; and for PC1 can be 28.9 dBc for IoT-NTN HPUE.

Proposal 4: For IoT-NTN power class 2, our results from scenario 4 suggest ACLR as 24.5 dBc.

Proposal 5: For IoT-NTN power class 1, our reuslts from scenario 4 suggest ACLR as 28.9 dBc.

Observation 3: The above results considered the baseline assumptions, and they should be further discussed together with the submitted results based on other optional assumptions.

4	Uplink power control options

Besides the baseline assumptions that [1] agreed, there are also several key optional assumptions which will impact the co-ex study results and worth to be further discussed. And the uplink power control options is one of them.

In [1], there are one uplink power control option proposed as below:

Issue 2-4-4: NTN HPUE uplink power control - Agreement - To update the existing uplink power control parameters considering different power classes and bandwidth (Samsung, Qualcomm, Huawei) - Update Pmax to 26/29/31 for PC2/1.5/1; - Update Rmin to -66/-69/-71 assuming Pmin as -40; - Update BW to corresponding transmission BW. - Agree on the following detailed parameters where: - Pmax = 26dBm for PC2, 29 dBm for PC1.5, 31 dBm for PC1. - Rmin = -66 dB for PC2, -69 dB for PC1.5, -71 dB, assuming -40dBm minimum output power, - CLx-ile and γ are set as following: - CLx-ile = 10*log10(Pmax) – (SNRtarget + 10*log10(kTB) + NF ), - Where: - SNRtarget and BW For NR-NTN HPUE: - Option 1: 3 dB SNR target and 5MHz UL BW (One company proposed for LEO only) - Option 2: 15 dB SNR target and 2RB UL BW as baseline assumption - NF is the SAN noise figure, i.e. 4.3dB. - 10log10(kT) = -174dBm/Hz. - γ = 1 For uplink scenario.

If we take the Coupling loss calibration table [4] from Rel-17 studies as a reference, the LEO-600 UL cablirated coupling loss from multiple companies is shown in the table below:

And we take a simple link budget analysis for the baseline option (SNR target 15dB and UL BW 2RB) and the optional one (SNR target 3dB and UL BW 5MHz), it can be found as below:

| Options | Baseline |  |  | Optional |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BW | 2RB |  |  | 5MHz |  |  |
| Noise | -118.44 dBm |  |  | -107.01 dBm |  |  |
| Satellite NF | 4.3 dB |  |  |  |  |  |
| Coupling loss | 118.85 | 125.6 | 132.41 | 118.85 | 125.6 | 132.41 |
| SNR (23dBm) | 18.27 | 11.57 | 4.73 | 6.86 | 0.11 | -6.70 |
| SNR (26dBm) | 21.27 | 14.57 | 7.73 | 9.86 | 3.11 | -3.70 |
| SNR (29dBm) | 24.27 | 17.57 | 10.73 | 12.86 | 6.11 | -0.70 |
| SNR (31dBm) | 26.27 | 19.57 | 12.73 | 14.86 | 8.11 | 1.3 |

Observation 4: It can be seen from above analysis, that adopting the wider UL Tx bandwidth would compensate the benefits from incorporating a lower target SNR. The bold numbers in the table still requires a total burst of Tx power to reach the SNR target in baseline and optional cases repsectively.

Observation 5: From SLS point of view, our preliminary observation is that this optional SNR target as 3 dB and UL BW as 5MHz option will not dramatically reduce the UL Tx power distribution in the studied power classes, and hence it will not impact the co-ex study results much.

Proposal 6: Based on our observations 4/5, the differences from uplink power control options (w.r.t. SNR targets and UL bw) in co-ex study resutls could be marginal. It is proposed to use the baseline assumption for co-ex studies to align with previous studies.

5	Conclusion

For the NTN HPUE co-ex studies, we propose the following.

Observation 1: The results from scenario 4 suggest the ACLR for PC3 can be 24.8 dBc; PC2 can be 27.6 dBc; and for PC1.5 can be 30.5 dBc, and 33.2 dBc for PC1 for NR-NTN HPUE.

Proposal 1: For NR-NTN power class 2, our results from scenario 4 suggest ACLR as 27.6 dBc.

Proposal 2: For NR-NTN power class 1.5, our results from scenario 4 suggest ACLR as 30.5 dBc.

Proposal 3: For NR-NTN power class 1, our reuslts from scenario 4 suggest ACLR as 33.2 dBc.

Observation 2: The results from scenario 4 suggest the ACLR for PC3 can be 21.9 dBc; PC2 can be 24.5 dBc; and for PC1 can be 28.9 dBc for IoT-NTN HPUE.

Proposal 4: For IoT-NTN power class 2, our results from scenario 4 suggest ACLR as 24.5 dBc.

Proposal 5: For IoT-NTN power class 1, our reuslts from scenario 4 suggest ACLR as 28.9 dBc.

Observation 3: The above results considered the baseline assumptions, and they should be further discussed together with the submitted results based on other optional assumptions.

Observation 4: It can be seen from above analysis, that adopting the wider UL Tx bandwidth would compensate the benefits from incorporating a lower target SNR. The bold numbers in the table still requires a total burst of Tx power to reach the SNR target in baseline and optional cases repsectively.

Observation 5: From SLS point of view, our preliminary observation is that this optional SNR target as 3 dB and UL BW as 5MHz option will not dramatically reduce the UL Tx power distribution in the studied power classes, and hence it will not impact the co-ex study results much.

Proposal 6: Based on our observations 4/5, the differences from uplink power control options (w.r.t. SNR targets and UL bw) in co-ex study resutls could be marginal. It is proposed to use the baseline assumption for co-ex studies to align with previous studies.

6	Reference

[1] R4-2414452, WF on UE RF requirements for NTN HPUE, Samsung

[2] TR 38.863

[3] R4-2217473

[4] R4-2112016, NR-NTN calibration summary and observations, Samsung