3GPP TSG-RAN4 Meeting #112bis	R4-2416295

# Hefei, Anhui, China, 14th – 18th October, 2024

Title:	Discussion on NR-NTN HPUE Rx requirements

Source:	Samsung

Agenda item:	6.8.2.2.2

Document for:	Approval

1	Introduction

In the RAN4#112 meeting, the WF R4-2414452 [1] had initially discussed the reference sensitivity degradation (RSD) requirements for NTN HPUE as following:

Issue 4-2: RSD for NR-NTN - Agreement - Reference sensitivity degradation (RSD) requirements is needed for NTN HPUE. - Further discuss the following aspects: - Use similar way in TN spec by introducing a new table of RSD compared to the PC3 - Whether default Tx-Rx frequency separation is used for test point

In this paper, we continue to discuss the RSD requirements for NR-NTN HPUE.

2	Discussions

The power classes of HPUEs, including PC3, 2, 1.5 and 1, generally impact the reference sensitivity as discussed. The factors of the RSD include the RFIC characteristics and duplexer and PA characteristics impact by bearing higher power level for HPUE.

The existing TN spec already considered the noise floor increase by above mentioned factors, and the degradation is allowed by introducing a table to relax the referense sensitivity requirements with certain levels for each power classes.

The Tx-Rx frequency separation for the currently defined NR-NTN bands is listed as follows:

5.4.4 TX–RX frequency separation The default TX channel (carrier centre frequency) to RX channel (carrier centre frequency) separation for operating bands is specified in Table 5.4.4-1 for FR1-NTN. Table 5.4.4-1: UE TX-RX frequency separation (FR1-NTN) NTN Satellite Operating Band; TX – RX carrier centre frequency separation. n256; 190 MHz. n255; -101.5 MHz. n254; 862 – 885 MHz.

The Tx-Rx frequency separation in some of the NR-NTN bands, as showed above, is also quite close, similar as the TN bands, for example n3. The Tx/Rx isolation in NR-NTN band has the same challenge as in NR bands due to small frequency separations.

Observation 1: The factors impacting the NTN HPUE is similar to the previous discussion in TN HPUE, including the PA, duplexer, RFIC, non-linearity due to higher power.

In TN spec, the refsens degradation is introduced as follows:

For power class 2 UEs, certain degradation of the reference sensitivity in Table 7.3.2-1a is allowed. The maximum amount of degradation is specified in Table 7.3.2-1c, and in Table 7.3.2-1d for a UE that indicates txDiversity-r16 or txDiversity2Tx-r18 [15]. Table 7.3.2-1c Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE not supporting Tx Diversity Operating Band; 5 MHz (dB); 10 MHz (dB); 15 MHz (dB); 20 MHz (dB); 25 MHz (dB); 30 MHz (dB); 35 MHz (dB); 40 MHz (dB); 45 MHz (dB); 50 MHz (dB). n1; 0; 0; 0; 0; 0; 0; -; 0; 0; 0. n2; 0.8; 0.9; 1.1; 1.2; 1.3; 2.7; 2.8; 3.5. n3; 0.5; 0.5; 0.5; 0.5; 0.6; 0.8; 1.1; 1.5; 2.3; 2.8. n7; 0.5; 0.5; 0.5; 0.5; 0.5; 0.5; 0.5; 0.5; 2.0. n8; 0.5; 0.7; 0.8; 2.3; 2.8; 3.2; 3.1. n13; 0.8; 0.9. n14; 0.6; 0.8. n25; 0.8; 0.8; 0.9; 1.1; 1.3; 2.7; 2.8; 3.5; 3.7. n66; 0; 0; 0; 0; 0; 0; 0; 0; 0. n71; 0.5; 0.9; 0.9; 2.2; 2.42 2.53; 2.52 2.43; 2.92 3.13. n70; 0; 0; 0; 0; 0. n85; 0.6; 1.0; 2.6. NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 2: Applies to UEs that support a maximum uplink BW of 20 MHz in this band. NOTE 3: Applies to UEs that support optional symmetric UL/DL for this BW..

Considering the similar frequency range, if we take n255, n256 as an example, and Tx-Rx separation distances, the RSD values from n1, n3 can be used as a reference or starting point.

Proposal 1: The PC2 RSD values of NR-TN n1 can be used as a candidate for n256 RSD discussion.

Proposal 2: The PC2 RSD values of NR-TN n3 can be used as a candidate for n255 RSD discussion.

3	Conclusion

For the HPUE Rx requirements, we propose the following.

Observation 1: The factors impacting the NTN HPUE is similar to the previous discussion in TN HPUE, including the PA, duplexer, RFIC, non-linearity due to higher power.

Proposal 1: The PC2 RSD values of NR-TN n1 can be used as a candidate for n256 RSD discussion.

Proposal 2: The PC2 RSD values of NR-TN n3 can be used as a candidate for n255 RSD discussion.

4	Reference

[1] R4-2414452, WF on UE RF requirements for NTN HPUE, Samsung