3GPP TSG-RAN4 Meeting #112bis	R4-2416296

# Hefei, Anhui, China, 14th – 18th October, 2024

Title:	Discussion on IoT-NTN HPUE Tx requirements

Source:	Samsung

Agenda item:	6.8.2.3.1

Document for:	Approval

1	Introduction

In the RAN4#112 meeting, the WF R4-2414452 [1] had achieved agreements for IoT-NTN HPUE Tx requirements. And in this paper, we continue to discuss this issue.

2	Discussions

ACLR:

In [1], the feasibility discussion had concluded that it is feasible to support PC2 for both handheld and non-handheld devices types.

Issue 3-2-1: NR-NTN HPUE feasibility for different power classes Issue 3-2-2: NB-IoT based IoT-NTN HPUE feasibility for different power classes Issue 3-2-3: eMTC based IoT-NTN HPUE feasibility for different power classes - Agreement of Issue 3-2-1 to Issue 3-2-3: - For PC2, it is feasible to support both handheld and non-handheld devices. - FFS other power classes for applicable device type.

In our another paper, our co-ex study results suggested the required ACLR can be as following:

Table. Suggested IoT-NTN HPUE ACLR for PC 2/1 from Scenario 4 Power Class; Suggested ACLR (dBc). 3; 21.9. 2; 24.5. 1; 28.9.

Observation 1: The suggested ACLR for PC 2 is 24.5 dBc; and 28.9 dBc for PC1 from our study results.

In Rel-18, the IoT-NTN UE ACLR/ACS were agreed to use the IoT-TN UE ACLR and ACS requirements, and it was based on the assumptions that the IoT-NTN UE (PC3) will re-use the legacy NB-IoT or eMTC UEs, and its RF architectures and designs.

However, with the further development and enhancement of IoT-NTN in last and current on-going releases, the HPUE NTN in this WI may not re-use the existing HPUE TN devices and its RF architectures/components.

Moreover, in TN spec, there is no HPUE for IoT, so there’s no existing ACLR/ACS requirements for PC2 or PC1.

Proposal 1: Based on our co-ex study results, we suggest ACLR for NB IoT-NTN HPUE PC2 can be 24.5 dBc. And this can apply to both handheld and non-handheld devices.

MPR:

In last meeting, we have agreed on the principles to discuss the MPR requirements for IoT-NTN HPUE, which are as follows:

Issue 3-3-2: MPR - Agreement: - FFS the MPR requirements. Following potential factors to be further discussed: - ACLR from co-existence study - Current TN MPR requirements - MPR simulation assumptions - Other factors not precluded if identified.

Given in our proposal 1, we have proposed to adopt 24.5 dBc, which is a lower value compared to the IoT-NTN PC3 ACLR requirements in 36.102 as shown below.

Table 6.5B.3.4-1: category NB1 and NB2 UE ACLR requirements GSMACLR; UTRAACLR. ACLR; 20 dB; 37 dB. Adjacent channel center frequency offset from category NB1 or NB2 Channel edge; ±200 kHz; ±2.5 MHz. Adjacent channel measurement bandwidth; 180 kHz; 3.84 MHz. Measurement filter; Rectangular; RRC-filter α=0.22. Category NB1 and NB2 channel measurement bandwidth; 180 kHz; 180 kHz. Category NB1 and NB2 channel Measurement filter; Rectangular; Rectangular.

Observation 2: Our study suggests a lower ACLR requirement for PC2 compared to the IoT-NTN PC3 ACLR requirements, which does not introduce challenge to meet ACLR.

In this case, with higher Tx power for PC2 to meet a lower ACLR requirements, our view is that the existing IoT-NTN PC3 MPR requirements can still apply, if there’s no other technical difficulties identified.

Proposal 2: To re-use the IoT-NTN PC3 MPR requirements for IoT-NTN PC2 as starting point.

SEM:

In last meeting, [1] discussed whether the existing PC3 SEM requirements can be applied to other PCs.

In our view, the introduction of higher power class does not necessarily change the SEM requirements, especially given the requirements generally was made to meet existing regulatory requirements. Hence, we suggest to re-use existing SEM requirements from PC3 to other PCs.

Proposal 3: To re-use existing SEM requirments from PC3 to other PCs.

SAR:

In last meeting, [1] agrees on the SAR solutions for IoT-NTN HPUE handheld devices as following:

Issue 3-3-4: SAR for handheld Agreement: - Using P-MPR as the starting point - The other solutions are not precluded - E.g., the solution similar to TN duty cycle based on UE capability and network scheduling

The left-over issue is whether a SAR solution, other than P-MPR, can be introduced for the SAR issue for a hand-held NTN HPUE device.

In our view, the P-MPR can be a baseline option, as it was widely adopted and implemented in the current UEs. And if we consider some new methods, for example the duty cycle based SAR solution, we need to first identify the new demands in addition to the P-MPR solutions.

When compared to the existing TN deployments, NTN becomes critical in adverse conditions such as severe weather events or natural disasters, where or when the terrestrial deployments are not present or temporally unavailable. However, satellite communication involves transmitting signals over long distances, which results in larger path loss. Thus increasing the UE maximum transmit power will significantly improve UL performance in terms of coverage, availability and throughput, which would be needed to meet the above market demands associated with fixed wireless, public safety and automotive usage. HPUE for NTN is feasible. For a number of NTN use cases, including automotive, maritime, logistics, outdoor industrial monitoring and other scenarios not reliant on battery power, such that the higher power requirements of PC2, PC1.5 and PC1 are not with a battery power issue. Regarding the regulation, the existing regulation and specifications used by the current satellite services (primarily MSS), such as ECC/CEPT and FCC, typically allow significantly higher UE transmit powers even for handheld (e.g. satellite phones and trackers, which are different from smartphones) and automotive UE, and operate in narrow-band spectrum. Therefore, regulation is not expected to be a barrier. To address the peak spatial-average specific absorption rate (SAR) restriction for some UE (e.g. smartphones) the proper solution similar to TN HPUE would be needed.

The NTN HPUE objectives, according to the WIDs above, show that the use cases of the NTN HPUE include to provide service coverage in ”severe weather events or natural disasters”. And unike the TN HPUE case, the current satellite smartphone devices were allowed to tarnsmit in higher power to make sure its connectivity/availability to the network during those cases.

Observation 4: The NTN HPUE use cases including to provide service during severe weather conditions and natural disasters, which requires higher power to compensate the increasing pathloss due to longer distance and potential additional rain/cloud fadings during those conditions.

Though IoT-NTN bands are currently all FDD bands, a duty cycle based method can be applicable because the IoT-NTN coverage can be a TDM manner as one NTN SAN would cover multiple cells/beams and in a TDM manner.

Observation 5: The NTN satellite will cover several cells/beams in a large area, and the service over each cell/beam can be a TDM-like manner.

If we look at the current TN duty cycle based SAR solution, it requires HPUE to report its max uplink duty cycle capability and to receive the current network uplink configurations to understand the uplink transmission ratio in time domain and peroform power class back off if curretn network uplink configuration is higher than the reported capability of this HPUE.

The following issues remains for NTN to adopt a duty cycle based SAR:

- the NTN FDD bands does not provide such TDD UL/DL slot configuration which is for a TDD band, that can indicate the uplink transmission ratio in a semi-static way;

- the NTN TDM-like service over all its serving beam/cell can be implmentation based, then the uplink ratio for a serving cell or beam or the particular HPUE can be different from time to time;

- Compared to the SAR regulatory requirements, which accumulate the SAR exposure in a consecutive 6 mins time window, the NTN HPUE uplink transmission can be comparably short but critical if it’s a public safety or life threatening use case.

Observation 6: The NTN network may work in a TDM-manner to cover all its serving cells and beams, but the SAN coverage over each of its serving cells/beams can be implmenetation based. In this case, the SAN may not be able to send its configured uplink ratio in each cell or beam to the UE, or it may not be a semi-static configurations.

The SAR exposure time window is 6 mins. And the HPUE uplink transmission can be occasional and short compared to this time window, and the uplink transmission can be critical if its a life threatening or public safety related transmissions.

Observation7: Even if such uplink ratio configuration can be send to UE/cell/beam, the HPUE should be allowed to transmit as much as possible when performing its duty.

Proposal 4-1: The duty cycle based SAR solution may not be applicable to IoT-NTN cases, as the IoT-NTN uplink duty cycle for each UE (or for each cell/beam) can be up to network implementation.

Proposal 4-2: If the SAN can send its configured uplink ratio for a particular UE or cell or beam, the NTN HPUE should be allowed to transmit with its max Tx power as much as possible, considering the uplink transmission in a NTN scenario can be critical but short in time compared to 6mins SAR regulatory evaluation period.

3	Conclusion

For the IoT-NTN HPUE Tx requirements, we propose the following.

Observation 1: The suggested ACLR for PC 2 is 24.5 dBc; and 28.9 dBc for PC1 from our study results.

Proposal 1: Based on our co-ex study results, we suggest ACLR for NB IoT-NTN HPUE PC2 can be 24.5 dBc. And this can apply to both handheld and non-handheld devices.

Observation 2: Our study suggests a lower ACLR requirement for PC2 compared to the IoT-NTN PC3 ACLR requirements, which does not introduce challenge to meet ACLR.

Proposal 2: To re-use the IoT-NTN PC3 MPR requirements for IoT-NTN PC2 as starting point.

Proposal 3: To re-use existing SEM requirments from PC3 to other PCs.

Observation 4: The NTN HPUE use cases including to provide service during severe weather conditions and natural disasters, which requires higher power to compensate the increasing pathloss due to longer distance and potential additional rain/cloud fadings during those conditions.

Observation 5: The NTN satellite will cover several cells/beams in a large area, and the service over each cell/beam can be a TDM-like manner.

Observation 6: The NTN network may work in a TDM-manner to cover all its serving cells and beams, but the SAN coverage over each of its serving cells/beams can be implmenetation based. In this case, the SAN may not be able to send its configured uplink ratio in each cell or beam to the UE, or it may not be a semi-static configurations.

Observation7: Even if such uplink ratio configuration can be send to UE/cell/beam, the HPUE should be allowed to transmit as much as possible when performing its duty.

Proposal 4-1: The duty cycle based SAR solution may not be applicable to IoT-NTN cases, as the IoT-NTN uplink duty cycle for each UE (or for each cell/beam) can be up to network implementation.

Proposal 4-2: If the SAN can send its configured uplink ratio for a particular UE or cell or beam, the NTN HPUE should be allowed to transmit with its max Tx power as much as possible, considering the uplink transmission in a NTN scenario can be critical but short in time compared to 6mins SAR regulatory evaluation period.

4	Reference

[1] R4-2414452, WF on UE RF requirements for NTN HPUE, Samsung