3GPP TSG-RAN WG4 Meeting #112bis	R4-2416308

Hefei, China, Oct 14 – Oct 18, 2024

Source:	Nokia

Title:	Core requirement for Expected EIRP mask

Agenda item:       6.11.2 Expected EIRP mask for upper 6GHz			 [NR_BS_RF_req_evo-Core]

Document for:	Approval

Introduction

TSG RAN#103 meeting approved new Rel-19 work item “NR base station (BS) RF requirement evolution for FR1/FR2 and testing” [1], where one objective is related to expected EIRP mask for upper 6 GHz.

WRC-23 has identified the frequency range 6425 – 7125 MHz which is 3GPP band n104 for IMT as in the RR Footnotes 5.457D, 5.457E and 5.457F. The IMT identification comes with some technical conditions which includes the limits on the expected equivalent isotropically radiated power (e.i.r.p.) spectral density of IMT base stations for protecting Earth-to-space fixed satellite services (FSS) as contained in Resolution 220 (WRC-23) [2].

Core part of expected EIRP mask objective of work item [1] is as follow:

Expected EIRP mask for upper 6GHz - Introduce the concept of “limits on expected EIRP for angles above horizon” into the RAN4 BS specifications and specify related BS requirement for 6425-7125 MHz based on the WRC-23 outcome. - Note: target completion date of this objective is December 2024.

Last RAN4#112 meeting discussions were concluded in agreed WF [3].

For core requirements draft CR [4] to BS core specification TS 38.104 was agreed. Based on this a Text Proposal to Technical Report 38.908 for core requirement was agreed as well in [5].

In this contribution we discuss some details and propose updates to draft CR to BS core specification.

Discussion

In draft CR [5] we provide some proposal to improve agreed draft CR for Expected EIRP mask introduction.

Agreed draft CR [4] included in definition added term “over time”:

#### 9.9.2.1 Minimum requirement for BS type 1-H and BS type 1-O For BS type 1-H and BS type 1-O operating in band n104, the Expected EIRP (EEIRP) in the frequency range 6425 – 7075 MHz , shall not exceed the values specified in table 9.9.2.1-1. Expected EIRP (EEIRP) is defined as the average value of the EIRP, with the averaging being performed over the supported weighted beamforming directions over time within the BS horizontal and vertical steering range and the averaging being performed over horizontal angles from −180° to +180° and the specified elevation angle range θHL ≤ θ < θHH in Table 9.9.2-1.

Additionally in TP to TR 38.908 a note with clarification was added:

# 5 RF requirement OTA spatial emission requirements are defined to set upper limits on radiated power in specific directions. This requirement shall be applied to BS operating in band n104 to protect FSS (Earth-to-space) satellite receiver. For BS type 1-H and BS type 1-O operating in band n104, the Expected EIRP (EEIRP) in the frequency range 6425 – 7075 MHz, shall not exceed the values specified in table 5-1. Expected EIRP (EEIRP) is defined as the average value of the EIRP, with the averaging being performed over the supported weighted beamforming directions over time within the BS horizontal and vertical steering range and the averaging being performed over horizontal angles from −180° to +180° and the specified elevation angle range θHL ≤ θ < θHH in table 5-1. Note: The beamforming directions over time means directions are measured sequentially in the test.

As technical report is only informative document and is not normative, we think that similar note should be added to draft CR to BS core specification in TS 38.104 as well, otherwise, term “over time” is not clear and introduce ambiguity.

It is propose to introduce additional note with clarification of term “over time” into TS 38.104 EEIRP definition.

We also provide some updates and corrections to draft CRs in [6] that introduces updates to references, clause with core requirements.

Conclusion

In this contribution we discuss some details and propose updates to draft CR to BS core specification.

The following Observations and Proposals were made:

Proposal 1: It is propose to introduce additional note with clarification of term “over time” into TS 38.104 EEIRP definition.

We also provide some updates and corrections to draft CRs in [6] that introduces updates to references, clause with core requirements.

- References

- RP-240829, New WID: NR base station (BS) RF requirement evolution for FR1/FR2 and testing, RAN4 Chair (Huawei)

- ITU WRC-23, Final Acts, Resolution 220

- R4-2413509, WF for [112][305] NR_BS_RF, ZTE Corporation

- R4-2413574, Draft CR for introduction of U6GHz EIRP mask, ZTE Corporation

- R4-2413576, TP to TR 38.908 U6GHz EEIRP mask requirement, Samsung

- R4-2416309, Draft CR to TS 38.104 for introduction of expected EIRP mask core requirement, Nokia