3GPP TSG-RAN WG4 Meeting #112bis	R4-2416310

Hefei, China, Oct 14 – Oct 18, 2024

Source:	Nokia

Title:	On conformance testing of Expected EIRP mask requirement

Agenda item:       6.11.4 BS conformance testing	[NR_BS_RF_req_evo-Perf]

Document for:	Approval

- Introduction

TSG RAN#103 meeting approved new Rel-19 work item “NR base station (BS) RF requirement evolution for FR1/FR2 and testing” [1], where first objective is related to expected EIRP mask for upper 6 GHz.

WRC-23 has identified the frequency range 6425 – 7125 MHz which is 3GPP band n104 for IMT as in the RR Footnotes 5.457D, 5.457E and 5.457F. The IMT identification comes with some technical conditions which includes the limits on the expected equivalent isotropic radiated power (e.i.r.p.) spectral density of IMT base stations for protecting Earth-to-space fixed satellite services (FSS) as contained in ITU-R Resolution 220 (WRC-23) [2].

Performance part of expected EIRP mask objective of work item [1] is as follow:

Expected EIRP mask for upper 6GHz - Specify manufacturer declarations, conformance test descriptions, test tolerances and test requirements. - Note: target completion date of this objective is December 2024.

Last RAN4#112 meeting discussions were concluded in agreed WF [3]. In this contribution we continue discussions for conformance testing for expected EIPR mask requirement for the upper 6 GHz.

We provide also text proposal (TP) with for clause 6 [4] to TR 38.908 (Protection of fixed satellite service (FSS) UL within 6425 to 7125 MHz).

In addition, we provide draft CR that includes test procedure for BS conformance specification TS 38.141-2 in contribution [5] and draft CR for Annex N.

- Discussion

In WF [3] following open issues are collected for further discussions are presented in table below. On top of these issues, we also provide our views and proposal for some other issues and details related to the testing of OTA spatial emission requirements.

# Open issues for U6GHz EIRP mask Issue 4-2: Test vectors for EEIRP mask Agreement: Companies are encourage to provide the inputs for test beams for EERIP measurement; Issue 4-3: Measurement grid over elevation angle and azimuth angle Agreement: Companies are encourage to provide the inputs for measurement step size for EERIP measurement; Issue 4-4: RF channels for the conformance testing - Proposal 1: B, T [Nokia] - Proposal 2: M. [Huawei] - Proposal 3: T [Ericsson] Agreement: Companies are encourage to provide the inputs for RF channels for conformance testing Issue 4-6: Test procedures - Proposal 1: RAN4 need to capture the high-level test procedure above in BS RF conformance test specification in the new sub-clause for OTA spatial emission. - Proposal 2: RAN4 need to create entry in TR 37.941 for OTA spatial emission requirement and corresponding conformance test aspects, such as measurement procedure, calibration procedure and measurement uncertainty evaluation. Agreement: Companies are encourage to provide the inputs for test procedures for EEIRP measurement. Issue 4-7: Measurement uncertainty and CI - Proposal 1: EEIRP summation Error is 0.75dB [Huawei] - Proposal 2: follow the legacy approach adopted for TRP accuracy requirements as baseline [ZTE, Huawei] - Proposal 3 : MU and CIs are different measures and one cannot exchange one for the other but MU needs to be taken into account when estimating the CI [Spark] - Proposal 4: If the numbers of samples (N) is large say >20 the students t distribution and Normal distribution CIs are similar, and any can be chosen. [Spark] - Proposal 6: RAN5 to consider defining the estimation/measurement accuracy error as the difference between the expected EIRP over the oversampling factors (Oazim, Oelev) and (Lazim, Lelev) relative to the largest oversampling factor allowed (where testing is done). Agreement: Companies are encourage to provide the inputs for MU and CI values based on the assumption of test beams and measurement grid size. Issue 4-9: Others - Proposal 1: unless there are clear benefits identified to align the coordinate system between EEIRP mask and legacy 3GPP coordinate system, it’s not necessary to have such kind of converting. [ZTE]

  - Test vectors

The EEIRP calculation involves calculating the average EIRP over a defined set of elevation angles (around the base station, i.e. 360° azimuth) for the full range of beamforming directions. An example of the full range of beamforming directions is shown in Figure 2.1-1, with the electrical steering ranges set to 0° ≤ θetilt ≤ 16° and -60° ≤ ϕescan ≤ +60°. The number of beamforming directions for this example is 2057 for a 1° steering resolution (i.e. 17 x 121).

Figure 2.1-1: Reference Set of beamforming directions

Theoretically, this full range of beamforming directions are meant to be used for the EEIRP calculation, however, a representative subset of these beamforming directions can be considered as long as the overall averaging of the EEIRP is sufficiently accurate (as described in the Annex to the ITU Resolution 220). The motivation to create a small subset of beamforming directions (or Test Beams) is to reduce the test effort needed to calculate the EEIRP.

    - 2.1.1 Subset of beamforming directions

It is undoubtably that using the full range of beamforming directions will amount to extensive test effort as the EIRP for the entire upper hemisphere (around the base station) needs to be measured for every beamforming direction. Simplistically, halving the number of beamforming directions could potentially half the amount of effort needed to comply with the EEIRP requirement. Therefore, it is important to find a representative subset of these beamforming directions (Test Beams set) for the EEIRP calculation.

Three different sets of Test Beams are investigated to evaluate the accuracy compared to the Reference Set (i.e. full range of beamforming directions). The three sets are shown in Figure 2.1.1-1 to Figure 2.1.1-3 with different number of Test Beams and locations. This is based on the example electrical steering ranges shown in Figure 2.1-1.

Figure 2.1.1-1: Set 1 (25 Beams)

Figure 2.1.1-2: Set 2 (9 Beams – rectangular)

Figure 2.1.1-3: Set 3 (9 Beams – diamond)

It is important to note that no weighting is needed for each of these Test Beams as each Test Beams set is intended to be a representation of the Reference Set shown in Figure 2.1-1. The Reference Set fulfils the requirement that the EEIRP is calculated over the entire electrical steering range.

    - 2.1.2 Antenna configurations for evaluation

These three different Test Beams sets are evaluated over three different antenna configurations to show how suitable these Test Beams sets are. The antenna parameters for these configurations are shown in Table 6.3.2-1 where Configurations 1 and 2 are for the non-sub-array model, whilst Config 3 is for the sub-array model.

| Parameter |  | Value |  |  |
| --- | --- | --- | --- | --- |
|  |  | Config 1 | Config 2 | Config 3 |
| Element | Vertical HPBW (°) | 90 | 90 | 65 |
|  | Horizontal HPBW (°) | 90 | 90 | 90 |
|  | Element gain (dBi) | 5.5 | 5.5 | 6.4 |
|  | Front-to-back ratio (H/V) (dB) | 30 | 30 | 30 |
|  | Element power (dBm/100 MHz) | 22 | 16 | 21 |
| Array configuration | Number of rows | 16 | 32 | 8 |
|  | Number of columns | 8 | 16 | 16 |
|  | Sub-array | - | - | 3x1 |
|  | Sub-array pre-tilt: (°) | - | - | 6 |
|  | Vertical element separation (λ) | 0.5 | 0.5 | 2.1 |
|  | Horizontal element separation (λ) | 0.5 | 0.5 | 0.5 |

Table 2.1.2-1: Antenna parameters

These parameters are just some example antenna configurations for the purpose of evaluating the suitability of the Test Beams sets rather than for evaluating against the specific EEIRP values required for the different elevation ranges. In addition, no specific antenna techniques such as sidelobe suppression are used where these techniques could potentially lower the EIRP of the peak of the main lobe and sidelobes.

The antenna patterns generated from these parameters are shown in Figure 2.1.2-1 to Figure 2.1.2-3.

Figure 2.1.2-1: Antenna pattern of Configuration 1

The antenna configuration in Figure 2.1.2-1 is based on the parameters which was provided to ITU-R for the used in sharing studies which led to ITU Resolution 220.

Figure 2.1.2-2: Antenna pattern of Configuration 2

The antenna configuration in Figure 2.1.2-2 is based on Configuration 1 with doubling the number of antenna elements in the horizontal and vertical domain and adjusting the power per element accordingly.

Figure 2.1.2-3: Antenna pattern of Configuration 3

The antenna configuration in Figure 2.1.2-3 is based on a sub-array model with a sub-array pre-tilt of 6°.

    - 2.1.3 Evaluation of Test Beams set

The Test Beams sets in clause 2.1.1 are used to evaluate against the antenna configurations in clause 2.1.2. The Reference Set illustrated in Figure 2.1.1-1 are based on 1° steering resolution, and the calculation of the EEIRP above the horizon (upper hemisphere) is based on 0.5° sampling resolution in order to capture the beam as accurately as possible.

The antenna pattern generated from these configurations were normalized, and just to note that the normalization factor is only up to around 1.5dB. The results shown in Table 2.1.3-1 to Table 2.1.3-3 is in the form of difference (delta) in dB comparing the EEIRP generated from those Test Beams set to the Reference Set as shown in Figure 2.1-1.

| Elevation Bin | Elevation angular range | Delta (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Set 1 (25 Beams) |  |  |
|  |  | Config 1 | Config 2 | Config 3 |
| 1 | 0<q<5 | 1.5042 | 2.9003 | 2.8642 |
| 2 | 5<q<10 | 0.3517 | 0.9653 | 1.6473 |
| 3 | 10<q<15 | 0.59 | -0.2142 | 0.978 |
| 4 | 15<q<20 | 0.0508 | 0.7039 | -0.2957 |
| 5 | 20<q<30 | -0.1745 | -0.1444 | 0.7201 |
| 6 | 30<q<60 | -0.655 | -0.6488 | -0.0819 |
| 7 | 60<q<90 | -0.7833 | -0.8611 | -0.0944 |

Table 2.1.3-1: Difference of Test Beams set 1 from Reference Set

For Test Beams set 1, the largest difference is around 2.9 dB of over-estimation in elevation bin 1 (Configuration 2 and Configuration 3), and up to 0.9 dB of under-estimation in elevation bin 7 (Configuration 2).

| Elevation Bin | Elevation angular range | Delta (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Set 2 (9 Beams - rectangular) |  |  |
|  |  | Config 1 | Config 2 | Config 3 |
| 1 | 0<q<5 | 3.2695 | 4.8185 | 4.7302 |
| 2 | 5<q<10 | -0.1618 | 1.8493 | 3.1198 |
| 3 | 10<q<15 | 0.903 | 0.1029 | 2.4126 |
| 4 | 15<q<20 | 1.1424 | 0.8559 | -4.7315 |
| 5 | 20<q<30 | -0.9639 | 0.074 | 1.7426 |
| 6 | 30<q<60 | -1.3376 | -1.6784 | -0.5279 |
| 7 | 60<q<90 | 0.0858 | 0.5385 | 3.0346 |

Table 2.1.3-2: Difference of Test Beams set 2 from Reference Set

For Test Beams set 2, the largest difference is around 4.8 dB of over-estimation in elevation bin 1 (Configuration 2), and up to 4.7 dB of under-estimation in elevation bin 4 (Configuration 3).

| Elevation Bin | Elevation angular range | Delta (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Set 3 (9 Beams - diamond) |  |  |
|  |  | Config 1 | Config 2 | Config 3 |
| 1 | 0<q<5 | -0.3157 | 0.7923 | 0.7893 |
| 2 | 5<q<10 | -0.0121 | 0.0559 | 0.0498 |
| 3 | 10<q<15 | 0.1368 | -0.5011 | -0.8413 |
| 4 | 15<q<20 | -0.2029 | 0.5704 | 0.4144 |
| 5 | 20<q<30 | -0.0294 | -0.3323 | -0.4852 |
| 6 | 30<q<60 | -0.0323 | 0.0938 | 0.2308 |
| 7 | 60<q<90 | 0.7835 | 1.029 | 3.1612 |

Table 2.1.3.3: Difference of Test Beams set 3 from Reference Set

For Test Beams set 3, the largest difference is around 3.2 dB of over-estimation in elevation bin 7 (Configuration 3), and up to 0.8 dB of under-estimation in elevation bin 3 (Configuration 3).

Observation 1: Different Test Beams set (which is a subset of the Reference Set) will result in different accuracies to the Reference Set (for different elevation bins). However, based on the different antenna configurations, more Test Beams does not necessarily lead to better accuracy.

Observation 2: From the antenna configurations that was tested, the Test Beams pattern Set 3 (9 Beams - diamond) results to smaller EIRP under-estimation from the Reference Set.

Proposal 1: RAN4 to consider a suitable Test Beams set pattern to fulfil the requirement.

Observation 3: The difference resulting from the Test Beams set and the Reference Set can be quantified as over-estimation or under-estimation and the magnitude of these are different. However, from the regulatory perspective, under-estimation is more concerning than over-estimation of the measured EEIRP.

  - Measurement grid

We already presented analysis on angular step size in our contribution [6].

Observation 4: The ability to define separate elevation and azimuthal step size for the different elevation bin could reduce the number of measurement points whilst not impacting the averaging accuracy.

Observation 5: The maximum elevation step size for a particular bin should not be bigger than the elevation bin size nor the 3 dB beamwidth of the beam.

Observation 6: The lowest elevation bin is impacted the most from the elevation step size.

Proposal 2: RAN4 to consider defining the maximum elevation step size for a particular elevation bin to be smaller or equal to the elevation bin size or the 3 dB beamwidth of the beam.

  - Test signal configuration

Current requirements in radiated conformance specification TS 38.141-2 are using following signals to build test configurations:

Table 4.7.2.1-1: Signal to be used to build NR TCs for BS type 1-H and BS type 1-O

| Operating band characteristics |  | FDL_high – FDL_low < 100 MHz | FDL_high – FDL_low ≥ 100 MHz |
| --- | --- | --- | --- |
| TC signal | BWchannel | 5 MHz (Note) | 20 MHz (Note) |
| characteristics | Subcarrier spacing | Smallest supported subcarrier spacing declared per operating band (D.7) |  |
| NOTE: If this BS channel bandwidth is not supported, the narrowest supported BS channel bandwidth declared per operating band (D.7) shall be used. |  |  |  |

As band n104 is wider than 100 MHz according to current specification 20MHz BS channel bandwidth should be used or alternatively narrowest channel bandwidth if 20 MHz CBW is not supported. However, for Expected EIRP mask 20 MHz BS channel bandwidth may not be the case with highest transmitted power which is the most important for expected EIRP mask testing. Thus, narrowest supported BS channel bandwidth should be used which give maximum transmitted output power.

Proposal 3: It is proposed to use narrowest supported BS channel bandwidth signal with highest transmitted output power for test configuration for OTA spatial emission requirement.

  - Test model

Most of NR RF requirements except those related to mostly transmission quality (Dynamic range, EVM, Frequency Error) are using NR test model 1 i.e. NR-FR1-TM1.1 for frequency range 1. OTA spatial emission requirement can also reuse this test model type.

Proposal 4: It is proposed to reuse NR-FR1-TM1.1 test model for OTA spatial emission requirement.

  - Test configuration

For test configuration for OTA spatial emission one proposal could be to use single carrier (SC) that should be configured with supported highest PSD which is declared by manufacturer.

However, this does not necessary provide maximum output power.

Observation 7: Usage of single carrier (SC) that should be configured with supported highest PSD does not necessary provide maximum output power.

Proposal 5: It is proposed to use single carrier (SC) which provides highest maximum transmitted output power for OTA spatial emission requirement.

  - RF channels

Number of frequencies for OTA spatial emission conformance testing should be performed with appropriate frequencies selected. Many tests in BS conformance specifications are performed with appropriate frequencies in the bottom, middle and top channels of the supported frequency range of the BS. These are denoted as RF channels B (bottom), M (middle) and T (top). During last RAN4 meeting companies had similar views that number of RF channels for conformance testing can be lower. Following 3 options were proposed:

Proposal 1: B, T

Proposal 2: M

Proposal 3: T

Observation 8: Number of frequencies to measure in operating band will impact test effort.

Due to nature of OTA spatial emission requirement as a protection for co-existing with FSS UL services it seems reasonable to set upper frequency range extreme i.e. top (T) RF channel. That will assure respective test coverage taking into account reasonable test effort.

Proposal 6: It is proposed to use top (T) channel as frequency to measure when defining conformance testing for Expected EIRP.

  - Measurement uncertainty and CI

Resolution 220 of the ITU Radio Regulation [2] contains the EEIRP technical condition for the protection of the FSS (Earth-to-space) and the Annex accompanying this Resolution outlines the theoretical calculation of this EEIRP. This Annex describes that the EEIRP is calculated in two steps where the beamforming directions are averaged in step 1 while step 2 describes the averaging of the required horizontal and vertical angles. The averaging of these two steps needs to be accurate and this can be done in different ways where one example suggested in the Resolution is by means of using confidence interval of 95%. This does not exclude other metrics to evaluate the accuracy of the EEIRP calculation that is done in step 1 and step 2.

For conformance testing, it could be more practical to define the accuracy by means that has already be done such as measurement uncertainty or even evaluating the over-estimation or under-estimation of the EEIRP calculation due to sampling of the EIRP when the measurements are done and/or using a representative Test Beams set. As described, the Test Beams set is a subset of all the beamforming directions within the steering range of the base station and this Test Beams set can be considered as long as the overall averaging of the EEIRP is sufficiently accurate.

From the regulation perspective, under-estimation of the EEIRP compared to the required limits is more concerning than over-estimation. Therefore, evaluating the accuracy of the EEIRP calculation can be based solely on the under-estimation of the EEIRP calculation.

Proposal 7: RAN4 to consider defining the accuracy of the EEIRP calculation required by the regulation based only on the under-estimation of the calculation.

- Conclusion

In this contribution we continue discussions for conformance testing for expected EIPR mask requirement for the upper 6 GHz.

We provide also text proposal (TP) with for clause 6 [4] to TR 38.908 (Protection of fixed satellite service (FSS) UL within 6425 to 7125 MHz). In addition, we provide draft CR that includes test procedure for BS conformance specification TS 38.141-2 in contribution [5] and draft CR for Annex N.

We have made following observations and proposals:

Observation 1: Different Test Beams set (which is a subset of the Reference Set) will result in different accuracies to the Reference Set (for different elevation bins). However, based on the different antenna configurations, more Test Beams does not necessarily lead to better accuracy.

Observation 2: From the antenna configurations that was tested, the Test Beams pattern Set 3 (9 Beams - diamond) results to smaller EIRP under-estimation from the Reference Set.

Proposal 1: RAN4 to consider a suitable Test Beams set pattern to fulfil the requirement.

Observation 3: The difference resulting from the Test Beams set and the Reference Set can be quantified as over-estimation or under-estimation and the magnitude of these are different. However, from the regulatory perspective, under-estimation is more concerning than over-estimation of the measured EEIRP.

Observation 4: The ability to define separate elevation and azimuthal step size for the different elevation bin could reduce the number of measurement points whilst not impacting the averaging accuracy.

Observation 5: The maximum elevation step size for a particular bin should not be bigger than the elevation bin size nor the 3 dB beamwidth of the beam.

Observation 6: The lowest elevation bin is impacted the most from the elevation step size.

Proposal 2: RAN4 to consider defining the maximum elevation step size for a particular elevation bin to be smaller or equal to the elevation bin size or the 3 dB beamwidth of the beam.

Proposal 3: It is proposed to use narrowest supported BS channel bandwidth signal with highest transmitted output power for test configuration for OTA spatial emission requirement.

Proposal 4: It is proposed to reuse NR-FR1-TM1.1 test model for OTA spatial emission requirement.

Observation 7: Usage of single carrier (SC) that should be configured with supported highest PSD does not necessary provide maximum output power.

Proposal 5: It is proposed to use single carrier (SC) which provides highest maximum transmitted output power for OTA spatial emission requirement.

Observation 8: Number of frequencies to measure in operating band will impact test effort.

Proposal 6: It is proposed to use top (T) channel as frequency to measure when defining conformance testing for Expected EIRP.

Proposal 7: RAN4 to consider defining the accuracy of the EEIRP calculation required by the regulation based only on the under-estimation of the calculation.

- 4. References

- RP-241284, Revised WID: NR base station (BS) RF requirement evolution for FR1/FR2 and testing, ZTE Corporation

- ITU WRC-23, Final Acts, Resolution 220

- R4-2413509, WF for [112][305] NR_BS_RF, ZTE Corporation

- R4-2416313, TP to TR 38.908 clause 6 – Conformance testing, Nokia

- R4-2416311, Draft CR to TS 38.141-2 with EEIRP mask testing procedure, Nokia

- R4-2412899, Expected EIRP mask testing, Nokia