3GPP TSG-RAN WG4 Meeting #112bis	R4-2416315

Maastricht, Netherlands, August 19th – 23rd 2024

Agenda Item:	6.17.2.2

Source:	Ericsson

Title:	Model Sharing

Document for:	Discussion

# Introduction

As part of the study on interoperability and testability for the AI/ML CSI compression use case, one of the options under consideration is a standardized ML model for testing. Thus, a first preliminary experiments have been decided to see how well the different companies can converge on a common model. To agree on a common model, the models of different companies need to be shared offline notably to verify the performances and can derive the counterpart model based on the test model (e.g., can derive an encoder based on a shared test decoder). For this use case only the ML weights need to be shared.

For the current discussion, the main topics are the amount of data that needs to be stored or transferred, the need to retain data and formats for dataset and model sharing. In this contribution, we consider model sharing in the immediate term and conclude that ONNX may be the only way forward. For the longer term, if a model is to be standardized, or if there would be an ongoing need for model sharing, then the best way to capture a model should be considered further. Some views are also provided on important aspects to consider for a standardized model format in the longer term.

# 2	Dataset size and retention policy

Dataset size and retention policy have been discussed to some extent offline, although there was no firm conclusion on retention policy.

Currently, all TDOCs, TRs, TS etc submitted to or created by 3GPP are in general retained, although there is a procedure for withdrawing TR or TS. In principle, the lowest risk retention policy for AI models or datasets would be the same as for TDOCs; i.e. indefinite retention. However, the cost associated with indefinite retention have the potential to be non-trivial.

In order to decide on a retention policy, it would be good to get an idea how significant the costs are.

Strive to further understand the relative costs of different retention policies

Retention policy may differ for different types of dataset or model. At least the following may be considered:

- Datasets or models shared between companies on a meeting to meeting basis as part of developing a study or work item

- Datasets or models submitted as reference models

- Datasets or models that are formally agreed to be references for deriving requirements

- Datasets or models that are formally captured as part of a TS

For datasets or models that are shared between companies but not agreed to be used, if retention is expensive then the retention policy could be made short term.

For datasets or models submitted as reference models, potentially the retention policy could be the duration of a release.

For datasets or models agreed to be used as references for deriving requirements, the retention policy should be discussed and should be one or more releases.

For datasets or models captured in a TS, in principle the retention policy should be indefinite. Potentially a procedure may be needed for formal withdrawal. In addition, a policy may be needed relating to how long to retain in the event of e.g. a model in a TS being superseded by another agreed model.

Consider retention in the light of the following uses for dataset/models, including whether each usage type should have a different retention policy:

- Datasets or models shared between companies on a meeting to meeting basis as part of developing a study or work item

- Datasets or models submitted as reference models

- Datasets or models that are formally agreed to be references for deriving requirements

- Datasets or models that are formally captured as part of a TS

# 3	How to share ML models offline during the current study

We would like to emphasize again that, the scope of the discussion and any agreements should be limited to the current RAN4 exploration of the alignment of ML model for CSI estimation.

Current agreements on model format should be limited in scope to what is needed in the short term for the RAN4 study. Long term ways to capture model formats should be discussed separately (TBC whether RAN4 or RAN1).

The next section is dedicated give some insight for the long-term view on how to share model within 3GPP. There are several ways of sharing model for the current RAN4 activity,

- Sharing a minimal code example used to create the architecture and the weights.

  - This should come with a detailed description of the python requirements and minimal code to run an example,

  - Or/and a Docker file to recreate an image to reproduce the result.

  - The format used for sharing model can be restricted, for instance only code in Python or only models in Pytorch or TensorFlow.

- Using an open format, such as ONNX, and share both model and weights in ONNX format.

The option A is not very practical as it will require lot of effort that can’t be easily automated to test each model. Even with a complete Docker Image, the integration of each model into simulator to verify performances might be complex.

The option B solves the previous issue as ONNX version of models can be run directly to verify performances and can be converted to several ML format (PyTorch, TensorFlow, …).

In the short term, sharing model through ONNX format seems the most practical direction. The version of ONNX need to be decided. To allow for a potential retraining and the encoder and decoder should be provided separately and without quantization in the latent space.

Additionally, some test examples should be provided to ensure that the model behave as expected: some simple random inputs and the expected output should be sufficient to verify that the model is similar when retrieved.

ONNX appears to be the most suitable for short term model-sharing, but some experiments to ensure that it can be made to work may be needed.

# 4. How to represent a model in the standard

In addition to the short-term discussion for the current offline model sharing, the feasibility of representing a standardized reference model (structure + parameters) and/or a standardized reference model structure in a 3GPP specification document(s) should also be considered as part of the study. We present some considerations here, emphasizing that these considerations are longer term than the immediate need for model/dataset sharing within the study.

There are multiple options to describe an AI/ML model in specification. We find the following aspects to be important:

- That the format can accurately and precisely describe the complete model (structure + parameters). This is important for both a standardized reference model, but also for model transfer, as the UE needs to interpret the transferred parameters. For example, RAN1 has seen “Transformers” with and without cross-attention; it needs to be specified if matrix of parameters is interpreted row-wise or column-wise; it needs to be specified if the model parameters and input are real- or complex-valued, and for real-valued models how to concatenate real-parts and imaginary-parts.

- That the format is flexible enough so that it can be adapted when new AI/ML technologies emerge. 3GPP will only be able to specify models that can be readily described in the format. Thus, 3GPP may want to ensure that the format is developed in a suitable direction. This includes also existing technologies, e.g., pre-processing with FFT.

- That the format is open enough to interface with different software stacks for practical implementation. Different companies are, likely, going to use different toolchains, which is out of scope for 3GPP to dictate, and the format may therefore need to interface a large number of such toolchains, including partially proprietary ones. Even if the model is completely specified, there are still flexibilities and vendor differentiation in how it is compiled to hardware, e.g., number formats, accelerators, read-/write- capabilities, scheduling, which is outside of 3GPP scope.

- That the format is stable and reasonably versioned. It is likely that UEs supporting a release where models may be specified in the format will exist in the network for a long time into the future. Potentially the conclusions in this study could even serve as the foundations for 6G. Hence, backward compatibility is going to be paramount, because the alternative is that either old UEs can lose AI/ML features over time, or that UE/Chipset vendors get a large burden on upgrading firmware for legacy UEs, which should then undergo new rounds of testing to ensure compliance and conformance. However, backward compatibility cannot neither be designed in such a way that it poses an excessively large burden on the NW.

- The effort of implementing and maintaining the standard is reasonable. This can be expressed both as a reasonable standardization effort in 3GPP, as well as a reasonable effort in product development. The latter is important since specialization in too many cases may cause the feature to be too expensive to implement.

There are certain options that have been discussed for how a specification of an AI/ML model can be captured. We address some of the below:

- Using an existing AI/ML framework such as, e.g., TensorFlow or PyTorch. While fulfilling aspect 1 there are problems with all other aspects. For aspect 2 the development is the outside of 3GPP control, leaving external actors effectively deciding how the AI/ML applications in 3GPP should evolve. Moreover, it is a historic fact that some AI/ML toolchains starts to disappear when something new evolves, which would violate both 2, as no new development of such a format will hinder development in 3GPP; 5, the risk of having to maintain a sunsetting AI/ML framework, without a priori agreed procedures, is a too large risk for 3GPP technologies. Moreover, 3, the entry point of the software stack would specified, which may not be flexible enough for vendors; 4, 3GPP would have to agree on what version to support for each release (with potentially more software dependencies) or vendors would need to continuously update products already on the market; 5, having to possibly run multiple versions of an external AI/ML framework can be costly for product development.

- Using an open format such as, e.g., ONNX. This way of representing an AI/ML model shares most issues with the above point. However, it can be assumed that the impact on 3 is less, although not non-existent, as the nature of the format is that it should be able to interface multiple software stacks. Nevertheless, the issues around needing that the format is maintained and developed, while not being able to directly influence the development, is a major risk. With respect to 4 it is not obvious who to decide on what release of the format should be used in a 3GPP release, as the two can have very different release cycles. Thus, while an external format may be perceived as quick in adding new AI/ML technology components, it is not clear to what extent the 3GPP work can utilize it.

- Have 3GPP specify its own format. For this way of describing an AI/ML model, one would have to assume that 3GPP can handle 1, 3, 4, and partially 5, as these types of aspects are present today in the normal work with a 3GPP release. Specifically, 1, the standard is precise as demonstrated by it working; 3, there is an open market of companies implementing the standard; 4, 3GPP is handling releases; 5, products are being implemented at scale. However, for 5, the effort is to standardize and maintain an AI/ML format could be significant and it is difficult to estimate as it is something 3GPP has not done. It is also uncertain which 3GPP working group would get the responsibility, and what scope it should cover. Moreover, regarding 2, while many companies are showing AI/ML excellency, 3GPP as an organization has yet to demonstrate its capacity. Furthermore, if the average 3GPP release is 18 months long, that could be the lead-time between identifying a AI/ML technology component and the possibility to start implementing it.

- Use a format co-ordinated by the two partaking sides, outside of standardization. For this of describing an AI/ML model one would have to assume that 1, 2, partially 3, and partially 4, are fulfilled. For aspect 3 it would be easy to assume that the two vendors would ensure that the format fits them both. However, different companies have different software stacks, and the multi-vendor aspect makes it even more difficult to confidently say that 3 is fulfilled. This ties into 5, where the multi-vendor aspect can create complications: Different companies may have different preferences, which can either induce high costs on parallel tracks and toolchains, or effectively mean that some companies would have to choose to either adapt or not be part of the feature. In other words, there is a risk for a de-facto-standard being established outside of the common standardization forum. It could also lead to that the feature is so costly to implement, with limited applicability, and thus never gets used. Moreover, this way of expressing an AI/ML model is, likely, the most complex one when it comes to compliance- and conformance-testing. Regarding aspect 4 there is a risk that the versioning may be complicated for the NW to handle, especially in the multi-vendor setting where there may be a large number of versions for different UEs and different UE/chipset vendors.

In general, the desire that the format can develop fast to adapt to new technologies, as captured in aspect 2, needs to be carefully balanced against 4 and 5, as products involve physical devices, developed by multiple companies, and that may stay in the field for a long time.

# Conclusions

Proposal 1	Strive to further understand the relative costs of different retention policies

Proposal 2	Consider retention in the light of the following uses for dataset/models, including whether each usage type should have a different retention policy:

- Datasets or models shared between companies on a meeting to meeting basis as part of developing a study or work item

- Datasets or models submitted as reference models

- Datasets or models that are formally agreed to be references for deriving requirements

- Datasets or models that are formally captured as part of a TS

Proposal 3	Current agreements on model format should be limited in scope to what is needed in the short term for the RAN4 study. Long term ways to capture model formats should be discussed separately (TBC whether RAN4 or RAN1).

Proposal 4	ONNX appears to be the most suitable for short term model-sharing, but some experiments to ensure that it can be made to work may be needed.