3GPP TSG-RAN WG4 Meeting # 112bis 	    R4-2416327

Hefei, CN, 14  – 18 October 2024

Agenda Item:	4.2.2.5

Source: 	Ericsson

Title:	On performance requirement for bandwidth aggregation for positioning measurements

Document for:	Information

# Introduction

- In this contribution accuracy requirement for RSTD and UE Rx-Tx measurements performed by UE by aggregating PRS resources from multiple PFLs are presented. In annex, complementing to simulation results previously submitted by Ericsson, additional simulation results for PRS aggregation are presented. The simulation results are for RSTD and UE Rx-Tx measurement accuracies in AWGN channel condition for 2 and 3 PFLs scenarios for the agreed side conditions.

# RSTD accuracy requirement

Summary of simulation results submitted by volunteering companies for RSTD accuracy requirements are summarized in our companion paper R4-2416328. Based on the simulation summary, we propose to define accuracy requirement for RSTD measurements performed by UE by aggregating PRS resources from multiple PFLs as follows.

Proposal 1: Define 4-sample RSTD absolute accuracy in FR1 for AWGN channel as:

| 3 PFLs (Tc) | 2 PFLs (Tc) | SINR | SCS (kHz) | PRS bandwidth per PFL (PRBS) |
| --- | --- | --- | --- | --- |
| 16 | 20 | (PRS Ês/Iot)ref ≥-6dB (PRS Ês/Iot)i ≥-13dB | 15 | ≥ 104 |
| 6 | 11 |  | 30 | ≥ 132 |
| 4 | 6 |  |  | 272 |
| 6 | 11 |  | 60 | ≥ 64 |
| 4 | 5 |  |  | 132 |

Proposal 2: Define 4-sample RSTD absolute accuracy in FR2 for AWGN channel as:

| 3 PFLs (Tc) | 2 PFLs (Tc) | SINR | SCS (kHz) | PRS bandwidth per PFL (PRBS) |
| --- | --- | --- | --- | --- |
| 6 | 11 | (PRS Ês/Iot)ref ≥-6dB (PRS Ês/Iot)i ≥-13dB | 60 | ≥ 64 |
| 4 | 5 |  |  | ≥ 132 |
| 4 | 6 |  | 120 | ≥ 64 |
| 2 | 3 |  |  | ≥ 128 |

Proposal 3: Define 4-sample RSTD absolute accuracy in FR1 for fading channel as:

| 3 PFLs (Tc) | 2 PFLs (Tc) | SINR | SCS (kHz) | PRS bandwidth per PFL (PRBS) |
| --- | --- | --- | --- | --- |
| 35 | 45 | (PRS Ês/Iot)ref ≥-6dB (PRS Ês/Iot)i ≥-13dB | 15 | ≥ 104 |
| 29 | 30 |  | 30 | ≥ 132 |
| 27 | 27 |  |  | 272 |
| 35 | 31 |  | 60 | ≥ 64 |
| 33 | 27 |  |  | 132 |

Proposal 4: Define 4-sample RSTD absolute accuracy in FR2 for fading channel as:

| 3 PFLs (Tc) | 2 PFLs (Tc) | SINR | SCS (kHz) | PRS bandwidth per PFL (PRBS) |
| --- | --- | --- | --- | --- |
| 50 | 72 | (PRS Ês/Iot)ref ≥-6dB (PRS Ês/Iot)i ≥-13dB | 60 | ≥ 64 |
| 39 | 55 |  |  | ≥ 132 |
| 49 | 73 |  | 120 | ≥ 64 |
| 51 | 56 |  |  | ≥ 128 |

Proposal 5: Define RSTD absolute accuracy in FR1 for AWGN channel with reduced number of samples as:

| 3 PFLs (Tc) | 2 PFLs (Tc) | SINR | SCS (kHz) | PRS bandwidth per PFL (PRBS) |
| --- | --- | --- | --- | --- |
| 20 | 24 | (PRS Ês/Iot)ref ≥-3dB (PRS Ês/Iot)i ≥-6dB | 15 | ≥ 104 |
| 7 | 13 |  | 30 | ≥ 132 |
| 5 | 6 |  |  | 272 |
| 7 | 13 |  | 60 | ≥ 64 |
| 5 | 6 |  |  | 132 |

Proposal 6: Define RSTD absolute accuracy in FR2 for AWGN channel with reduced number of samples as:

| 3 PFLs (Tc) | 2 PFLs (Tc) | SINR | SCS (kHz) | PRS bandwidth per PFL (PRBS) |
| --- | --- | --- | --- | --- |
| 7 | 13 | (PRS Ês/Iot)ref ≥-3dB (PRS Ês/Iot)i ≥-6dB | 60 | ≥ 64 |
| 5 | 6 |  |  | ≥ 132 |
| 4 | 6 |  | 120 | ≥ 64 |
| 2 | 3 |  |  | ≥ 128 |

# UE Rx-Tx accuracy requirement

Summary of simulation results submitted by volunteering companies for UE Rx-Tx measurement accuracy requirements are summarized in our companion paper R4-2416328. Based on the simulation summary, we propose to define accuracy requirement for UE Rx-Tx measurements performed by UE by aggregating PRS resources from multiple PFLs as follows.

Proposal 7: Define 4-sample UE Rx-Tx time difference measurement accuracy in FR1 in AWGN as:

| 3 PFLs (Tc) | 2 PFLs (Tc) | SINR | PRS bandwidth per PFL (PRBS) | SCS (kHz) |
| --- | --- | --- | --- | --- |
| ± 10+𝛅 | ± 12+𝛅 | -3dB | ≥104 | 15 |
| ± 3+ 𝛅 | ± 6+𝛅 |  | ≥132 | 30 |
| ± 2+ 𝛅 | ± 3+𝛅 |  | 272 |  |
| ± 3+ 𝛅 | ± 6+𝛅 |  | ≥64 | 60 |
| ± 2+𝛅 | ± 3+𝛅 |  | ≥ 132 |  |
| ± 13+𝛅 | ± 14+𝛅 | -13dB | ≥104 | 15 |
| ± 4+𝛅 | ± 8+𝛅 |  | ≥132 | 30 |
| ± 3+𝛅 | ± 4+𝛅 |  | 272 |  |
| ± 4+𝛅 | ± 9+𝛅 |  | ≥ 64 | 60 |
| ± 3+𝛅 | ± 4+𝛅 |  | ≥ 132 |  |

Proposal 8: Define UE Rx-Tx time difference measurement accuracy in FR1 in AWGN with reduced number of samples as:

| 3 PFLs (Tc) | 2 PFLs (Tc) | SINR | PRS bandwidth per PFL (PRBS) | SCS (kHz) |
| --- | --- | --- | --- | --- |
| ±9+𝛅 | ±11+𝛅 | 0dB | ≥104 | 15 |
| ±3+𝛅 | ±5+𝛅 |  | ≥132 | 30 |
| ±2+𝛅 | ±3+𝛅 |  | 272 |  |
| ±3+𝛅 | ±5+𝛅 |  | ≥64 | 60 |
| ±2+𝛅 | ±3+𝛅 |  | ≥ 132 |  |
| ±13+𝛅 | ±14+𝛅 | -6dB | ≥104 | 15 |
| ±4+𝛅 | ±9+𝛅 |  | ≥132 | 30 |
| ±3+𝛅 | ±4+𝛅 |  | 272 |  |
| ±4+𝛅 | ±9+𝛅 |  | ≥ 64 | 60 |
| ±3+𝛅 | ±4+𝛅 |  | ≥ 132 |  |

Proposal 9: Define UE 4-sample Rx-Tx time difference measurement accuracy in FR1 for fading channel as:

| 3 PFLs (Tc) | 2 PFLs (Tc) | SINR | PRS bandwidth per PFL (PRBS) | SCS (kHz) |
| --- | --- | --- | --- | --- |
| ± 39+𝛅 | ± 43+𝛅 | -3dB | ≥104 | 15 |
| ± 32+𝛅 | ± 36+𝛅 |  | ≥132 | 30 |
| ± 32+𝛅 | ± 33+𝛅 |  | 272 |  |
| ± 33+𝛅 | ± 37+𝛅 |  | ≥64 | 60 |
| ± 33+𝛅 | ± 32+𝛅 |  | ≥ 132 |  |
| ± 40+𝛅 | ± 47+𝛅 | -13dB | ≥104 | 15 |
| ± 34+𝛅 | ± 36+𝛅 |  | ≥132 | 30 |
| ± 32+𝛅 | ± 33+𝛅 |  | 272 |  |
| ± 37+𝛅 | ± 37+𝛅 |  | ≥ 64 | 60 |
| ± 35+𝛅 | ± 34+𝛅 |  | ≥ 132 |  |

Proposal 10: Define 4-sample UE Rx-Tx time difference measurement accuracy in FR2 in AWGN as:

| 3 PFLs (Tc) | 2 PFLs (Tc) | SINR | PRS bandwidth per PFL (PRBS) | SCS (kHz) |
| --- | --- | --- | --- | --- |
| ± 3+𝛅 | ± 6+𝛅 | -3dB | ≥64 | 60 |
| ± 2+𝛅 | ± 3+𝛅 |  | ≥132 |  |
| ± 2+𝛅 | ± 3+𝛅 |  | ≥64 | 120 |
| ± 1+𝛅 | ± 2+𝛅 |  | ≥128 |  |
| ± 4+𝛅 | ± 9+𝛅 | -13dB | ≥64 | 60 |
| ± 3+𝛅 | ± 4+𝛅 |  | ≥132 |  |
| ± 3+𝛅 | ± 4+𝛅 |  | ≥64 | 120 |
| ± 1+𝛅 | ± 2+𝛅 |  | ≥128 |  |

Proposal 11: Define UE Rx-Tx time difference measurement accuracy in FR2 in AWGN with reduced number of samples as:

| 3 PFLs (Tc) | 2 PFLs (Tc) | SINR | PRS bandwidth per PFL (PRBS) | SCS (kHz) |
| --- | --- | --- | --- | --- |
| ± 3+𝛅 | ± 5+𝛅 | 0dB | ≥64 | 60 |
| ± 2+𝛅 | ± 3+𝛅 |  | ≥132 |  |
| ± 2+𝛅 | ± 3+𝛅 |  | ≥64 | 120 |
| ± 1+𝛅 | ± 2+𝛅 |  | ≥128 |  |
| ± 4+𝛅 | ± 9+𝛅 | -6dB | ≥64 | 60 |
| ± 3+𝛅 | ± 4+𝛅 |  | ≥132 |  |
| ± 3+𝛅 | ± 4+𝛅 |  | ≥64 | 120 |
| ± 1+𝛅 | ± 2+𝛅 |  | ≥128 |  |

Proposal 12: Define 4-sample UE Rx-Tx time difference measurement accuracy in FR2 for fading channel as:

| 3 PFLs (Tc) | 2 PFLs (Tc) | SINR | PRS bandwidth per PFL (PRBS) | SCS (kHz) |
| --- | --- | --- | --- | --- |
| ± 35+𝛅 | ± 55+𝛅 | -3dB | ≥64 | 60 |
| ± 38+𝛅 | ± 35+𝛅 |  | ≥132 |  |
| ± 40+𝛅 | ± 44+𝛅 |  | ≥64 | 120 |
| ± 42+𝛅 | ± 43+𝛅 |  | ≥128 |  |
| ± 40+𝛅 | ± 69+𝛅 | -13dB | ≥64 | 60 |
| ± 40+𝛅 | ± 43+𝛅 |  | ≥132 |  |
| ± 45+𝛅 | ± 54+𝛅 |  | ≥64 | 120 |
| ± 51+𝛅 | ± 47+𝛅 |  | ≥128 |  |

# Summary

The following proposals are made to define accuracy requirement for RSTD and UE Rx-Tx measurement performed by UE by aggregating PRS resources from multiple PFLs:

Proposal 1: Define 4-sample RSTD absolute accuracy in FR1 for AWGN channel as:

| 3 PFLs (Tc) | 2 PFLs (Tc) | SINR | SCS (kHz) | PRS bandwidth per PFL (PRBS) |
| --- | --- | --- | --- | --- |
| 16 | 20 | (PRS Ês/Iot)ref ≥-6dB (PRS Ês/Iot)i ≥-13dB | 15 | ≥ 104 |
| 6 | 11 |  | 30 | ≥ 132 |
| 4 | 6 |  |  | 272 |
| 6 | 11 |  | 60 | ≥ 64 |
| 4 | 5 |  |  | 132 |

Proposal 2: Define 4-sample RSTD absolute accuracy in FR2 for AWGN channel as:

| 3 PFLs (Tc) | 2 PFLs (Tc) | SINR | SCS (kHz) | PRS bandwidth per PFL (PRBS) |
| --- | --- | --- | --- | --- |
| 6 | 11 | (PRS Ês/Iot)ref ≥-6dB (PRS Ês/Iot)i ≥-13dB | 60 | ≥ 64 |
| 4 | 5 |  |  | ≥ 132 |
| 4 | 6 |  | 120 | ≥ 64 |
| 2 | 3 |  |  | ≥ 128 |

Proposal 3: Define 4-sample RSTD absolute accuracy in FR1 for fading channel as:

| 3 PFLs (Tc) | 2 PFLs (Tc) | SINR | SCS (kHz) | PRS bandwidth per PFL (PRBS) |
| --- | --- | --- | --- | --- |
| 35 | 45 | (PRS Ês/Iot)ref ≥-6dB (PRS Ês/Iot)i ≥-13dB | 15 | ≥ 104 |
| 29 | 30 |  | 30 | ≥ 132 |
| 27 | 27 |  |  | 272 |
| 35 | 31 |  | 60 | ≥ 64 |
| 33 | 27 |  |  | 132 |

Proposal 4: Define 4-sample RSTD absolute accuracy in FR2 for fading channel as:

| 3 PFLs (Tc) | 2 PFLs (Tc) | SINR | SCS (kHz) | PRS bandwidth per PFL (PRBS) |
| --- | --- | --- | --- | --- |
| 50 | 72 | (PRS Ês/Iot)ref ≥-6dB (PRS Ês/Iot)i ≥-13dB | 60 | ≥ 64 |
| 39 | 55 |  |  | ≥ 132 |
| 49 | 73 |  | 120 | ≥ 64 |
| 51 | 56 |  |  | ≥ 128 |

Proposal 5: Define RSTD absolute accuracy in FR1 for AWGN channel with reduced number of samples as:

| 3 PFLs (Tc) | 2 PFLs (Tc) | SINR | SCS (kHz) | PRS bandwidth per PFL (PRBS) |
| --- | --- | --- | --- | --- |
| 20 | 24 | (PRS Ês/Iot)ref ≥-3dB (PRS Ês/Iot)i ≥-6dB | 15 | ≥ 104 |
| 7 | 13 |  | 30 | ≥ 132 |
| 5 | 6 |  |  | 272 |
| 7 | 13 |  | 60 | ≥ 64 |
| 5 | 6 |  |  | 132 |

Proposal 6: Define RSTD absolute accuracy in FR2 for AWGN channel with reduced number of samples as:

| 3 PFLs (Tc) | 2 PFLs (Tc) | SINR | SCS (kHz) | PRS bandwidth per PFL (PRBS) |
| --- | --- | --- | --- | --- |
| 7 | 13 | (PRS Ês/Iot)ref ≥-3dB (PRS Ês/Iot)i ≥-6dB | 60 | ≥ 64 |
| 5 | 6 |  |  | ≥ 132 |
| 4 | 6 |  | 120 | ≥ 64 |
| 2 | 3 |  |  | ≥ 128 |

Proposal 7: Define 4-sample UE Rx-Tx time difference measurement accuracy in FR1 in AWGN as:

| 3 PFLs (Tc) | 2 PFLs (Tc) | SINR | PRS bandwidth per PFL (PRBS) | SCS (kHz) |
| --- | --- | --- | --- | --- |
| ± 10+𝛅 | ± 12+𝛅 | -3dB | ≥104 | 15 |
| ± 3+ 𝛅 | ± 6+𝛅 |  | ≥132 | 30 |
| ± 2+ 𝛅 | ± 3+𝛅 |  | 272 |  |
| ± 3+ 𝛅 | ± 6+𝛅 |  | ≥64 | 60 |
| ± 2+𝛅 | ± 3+𝛅 |  | ≥ 132 |  |
| ± 13+𝛅 | ± 14+𝛅 | -13dB | ≥104 | 15 |
| ± 4+𝛅 | ± 8+𝛅 |  | ≥132 | 30 |
| ± 3+𝛅 | ± 4+𝛅 |  | 272 |  |
| ± 4+𝛅 | ± 9+𝛅 |  | ≥ 64 | 60 |
| ± 3+𝛅 | ± 4+𝛅 |  | ≥ 132 |  |

Proposal 8: Define UE Rx-Tx time difference measurement accuracy in FR1 in AWGN with reduced number of samples as:

| 3 PFLs (Tc) | 2 PFLs (Tc) | SINR | PRS bandwidth per PFL (PRBS) | SCS (kHz) |
| --- | --- | --- | --- | --- |
| ±9+𝛅 | ±11+𝛅 | 0dB | ≥104 | 15 |
| ±3+𝛅 | ±5+𝛅 |  | ≥132 | 30 |
| ±2+𝛅 | ±3+𝛅 |  | 272 |  |
| ±3+𝛅 | ±5+𝛅 |  | ≥64 | 60 |
| ±2+𝛅 | ±3+𝛅 |  | ≥ 132 |  |
| ±13+𝛅 | ±14+𝛅 | -6dB | ≥104 | 15 |
| ±4+𝛅 | ±9+𝛅 |  | ≥132 | 30 |
| ±3+𝛅 | ±4+𝛅 |  | 272 |  |
| ±4+𝛅 | ±9+𝛅 |  | ≥ 64 | 60 |
| ±3+𝛅 | ±4+𝛅 |  | ≥ 132 |  |

Proposal 9: Define UE 4-sample Rx-Tx time difference measurement accuracy in FR1 for fading channel as:

| 3 PFLs (Tc) | 2 PFLs (Tc) | SINR | PRS bandwidth per PFL (PRBS) | SCS (kHz) |
| --- | --- | --- | --- | --- |
| ± 39+𝛅 | ± 43+𝛅 | -3dB | ≥104 | 15 |
| ± 32+𝛅 | ± 36+𝛅 |  | ≥132 | 30 |
| ± 32+𝛅 | ± 33+𝛅 |  | 272 |  |
| ± 33+𝛅 | ± 37+𝛅 |  | ≥64 | 60 |
| ± 33+𝛅 | ± 32+𝛅 |  | ≥ 132 |  |
| ± 40+𝛅 | ± 47+𝛅 | -13dB | ≥104 | 15 |
| ± 34+𝛅 | ± 36+𝛅 |  | ≥132 | 30 |
| ± 32+𝛅 | ± 33+𝛅 |  | 272 |  |
| ± 37+𝛅 | ± 37+𝛅 |  | ≥ 64 | 60 |
| ± 35+𝛅 | ± 34+𝛅 |  | ≥ 132 |  |

Proposal 10: Define 4-sample UE Rx-Tx time difference measurement accuracy in FR2 in AWGN as:

| 3 PFLs (Tc) | 2 PFLs (Tc) | SINR | PRS bandwidth per PFL (PRBS) | SCS (kHz) |
| --- | --- | --- | --- | --- |
| ± 3+𝛅 | ± 6+𝛅 | -3dB | ≥64 | 60 |
| ± 2+𝛅 | ± 3+𝛅 |  | ≥132 |  |
| ± 2+𝛅 | ± 3+𝛅 |  | ≥64 | 120 |
| ± 1+𝛅 | ± 2+𝛅 |  | ≥128 |  |
| ± 4+𝛅 | ± 9+𝛅 | -13dB | ≥64 | 60 |
| ± 3+𝛅 | ± 4+𝛅 |  | ≥132 |  |
| ± 3+𝛅 | ± 4+𝛅 |  | ≥64 | 120 |
| ± 1+𝛅 | ± 2+𝛅 |  | ≥128 |  |

Proposal 11: Define UE Rx-Tx time difference measurement accuracy in FR2 in AWGN with reduced number of samples as:

| 3 PFLs (Tc) | 2 PFLs (Tc) | SINR | PRS bandwidth per PFL (PRBS) | SCS (kHz) |
| --- | --- | --- | --- | --- |
| ± 3+𝛅 | ± 5+𝛅 | 0dB | ≥64 | 60 |
| ± 2+𝛅 | ± 3+𝛅 |  | ≥132 |  |
| ± 2+𝛅 | ± 3+𝛅 |  | ≥64 | 120 |
| ± 1+𝛅 | ± 2+𝛅 |  | ≥128 |  |
| ± 4+𝛅 | ± 9+𝛅 | -6dB | ≥64 | 60 |
| ± 3+𝛅 | ± 4+𝛅 |  | ≥132 |  |
| ± 3+𝛅 | ± 4+𝛅 |  | ≥64 | 120 |
| ± 1+𝛅 | ± 2+𝛅 |  | ≥128 |  |

Proposal 12: Define 4-sample UE Rx-Tx time difference measurement accuracy in FR2 for fading channel as:

| 3 PFLs (Tc) | 2 PFLs (Tc) | SINR | PRS bandwidth per PFL (PRBS) | SCS (kHz) |
| --- | --- | --- | --- | --- |
| ± 35+𝛅 | ± 55+𝛅 | -3dB | ≥64 | 60 |
| ± 38+𝛅 | ± 35+𝛅 |  | ≥132 |  |
| ± 40+𝛅 | ± 44+𝛅 |  | ≥64 | 120 |
| ± 42+𝛅 | ± 43+𝛅 |  | ≥128 |  |
| ± 40+𝛅 | ± 69+𝛅 | -13dB | ≥64 | 60 |
| ± 40+𝛅 | ± 43+𝛅 |  | ≥132 |  |
| ± 45+𝛅 | ± 54+𝛅 |  | ≥64 | 120 |
| ± 51+𝛅 | ± 47+𝛅 |  | ≥128 |  |

# Annex

## Additional RSTD Results

90%-ile value for accuracy for 4 sample RSTD measurement in FR1 under AWGN propagation condtion when SINR = (-6, -13, -13)dB.

| SCS (kHz) | BW per PFL (PRBs) | Accuracy (Tc) |  |
| --- | --- | --- | --- |
|  |  | 2 PFL | 3 PFL |
| 15 | 104 | 23 | 23 |
| 30 | 132 | 12 | 6 |
|  | 272 | 6 | 4 |
| 60 | 64 | 12 | 6 |
|  | 132 | 6 | 3 |

90%-ile value for accuracy for reduced sample RSTD measurement in FR1 under AWGN propagation condition when SINR = (-3, -6, -6)dB.

| SCS (kHz) | BW per PFL (PRBs) | Accuracy (Tc) |  |
| --- | --- | --- | --- |
|  |  | 2 PFL | 3 PFL |
| 15 | 104 | 27 | 23 |
| 30 | 132 | 12 | 7 |
|  | 272 | 6 | 4 |
| 60 | 64 | 12 | 6 |
|  | 132 | 6 | 3 |

90%-ile value for accuracy for 4 sample RSTD measurement in FR2 under AWGN propagation condition when SINR = (-6, -13, -13)dB.

| SCS (kHz) | BW per PFL (PRBs) | Accuracy (Tc) |  |
| --- | --- | --- | --- |
|  |  | 2 PFL | 3 PFL |
| 60 | 64 | 12 | 6 |
|  | 132 | 6 | 3 |
| 120 | 64 | 6 | 3 |
|  | 128 | 3 | 2 |

90%-ile value for accuracy for reduced  sample RSTD measurement in FR2 under AWGN propagation condition when SINR (-3, -6, -6)dB.

| SCS (kHz) | BW per PFL (PRBs) | Accuracy (Tc) |  |
| --- | --- | --- | --- |
|  |  | 2 PFL | 3 PFL |
| 60 | 64 | 12 | 6 |
| 60 | 132 | 6 | 4 |
| 120 | 64 | 6 | 3 |
| 120 | 128 | 3 | 2 |

## Additional UE Rx-Tx Results

90%-ile value for accuracy in FR1 for 4 sample UE Rx-Tx measurement under AWGN propagation condition.

| SCS (kHz) | BW per PFL (PRBs) | 2 PFL Accuracy (Tc) |  | 3 PFL Accuracy (Tc) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | -3dB | -13dB | -3dB | -13dB |
| 15 | 104 | 15 | 15 | 15 | 15 |
| 30 | 132 | 8 | 8 | 4 | 4 |
|  | 272 | 4 | 4 | 2 | 2 |
| 60 | 64 | 8 | 8 | 4 | 4 |
|  | 132 | 4 | 4 | 2 | 2 |

90%-ile value for accuracy in FR1 for reduced sample UE Rx-Tx measurement under AWGN propagation condition.

| SCS (kHz) | BW per PFL (PRBs) | 2 PFL Accuracy (Tc) |  | 3 PFL Accuracy (Tc) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 0dB | -6dB | 0 dB | -6 dB |
| 15 | 104 | 15 | 15 | 15 | 15 |
| 30 | 132 | 8 | 8 | 4 | 4 |
|  | 272 | 4 | 4 | 2 | 2 |
| 60 | 64 | 8 | 8 | 4 | 4 |
|  | 132 | 4 | 4 | 2 | 2 |

90%-ile value for accuracy in FR2 for 4-sample UE Rx-Tx measurement under AWGN propagation condition.

| SCS (kHz) | BW per PFL (PRBs) | 2 PFL Accuracy (Tc) |  | 3 PFL Accuracy (Tc) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | -3dB | -13dB | -3dB | -13dB |
| 60 | 64 | 8 | 8 | 4 | 4 |
|  | 132 | 4 | 4 | 2 | 2 |
| 120 | 64 | 4 | 4 | 2 | 2 |
|  | 128 | 2 | 2 | 1 | 1 |

90%-ile value for accuracy in FR2 for reduced sample UE Rx-Tx measurement under AWGN propagation condition.

| SCS (kHz) | BW per PFL (PRBs) | 2 PFL Accuracy (Tc) |  | 3 PFL Accuracy (Tc) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 0dB | -6dB | 0dB | -6dB |
| 60 | 64 | 8 | 8 | 4 | 4 |
|  | 132 | 4 | 4 | 2 | 2 |
| 120 | 64 | 4 | 4 | 2 | 2 |
|  | 128 | 2 | 2 | 1 | 1 |