3GPP TSG-RAN WG4 Meeting # 112bis 	    R4-2416333

Hefei, CN, 14  – 18 October 2024

Agenda Item:	6.18.3

Source: 	Ericsson

Title:	On testability issues related to AI/ML for mobility

Document for:	Approval

# Introduction

The updated study item description identifies two objectives for RAN4. The first objective is to study impact on RAN4 requirements based on RAN2 assumptions for AI/ML based mobility. The second objective is to study testability and interoperability issues based on the RAN2 framework for AI/ML based mobility.  The study item objectives are copied below for reference and in the section to follow, testability issues related to AI/ML for mobility are highlighted and discussed.

Text from SID for AI/ML mobility [1].

The study will focus on mobility enhancement in RRC_CONNECTED mode over air interface by following existing mobility framework, i.e., handover decision is always made in network side. Mobility use cases focus on standalone NR PCell change. UE-side and network-side AI/ML model can be both considered, respectively.

Study and evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, considering the following aspects:

- AI/ML based RRM measurement and event prediction,

  - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2]

    - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2]

  - HO failure/RLF prediction (UE sided model) [RAN2]

  - Measurement events prediction (UE sided model) [RAN2]

- Study the need/benefits of any other UE assistance information for the network side model [RAN2]

- The evaluation of the AI/ML aided mobility benefits should consider HO performance KPIs (e.g., Ping-pong HO, HOF/RLF, Time of stay, Handover interruption, prediction accuracy, and measurement reduction) etc.) and complexity tradeoffs [RAN2]

  - NOTE: Simulation assumption and methodology can leverage TR 38.901, 38.843 and 36.839. And leave the detail discussion to RAN2

- Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2]

  - NOTE: This would only be treated after sufficient progress is made in the Rel-19 AI/ML air interface WID

- Potential specification impacts of AI/ML aided mobility [RAN2]

- Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4]

  - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed

  - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.)

  - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability.

  - NOTE 5: Avoid the overlaps with RAN2 work for evaluation

- NOTE 1: RAN1/3 work can be triggered via LS

- NOTE 2: To avoid duplicate study with “AI/ML for NG-RAN” led by RAN3

- NOTE 3: Two-sided model is not included

# Discussions

The updated SID on AI/ML for mobility has study of the testability and interoperability issues as one of the objectives for RAN4 WG. In our view RAN4 should study these aspects in relation to the scenarios identified/agreed by RAN2 for the evaluation of AI/ML based mobility.

Table 1. AI/ML mobility scenarios considered by RAN2.

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

In the agreed scenarios in Table 1, scenarios 2, 3, and 4 are identified as high priority cases, scenarios 1 and 5 subject to further discussions and agreements, and scenario 6 is identified as a middle priority case. In our view discussions on testability and interoperatibility issues in RAN4 should be aligned with the RAN2 prioritization of scenarios for evaluation.

Proposal 1: RAN4 to prioritize discussions on testability and interoperability issues for case numbers 2,3, and 4 in Table 1.

In our companion paper, R4-2416332, we highlighted the issue related to the scope/definition of temporal prediction, where we proposed to define temporal prediction as a process of using a trained AI/ML model to produce a set of outputs that are valid for the future reference of time based on a set of inputs. Based on this definition it is important to understand the time window aspects related to temporal prediction before an evaluation of testability aspects can be performed by RAN4. It is therefore our view to start discussing testability of the scenario 2 and 4 to start once the details related to the time window for prediction is further progressed or settled in RAN2.

Proposal 2: RAN4 to start discussing testability of AI/ML based mobility for scenarios 2 and 4 after the details related to the time window for prediction is further progressed or settled in RAN2.

# Summary

In this contribution Ericsson’s view on testability issues related to AI/ML for mobility are highlighted and discussed. The proposals below summarize the discussion presented in this contribution.

Proposal 1: RAN4 to prioritize discussions on testability and interoperability issues for case numbers 2,3, and 4 in Table 1.

Proposal 2: RAN4 to start discussing testability of AI/ML based mobility for scenarios 2 and 4 after the details related to the time window for prediction is further progressed or settled in RAN2.

# References

- RP-242393, Revised SID on AIML for mobility in NR, OPPO (rapporteur).