3GPP TSG RAN WG4 Meeting #112bis	R4-2416334

Hefei, China, 14 Oct. 2024 – 18 Oct. 2024

Agenda Item:	6.19.3.1

Source: 	Ericsson

Title:	Discussion on UE initiated BM requirements

Document for:	Discussion

# Introduction

In RP-234007, WID for MIMO phase 5 is agreed. In RAN4#112 meeting, RRM requirements impact was analyzed and the following WF [1] is agreed.

| Topic | Description | RRM impact |
| --- | --- | --- |
| UE-initiated/event-driven beam management | UE-initiated/event-driven beam management | YES |
| CSI enhancement | Type-I codebook refinement supporting up to a total of 128 CSI-RS ports | NO |
|  | Type-II codebook refinement supporting up to a total of 128 CSI-RS ports | NO |
|  | CRI-based CSI refinement for up to 128 CSI-RS ports | NO |
|  | Aperiodic standalone CJT calibration reporting | FFS |
| 3TX | 3-antenna-port codebook-based UL transmission | FFS |
| Asymmetric DL sTRP/UL mTRP |  | FFS |

In this contribution we analyze the RRM requirements impact for UE-initiated/event-driven beam management.

# Discussion

In last meeting RAN4 started discussing event triggered L1 measurement reporting requirements. Following WF is agreed [1].

Issue 2-1-2: Scope of the scenario of event-triggered L1 beam reporting Agreement: - Intra-cell and inter-cell for FR2. - Note: If specific requirements cannot be applicable, discuss it case by case. Issue 2-1-3: Whether/how to define new delay requirements such as measurement reporting delay/measurement period? Agreement: - RAN4 will define Event triggered measurement reporting delay requirement - Further discuss: - Whether to support both mode A and mode B - Whether to differentiate mode A and mode B - Other related aspects Issue 2-1-5: measurement metrics for beam report? Agreement: - Based on L1-RSRP - Note: others can be revisited if RAN1 have further agreement Issue 2-1-6: L1-RSRP for CSI-RS measurement? Enhancement for UE-initiated/event-driven beam management Agreement: Define CSI-RS based L1-RSRP measurement requirements. CSI-RS is periodic CSI-RS. Issue 2-1-7: Whether to specify new unified TCI state switching delay? <Way forward> - FFS on whether to define new TCI state switching delay or existing requirements are sufficient Issue 2-1-8: Whether to filter L1-RSRP? <Way forward> - FFS: RRM requirements for event-triggered L1 reporting without time-window-based filtering or other filtering configured. Issue 2-1-9: Impact of beam on known/unknown conditions for TCI states? <Way forward> - FFS. Depends on RAN1 further progress. Issue 2-1-10: Whether to enhance beam sweeping in event triggered L1 beam reporting? <Way forward> - FFS: The assumption of N = 8 fine beam sweeping can be enhanced to realize more efficient BM. For example, the combination of coarse Rx beam and fine Rx beam would facilitate faster Tx and Rx beam refinement. Issue 2-1-11: Whether to define new accuracy requirements by Enhancement for UE-initiated/event-driven beam management? <Way forward> - Option 1: No new accuracy requirements for existing L1 measurements are required. Option 2: RAN4 need to discuss whether to update legacy accuracy requirements

Based on the last meeting WF we divide discussion into following sections.

- Scope of the requirements for UE initiated beam management (IBM)

- Event triggered measurement requirements

- Impact on the unified TCI state switching delay

- Other enhancements

Scope of the requirements for UE IBM

Whether to consider intra-cell and inter-cell scenario for defining event triggered reporting for CSI-RS is discussed in last meeting. We provide our view on the inter-cell scenario in this section.

In Rel-15, SSB based L1-RSRP requirements were defined for intra-cell scenario where all the SSB in a cell are associated with single PCI. In Rel-17, RAN1 introduced multi-TRP scenario where SSB of the cell can be associated with different PCI, where the SSB with different PCI are typically transmitted from different TRP associated with same serving cell. The scenario is referred as inter-cell multi-TRP. In Rel-17, RAN4 defined L1-RSRP measurement requirements for inter-cell multi-TRP scenario where one TRP can be associated with serving cell PCI and other TRP can be associated with different PCI than the serving cell PCI. However, in that WI, only SSB was considered, and CSI-RS based inter-cell L1-RSRP measurements were not considered during that WI.

CSI-RS requirements are defined in Rel-15 and SSB is assumed to be QCL’ed with CSI-RS. As per the applicability of CSI-RS requirements shown below, SSB which is QCLed with CSI-RS can be intra-cell and inter-cell.

For periodic CSI-RS resources in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply if qcl-InfoPeriodicCSI-RS is configured for all the resources in the resource set and for each resource one RS has QCL-TypeD with - SSB for L1-RSRP measurement, or - another CSI-RS in resource set configured with repetition ON. - For periodic CSI-RS resources in a resource set configured with higher layer parameter repetition set to ON, N=ceil(maxNumberRxBeam / Nres_per_set) for Table 9.5.4.2-2, where Nres_per_set is number of resources in the resource set. The requirements apply provided qcl-InfoPeriodicCSI-RS is configured with QCL-TypeD for all resources in the resource set. For semi-persistent CSI-RS resources in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply provided TCI state is provided for all resources in the resource set in the MAC CE activating the resource set and for each resource one RS has QCL-TypeD with - SSB for L1-RSRP measurement, or - another CSI-RS in resource set configured with repetition ON.

One of the first agreement RAN1 made for this work item is shown below. Since supporting intra-cell and inter-cell scenario was one of the first agreement by RAN1, it signifies the importance of considering both the scenarios.

Agreement

On UE-initiated/event-driven beam reporting, at least support L1-RSRP as a measurement quantity on SSB for intra-cell and inter-cell, and periodic CSI-RS for beam management

- Notes: measurement results may be contained in the beam report and/or used as quality metric(s) to initiate/trigger the reporting.

- FFS: Semi-persistent CSI-RS and aperiodic CSI-RS.

- FFS: Whether/how to support L1-SINR measurement, assuming legacy RS or RS combination (e.g., CMR only, CMR+ZP/NZP-IMR) for Rel-16 SINR is reused.

- FFS: Whether/how to specify filtering operation for L1-RSRP.

- Event triggered measurement report requirements are applicable for intra and inter-cell scenario for SSB and CSI-RS.

When UE is configured with CSI-RS measurements in inter-cell scenario, SSB associated with CSI-RS may be from different TRP (different PCI) and in FR2, they may need to be received with different QCL Type D. Since multi-Rx may not always be assumed, UE may be able to measure only one TRP CSI-RS at a time when the CSI-RS from different TRP are overlapping in time. RAN4 may need to define sharing or scaling rules for the inter-cell CSI RS measurements. One may argue that SSB based inter-cell L1-RSRP measurements are defined during a dedicated WI in Rel-17 and may contain big workload to define CSI-RS based inter-cell L1-RSRP measurement requirements. We think that RAN4 can reuse the principles of sharing agreed for SSB based measurements to derive the requirements for CSI-RS based inter-cell L1-RSRP measurements.

- RAN4 to define CSI-RS based L1-RSRP measurements requirements for inter-cell mTRP scenario in FR2.

- RAN4 to use principles of sharing agreed for SSB based L1-RSRP for inter-cell CSI-RS based L1-RSRP measurements.

Event triggered measurement requirements

Based on the RAN1 agreements, we summarize the framework for UE initiated beam report in figure 1.

Figure 1: Framework for UE initiated beam reporting

As shown in figure 1, UE initiated beam management framework can be divided into configuration aspects and UE actions. From the RAN1 agreements we can observe that RAN1 is still discussing the resource configuration, event configuration, events definition, and the report configuration. As the configuration aspects are specified in RAN1/2, RAN4 does not need to discuss the configuration aspects and RAN1 progress may not have impact on RAN4.

UE actions: From the figure 1, we think following UE actions may have impact on the RAN4 requirements and needs to be discussed in RAN4.

- L1 measurements for event triggered beam report

- Event evaluation

- Measurement Report transmission

  - Transmission on first UL channel

  - Obtain resource for second UL channel

  - Transmit on second UL channel

In previous RAN4 meeting following WF is agreed.

Whether/how to define new delay requirements such as measurement reporting delay/measurement period?

Agreement:

•	RAN4 will define Event triggered measurement reporting delay requirement

•	Further discuss:

- Whether to support both mode A and mode B

- Whether to differentiate mode A and mode B

- Other related aspects

Event triggered measurement delay:

The legacy measurement reporting delay for FR1 is copied below for the reference. UE measurement occasions for L1-RSRP depend on following factors.

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: K = 1 when TSSB ≤ 40 ms and highSpeedMeasFlag-r16 or highSpeedMeasCA-Scell-r17 are configured; otherwise K = 1.5. Note 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16. or measurementEnhancementCA-r17 |  |

- Report periodicity (TReport)

- SSB periodicity (TSSB)

- Number of samples (M)

- Sharing factor (P Sharing with inter-cell L1 and L3)

- DRX period (TDRX)

As per the existing L1 measurement delay requirement, if the reporting periodicity is 80ms and SSB periodicity is 20ms, in FR1, when M=1, UE at least need to measure SSB every 80ms though the SSB periodicity is 20ms. This is mainly because, even if UE measures every 20ms, UE need to report it every 80ms. However, same is not true for L1 event triggered reporting as UE may not know when the event is going to be met, and reporting periodicity should not be taken into the account for event triggered reporting delay. For the event triggered reporting, we think all other parameters than the TReport in the measurement delay can be considered same. In other words, for the L1 event triggered reporting, UE need to measure every measurement occasion which result for L1-RSRP measurement after considering the sharing with L1 and L3 measurements and sharing among the serving cell and additional PCI. One may argue that this may not be the case for L3 event triggered reporting. We think for the L3 based event triggered measurement, UE needs to send one L1 measurement to L3 layer which is averaged over 5 samples and the UE L1 sampling frequency is minimum of 200ms. However, same do not holds for L1 event triggered reporting as RAN1 did not agree on UE filtering for L1 event triggered measurements.

- SSB based L1-RSRP measurement delay for event triggered measurement report is TL1-RSRP_Measurement_Period_SSB (ms) with TReport as 0.

- CSI-RS based L1-RSRP measurement delay for event triggered measurement report is TL1-RSRP_Measurement_Period_CSI-RS with TReport as 0.

Event evaluation:

L1-RSRP is used for beam management and periodic L1-RSRP can be configured with periodicity of the SSB or CSI-RS. Since the report is used for beam management, UE should be able to send the report as soon as event condition is met. Event triggering conditions can be changed as fast as the measurement occasions. UE is expected to evaluate the event after every measurement occasion to check if any of the configured event is met. Once the event is met as shown in figure 1, UE needs to send the first UL signal to the NW to indicate to the NW that event is met. If the time between measurement occasion and first UL channel is short UE may not be ready to transmit on the first UL channel. In this case UE may need to wait for next transmission opportunity of the first UL channel. To avoid the scheduling of the first UL channel so close to the measurement occasions, RAN4 should define the event evaluation time.

- RAN4 to discuss the event evaluation time (i.e., the minimum time required to complete the event evaluation and be ready to send on first UL channel from the measurement occasion).

Measurement Report transmission

Either Mode-A or Mode-B discussed in RAN1 can contain following three steps.

- Transmission on first UL channel

- Obtain resource for second UL channel

  - Mode A: Obtain the resource though Aperiodic CSI request

  - Mode B: Pre-configured Type-1 CG-PUSCH (at least other types may be FFS)

- Transmit on second UL channel

The above three steps depend on the configuration and NW scheduling. We may not need to specify RAN4 requirements for these steps.

- RAN4 may not need to specify requirements for Transmission on first UL channel, obtain resource for second UL channel and transmit on second UL channel.

Impact on unified TCI state switching delay

In last meeting there was a discussion on whether to define new TCI state switching delay or existing requirements are sufficient for the unified TCI state switching based on event triggered reporting. To analyze the requirements, we first look at the events agreed in RAN1 to discuss whether there is an impact on the unified TCI state switch delay.

RAN1 agreed on following events

- Event-1: Quality of the current beam is worse than a certain threshold.

- Event-2: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam.

- Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the M-th best quality.

  - M is RRC configured with subjective to UE capability signalling

    - UE may only indicate a single candidate value or not support Event-7.

The intention or use case of each of the event is provided below

- Event-1: when the quality of the current beam is lower than certain threshold NW may request the aperiodic report and schedule a beam change.

- Event 2: when quality of the new beam is better than the current beam, NW can trigger the beam switch based on the report from the UE.

- Event-7: When a new beam which was not in the active TCI state list is better than beams in active TCI state list, based on the UE report, NW update the active TCI state list.

Currently RAN1 supports two alternatives for unified TCI state activation. Based on the use case for which the events are defined, we think the RAN1 defined events support the existing TCI state activation framework listed below.

Alternative 1:

- Step 1: RRC configuring large pool of TCI states

- Step 2: MAC CE activating some of the TCI states from the RRC configured TCI states

- Step 3: DCI indicating one of the MAC CE activated TCI states.

Alternative 2:

- Step 1: RRC configuring large pool of TCI states

- Step 2: MAC CE directly activating a TCI state from the RRC configured TCI states

In some implementations, after receiving the measurement report when the event 2 trigger conditions are met, if a new beam which was not in the active TCI state list is better than the current beam, then NW may decide to switch the current beam to the new beam. In this scenario, Alternative 1 may cause extra delay for switching the current beam to best beam. However, we think that this can be supported with Alternative 2 as described above. Hence, we do not see a need for new requirements definition.

- Existing unified TCI state switching delay requirements are applicable for event triggered measurement reporting based TCI state switching.

Other enhancements

Following two issues are brought as part of the other enhancements during last meeting. Though these are good to have, we think both of the issues are out of scope for this WI and do not see a need for specifying them in Rel-19 as they are not specific to event triggered beam reporting.

Whether to enhance beam sweeping in event triggered L1 beam reporting?

<Way forward>

•	FFS: The assumption of N = 8 fine beam sweeping can be enhanced to realize more efficient BM. For example, the combination of coarse Rx beam and fine Rx beam would facilitate faster Tx and Rx beam refinement.

Whether to define new accuracy requirements by Enhancement for UE-initiated/event-driven beam management?

<Way forward>

•	Option 1: No new accuracy requirements for existing L1 measurements are required.

•	Option 2: RAN4 need to discuss whether to update legacy accuracy requirements.

- RAN4 not to specify new Rx beam sweeping factor or new accuracy requirements as part of the event triggered measurement requirements.

# Summary and Conclusion

In this contribution we analyzed potential RAN4 RRM requirements impact to UE IBM and made following observations and proposals.

- Event triggered measurement report requirements are applicable for intra and inter-cell scenario for SSB and CSI-RS.

- RAN4 to define CSI-RS based L1-RSRP measurements requirements for inter-cell mTRP scenario in FR2.

- RAN4 to use principles of sharing agreed for SSB based L1-RSRP for inter-cell CSI-RS based L1-RSRP measurements.

- SSB based L1-RSRP measurement delay for event triggered measurement report is TL1-RSRP_Measurement_Period_SSB (ms) with TReport as 0.

- CSI-RS based L1-RSRP measurement delay for event triggered measurement report is TL1-RSRP_Measurement_Period_CSI-RS with TReport as 0.

- RAN4 to discuss the event evaluation time (i.e., the minimum time required to complete the event evaluation and be ready to send on first UL channel from the measurement occasion).

- Existing unified TCI state switching delay requirements are applicable for event triggered measurement reporting based TCI state switching.

- RAN4 not to specify new Rx beam sweeping factor or new accuracy requirements as part of the event triggered measurement requirements.

# References

[1]		R4-2414035, “WF on RRM requirements for NR_MIMO_Ph5”, Samsung

[2] 	RP-234007, New WID: NR MIMO Phase 5