3GPP TSG RAN WG4 Meeting #112bis	R4-2416336

Hefei, China, 14 Oct. 2024 – 18 Oct. 2024

Agenda Item:	6.24.2.1

Source: 	Ericsson

Title:	Discussion on measurements enhancements for purpose of supporting LTM

Document for:	Discussion

# Introduction

In this contribution we discuss Event triggered measurement requirements and CSI-RS based L1 measurement requirements.

# Discussion

## Event triggered measurement requirements

### Impact of event triggered to reporting to LTM procedures

In Rel-19, RAN2 is introducing event triggered L1 measurement reporting to reduce the signalling overhead in the UL due to the periodic LTM CSI reports in the UL. RAN2 agreed on introducing the following events for the purpose of triggering measurement reporting and the reporting content for the event triggered reporting is under further study in RAN2.

| Event | Description |
| --- | --- |
| Event LTM2 | Beam of serving cell becomes worse than absolute threshold; |
| Event LTM3 | Beam of candidate cell becomes amount of offset better than beam of serving cell; |
| Event LTM4 | Beam of candidate cell becomes better than absolute threshold; |
| Event LTM5 | Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2. |

Before discussing the RAN4 RRM requirements scope, we look at the intention or use case of the RAN2 introduced events.

- Event LTM2: As the name suggests this event is similar to event A2 for the event triggered L3 measurements. When the beam of the serving cell becomes worse than absolute threshold, NW may request an aperiodic report to schedule early TCI state activation or PDCCH order based RACH.

- Event LTM3: As the name suggests this event is similar to event A3 for the event triggered L3 measurements. When beam of candidate cell becomes amount of offset better than beam of serving cell, NW can trigger the cell switch based on the event triggered report from the UE.

- Event LTM4: As the name suggests this event is similar to event A4 for the event triggered L3 measurements. When beam of candidate cell becomes better than absolute threshold, based on the UE report, NW may activate the TCI states of the candidate cell or update the active TCI state list.

- Event LTM5: As the name suggests this event is similar to event A5 for the event triggered L3 measurements. When Beam of serving cell becomes worse than absolute threshold1 AND beam of candidate cell becomes better than another absolute threshold2, based on the UE report, NW may activate the TCI states of the candidate cell or update the active TCI state list or perform LTM cell switch.

Based on the above analysis we can observe that RAN2 agreed events are designed to request a candidate cell L1 measurement report to perform early DL/UL synchronization on candidate cells or trigger an LTM cell switch.

How to use these events and how many events to configure is up to NW implementation. The above events may be configured with other events or a single event (e.g., NW may configure only event LTM3). Assuming NW may trigger LTM cell switch based on the LTM3, to utilize the potential of LTM, prior to the LTM3 event entering condition is met, NW may have to schedule early TCI state activation and PDCCH order based RACH. Early TCI state activation can be based on L3 measurement reports or based on other L1 event triggered reporting. If the early TCI state activation is based on L3 measurement report and cell switch is based on the event triggered L1 report, current TCI state known conditions may not be valid as the existing known conditions (at least for TCI state activation) are defined based on the periodic L1-RSRP report.

Rel-18 candidate cell TCI state activation delay requirements are applicable for known TCI states and for unknown TCI states under certain scenarios. Rel-18 TCI state known conditions and conditions under which TCI state activation requirements are applicable are mentioned in below table.

Known conditions of the TCI state: The candidate TCI state in the TCI state activation list is known if the following conditions are met: - During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target downlink TCI state to the completion of active downlink TCI state activation, where the RS resource for L1-RSRP measurement is the RS in target downlink TCI state or QCLed to the target downlink TCI state - The MAC CE command indicating the activation of downlink TCI state is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement - The UE has sent at least 1 valid L1-RSRP report for the target downlink TCI state before the MAC CE command is received. - The target downlink TCI state remains detectable during the downlink TCI state activation period - The SSB associated with the downlink TCI state remain detectable during the downlink TCI activation period - SNR of the downlink TCI state ≥ -3dB Otherwise, the candidate TCI state is unknown. TCI state activation requirements are applicable for: Known TCI state in FR1 and FR2 specifed in clause 8.25.2, and - Unknown TCI state in FR1 specifed in clause 8.25.2, provided the following conditions are met: - UE has reported L3 measurement result with the associated SSB index of the TCI state within 5 seconds before the LTM TCI state activation command. - SNR of the associated SSB is above -3dB.

We may need to consider a scenario where event triggered reporting is configured but the event is not triggered. In this scenario, though UE was measuring the RS, since the report was not sent, as per the existing known condition defintion, TCI state is considered unknown. If the TCI state is unknown, cell switch delay requirements are not applicable as per Rel-18 agreements. Due to this, we think TCI state known condition defintion needs to be modified. Moreover, TCI state activation requirements are still under discussion for Rel-18 LTM. We may need to look at the impact of TCI state activation requirements with the introduction event triggered reporting.

- RAN4 to revisit following requirements to accommodate event triggered LTM reporting

  - Defintion of TCI state known condition

  - TCI state activation requirements

RAN2 agreed to support at least L1-RSRP for the event triggered beam report. Further, as part of the measurement enhancements, Rel-19 needs to specify the CSI-RS based measurement requirements. Since the event framework and event evaluation criteria should be same for CSI-RS and SSB, we think RAN4 requirements for event triggered LTM reporting should be applicable for SSB and CSI-RS.

- RAN4 to specify the requirements for SSB based event triggered L1-RSRP reporting, and CSI-RS based event triggered L1-RSRP reporting.

### L1-RSRP requirements for event triggered reporting

In previous meeting, many companies suggested taking event triggered L3 measurement requirements as baseline for L1 event triggered measurement requirements. Existing L3 event triggered reporting delay requirements are defined considering RRC reconfiguration as a starting point keeping the testability aspect in mind. That means, when the test is started, the carrier is not detectable (lets say during T0 in Figure 1, carrier is not detectable) and after a certain point (e.g., during T1), the signal level of the carrier is increased to an extent that carrier is detectable and event entering condition is met.

The event triggered measurement requirement for L3 measurement (shown in below table).

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on. The event triggered measurement reporting delay, measured without L3 filtering shall be less than Tidentify intra with index or T identify intra without index defined in clause 9.2.5.1 or clause 9.2.6.2. When L3 filtering is used an additional delay can be expected. In EN-DC and NE-DC operation, when the UE is configured to perform E-UTRA SRS carrier-based switching an additional delay can be expected in FR1 if the UE is capable of per-FR gap, or an additional delay can be expected in both FR1 and FR2 if the UE is not capable of per-FR gap. If a cell which has been detectable at least for the time period Tidentify intra without index or Tidentify intra with index defined in clause 9.2.5.1 or clause 9.2.6.2 becomes undetectable for a period ≤ 5 seconds and then the cell becomes detectable again with the same spatial reception parameter and triggers an event, the event triggered measurement reporting delay shall be less than TSSB_measurement_period_intra provided the timing to that cell has not changed more than 3200/2µ Tc while the measurement gap has not been available and L3 filtering has not been used, where µ is the SCS configuration as defined in clause 4.2 of TS 38.211 [3]. When L3 filtering is used, an additional delay can be expected. In EN-DC and NE-DC operation, when the UE is configured to perform E-UTRA SRS carrier-based switching an additional delay can be expected in FR1 if the UE is capable of per-FR gap, or an additional delay can be expected in both FR1 and FR2 if the UE is not capable of per-FR gap.

Figure 1: L3 event triggered measurement period

The requirement highlight in yellow represents the scenario where carrier is configured/detected for the first time or re-detected after 5 secs from last measurement. In this scenario, UE needs to perform cell search with SSB index detection or without SSB index detection based on the report configuration and perform SSB based measurement. The point at which UE realise the event is met is Point C.

The requirement shown in cyan represents the scenario where UE detected the cell within last 5 seconds. In this scenario, the maximum amount of time UE takes to detect event met condition is less than SSB measurement period (in some cases, if UE implemented sliding window, UE may detect the event with one additional sample after the event is met in channel or 5 additonal samples depending on the UE measurement window). The point at which UE realise the event is met is Point B or A based on the UE implementation.

The requirement highlighted in green shows that maximum uncertaininty to insert the report into the DCCH. This can be considered as the report preparation time after the UE realises that the event is met.

- L3 event triggered reporting requirements are specified for unknown cell and known cell.

In Rel-18, LTM L1-RRSP measurements are defined for the known cell scenario. That means UE has measured or sent a L3 measurement report within last 5 seconds. It may be reasonable to keep the same assumption for the event triggered measurement reporting too.

- RAN4 to define the event triggered measurement report for known cell scenario.

In Rel-18, LTM L1-RSRP report is single shot measurement (i.e., M is 1). However, event triggered measurement report main purpose is to reduce the signalling overhead in the UL. To avoid false event triggering reports due to fast fading, RAN1 is discussing filtering for the event triggered reporting. If the filtering is agreed, we need to define new measurement delay with higher number of samples than single shot measurement.

- To define event triggered measurement delay for L1-RSRP, RAN4 to wait for RAN1 conclusion on filtering.

Since the LTM L1 measurement report is used for mobility decisions, UE should be able to send the report as soon as event condition is met. Event triggering conditions may be changed as fast as the measurement occasions. UE is expected to evaluate the event after every measurement occasion to check if any of the configured event is met. If the time between measurement occasion and reporting occasion is short UE may not be ready to transmit on the first reporting occasion. In this case UE may need to wait for next transmission opportunity of the reporting occasion. To avoid the scheduling of the reporting occasion so close to the measurement occasions, RAN4 should define the event evaluation time.

- RAN4 to discuss and define the event evaluation time (i.e., the minimum time required to complete the event evaluation and be ready to send on the first reporting occasion from the measurement occasion).

Measurement Report transmission

The reporting procedure and the steps involved in the reporting procedure is FFS and being discussed in other WG. The impact of the reporting procedure on the RAN4 requirements can be revisited after further progress in RAN1/2.

## CSI-RS based L1 measurement requirements

In last meeting started initial discussion on the CSI-RS based measurement requirements. Following requirements are identified as the set of requirements that may need to be defined.

- The following requirements need to be discussed for CSI-RS based L1 RSRP measurement on LTM candidate cell(s)

  - Measurement delay

  - Measurement reporting requirements

  - Measurement restrictions

  - Scheduling availability

- FFS on other potential impact.

Minimum resource that can be configured for L1-RSRP or L3-RSRP is 24 RB. However, RAN4 requirements for serving cell L1-RSRP and L3-RSRP measurements are defined for CSI-RS resource configuration of {D=3, PRB≧48}. In last RAN1 meeting, at least CSI-RS for BM is agreed. Hence, we think the same resource configuration can be reused.

- RAN4 to use CSI-RS configuration of D=3, PRB≧48 for the accuracy requirements definition.

- RAN4 to discuss the CSI-RS accuracy requirements during performance part.

### Intra-frequency and Inter-frequency defintion

Though RAN1 did not complete the CSI-RS configuration aspects for L1-RSRP on the candidate cells, RAN4 started discussing intra-frequency and inter-frequency definition. Following options were discussed in last meeting.

- Option 1: following CSI-RS based L3 measurement, (Apple) - A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that: - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of the CSI-RS resource on the serving cell indicated for L1 measurement, and - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of the CSI-RS resource of the serving cell indicated for L1 measurement, and - It is applied for SCS = 60KHz - the centre frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of the CSI-RS resource of the serving cell indicated for L1 measurement - Option 2: Categorize CSI-RS based L1-RSRP measurement into CSI-RS based L1-RSRP measurement within active BWP and outside active BWP for further discussion. (MTK) - Option 3: For CSI-RS based L1 measurement, RAN4 not to introduce definition of intra-frequency/inter-frequency candidate cell measurement. (vivo)

Option 1 was motivated from the CSI-RS based L3 measurement intra-frequency defintion. One disadvantage we see in option 1 is that NW always need to configure serving cell CSI-RS based L1 measurements for the comparison of intra-frequency defintion, though NW may not use CSI-RS for L1-RSRP in the serving cell (that means NW do not use fine beams in serving cell but may plan to use them in candidate cells). As per option 1, if NW do not configure CSI-RS based L1 measurement, though the CSI-RS of the neighbour cell is in the active BWP, it is considered as inter-frequency measurement and may need to be measured with GAP.

Option 2 and option 3 do not consider defining the measurement as an intra-frequency or inter-frequency measurement but defines whether the UE needs measurement gap or not. We think option 2 may be better suited for CSI-RS based candidate cell L1 measurements.  However, intra-frequency or inter-frequency measurement defintion is not only used for defining the measurement with gap or without gap but also to count the number of frequency layers that UE needs to measure and to define the sharing or scaling rules for the measurement delay requirements. Unlike SSB, CSI-RS is measurements are expected to be performed using the data FFT (SSB measurements are assumed to be performed using measurement engine). When CSI-RS BW is within active BWP, SCS is same as the serving cell PDSCH, CP type are same serving cell, UE should be able to measure them as Intra frequency measurements without gap.

- RAN4 to not define intra-frequency and inter-frequency defintion for CSI-RS based L1 measurements on candidate cells. Instead, categorize CSI-RS based L1 measurement into CSI-RS based L1-RSRP measurement within active BWP and outside active BWP

- RAN4 to define frequency layer to define the measurement delay requirements and applicability requirements

### Defintion of the frequency layer

If two CSI-RS resources belonging to same frequency layer, UE is expected to measure them at the same time using the same FFT. UE can measure the two CSI-RS resources at the same time using a same FFT if their SCS, CP type, BW, center frequency, and has overlapping periodicity.

- Two CSI Resource are of same frequency layer if they have same SCS, CP type, BW, center frequency, and has overlapping periodicity.

### Supported measurement types

When a measurement is within the active BWP it can be assumed to be performed without measurement gap and if the measurement is outside active BWP, it can be assumed to be performed with measurement gap. Since CSI-RS is used for L1-RSRP of candidate cells for the mobility purpose, RTD > CP shall be considered. In short, measurements inside BWP, outside BWP, RTD > CP and RTD < CP shall be considered.

- CSI-RS based L1 measurements for candidate cells shall support

- Measurement within active BWP, Measurements outside active BWP

- RTD < CP, and RTD > CP

### Measurement behaviour

In the previous meeting, the following options were discussed. We provide our views on the measurement behaviour for CSI-RS based L1 measurements.

- P1: Consider SSB based L3 measurement as the pre-requisite condition to determine the target cells for CSI-RS based L1-RSRP measurement. (Apple) - P1a: In FR1, UE shall first perform SSB based L3 measurement on candidate neighbour cells. Then UE performs configured CSI-RS L1 measurement on these candidate neighbour cells. (HW) - P2: For L1 CSI-RS measurement on candidate neighbour cells in FR2, RAN4 needs to further discuss two possible options: (HW) - Option 1: UE performs SSB based L3 measurements and acquires SSB index information of the candidate cells. And then UE receives the configured CSI-RS resources and perform L1 CSI-RS measurement with Rx beam sweeping on candidate cells. - Option 2: UE performs SSB based L3 measurements and acquires SSB index information of the candidate cells. And then UE performs SSB based L1 measurement to refine the RX beam (with RX beam sweeping) which is the same as R18 LTM. Afterwards UE perform measurements without RX beam sweeping on the configured CSI-RS resources of candidate cells where each CSI-RS resource is QCL-typeD with SSB for L1-RSRP measurement. - P3: For CSI-RS resources with repetition OFF, L1-RSRP measurement is performed only after UE has performed L1-RSRP measurement on the associated SSB. (MTK)

In last meeting at least periodic CSI-RS is agreed in RAN1. One potential way of using the CSI-RS on candidate cells to achieve better gain may be periodic or semi-persistent CSI-RS combined with aperiodic or semi-persistent CSI report. After the early sync, NW may request the aperiodic/semi-persistent report with repetition OFF and/or repetition ON (pending based on RAN1 agreement) to make better use of the feature. Hence, it is reasonable to assume that CSI-RS report may be requested by NW after the SSB based measurement configuration or reports. Since we cannot put requirement on the NW configuration, we can say that CSI-RS requirements are applicable if the SSB based measurement is performed at least X seconds before the CSI-RS report request.

- CSI-RS based measurement requirements are applicable if the UE measured SSB based L3 measurements less than X seconds before the CSI-RS measurement.

Since the CSI-RS needs to be QCLed with SSB for the timing and QCL assumption, it is reasonable to assume that UE needs to measure the SSB at least X ms before CSI-RS measurement occasion. In FR1, UE needs to acquire the timing from the SSB, UE needs to measure the SSB at least once 160ms before the CSI-RS measurement occasion.

- In FR1, if the SSB L3 measurement period is more than 160ms, UE may need additional SSB before the CSI-RS measurement is performed.

In FR2, UE may need to perform RX beam sweeping. The measurement flow depends on whether CSI-RS is configured with repetition ON/OFF.

If the repetition is not configured, and SSB based L1-RSRP is configured on the candidate cell, our understanding of UE measurement behaviour is provided below.

- UE performs SSB based L3 measurements with RX beam sweeping in FR2.

- SSB based L1 measurements are performed with Rx beam sweeping in FR2.

- UE performs the CSI-RS measurement without Rx beam sweeping.

If the repetition is not configured, and SSB based L1-RSRP is not configured on the candidate cell, our understanding of UE measurement behaviour is provided below.

- UE performs SSB based L3 measurements with RX beam sweeping in FR2.

- UE performs the CSI-RS measurement without Rx beam sweeping.

If the repetition is configured, and SSB based L1-RSRP is configured on the candidate cell, our understanding of UE measurement behaviour is provided below.

- UE performs SSB based L3 measurements with RX beam sweeping in FR2.

- SSB based L1 measurements are performed with Rx beam sweeping in FR2.

- UE performs the CSI-RS measurement with Rx beam sweeping in FR2.

If the repetition is configured, and SSB based L1-RSRP is not configured on the candidate cell, our understanding of UE measurement behaviour is provided below.

- UE performs SSB based L3 measurements with RX beam sweeping in FR2.

- UE performs the CSI-RS measurement with Rx beam sweeping.

Based on the above analysis, in FR2, if the SSB based L1-RSRP is not configured, UE do not need to perform SSB based L1-RSRP before performing CSI-RS based L1-RSRP measurement. The main reason for this is, when repetition is OFF, the Rx beam used for SSB based L1 measurement and CSI-RS L1 measurement need not be different, and UE may use the same beam for SSB based L1-RSRP  and CSI-RS based L1-RSRP. However, SSB based L3 measurement beam and SSB based L1 measurement beam can be different. We think UE can directly measure CSI-RS to acquire the fine beam for L1-RSRP measurement.

- In FR2, if the SSB based L1-RSRP is not configured, UE do not need to perform SSB based L1-RSRP before performing CSI-RS based L1-RSRP measurement.

### Known cell conditions

SSB based L1-RSRP measurement are defined for known cell and the known cell definition is agreed as following. We think same assumption can be agreed for CSI-RS based L1-RSP requirements.

The cell is considered as known if the following conditions are met in this requirement: - The UE has performed L3 measurement on the target cell during the last 5 seconds, and - The SSB from the target cell configured for L1 measurement remains detectable according to the cell identification requirements specified in clause 9.2 remains detectable according to the cell identification requirements specified in clause 9.2. Otherwise, the cell is unknown.

- CSI-RS based L1-RSRP on neighbour cells to be defined for known cell scenario and an LTM candidate cell is known if the following conditions are met

  - The UE has performed L3 measurement on the target cell during the last 5 seconds, and

  - The CSI-RS from the target cell configured for L1 measurement remains measurable.

### Measurement gaps

In last meeting RAN1 agreed on using at least CSI-RS for BM as RS for CSI-RS measurements on candidate cells. The type of MG to be used may depend on the type of CSI-RS RAN1 agrees on.  Since RAN1 has just started configuration aspects for CSI-RS measurements on candidate cells, RAN4 can wait for further RAN1 progress to discuss the type of measurement gaps to use.

- For the decision on MG to use, RAN4 to wait for further RAN1 progress

# Summary and Conclusion

In this contribution we analyzed potential RAN4 RRM requirements impact and made following observations and proposals.

- RAN4 to revisit following requirements to accommodate event triggered LTM reporting

  - Defintion of TCI state known condition

  - TCI state activation requirements

- RAN4 to specify the requirements for SSB based event triggered L1-RSRP reporting, and CSI-RS based event triggered L1-RSRP reporting.

- RAN4 to define the event triggered measurement report for known cell scenario.

- To define event triggered measurement delay for L1-RSRP, RAN4 to wait for RAN1 conclusion on filtering.

- RAN4 to discuss and define the event evaluation time (i.e., the minimum time required to complete the event evaluation and be ready to send on the first reporting occasion from the measurement occasion).

- RAN4 to use CSI-RS configuration of D=3, PRB≧48 for the accuracy requirements definition.

- RAN4 to discuss the CSI-RS accuracy requirements during performance part.

- RAN4 to not define intra-frequency and inter-frequency defintion for CSI-RS based L1 measurements on candidate cells. Instead, categorize CSI-RS based L1 measurement into CSI-RS based L1-RSRP measurement within active BWP and outside active BWP

- RAN4 to define frequency layer to define the measurement delay requirements and applicability requirements

- Two CSI Resource are of same frequency layer if they have same SCS, CP type, BW, center frequency, and has overlapping periodicity.

- CSI-RS based L1 measurements for candidate cells shall support

- Measurement within active BWP, Measurements outside active BWP

- RTD < CP, and RTD > CP

- CSI-RS based measurement requirements are applicable if the UE measured SSB based L3 measurements less than X seconds before the CSI-RS measurement.

- In FR1, if the SSB L3 measurement period is more than 160ms, UE may need additional SSB before the CSI-RS measurement is performed.

- In FR2, if the SSB based L1-RSRP is not configured, UE do not need to perform SSB based L1-RSRP before performing CSI-RS based L1-RSRP measurement.

- CSI-RS based L1-RSRP on neighbour cells to be defined for known cell scenario and an LTM candidate cell is known if the following conditions are met

  - The UE has performed L3 measurement on the target cell during the last 5 seconds, and

  - The CSI-RS from the target cell configured for L1 measurement remains measurable.

- For the decision on MG to use, RAN4 to wait for further RAN1 progress.

# References

[1] 	RP-241515, New WID: NR mobility enhancements Phase 4

[2] 	R4-2414060, WF on NR mobility enhancements Phase 4