# 3GPP TSG-RAN WG4 Meeting # 112bis                                R4-2416342

# Hefei, China, 14 – 18 October, 2024

# Title: 	Discussion on MIMO layer evaluation for 6Rx UE

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	6.1.1.3.2

# Document for:	Approval

# 1 Introduction

In RAN #103 meeting, the new WID of UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4 was approved in [1], wherein the objectives related to 6Rx for handheld UE and FWA UE are listed as below:

6Rx for handheld and FWA UE - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104 - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers - Specify the Rx requirements including reference sensitivity requirements for support 6Rx - Note: the specified requirements can be applicable to both handheld UE and FWA devices - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, t4r6 depending on UE capability - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

During the last meeting, some conclusions has been reached for 6Rx UE MIMO layer [2]. In the following section, we will provide the detailed discussions for MIMO layer evaluation with 6Rx.

# 2 Discussion

6-layer Support

Way Forward: RAN4 to further discuss the following options. - Option 1: 6-layer support is feasible for handheld and FWA based on some company evaluations considering realistic antenna correlation assumptions and deployment scenarios. - Option 2: 6-layer support is feasible for FWA devices. - Option 3: Companies to align on the feasibility criteria and simulation assumptions to evaluate 6-Layer feasibility using the following as a starting point. - Feasibility criteria - Consider 4-layer vs 6-layer throughput performance. - Simulation assumptions - Only consider link-level simulation for performance evaluations. - Simulation assumptions provided in Annex A as a starting point. Companies to clarify the settings used in the simulation assumptions for BS correlation matrix and Tx EVM. - Option 4: 6-layer support is not feasible for handheld UEs.

In last meeting, RAN4 discussed the detailed simulation assumptions for 6Rx, as shown in the appendix. Regarding the assumptions, the most critical is the UE correlation matrix, which was also the subject of extensive discussion at the time. Ultimately, RAN4 presented two candidate options for evaluation. It is obvious that there are still significant difference between these two correlation matrices. We understand that the antenna correlation depends on the implementation, being related to the design and the placement of the antennas, as well as the type of the handle UE. However, the gap between these varying implementations might lead to divergent in the evaluation results, which is also related to BS correlation matrix. There is no doubt that the conclusion of a feasibility study for 6 MIMO layers would also vary.

Observation 1. There are significant differences between the antenna correlation matrices.

Regarding antenna correlation matrix, from legacy testing perspective, RAN4 only defined low, medium A and B, high MIMO correlation matrices for ULA and X-pol from LTE. From our understanding, the correlation matrix currently employed by RAN4 is based on the Kronecker channel model, where the correlation between the transmitter and receiver can be decoupled. Consequently, the channel spatial matrix can be decomposed into the Kronecker form of the transmitter side and receiver side correlation matrices, we merely need to calculate the corresponding correlation matrices according to parameters α and β, which defined in TS38.101-4 section B.2.

And now, we cam directly derive the correlation matrix based on the results of actual measurements. We understand that is due to the specific implementation method. However, different types have varying implementation approaches, which also means that the antenna correlation matrices are not same. From RAN4 testability perspective, this will significantly increase the complexity and burden of testing. To avoid the potential testing issues, RAN4 might need to seek a compromise for addressing the antenna correlation issues. On the other side, if requirements ultimately defined based on practical correlations, will future requirements for multiple antennas also need to be specified according to implementation details rather than following the conventional methods of RAN4.

Observation 2. Different UE types have different implementation methods, which also have different antenna correlation.

Proposal 1. From testing perspective, RAN4 might need to find a comprise for addressing the antenna correlation issues.

We also evaluated the performance of the 4-layer ans 6 MIMO layer with different antenna correlation, which as shown in following Table 2-1 to Table 2-4. Based on the simulation results, the overall throughput of the 6-layer is higher than that of the 4-layer, even though the number of DMRS symbols for the 6-layer is twice that of the 4-layer. Undoubtedly, the actual SNR point associated with the 6-layer achieves 70% of its maximum throughput is high than that of 4-layer. Regarding the SNR point, we think that the current SNR value are feasibility on low modulation order.

Proposal 2. From our simulation results, 6 MIMO Layers could be considered as an optional feature.

More importantly, there is no significant difference in the outcomes for different UE antenna correlation matrices. Based on our analysis, this is due to the assumption that the BS side is low correlation. As we all know, the most crucial step in deriving the correlation matrix is RgNB RUE, given that RgNB is the Identity matrix, the resulting Rspat matrix ends up with only a small portion of its elements being non-zero. After multiplying this with the channel matrix, the impact of correlation is minimal.

Regarding the other simulation assumptions, as mentioned before, we assumed low correlation matrix in BS side and Tx EVM is 6%, which assumed the same assumption with R-18 8Rx demodulation requirements.

| BS correlation matrix | ULA Low |
| --- | --- |
| EVM | 16QAM/64QAM: 6% |

Observation 3. The overall throughput of the 6-layer is higher than that of the 4-layer.

Observation 4. The SNR point of 70% of the maximum throughput of 6-layer is higher than 4-layer.

Observation 5. There is no significant difference in the outcomes for different UE antenna correlation matrices.

Table 2-1: SNR corresponding to 70% maximum throughput for correlation matrix with option 1

| Duplex CBW/SCS (RB allocation) | Antenna configuration | Rank, MCS | Channel model | SNR@70% maxTP |
| --- | --- | --- | --- | --- |
| 40M/30kHz | 8T6R | Rank 4, MCS 13 | TDLA 30-10 | 8.9 |
|  |  | Rank 6, MCS 13 | TDLA 30-10 | 14.2 |
|  |  | Rank 4, MCS 17 | TDLA 30-10 | 12.6 |
|  |  | Rank 6, MCS 17 | TDLA 30-10 | 19.5 |
|  |  | Rank 4, MCS 13 | TDLC 300-100 | 10.7 |
|  |  | Rank 6, MCS 13 | TDLC 300-100 | 16.6 |
|  |  | Rank 4, MCS 17 | TDLC 300-100 | 14.8 |
|  |  | Rank 6, MCS 17 | TDLC 300-100 | 23.3 |

Table 2-2: Throughput curves for correlation matrix with option 1

| (1) TDD 40M/30k 8T6R MCS 13 TDLA30-10 | (2) TDD 40M/30k 8T6R MCS 13 TDLA30-10 |
| --- | --- |
| (3) TDD 40M/30k 8T6R MCS 13 TDLC300-100 | (4) TDD 40M/30k 8T6R MCS 17 TDLC300-100 |

Table 2-3: SNR corresponding to 70% maximum throughput for correlation matrix with option 2

| Duplex CBW/SCS (RB allocation) | Antenna configuration | Rank, MCS | Channel model | SNR@70% maxTP |
| --- | --- | --- | --- | --- |
| 40M/30kHz | 8T6R | Rank 4, MCS 13 | TDLA 30-10 | 9.0 |
|  |  | Rank 6, MCS 13 | TDLA 30-10 | 14.3 |
|  |  | Rank 4, MCS 17 | TDLA 30-10 | 12.6 |
|  |  | Rank 6, MCS 17 | TDLA 30-10 | 19.5 |
|  |  | Rank 4, MCS 13 | TDLC 300-100 | 10.8 |
|  |  | Rank 6, MCS 13 | TDLC 300-100 | 16.6 |
|  |  | Rank 4, MCS 17 | TDLC 300-100 | 15.0 |
|  |  | Rank 6, MCS 17 | TDLC 300-100 | 24.4 |

Table 2-4: Throughput curves for correlation matrix with option 2

| (1) TDD 40M/30k 8T6R MCS 13 TDLA30-10 | (2) TDD 40M/30k 8T6R MCS 13 TDLA30-10 |
| --- | --- |
| (3) TDD 40M/30k 8T6R MCS 13 TDLC300-100 | (4) TDD 40M/30k 8T6R MCS 17 TDLC300-100 |

# 3 Conclusion

In this contribution, we give some discussions on the MIMO layers related to 6Rx, The conclusions are:

Observation 1. There are significant differences between the antenna correlation matrices.

Observation 2. Different UE types have different implementation methods, which also have different antenna correlation.

Observation 3. The overall throughput of the 6-layer is higher than that of the 4-layer.

Observation 4. The SNR point of 70% of the maximum throughput of 6-layer is higher than 4-layer.

Observation 5. There is no significant difference in the outcomes for different UE antenna correlation matrices.

Proposal 1. From testing perspective, RAN4 might need to find a comprise for addressing the antenna correlation issues.

Proposal 2. From our simulation results, 6 MIMO Layers could be considered as an optional feature.

# 4 References

- RP-240828, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, RAN4 chair (Huawei).

- R4-2414285, WF on 6Rx UE requirements, AT&T.

# 5 Annex

- Table: Feasibility Study Link-Level Simulation Assumptions

| Parameter |  | Value |
| --- | --- | --- |
| Duplex mode |  | TDD |
| Bandwidth |  | 40MHz or 100MHz |
| SCS |  | 30 |
| Antenna configuration |  | Option 1: 16Tx, 6Rx Option 2: 8Tx, 6Rx |
| Propagation channel |  | Option 1: TDLA30-5 Option 2: TDLA30-10 Option 3: TDLC300-100 |
| Rank |  | Fixed rank [4,5,6] |
| MCS |  | Option 1: Adaptive MCS (target BLER 10%) Option 2: MCS13, MCS17 from Table 1 Other MCS options are not precluded |
| PDSCH configuration | Mapping type | Type A |
|  | Starting symbol | 2 |
|  | Length | 12 |
|  | PRB bundling size | 2 |
|  | VRB-to-PRB mapping type | Non-interleaved |
|  | Precoding | Rel-15 Type I |
| PDSCH DMRS configuration | DMRS Type | Type 1 |
|  | DMRS Configurations | Option 1: [1+1(4L), 2+0(6L)] Option 2: [2+2(4L), 2+2(6L)] Option 3: [1+1 (4L), 2+2 (6L)] |
|  | Precoding | Random (further check) |
| UE Correlation matrix |  | Option 1: Option 2: 1.000.100.101.000.300.10 0.300.150.100.141.000.12 0.130.120.300.200.200.120.150.140.130.300.120.20 0.121.000.200.110.120.30 0.110.301.000.100.101.00 Option 3: Other options are not precluded. |