# 3GPP TSG-RAN WG4 Meeting # 112bis                                 R4-2416346

# Hefei, China, 14 – 18 October, 2024

# Title: 	Simulation results on UE demodulation performance requirements for 8Rx with MMSE-IRC

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	6.16.2

# Document for:	Discussion

# 1 Introduction

In RAN #104 meeting, the new WID of UE performance requirements for 8Rx CPE/FWA/vehicle/industrial devices, which was approved in [1], wherein the objectives related to 8Rx for CPE/FWA /vehicles are listed as below:

- Define the following UE performance requirements for 8Rx CPE/FWA/vehicle/industrial devices: - PDSCH demodulation and CQI reporting requirements with inter-cell interference with MMSE-IRC receiver - Reuse the Rel-17 interference model for 2/4Rx UE as a baseline - PDSCH demodulation requirements with intra-cell inter-user interference with MMSE-IRC receiver - Reuse the existing interference model for 2/4Rx UE as a baseline

In this contribution, we give some initial simulation results on the UE performance requirements for 8Rx CPE/FWA/vehicle/industrial device, which include inter-cell interference cancellation based on the lasting meeting conclusion [2].

# 2 Discussion

In last meeting, RAN4 discussed whether introduce two set requirements for baseline and simplified receivers. Considering that there may be differences in performance of different receivers. Consequently, we conducted numerous simulations to substantiate whether there exists a performance disparity between the baseline and simplified receivers.

The detail simulation assumptions as shown in following Table 2-1.

We evaluated HomNet and HetNet scenarios with different MCS, propagation conditions and different Rank, eta.

The SNR corresponding to 70% maximum throughput is shown in Table 2-2 and Table 2-5.

The throughput curves are shown in Table 2-3 and Table 2-4 for HomNet scenarios, Table 2-6 and Table 2-7 for HetNet scenarios, .

Table 2-1: Test parameters

| Parameter |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 3 |
|  |  |  | Enabled | Enabled | Enabled Disabled for HetNet |
| Duplex mode |  |  | FDD |  |  |
| Active DL BWP index |  |  | 1 |  |  |
| Physical cell ID |  |  | 0 | 1 | 2 |
| Transmission rank |  |  | 1 | Random rank with 70% and 30% probability for rank 1 and rank 2 | Random rank with 70% and 30% probability for rank 1 and rank 2 for HomNet N/A for HetNet |
| Time offset to Cell1 |  | us | N/A | 3 | -1 |
| Frequency offset to Cell 1 |  | Hz | N/A | 300 | -100 |
| INR |  | dB | N/A | 7.77 for HomNet 7.58 for HetNet | 2.29 for HomNet |
| PDSCH configuration | Mapping type |  | Type A |  |  |
|  | k0 |  | 0 |  |  |
|  | Starting symbol (S) |  | 2 |  |  |
|  | Length (L) |  | 12 |  |  |
|  | PDSCH aggregation factor |  | 1 |  |  |
|  | PRB bundling type |  | Static |  |  |
|  | PRB bundling size |  | 2 |  |  |
|  | Resource allocation type |  | Type 0 |  |  |
|  | RBG size |  | Config2 |  |  |
|  | VRB-to-PRB mapping type |  | Non-interleaved |  |  |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |  |  |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |  |  |
|  | Number of additional DMRS |  | 1 |  |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |  |  |
| Number of HARQ Processes |  |  | 4 |  |  |

Table 2-2: SNR corresponding to 70% maximum throughput for HomNet

| Duplex CBW/SCS (RB allocation) | Antenna configuration | Rank, MCS | Channel moel | Baseline (SNR@70% maxTP) | Simplified (SNR@70% maxTP) |
| --- | --- | --- | --- | --- | --- |
| 10M/15kHz | 2T8R | Rank 2, MCS 13 | TDLA 30-10 Low | 11.2 | 12.6 |
|  |  |  | TDLC 300-100 Low | 15.0 | 16.2 |
|  |  | Rank 2, MCS 17 | TDLA 30-10 Low | 13.6 | 15.2 |
|  |  |  | TDLC 300-100 Low | 18.1 | 19.2 |
|  | 4T8R | Rank 4, MCS 13 | TDLA 30-10 Low | 17.3 | 21.1 |
|  |  |  | TDLC 300-100 Low | 19.1 | 23.0 |
|  |  | Rank 4, MCS 17 | TDLA 30-10 Low | 21.0 | 26.2 |
|  |  |  | TDLC 300-100 Low | 22.7 | 27.8 |

Table 2-3: Throughput curves for HomNet under Rank 2

| - FDD Rank 2, MCS13, TDLA30-10 Low, 2T8R | - FDD Rank 2, MCS13, TDLC300-100 Low, 2T8R |
| --- | --- |
| - FDD Rank 2, MCS17, TDLA30-10 Low, 2T8R | - FDD Rank 2, MCS17, TDLC300-100 Low, 2T8R |

Table 2- 4: Throughput curves for HomNet under Rank 4

| - FDD Rank 4, MCS13, TDLA30-10 Low, 4T8R | - FDD Rank 4, MCS13, TDLC300-100 Low, 4T8R |
| --- | --- |
| - FDD Rank 4, MCS17, TDLA30-10 Low, 4T8R | - FDD Rank 4, MCS17, TDLC300-100 Low, 4T8R |

Table 2-5: SNR corresponding to 70% maximum throughput for HetNet

| Duplex CBW/SCS (RB allocation) | Antenna configuration | Rank, MCS | Channel moel | Baseline (SNR@70% maxTP) | Simplified (SNR@70% maxTP) |
| --- | --- | --- | --- | --- | --- |
| 10M/15kHz | 2T8R | Rank 2, MCS 13 | TDLA 30-10 Low | 5.3 | 6.8 |
|  |  |  | TDLC 300-100 Low | 13.6 | 14.9 |
|  |  | Rank 2, MCS 17 | TDLA 30-10 Low | 7.5 | 10.1 |
|  |  |  | TDLC 300-100 Low | 16.8 | 18.1 |
|  | 4T8R | Rank 4, MCS 13 | TDLA 30-10 Low | 15.5 | 19.9 |
|  |  |  | TDLC 300-100 Low | 19.6 | 22.2 |
|  |  | Rank 4, MCS 17 | TDLA 30-10 Low | 19.3 | 25.2 |
|  |  |  | TDLC 300-100 Low | 21.2 | 26.8 |

Table 2-6: Throughput curves for HetNet under Rank 2

| (1) FDD Rank 2, MCS13, TDLA30-10 Low, 2T8R | - (2) FDD Rank 2, MCS13, TDLC300-100 Low, 2T8R |
| --- | --- |
| - FDD Rank 2, MCS17, TDLA30-10 Low, 2T8R | - FDD Rank 2, MCS17, TDLC300-100 Low, 2T8R |

Table 2-7: Throughput curves for HetNet under Rank 4

| (1) FDD Rank 4, MCS13, TDLA30-10 Low, 4T8R | - (2) FDD Rank 4, MCS13, TDLC300-100 Low, 4T8R |
| --- | --- |
| (3) FDD Rank 4, MCS17, TDLA30-10 Low, 4T8R | - (4) FDD Rank 4, MCS17, TDLC300-100 Low, 4T8R |

Observation 1. Simulation results show a significant performance difference between the baseline and simplified receivers, particularly in Rank 2 and Rank 4 scenarios.

Observation 2. Simulation results show a performance difference between the baseline and simplified receivers, particularly in Rank 2 about more than 1dB.

Observation 3. Simulation results show a significant performance difference between the baseline and simplified receivers, particularly in Rank 4 about more than 3dB.

Observation 4. Simulation results show a significant performance difference between the baseline and simplified receivers, particularly in HomNet and HetNet scenarios.

# 3 Conclusion

In this contribution, we give some discussions on UE demodulation performance requirements for 8Rx with MMSE-IRC, The conclusions are:

Observation 1. Simulation results show a significant performance difference between the baseline and simplified receivers, particularly in Rank 2 and Rank 4 scenarios.

Observation 2. Simulation results show a performance difference between the baseline and simplified receivers, particularly in Rank 2 about more than 1dB.

Observation 3. Simulation results show a significant performance difference between the baseline and simplified receivers, particularly in Rank 4 about more than 3dB.

Observation 4. Simulation results show a significant performance difference between the baseline and simplified receivers, particularly in HomNet and HetNet scenarios.

# 4 References

- RP-240831, New WID: WI resulting from discussion on Demodulation performance requirements enhancements.

- R4-2413570, WF on NR demodulation performance: Phase 5. China Telcom.