# 3GPP TSG-RAN WG4 Meeting #112bis                                 R4-2416348

# Hefei, Anhui, China, 14th – 18th October, 2024

# Title: 	Simulation results for BS demodulation performance requirements MMSE-IRC

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	6.16.3

# Document for:	Approval

# 1 Introduction

In RAN #104 meeting, the revised WID of NR demodulation performance requirements for NR BS with MMSE-IRC receiver was approved in [1], wherein the objectives related to NR BS MMSE-IRC receiver are listed as below:

- Define FR1 PUSCH performance requirements with inter-cell interference in homogeneous and heterogeneous deployment based on the following assumptions: - Receiver type: MMSE-IRC receiver - Scenarios: - FDD synchronous deployments - TDD synchronous deployments with aligned UL:DL configurations among cells - Slot-based transmission and aligned SCS among cells - Reuse LTE based interference profile in TR 36.884 as a starting point. Other interference profiles are not precluded.

In this contribution, we give some initial simulation results for NR BS MMSE-IRC receiver.

- Discussion

In RAN4 #112 meeting, some simulation assumptions are reached with some parameters pended.

Base on the conclusion in RAN4 # 112 meeting[2], we provide our initial simulation results with the simulation assumptions listed in Table 2-1.

Table 2-1 Simulation assumption for NR BS MMSE-IRC vs MMSE

| Parameter |  | Tested signal | Interferer 1 (Note 1) | Interferer 2 (Note 1) |
| --- | --- | --- | --- | --- |
| Maximum number of HARQ transmissions |  | 4 | N/A | N/A |
| RV sequence |  | 0, 2, 3, 1 | N/A | N/A |
| DIP (Note 2) | Set 1 | N/A | -1.11 | -10.91 |
|  | Set 2 | N/A | -0.43 | -13.78 |
| Interference model |  | N/A | 16QAM randomly modulated data | 16QAM randomly modulated data |
| Cyclic Prefix |  | Normal |  |  |
| Duplex mode |  | FDD |  |  |
| Network type |  | Synchronous scenario |  |  |
| Antenna configuration |  | 1 Tx 2/4/8 Rx |  |  |
| Number of interferers |  | 1 interferer for 2 Rx 2 interferers case for 4/8 Rx |  |  |
| SCS and BW |  | 15kHz SCS, 5MHz 30kHz SCS, 10MHz |  |  |
| MCS |  | MCS 13 |  |  |
| Waveform |  | CP-OFDM |  |  |
| Receiver type |  | MMSE MMSE-IRC |  |  |
| PUSCH mapping type |  | Type A and Type B |  |  |
| Test metric |  | 70% of maximum throughput |  |  |
| Propagation condition |  | TDLA30-10: serving cell, neighbour cell TDLB100-400:serving cell, neighbour cell |  |  |
| DMRS | DM-RS configuration type | 1 |  |  |
|  | DM-RS duration | single-symbol DM-RS |  |  |
|  | Additional DM-RS position | pos1 |  |  |
|  | Number of DM-RS CDM group(s) without data | 2 |  |  |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |  |  |
|  | DM-RS port(s) | 0 |  |  |
|  | DM-RS sequence generation | NID0=0, group hopping and sequence hopping are disabled |  |  |

Table 2-2 summarizes the demodulation performance with 70% max throughput operating SNR.

| index | Scenario | channel | SCS, BW | Mapping type | Rx | SNR(dB) MMSE | SNR(dB) MMSE-IRC | Gain(dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | HomNet | TDLA30-10 | 15kHz,5MHz | Type A | 2 | 14.1 | 10.5 | 3.6 |
| 2 | HomNet | TDLA30-10 | 15kHz, 5MHz | Type A | 4 | 12.3 | 7.7 | 4.6 |
| 3 | HomNet | TDLA30-10 | 15kHz, 5MHz | Type A | 8 | 10.8 | 5.2 | 5.6 |
| 4 | HomNet | TDLA30-10 | 15kHz, 5MHz | Type B | 2 | 14.6 | 10.7 | 3.9 |
| 5 | HomNet | TDLA30-10 | 15kHz, 5MHz | Type B | 4 | 12.3 | 7.7 | 4.6 |
| 6 | HomNet | TDLA30-10 | 15kHz, 5MHz | Type B | 8 | 11.1 | 5.4 | 5.7 |
| 7 | HomNet | TDLA30-10 | 30kHz,10MHz | Type A | 2 | 15.0 | 11.2 | 3.8 |
| 8 | HomNet | TDLA30-10 | 30kHz,10MHz | Type A | 4 | 12.4 | 7.6 | 4.8 |
| 9 | HomNet | TDLA30-10 | 30kHz,10MHz | Type A | 8 | 11.1 | 5.4 | 5.7 |
| 10 | HomNet | TDLA30-10 | 30kHz,10MHz | Type B | 2 | 15.1 | 11.3 | 3.8 |
| 11 | HomNet | TDLA30-10 | 30kHz,10MHz | Type B | 4 | 12.5 | 7.7 | 4.8 |
| 12 | HomNet | TDLA30-10 | 30kHz,10MHz | Type B | 8 | 11.1 | 5.4 | 5.7 |
| 13 | HomNet | TDLB100-400 | 15kHz, 5MHz | Type A | 2 | 13.9 | 11.2 | 2.7 |
| 14 | HomNet | TDLB100-400 | 15kHz, 5MHz | Type A | 4 | 11.6 | 8.3 | 3.3 |
| 15 | HomNet | TDLB100-400 | 15kHz, 5MHz | Type A | 8 | 10.5 | 6.4 | 4.1 |
| 16 | HomNet | TDLB100-400 | 15kHz, 5MHz | Type B | 2 | 14.2 | 11.5 | 2.7 |
| 17 | HomNet | TDLB100-400 | 15kHz, 5MHz | Type B | 4 | 12.0 | 8.5 | 3.5 |
| 18 | HomNet | TDLB100-400 | 15kHz, 5MHz | Type B | 8 | 10.8 | 6.7 | 4.1 |
| 19 | HomNet | TDLB100-400 | 30kHz,10MHz | Type A | 2 | 14.4 | 11.6 | 2.8 |
| 20 | HomNet | TDLB100-400 | 30kHz,10MHz | Type A | 4 | 12.0 | 8.5 | 3.5 |
| 21 | HomNet | TDLB100-400 | 30kHz,10MHz | Type A | 8 | 10.8 | 6.6 | 4.2 |
| 22 | HomNet | TDLB100-400 | 30kHz,10MHz | Type B | 2 | 14.6 | 11.9 | 2.7 |
| 23 | HomNet | TDLB100-400 | 30kHz,10MHz | Type B | 4 | 12.1 | 8.7 | 3.4 |
| 24 | HomNet | TDLB100-400 | 30kHz,10MHz | Type B | 8 | 10.9 | 6.7 | 4.2 |
| 25 | HetNet | TDLA30-10 | 15kHz, 5MHz | Type A | 2 | 18.7 | 14.3 | 4.4 |
| 26 | HetNet | TDLA30-10 | 15kHz, 5MHz | Type A | 4 | 16.3 | 10.9 | 5.4 |
| 27 | HetNet | TDLA30-10 | 15kHz, 5MHz | Type A | 8 | 15.3 | 9.1 | 6.2 |
| 28 | HetNet | TDLA30-10 | 15kHz, 5MHz | Type B | 2 | 14.6 | 11.9 | 2.7 |
| 29 | HetNet | TDLA30-10 | 15kHz, 5MHz | Type B | 4 | 12.1 | 8.7 | 3.4 |
| 30 | HetNet | TDLA30-10 | 15kHz, 5MHz | Type B | 8 |  |  |  |
| 31 | HetNet | TDLA30-10 | 30kHz,10MHz | Type A | 2 | 19.3 | 14.8 | 4.5 |
| 32 | HetNet | TDLA30-10 | 30kHz,10MHz | Type A | 4 | 16.7 | 11.1 | 5.6 |
| 33 | HetNet | TDLA30-10 | 30kHz,10MHz | Type A | 8 | 15.6 | 9.3 | 6.3 |
| 34 | HetNet | TDLA30-10 | 30kHz,10MHz | Type B | 2 |  |  |  |
| 35 | HetNet | TDLA30-10 | 30kHz,10MHz | Type B | 4 |  |  |  |
| 36 | HetNet | TDLA30-10 | 30kHz,10MHz | Type B | 8 |  |  |  |
| 37 | HetNet | TDLB100-400 | 15kHz, 5MHz | Type A | 2 | 18.7 | 14.3 | 4.4 |
| 38 | HetNet | TDLB100-400 | 15kHz, 5MHz | Type A | 4 | 16.3 | 10.9 | 5.4 |
| 39 | HetNet | TDLB100-400 | 15kHz, 5MHz | Type A | 8 | 15.3 | 9.1 | 6.2 |
| 40 | HetNet | TDLB100-400 | 15kHz, 5MHz | Type B | 2 | 19.1 | 14.7 | 4.4 |
| 41 | HetNet | TDLB100-400 | 15kHz, 5MHz | Type B | 4 | 16.6 | 11.2 | 5.4 |
| 42 | HetNet | TDLB100-400 | 15kHz, 5MHz | Type B | 8 | 15.7 | 9.4 | 6.3 |
| 43 | HetNet | TDLB100-400 | 30kHz,10MHz | Type A | 2 | 19.3 | 14.8 | 4.5 |
| 44 | HetNet | TDLB100-400 | 30kHz,10MHz | Type A | 4 | 16.7 | 11.1 | 5.6 |
| 45 | HetNet | TDLB100-400 | 30kHz,10MHz | Type A | 8 | 15.6 | 9.3 | 6.3 |
| 46 | HetNet | TDLB100-400 | 30kHz,10MHz | Type B | 2 | 19.6 | 15.1 | 4.5 |
| 47 | HetNet | TDLB100-400 | 30kHz,10MHz | Type B | 4 | 16.8 | 11.4 | 5.4 |
| 48 | HetNet | TDLB100-400 | 30kHz,10MHz | Type B | 8 | 15.8 | 9.5 | 6.3 |

# 3	Reference

- RP-241297, Revised WID on NR demodulation performance: Phase 5, .

- R4-2413570, WF on NR demodulation performance: Phase 5, China Telcom

# Annex

- Figure A-1 HomNet typeA MCS 13 TDLA30-10

- Operating SNR：

- 14.1755

- 10.4669

- 14.9621

- 11.1756

- Figure A-2 HomNet typeB MCS 13 TDLA30-10 1T2R

- Operating SNR：

- 14.5552

- 10.6886

- 15.0786

- 11.3012

- Figure A-3 HomNet typeA MCS 13 TDLA30-10 1T4R

- Operating SNR：

- 12.2742

- 7.6556

- 12.3971

- 7.6495

- Figure A-4 HomNet typeB MCS 13 TDLA30-10 1T4R

- Operating SNR：

- 12.2732

- 7.6652

- 12.4897

- 7.6992

- Figure A-5 HomNet typeA MCS 13 TDLA30-10 1T8R

- Operating SNR：

- 10.7637

- 5.1901

- 11.0627

- 5.3388

- Figure A-6 HomNet typeB MCS 13 TDLA30-10 1T8R

- Operating SNR：

- 11.0775

- 5.3815

- 11.1352

- 5.4010

- HetNet的type A和type B的数据完全一样，需要再次确认。

- 重新跑一组5MHz MMSE+MMSEIRC，typeA/B数据查看

- Figure A-7 HetNet typeA MCS 13 TDLA30-10 1T2R

- Operating SNR：

- 18.9941

- 11.8421

- 19.8869

- 12.6703

- Figure A-8 HetNet typeB MCS 13 TDLA30-10 1T2R

- Operating SNR：

- 18.9941

- 11.8421

- 19.8869

- 12.6703

- Figure A-9 HetNet typeA MCS 13 TDLA30-10 1T4R

- Operating SNR：

- 16.6877

- 8.3349

- 17.0976

- 8.5255

- Figure A-10 HetNet typeB MCS 13 TDLA30-10 1T4R

- Operating SNR：

- 16.6877

- 8.3349

- 17.0976

- 8.5255

- Figure A-11 HetNet typeA MCS 13 TDLA30-10 1T8R

- Operating SNR：

- 15.6003

- 6.2628

- 15.9168

- 6.4136

- Figure A-12 HetNet typeB MCS 13 TDLA30-10 1T8R

- Operating SNR：

- 15.6003

- 6.2628

- 15.9168

- 6.4136

- Figure A-13 HomNet typeA MCS 13 TDLA30-10 1T2R

- Operating SNR：

- 13.8780

- 11.2108

- 14.4225

- 11.6280

- Figure A-14 HomNet typeA MCS 13 TDLA30-10 1T4R

- Operating SNR：

- 11.6364

- 8.2720

- 11.9714

- 8.4978

- Figure A-14 HomNet typeA MCS 13 TDLA30-10 1T8R

- Operating SNR：

- 10.5165

- 6.4355

- 10.7993

- 6.5938

- Figure A-15 HomNet typeB MCS 13 TDLB100-400 1T2R

- Operating SNR：

- 14.2606

- 11.5409

- 14.6333

- 11.8743

- Figure A-16 HomNet typeB MCS 13 TDLB100-400 1T4R

- Operating SNR：

- 11.9383

- 8.5439

- 12.1121

- 8.6752

- Figure A-17 HomNet typeB MCS 13 TDLB100-400 1T8R

- Operating SNR：

- 10.8020

- 6.6558

- 10.9196

- 6.7031

- Figure A-18 HetNet typeB MCS 13 TDLB100-400 1T2R

- Operating SNR：

- 19.0810

- 14.6914

- 19.5655

- 15.1446

- Figure A-19 HetNet typeB MCS 13 TDLB100-400 1T4R

- Operating SNR：

- 16.6452

- 11.1865

- 16.8444

- 11.3791

- Figure A-20 HetNet typeB MCS 13 TDLB100-400 1T4R

- Operating SNR：

- 15.6558

- 9.4037

- 15.7636

- 9.4826