# 3GPP TSG-RAN WG4 Meeting # 112bis                                R4-2416349

# Hefei, China, 14 – 18 October, 2024

# Title: 	Discussion on testability and interoperability issues for AI-CSI

# Source: 	ZTE Corporation, Sanechips

# Agenda item:

# Document for:	Approval

# 1 Introduction

In RAN #102 meeting, the WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [1]. The objectives for RAN4 are shown in following table.

- Testability and interoperability [RAN4]: - Finalize the testing framework and procedure for one-sided models and further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift

During the last meeting, some conclusions has been reached for CSI compression and CSI prediction[2]. In the following section, we will provide the detailed discussions for CSI compression and prediction.

# 2 Discussion

Option 3 for 2-side model

Simulation results and alignment

- perform a round of alignment until RAN4#112-Bis - refined simulation parameters can be found in R4-2414447

At the last meeting, all companies provided the simulation results for SGCS and legacy eType II. Unfortunately, not only the results of AI compression have a large span, but the legacy eType II results also have a large span, which can be found in following Fig 1.

Regarding eType II simulation results, RAN4 never discussed the detailed assumptions and implementations, such as channel estimation and codebook quantization, which are dependent on implementation and have a significant impact on performance. If RAN4 wishes to study the eType II performance, it may require substantial time and effort. However, this is inconsistent with RAN4’s current objectives at this stage. Therefore, we believe that the eType II results should only be considered as a reference. Even if results from various companies exhibit substantial difference, RAN4 still needs to continue its research on how to align the AI compression results.

Fig 1. AI compression and eType II SGCS simulation results

In RAN4 #112 meeting, considering the significant difference are observed among results from different companies, RAN4 further study the hyper parameters in the AI/ML training process, as well as parameters related to the training data, with specifics illustrated in the appendix. During these two meetings, RAN4 made assumptions about the AI model (e.g. CNN structure) and various parameters involved in the training process, virtually encompassing all parameters associated with AI/ML. Should the AI compression results still exhibit significant disparities in the upcoming 112bis meeting, RAN4 may need to consider that there discrepancies could be attributed to difference in the generation or pre-processing of the training data.

Our SGCS simulation results as shown in the following Table 1, with the size of the test data being 20k samples. The quantization aware training result is 0.7160, and we used fixed uniform quantization, e.g. 2 bit scalar quantization. And the CDF of the SGCS across sub-bands as shown in Fig 2. From the CDF curves, we can observe that the probability of SGCS being equal to 0.7 is approximately around 50%.

Table 1. Simulation results for Rank1

| Model structure |  | CNN ( Quantization aware training: fixed uniform quantization, e.g. 2bit scalar quantization ) |  | CNN (Quantization non-aware training) | eType II |
| --- | --- | --- | --- | --- | --- |
| CSI feedback bits | Training dataset | Test dataset |  | Test dataset | / |
|  |  | UMa |  | UMa | UMa |
| 64 bits | UMa | 0.7160 |  | 0.7876 | 0.6803 |
| FLOPS and size |  |  | FLOPS: Encoder: 46.966M Decoder: 739.965M Parameters Num: 1.928M |  | / |

Furthermore, we provided the simulation results with quantization non-aware training in Table 1, which show superior performance compared to those with quantization aware training. We believe that the phenomenon is also reasonable, as quantization is a lossy compression process. And the CDF of the SGCS across sub-bands as shown in Fig 3.

Observation 1. The results of quantization non-aware training is superior to quantization aware training.

| Fig 2. CDF curves for CSI compression (Quantization aware training) | Fig 3. CDF curves for CSI compression (Quantization non-aware training) |
| --- | --- |

Based on the simulation results of the CNN model, we would like to emphasize the importance of the AI model’s FLOPS, which determining the feasibility of the hardware implementation. As shown in Table 1, the total FPLOS of the CNN model is 786.931M. Compared to other AI models like transformer and MLP, the FLOPS of the CNN is significantly higher, thereby imposing higher requirements for hardware implementation. More importantly, the performance of the CNN model is inferior to that of the transformer model. This is one critical reason why the CNN model is challenging to apply.

In summary, if RAN4 intends to define requirements for CSI compression in the future, FLOPS should be considered as one of the metrics.

Observation 2. The FLOPS of the CNN model is significantly higher than other models, e.g. transformer, MLP.

Observation 3. The performance of the CNN model is inferior to that of other models, e.g. transformer, MLP.

Next, we would like to discuss the issue of alignment when the results from among companies are not vastly different. As expected in RAN4, we aspire to the alignment of results among different companies. The first obvious issue is that the required span among different companies. It is evident that RAN4 never study the span for SGCS or other test metrics besides SNR value in RAN4 testing conformance. If RAN4 intends to achieve conclusions that can be aligned among different companies, it should first establish that the extent of the span that can be agreed among different companies. Based on our observations, when considering low overhead feedback, the variation in the number of bits has a significant impact on the outcome; conversely, with high overhead feedback, changes in bit count barely influence the outcome.

Moreover, regarding the demodulation requirements for AI CSI compression, even RAN4 agreed on using CNN model for alignment evaluation. However, this is not the final requirements. From RAN4 demodulation requirements perspective, conformance testing is based on minimum requirements, additionally requirements are based on implementation. We noticed that different companies provided different models in last meeting, even the same models still have different parameters configuration, which gives rise to different outcomes in theoretically. Unfortunately, RAN4 has not evaluated the extent of differences in results from different models or parameter configurations. From our understanding, RAN4 requires more evaluations to validate the impacts of such differences on the outcomes.

On the other side, model structures rely on companies implementation. It is well-known that AI is often perceived as a block box, making it challenging to analyze theoretically through formulas or principles. In the past, for other demodulation requirements, RAN4 has established strict guidelines to govern process from Release 15. For AI- driven work item, even more stringent regulations are imperative.

Observation 4. RAN4 needs to build a strict guidelines to govern process for Release 19 for AI compression test metric.

Observation 5. If RAN4 would like to study feasibility for CSI compression, new regulations should be considered for AI/ML-based KPI.

Proposal 1. RAN4 may need to investigate how large of a span can be considered aligned results.

Option 4 for 2- side model

At the previous meeting, RAN4 deliberated on potential candidate options for option 4, namely the means of achieving interoperability between DUT vendors and TE vendors. In TR 38.843, we defined partial knowledge in RAN4 specification for decoder to achieve interoperability. With regard to this partial knowledge of the decoder, it can bring more flexible approaches for TE vendors. To ensure interoperability, we need to define more knowledge between DUT and TE side. All candidate options as shown in following table.

- 4a: Dataset based: specify at least dataset based on which the test decoder can be implemented by TE vendors FFS whether anything else needs to be specified (e.g. model structure for decoder or a reference encoder) - 4b: Reference encoder based: encoder is documented in the specifications, used to derive a decoder to be implemented by TE vendors FFS whether training dataset (channel information) needs to be specified - 4c: do not specify either reference encoder or dataset. specify enough constraints on the decoder output ordering

For above options, we want to give some analysis from our side. Firstly, if RAN4 would like to standardize a training data, which can be derived from reference encoder/decoder according generation procedure. And reference encoder/ decoder is also full defined in RAN4 specification. DUT vendor and TE vendor could use the training data to train AI/ML model, e.g. encoder and decoder models. A TE vendor could use training data and partial knowledge to train their own encoder and decoder. A DUT vendor also could use training data and partial knowledge to design their own encoder and decoder. DUT vendor and TE vendor can guarantee the better performance on their side.

As mentioned before, RAN4’s motivation is to achieve interoperability, which means RAN4 must guarantee that any encoder paired with any TE decoder should result in the same performance. In other words, TE decoder should be paired with any DUT decoder during the test.

However, the test decoder with partial knowledge, considering more implementation aspects during its design phase. Even all TE vendors use same model structure, it is a challenging to expect the same performance during test. Also it is difficult to ensure that DUT vendor and TE vendor have same model parameters during training their own decoder and test decoder.

On the other hand, the construction of training data is intimately related to the implementation of the physical layer, such as channel estimation, and system simulation design. Even RAN4 has agreed some system level assumptions to generate training data. Ensuring consistency among the training data provided by various companies remains a challenge task unless RAN4 demonstrates that distinct training data can also lead to convergence in the final results. Actually, we expect to reach a conclusion on this issue in the next meeting, as RAN4 unanimously agreed to use CNN model for result calibration. If the outcomes from different companies cannot be aligned, this would imply training data discrepancies in partial knowledge are irreparable.

Figure 4. Training data based for option 4 test decoder

Base on above analysis, we acknowledge that the validity of the training data still requires validation in the partial knowledge for option 4. From the simulation results of option 3, factors affecting performance not only include dataset but also the structure of the AI model and hyper-parameters. Even through RAN4 only agreed some system simulation assumptions to generate the dataset, it is still challenging to demonstrate that the generation of datasets can overlook certain specific implementations, unless RAN4 can align results among companies based on assumptions made at the last meeting. This will demonstrate that the dataset actually has little impact on AI performance, with model structure and hyper-parameters being critical factors. As a consequence, RAN4 can relax assumptions about the dataset but should strictly define AI structures and hyper-parameters. In other words, for option 4, only specifying the dataset is insufficient; it is challenging for DUT and TE vendors to achieve interoperability and testsbility through the dataset alone. To accomplish this goal, RAN4 needs to specify the model structure or a reference encoder.

Mode structure for decoder: From our understanding, the model structure is akin to the CNN model structure in the current option 3. DUT and TE vendors develop the defined decoder and train them using the dataset; given the consistent model architecture of the decoder, the DUT vendor can ensure that their encoder is compatible with the specified decoder. However, we overlook the impact of hyper-parameters during the training process. From the results of the last meeting, simulations results based on the CNN model did not achieve alignment, partly due to issues with hyper-parameters like loss function, optimizer, learning rate, batch size, indicating that these factors also play a critical role in model performance.

Reference encoder: In the conclusion of the previous meeting, reference encoder/ decoder can be used to derive requirements, which means that the reference is a well-defined AI model that can be directly utilized for testing purpose. DUT and TE vendors can design their own decoders on the reference encoder and train them with the dataset, ensuring compatibility between the encoder and decoder. Of course, TE vendors can also develop encoders with better performance based on the reference.

In summary, if RAN4 intends to adopt a data based approach, we believe that the model structure or reference is indispensable. At the same time, for the model structure, factors such as hyper-parameters should also be considered, because this requires training by the TE vendors to derive the decoder. Therefore, for methods that require retraining, ensuring consistency of parameters throughput the training process is a prerequisite for achieving comparable performance.

And on the other hand, concerning the acquisition of datasets, which could be either provided by a specific company or aggregated datasets. In any case, RAN4 must evaluate these datasets to determine if they are suitable for defining requirements.

Next, we want to share our view from reference perspective. In previous meeting, reference encoder and decoder were agreed to use in RAN4 discussion as least for simulation alignment / requirements derivation for option 3. Reference decoder is the test decoder for option 3. From test perspective, we think reference encoder/decoder is still needed for requirements derivation, which can be assumed as baseline model structure. As we all known, option 4 is allowed to provide more flexibility to TE vendor or DT vendor. If RAN4 defines the reference encoder/decoder, the minimum performance requirements can be guaranteed. Even DUT or TE vendor can perform updates based on partial knowledge, it also ensures that the performance meets the minimum requirements. Since the reference encoder/ decoder is necessary to ensure that the test results meet or surpass the minimum requirements.

From our understanding, reference model is a pre-trained model. Assuming alignment with simulation assumptions, for the training results submitted by all companies, the TE vendor merely need s to export these files to derive the complete model, thereby skipping the training phase. Subsequently, the TE vendor can directly train the decoder based on the exported reference encoder model. During the training process, since the encoder model is already fixed, if TE vendors can train the decoder based on the parameters of the encoder, we believe the performance of the decoder should be close. However, if different TE vendors have varying implementations during the training process, such as in hyper-parameters or other aspects, the performance of the decoder may show some differences.

If RAN4 only standardizes reference encoder, it is a challenging to guarantee the interoperability and testability during test. Since the decoder is partial knowledge based on RAN4 specification for DUT vendor and TE vendor, the acquisition of the decoder relies on the training process. Foe two side model, the encoder and decoder are trained together in a paired form. Therefore, we believe that a datasets for training is indispensable.

Proposal 2. To consider reference encoder based method, and training datasets need to be specified.

# 3 Conclusion

In this contribution, we give some detailed discussions on testability and interoperability issues for AI-CSI , The conclusions are:

Observation 1. The results of quantization non-aware training is superior to quantization aware training.

Observation 2. The FLOPS of the CNN model is significantly higher than other models, e.g. transformer, MLP.

Observation 3. The performance of the CNN model is inferior to that of other models, e.g. transformer, MLP.

Observation 4. RAN4 needs to build a strict guidelines to govern process for Release 19 for AI compression test metric.

Observation 5. If RAN4 would like to study feasibility for CSI compression, new regulations should be considered for AI/ML-based KPI.

Proposal 1. RAN4 may need to investigate how large a span can be considered aligned results.

Proposal 2. To consider reference encoder based method, and training datasets need to be specified.

# 4 References

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface.

- R4-2414323, WF on AI/ML. Qualcomm Incorporated.

- 3GPP TS38.101-4.

# 5 Appendix

- System simulation assumption

Table 5-1. System simulation assumptions for training data

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) |
| Frequency Range |  | FR1 only, [2GHz, 4GHz] |
| Inter-BS distance |  | 200m |
| Channel model |  | According to TR 38.901 |
| Antenna setup and port layouts at gNB |  | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) |
| BS Tx power |  | 44dBm for 20MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | Baseline: 15kHz for 2GHz; |
| Simulation bandwidth |  | Baseline: 10 MHz for 15kHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU-MIMO |
| MIMO layers |  | Baseline: 1 Optional: 2 |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback): 5 ms (baseline) - Scheduling delay (from CSI feedback to time to apply in scheduling): 4 ms |
| Traffic load (Resource utilization) |  | Baseline: 50% |
| UE distribution |  | CSI compression: 80% indoor (3 km/h), 20% outdoor (30 km/h) |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | ideal channel estimation |

- AI model structure

- Parameters

| Parameter/Configurations | Options/Values | Company comments |
| --- | --- | --- |
| Channel/data-set related |  |  |
| UE antenna element assumptions | - Option 1: Omni-directional antenna elements (Ericsson) - Option 2: Directional antenna elements - Option 3: Other | [Apple] We provided 2 simulation results: Directional (both horizontal/vertical) Omni (both horizontal/vertical) [QC] Option 1 (both horizontal and vertical) [Intel] Option 1 [OPPO]:Option1 [CMCC] option 1 |
|  | Configuration for alignment: - Ideal omni-directional antenna elements following RAN1 assumptions |  |
| Sub-band channel averaging | - [Nokia, QC, Apple, vivo, Ericsson,OPPO] Average of covariance channel matrix per RBs in the sub-band before SVD - [ZTE]: Per sub-band - [CMCC]: Average of raw channel matrix per RBs in the sub-band before SVD | CATT: Prefer not to do the averaging before SVD. |
|  | Configuration for alignment: - Average of covariance channel matrices per PRBs in the sub-band - 1 sample per PRB is selected |  |
| Any other pre-processing | - To be listed if used - [QC, Ericsson] Normalize the phase of all entries of the eigenvector based on the phase of the 1st entry of the eigenvector - [OPPO] Normalize the phase of all entries of the eigenvector based on the phase of the last entry of the eigenvector - [Apple]: Normalize the phase of all entries of the eigenvector based on the phase of that entry that has the maximum amplitude averaged over the sub-bands | [Intel] prefer no other pre-processing |
|  | Configuration for alignment: - Apply phase normalization: - Normalize the phase of all entries of the eigenvector based on the phase of the 1st entry of the eigenvector - i.e. the phase of the first value of the eigenvector is 0. - L2 norm of the eigenvector is equal to 1. |  |
| Data collection assumptions | - [Nokia, CATT, ZTE, Ericsson, CMCC] Each channel realization is collected from independent UE drop - Multiple channel realization can be collected from the same UE - Other - [QC] Multiple channels realizations are collected from multiple UEs and multiple drops of same UE - [Apple, Intel, OPPO]: Multiple drops. Each drop collect data from multiple UEs. Additionally collect samples from channel realization from each UE spaced at de-correlated interval. |  |
|  | Configuration for alignment: - Strive to ensure that the samples are decorrelated, especially when collected from the same UEs. |  |
| Training dataset size | - [Nokia]: 630K - [QC]: 450K - [Samsung]: 25K - [R&S]: 614,440 - [MTK]: 700K - [Ericsson]: 800K - [Intel]: 200K - [CATT]: 102,600 - [ZTE]: 600k - [Apple]: 600K - [vivo]: 243000 - [OPPO]：600k - [CMCC]: 100K |  |
|  | Configuration for alignment: - Aim to use the dataset around 600K. - At least 100K sample to be used. |  |
| Dataset quantization | - Float32 |  |
| Other data-set characteristics/statistics | - Companies to propose |  |
|  | - Companies are encouraged to report some statistics about the dataset. - Statistics are FFS - Companies are encouraged to report the CDF of the SGCS across sub-bands and input samples. - If possible, also report CDF for eTypeII. |  |
| Model-related |  |  |
| Quantization of encoder output | - [Nokia]: Non-trainable quantization (bypassed during training) - [QC, CMCC] Quantization aware training - [Apple]: Quantization in forward and approximation in backward to enable learning. (avoid zero gradient of round function) - [vivo] VQ with quantization aware training - [Intel] Quantization aware training. Scalar quantization with non-trainable quantization grid. - [Ericsson] 2 bit scalar quantization, quantizer aware training | CATT: Consider to align the activation function and quantization method. |
|  | - Report results without quantization of the FC layer outputs for validation purposes only, i.e., float32 is sent in CSI report - Companies still need to report quantization results: - Companies to report whether/how quantization aware training is implemented (e.g., how quantization is treated at back propagation) - Companies can try quantization aware training with bypassing of the quantization in back propagation - Quantization options, companies to provide the details: - Fixed unform quantization with equal ranges - Trainable quantization |  |
| Normalization before quantization in the encoder | - [Nokia, Ericsson]: Normalization with tanh function after FC layer - [CATT, ZTE, vivo, CMCC]: Sigmoid function used after FC layer - [QC] Normalization not done due to the use of quantization aware training - [Apple]: Normalization with tanh function after FC layer. We don’t apply inverse of that in de-quantization - [Intel] Sigmoid after FC layer. Inverse sigmoid after de-quantization stage in decoder. |  |
|  | Configuration for alignment: - For the results with quantization, to use a sigmoid function in the encoder for normalization - Sigmoid in the decoder is optional. |  |
| Use and configuration of drop-out | - [Nokia]: Use of on single drop-out layer after FC layer in the encoder and decoder, Drop-out rate = 0.4. - [CATT, QC, ZTE, Apple, vivo, Intel, Ericsson, CMCC]: No drop-out used. |  |
|  | Configuration for alignment: - Not to use drop-out layers |  |
| Training loss function, use of regularization | - [Nokia]: - SGCS with L2 regularization with weight decay, λ = 10-2 - [QC]: SGCS + additional terms for quantization aware training - [Apple, CATT, CMCC]: 1- SGCS - [R&S, MTK, Intel]: SGCS - [Ericsson]: NMSE - [ZTE]: MSE - [vivo]: 1-SGCS plus VQ loss for encoder output (MSE between original and quantized encoder output) - [Intel]: SGCS, no L1/L2 regularization |  |
|  | Configuration for alignment: - Use (1-SGCS) as loss function as reference |  |
| Optimizer | - [Samsung, QCM, Apple, MTK, Intel, CATT, Ericsson, CMCC]: Adam - [Nokia]: RMSProp (Root Mean Square Propagation) - [ZTE, vivo]: AdamW |  |
|  | Configuration for alignment: - Use Adam |  |
| Learning rate | - [Nokia]: - [QC, R&S, CATT, Ericsson]: 10^-4 - [R&S]: Learning rate adaptation: Reduce on plateau by factor 0.5 with patience of 50 epochs. - [Apple]: 0.0025 - [Samsung, Intel, MTK, CMCC]: 0.001 - [ZTE]:1.5e-3 - [vivo]: Warm-up to initial lr 1e-3 and then linearly decay to final lr 8e-4, update after each batch |  |
|  | Configuration for alignment: - Fixed learning rate of 10^-4 |  |
| Batch size | - [Nokia]:1024 - [QC, MTK, CMCC]: 128 - [Apple]: 256 - [Samsung]: 200 - [R&S, CATT, vivo, Ericsson]: 512 - [Ericsson]: 32 - [Intel]: 10 - [ZTE]:400 |  |
|  | Configuration for alignment: - 256 |  |
| (Max) Number of epochs | - [QC]: 150 - [Apple, vivo, Intel]: 100 - [R&S, Nokia, CATT]: 200 - [MTK, ZTE]: 300 - [CMCC]: 500 |  |
|  | Configuration for alignment: - 150 epochs |  |
| Stopping criteria | - [Nokia, Apple, CATT, QC, ZTE, Intel, CMCC]: No change in metric over a given number of epochs - Validation patience: [Intel, Nokia] 10, [QC] 50 - + Ericsson with constraint on max Epochs - [vivo] Stop when reaching the configured number of epochs |  |
|  | Configuration for alignment: - No improvement over 25 epochs. |  |
| Weight initialization | - [Apple, CATT, QC, vivo, Intel, CMCC]: Default - Ericsson: Variance scaling |  |
|  | Configuration for alignment: - Default |  |
| Kernel Parameters: - Dilation - Padding Type The value of groups (VG) | - [CMCC, Samsung, Vivo, CATT]: Dilation:1, Padding type=0, VG= 1 - [QC, ZTE, Apple, Intel]: Default |  |
|  | Configuration for alignment: - Default |  |