3GPP TSG-RAN WG4 Meeting # 112-bis	R4- 24xxxx

# Hefei, China, 14th – 18th October, 2024

Source:	LG Electronics

Title:	Simultaneous operation of Ambient IoT and Uu in intermediate UE

Agenda item:	6.21.3.3

Document for:	Discussion

Introduction

RF requirements discussion started in RAN4 #112. We are also currently focusing on D1T1 and D2T2 scenarios, where the intermediate node comes from the D2T2 scenario. The intermediate node can be a relay, IAB node, UE, repeater, etc. which is capable of Ambient IoT as specified in TR38.848. So Intermediate User Equipment (UE) for Ambient IoT can operate with Uu interfaces such as NR and LTE. In this perspective, we want to share view on Ambient IoT Intermediate node

- Discussion

Intermediate User Equipment (UE) for Ambient IoT can operate with Uu interfaces such as NR and LTE. Therefore, the simultaneous operation of Ambient IoT and Uu can be considered a valid operational scenario. However, the receiver performance of the intermediate UE may be affected by the interference caused by both Ambient IoT and Uu. The concurrent operation of Ambient IoT and Uu resembles Carrier Aggregation (CA) operation, leading to similar interference scenarios, including harmonic distortion, intermodulation, and cross-band isolation. Examples 1 and 2 illustrate specific cases of interference in the intermediate UE

Example 1 Aggressor: A-IoT(CW or R2D), legacy service UL (ex.NR) Victim: legacy service DL

Example 2

Aggressor:Legacy service UL(ex. NR) Victim: A-IoT D2R

Observation: The concurrent operation of Ambient IoT and Uu resembles CA operation, leading to similar interference scenarios, including harmonic, intermodulation and cross-band isolation.

Proposal: It is necessary to study the interference caused by concurrent operation between A-IoT and NR UL/DL.

- Conclusion

In this contribution, we want to share view on Ambient IoT Intermediate node and proposal is made as following:

Observation: The concurrent operation of Ambient IoT and Uu resembles CA operation, leading to similar interference scenarios, including harmonic, intermodulation and cross-band isolation.

Proposal: It is necessary to study the interference caused by concurrent operation between A-IoT and NR UL/DL.

- 3. Reference

[1] TR38.848, “Study on Ambient IoT (Internet of Things) in RAN”