# 3GPP TSG-RAN WG4 Meeting #112bis                              R4-2416364

HeFei, China, 14th – 18rd October, 2024

Source:  ZTE Corporation, Sanechips

Title:   Discussion on the testability aspects of AI/ML positioning

Agenda Item:	6.17.5.2

Document for:	Approval

- Introduction

In RAN plenary #94 meeting, a new SID[1] for Rel-18 was approved to study the AI/ML for NR air interface. The objectives for RAN4 of this SID are listed as follows.

- Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2 - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable - Consider the need and implications for AI/ML processing capabilities definition Note 1: specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved. Note 2: The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced.

In this document, we will provide some views on the interoperability and testability for AI positioning from the general test framework perspective.

- Discussion

Based on the progresses in the previous meeting, there were several typical use cases incorporated in [2], which are listed below.

Agreement: - AI positioning - UE Direct positioning - UE Assisted positioning

For AI/ML based positioning, direct positioning and assisted positioning are considered. For direct AI/ML positioning, the output is the estimated/predicted location, on the contrary, for assisted positioning the output of the AI model is the input of the non-AI positioning method, and the estimated location will go through two steps as the figure shown:

Figure 1 AI/ML direct positioning

Figure 2 AI/ML assisted positioning

From RAN4 perspective, we need to evaluate the performance metrics which are agreed in RAN1 and in TR38.843 and the following aspects need to be studied:

Both direct AI/ML positioning and AI/ML assisted positioning are considered. For metrics for positioning requirements/tests, the candidate options include - Option 1: positioning accuracy: Ground truth vs. reported - only option available for direct positioning - Option 2: CIR/PDP, channel estimation accuracy - Option 3: ToA, RSTD and RSRP, and RSRPP - Option 4: others (e.g., intermediate KPIs, LoS/NLoS)/combinations of the above The feasibility and testability of different options should be further justified in WI.

In TR 38.843, following 5 cases of AI/ML based positioning accuracy enhancement are agreed for further study：

- Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

And in previous meeting, RAN4 has reached the related agreements for AI positioning as below:

Issue 3-2: Requirements for case 2a Agreement: RAN4 should not define any positioning accuracy requirements because positioning is LMF based Issue 3-3: Reported metrics for case 2a Agreement: - RAN4 will take existing requirements for existing metrics as baseline - FFS whether to revisit these requirements - RAN4 will study how to define requirement for any new reported metrics - FFS how to define requirements for LOS/NLOS

First of all, we can see the case 1, case 2a and case 2b as below:

| (a) Case 1 | (b) Case 2a |
| --- | --- |
| (c) Case 2b |  |

Figure 3 The illustration of Case1, Case2a and Case2b

2.1 Testability and interoperability for Case 1

Case 1 is the direct AI/ML positioning, UE-based positioning with UE-side model. UE measures the downlink PRS and the related measurement results can be the inference input. Then, the inference will be directly performed at UE side, the inference output is the AI/ML predicted UE location. After inference, the UE will report the model output to LMF. Thus, the test metric is the positioning accuracy. For test equipment, one main challenge is the accuracy of UE-based direct positioning. In several past meetings, RAN4 did not discuss the ground truth for AI/ML positioning. If the ground truth is known to test equipment, the accuracy can be quantified compared to defined ground truth, such as AI/ML positioning error. Then the model performance can be evaluated and the subsequent operation of model can be clarified. However, how to study or define the positioning ground truth and how to test case 1 are two key points.

Observation 1: If the ground truth is known to test equipment, the accuracy can be quantified compared to defined ground truth, such as AI/ML positioning error. Then the model performance can be evaluated and the subsequent operation of model can be clarified.

In previous meetings, we proposed several times for ground truth on AI/ML positioning which is PRU (Positioning reference unit). If the UE is the PRU, then the LMF/gNB has already known its location. To my understanding, the AI/ML predicted location can be compared to PRU and then the positioning accuracy error will be known. The UE location is the coordinate and the PRU will be set on anywhere, if we set PRU randomly, we do not think the accuracy can be accurately evaluated. Thus, several companies proposed the test map granularity and test map can be defined as below [3]:

Figure 2 Grid map providing a ground-truth information for inference evaluation

While UE moves from one location point to another location, the inference result with AI/ML functionality at each location is reported to the test equipment and compared to the ground-truth information for verification of positioning accuracy.

Also, we deem that if UEs have the GNSS capability, they also can be the reference UEs. Based on TR 38.171, the GNSS error are as below:

| Scenario Type | Weak satellite signal conditions | Ideal conditions | Satellites with rather different signal levels | Multi-path Scenario | Moving scenario with periodic update |
| --- | --- | --- | --- | --- | --- |
| Position Error | 100 m | 15 m | 100 m | 100 m | 50 m |

However, if we choose the reference GNSS UEs, the GNSS error shall be considered when evaluating the AI/ML positioning accuracy error. This kind of ground truth can be discussed.

Observation 2: The UE location is the coordinate and the PRU will be set on anywhere, if we set PRU randomly, we do not think the accuracy can be accurately evaluated.

Observation 3: If UEs have the GNSS capability, they also can be the reference UEs. However, if we choose the reference GNSS UEs, the GNSS error shall be considered when evaluating the AI/ML positioning accuracy error.

Proposal 1: The ground truth can be the PRU and the test map shall be considered. Reference GNSS UEs shall be considered.

2.1 Testability and interoperability for Case 2a

For Case 2a, RAN1 deems that LOS/NLOS indicator can be the input for model inference at LMF side. However, how to verify the indicator is one concern since RAN4 did not discuss the propagation conditions before. Whether the single path or multi channels shall be considered and which kind of channels shall be used shall be studied too. This intermediate KPI verification will ensure that the minimum performance requirements are met by the AI/ML model/functionality. To my understanding, a single path for LOS is enough based on TR 38.901. How many multiple channels will be used for NLOS shall be discussed based on which kind of channels will be used.

Observation 4: RAN1 deems that LOS/NLOS indicator can be the input for model inference at LMF side. However, how to verify the indicator is one concern since RAN4 did not discuss the propagation conditions before.

Proposal 2: Whether the single path or multi channels shall be considered and which kind of channels shall be used shall be studied.

Proposal 3: A single path for LOS is enough. How many multiple channels will be used for NLOS shall be discussed based on which kind of channels will be used.

- Conclusion

In this contribution, we have the following observations and proposals for the AI/ML positioning:

Observation 1: If the ground truth is known to test equipment, the accuracy can be quantified compared to defined ground truth, such as AI/ML positioning error. Then the model performance can be evaluated and the subsequent operation of model can be clarified.

Observation 2: The UE location is the coordinate and the PRU will be set on anywhere, if we set PRU randomly, we do not think the accuracy can be accurately evaluated.

Observation 3: If UEs have the GNSS capability, they also can be the reference UEs. However, if we choose the reference GNSS UEs, the GNSS error shall be considered when evaluating the AI/ML positioning accuracy error.

Proposal 1: The ground truth can be the PRU and the test map shall be considered. Reference GNSS UEs shall be considered.

Observation 4: RAN1 deems that LOS/NLOS indicator can be the input for model inference at LMF side. However, how to verify the indicator is one concern since RAN4 did not discuss the propagation conditions before.

Proposal 2: Whether the single path or multi channels shall be considered and which kind of channels shall be used shall be studied.

Proposal 3: A single path for LOS is enough. How many multiple channels will be used for NLOS shall be discussed based on which kind of channels will be used.

- Reference

- RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, 3GPP RAN Plenary

- R4-2310433, “WF on RAN4 requirements for NR AI/ML”, Qualcomm Incorporated

- R4-2412994, Nokia