# 3GPP TSG-RAN WG4 Meeting #112bis                               R4-2416367

# HeFei, China, 14th – 18rd October, 2024

Source:	ZTE Corporation, Sanechips

Title:	Discussion on core parts on LPWUS

Agenda item:	6.23.4.2

Document for:	Approval

Introduction

In RAN #103 meeting the latest WID has been agreed [1] on Low-power wake-up signal and it is the first meeting for RAN4 to discuss the core part in this meeting. The WID can be seen as below:

The objectives of the work item are the following: - To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4) - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information. - At least duty-cycled monitoring of LP-WUS is supported - For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4) - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4) - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point. - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2) - For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Check in RAN#105 for potential TU adjustment in RAN2 - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR - Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3. - Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode. - Specify the necessary RAN4 core requirement(s) to support the feature (RAN4). - Specify UE low-power wake-up receiver requirements, at least REFSENS, ACS and ASCS requirements with consideration of possible new methodology to assess the low-power wake-up receiver performance - Define guard RBs for ACS and ASCS cases - Study testability of above requirements - Consider impacts of different architecture and impairments, and set requirements that enable all types of reasonable implementation - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS. - Current NR BS requirements is baseline - Specify necessary RRM requirements

In this paper we will continue to discuss the related issues and provide our views.

Discussion

General aspects

In previous meetings, the basic scenarios were discussed with a few agreements. In this section, we mainly discuss the LP-WUS monitoring and the MR/LR measurements based on RAN1 and RAN2’s agreements.

Core requirements to be specified for LP-WUR measurement in IDLE/INACTIVE state

According to WI, it specifies that the RAN4 should define the core requirements to support the feature. In this part, we mainly analyze the UE camps on different receivers and related core requirements combining with the progress and agreements in other groups. In this part, we primarily study the different cases under entry/exit conditions and specify the related RRM requirements, especially MR relaxation requirements for serving/neighbour cell and LR measurement requirements.

| RRM measurement case index | MR serving cell measurement | MR neighbouring cell measurement | LR measurement |
| --- | --- | --- | --- |
| Case#1 | OFF | OFF | ON |
| Case#2 | ON, with measurement relaxation | OFF | ON |
| Case#3 | ON, with measurement relaxation | ON, with measurement relaxation | ON |
| Case#4 | OFF | ON, with measurement relaxation | ON |
| Case#5 | ON | ON | OFF |

Requirements on cases with/without configured High Priority Frequency

In the last meeting, RAN4 kicked off the discussion on case#3 and the basic scenarios have been confirmed. The agreements can be seen as below:

Issue 1-1-1: Cases/states to be considered for RRM relaxation and serving cell measurement offloading Agreement: - Support case #3 - For serving cell measurement, further discuss: - Option 1: Further discuss whether to combine the measurements across the two radios, i.e., MR and WUR - Option 2: From RAN4 requirement perspective, not consider combining the measurements across the two radios, i.e., MR and WUR - Option 3: It is up to UE implementation whether to combine the measurement across the two radios. - Other options are not precluded. RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #3 Relaxed case b; On with relaxation measurement; On with relaxation measurement; ON. - Note: In RAN4 understanding, the discussion of the related measurement criteria (i.e., whether to use the legacy or new criteria) is out of RAN4 responsibility. Recommendations: Suggest to discuss issues related to higher priority frequency layers under 1-1-11. Continue discuss case 2 and other issues.

In previous meetings, RAN4 has already agreed the case#1 (MR Fully offload to LR) without consideration of configured high priority frequency, that is, only LR performs the serving cell measurement and MR is OFF. In the last meeting, we also agreed that MR and LR are in ON state (MR partially offload to LR, MR+LR).

For case#1, the measurement only performed by LR, that is, the LR measurement result will decide the MR is ON or OFF.

For case#3, the remaining issue is whether to combine the measurements across the two radios or not since MR and LR both perform the serving cell measurement, only  MR performs neighbour cell measurement.

There are still case#2 and case#4 (measurements with high priority frequency) are not discussed and we will firstly analyze the remaining scenarios and then specify the RRM requirements for all cases including the remaining issue for case#3.

Firstly, we shall focus on the feasibility of Case#2 and the following figure can be seen:

Figure 1 Different cases

Based on the above figure, we find that the MR and LR is ON and both of them perform the serving cell measurement for Case#2. However, the main difference between Case#2 and Case#3 is that there is no neighbour cell measurement for Case#2. In general, Case#2 and Case#3 depends on the serving cell measurement results. If the service of the current cell is better than the threshold defined in existing spec, the UE will not do the neighbour cell measurement, however, the measurement result is below the threshold which can enter into Case#1, the UE is still in the MR+LR state (Case#2). Otherwise, the UE shall perform the neighbour cell measurement (Case#3). We deem that that is not a corner case and Case#2 is needed. However, we shall have a comprehensive understanding if we decide to study Case#2. To my understanding, whether to study Case#2 or not shall consider the following reasons:

- MR measurement relaxation and power consumption

- The accuracy of LR measurement

We know that if UE stays in MR+LR state, the scaling factor for MR measurement time period is needed. The MR measurement is more infrequent than LR measurement. And the MR measurement will cost more power. Some companies think that the accuracy of LR measurement is lower than MR measurement. However, there is no evidence can prove that and that is RAN1’s evaluation work, we shall wait for RAN1’s agreements. Also, the periodicity of LR is under discussion in RAN1 which maybe 160ms, 320ms etc, that is, LR will measure more samples than MR, the measurement results may not necessarily be inaccurate. If the accuracy of LR measurement is the key concern, we only need to consider how to keep similar accuracy between LR and MR, such as introduce the offset for LR measurement instead of studying another case. Thus, the Case#2 is not necessary to consider.

Observation 1: Whether to study Case#2 or not shall consider the following reasons:

- MR measurement relaxation and power consumption

- The accuracy of LR measurement

Proposal 1: Case#2 is not necessary to consider.

2.2.1 Cases/States without high priority frequency

We shall clarify one key point that only MR can perform the neighbour cell measurement and LR only performs serving cell measurement based on RAN2 agreements:

- RAN2 understand that the RRM measurement of the neighboring cell can only be performed by MR. Can discuss again if RAN1 inform us otherwise.

- Observation 2: MR and LR shall both perform serving cell measurement. Only MR performs neighbour cell measurement.

We shall clarify the whole procedure based on the serving cell measurement and the entry/exit condition. To my understanding, the whole procedure can be:

Fully MR state (including the relaxation) → MR+LR → Fully LR → MR+LR → Fully MR (including the relaxation)

In different states, UE will perform corresponding measurements and the related measurement requirements shall be discussed and defined.

- Proposal 2: RAN4 shall specify the whole procedure based on serving cell measurement and specify the UE behaviour when it satisfies the entry/exit condition:

- When the entry condition is satisfied:

- Option 1: Fully MR →MR+LR →Fully LR

- Option 2: Fully MR →Fully LR

- When the exit condition is satisfied:

- Option 1: Fully LR →MR+LR→ Fully MR

- Option 2: Fully LR→ Fully MR

- For Case#1 (MR fully offload to LR):

In this case, UE stays in the center of current cell and the service of serving cell is the best, that is, UE MR is OFF and only LR is ON. At this time, LR will perform the serving cell measurement based on LP-SS based RSRP and RSRQ or SSS based LP-SSS-RSRP and LP-SSS-RSRQ for OFDM-based LP-WUR which has already agreed in RAN1 as below:

Agreement For LP-SS based LP-RSRQ, LP-RSRP and LP-RSSI are measured within the same bandwidth. Agreement For LP-RSSI definition for LP-RSRQ, the following is agreed Option 1: LP-RSSI is the linear average of total received power in ON and OFF LP-SS OOK symbols. ote: Above does not constrain LP-SS sequence design for OOK Working Assumption From RAN1 perspective, for the RRM measurement metrics based on SSS for OFDM-based LP-WUR, use the same definition of SS-RSRP and SS-RSRQ for LP-SSS-RSRP and LP-SSS-RSRQ, respectively. Above is applicable for both time-domain processing or frequency-domain processing Above does not imply that RAN1 will introduce LP-SSS-RSRP and LP-SSS-RSRQ in the specifications

As for this case, UE stays in current serving cell and has good service. We find that there is no need for a neighbour cell measurement without a configured high priority frequency.

From my perspective, the LP-SS based serving cell measurement requirements shall be defined. Since the LP-SS periodicity is 320ms which has agreed in RAN4 meeting and it may also be 160ms, 640ms, 1280ms and 2560ms under RAN1’s discussion. In legacy, introducing SMTC is aimed at reducing the complexity of UE measurements. There is no need to introduce the time window to detect LP-SS like SMTC since LP-SS is only for serving cell measurement not for neighbour cell measurement. However, from RAN1 perspective, for the RRM measurement metrics based on SSS for OFDM-based LP-WUR, use the same definition of SS-RSRP and SS-RSRQ for LP-SSS-RSRP and LP-SSS-RSRQ, respectively. Thus, the legacy RRM requirements based on SSB can be the baseline when considering defining the concrete requirements based on LP-SS.

- Observation 3: When UE camps on fully LR state, MR is OFF and only LR is ON, it means that UE is in the center area of the current cell. UE LR performs serving cell measurement and there is no neighbour cell measurement without high priority frequency.

- Observation 4: In legacy, RAN1 introduced SMTC in order to reduce the complexity of UE measurements because of neighbour cell measurement.

Proposal 3: RAN4 shall define the serving cell measurement requirements based on LP-SS. However, we deem that the time window like SMTC may not be introduced.

Proposal 4: The legacy RRM requirements based on SSB can be the baseline when considering defining the concrete requirements based on LP-SS

- For Case#3 (MR+LR):

In this case, UE satisfies the entry condition to enter into LR coverage. UE stays in MR+LR state, that is, MR and LR is ON. We have already agreed this case without the detailed requirements. In this section, we mainly analyze the RRM requirements.

- Serving cell measurement (including MR RRM relaxation)

In this case, we find that the serving cell measurement is performed by LR and MR. Thus, we shall consider the MR measurement relaxation to reduce the power consumption. However, RAN2 only defines the relaxed measurement criterion for neighbour cell not for serving cell in IDLE/INACTIVE state in legacy. Thus, RAN4 shall wait for RAN2 progress about the relaxed measurement criteria for serving cell. But RAN4 can study the mobility performance to quantify the relaxation such as scaling factor for measurement time period.

Observation 5: In case#3, MR and LR are ON. The serving cell measurement is performed by LR and MR, MR measurement relaxation is aimed at reducing power consumption.

Observation 6: RAN4 only defines the relaxed measurement requirements for neighbour cell not for serving cell.

Proposal 5: RAN4 shall wait for RAN2 to define the relaxed measurement condition for serving cell measurement if it has.

Proposal 6: When UE stays in MR+LR state, RAN4 shall consider the MR serving cell measurement relaxation and also study the mobility performance to quantify the relaxation such as scaling factor for time period.

For serving cell measurement of case#3, the potential options can be seen as below:

- For serving cell measurement, further discuss: - Option 1: Further discuss whether to combine the measurements across the two radios, i.e., MR and WUR - Option 2: From RAN4 requirement perspective, not consider combining the measurements across the two radios, i.e., MR and WUR - Option 3: It is up to UE implementation whether to combine the measurement across the two radios. - Other options are not precluded.

The following illustration can be seen:

Figure 2 Measurements from MR and LR separately

As the figure shows us, LR based measurement measures 8 samples with the 320ms periodicity. MR based measurement measures 2 samples with the measurement relaxation. We see that different receivers measure independently and the measurement results do not conflict with each other. Then, we shall consider one thing which measurement results shall be trusted or used to compare with the threshold.

In previous meeting, RAN2 has already have some conclusions on entry/exit condition on LP-WUS monitoring as below:

- LP-WUS entry/exit condition RAN2#126 meeting: Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed. Baseline for exit condition definition: If the serving cell measurement result based on LR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS. RAN2#127 meeting: Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds.

Based on the above table, we find that RAN2 deems that the entry condition for LP-WUS monitoring can be the only MR measurement result or MR measurement result and LR measurement result. However, the exit condition for LP-WUS monitoring can only be the LR based measurement. That is, the entry condition for LP-WUS monitoring may have two thresholds. MR based measurement can compare to MR threshold and LR based measurement can compare to LR threshold. If that, one thing shall be considered which the relationship of the MR threshold and the LR threshold, the relationship can be “AND” or  “OR” . RAN4 shall not consider or study the combined measurement from two receivers whatever the relationship is.

To my understanding, this interpretation implies that the LR is ON or OFF is up to UE implementation. MR measurement shall be always be used and MR performs serving cell measurements without measurement relaxation. However, the accuracy of LR measurement is not confirmed, unless one offset for LR measurement is introduced which can reach the similar accuracy as MR based measurement. Thus,

- If the MR based measurement and LR based measurement can be used and each measurement result shall satisfy each threshold, the UE can perform the LP-WUS monitoring (enter into the LR coverage).

- If the MR based measurement and LR based measurement can be used and only MR based measurement result satisfies corresponding threshold, the UE can perform the LP-WUS monitoring (enter into the LR coverage).

Observation 7: RAN2 deems that the entry condition for LP-WUS monitoring can be the only MR measurement result or MR measurement result and LR measurement result. However, the exit condition for LP-WUS monitoring can only b e the LR based measurement.

Observation 8: The above interpretation implies that the LR is ON or OFF is up to UE implementation. MR measurement shall be always be used. However, the accuracy of LR measurement is not confirmed, unless one offset for LR measurement is introduced which can reach the similar accuracy as MR based measurement.

Proposal 7: RAN4 shall not consider or study the combined measurement from two receivers for LP-WUS monitoring whatever the relationship between LR based threshold and MR based threshold is.

To my understanding, the similar situation can be considered for Case#3 measurement and offloading.

For the entry condition, there are two cases which from Case#5 to Case#3 and from Case#1 or Case#2 which is not confirmed to Case#3. For the first case, the ahead analysis of LP-WUS monitoring can be considered. For the second case, only LR performs serving cell measurement, and the LR based measurement can be the only criteria to enter into Case#3, and the threshold can be the legacy neighbour cell triggering.

For the exit condition, Case#3 shall enter into Case#5 or Case#1. For exit condition, the MR and LR perform the serving cell measurement together. The ahead analysis of LP-WUS monitoring shall be the baseline. If network configures two different thresholds which are for MR based measurement and LR based measurement, the two measurements can compare to the different thresholds. However, the difference between enter condition for LP-WUS monitoring and the exit condition for Case#3 is MR measurement relaxation. In Case#5, MR does not perform serving cell measurement with relaxation, however, in Case#3 the MR shall perform the serving cell measurement with relaxation. Thus, we think the LR measurement can be trusted since LR can measure more samples without relaxation compared to MR based measurement. If the companies also have concern on accuracy of LR measurement, then one offset can be considered. Thus,

- If the MR based measurement and LR based measurement can be used and each measurement result shall satisfy each threshold, the UE can exit Case#3.

- If the MR based measurement and LR based measurement can be used and one of measurement results satisfy corresponding threshold, the UE can exit Case#3.

Overall, the combined measurements can not be agreed and the option 2 is fine for us.

Observation 9:

- For the entry condition, there are two cases which from Case#5 to Case#3 and from Case#1 or Case#2 which is not confirmed to Case#3.

- For the exit condition, Case#3 shall enter into Case#5 or Case#1.

Proposal 8:

- If the MR based measurement and LR based measurement can be used and each measurement result shall satisfy each threshold, the UE can exit Case#3.

- If the MR based measurement and LR based measurement can be used and one of measurement results satisfy corresponding threshold, the UE can exit Case#3.

Proposal 9: From RAN4 requirement perspective, not consider combining the measurements across the two radios, i.e., MR and WUR.

- Neighbour cell measurement (MR RRM relaxation)

If  Srxlev ≤ S(non)IntraSearchP or Squal ≤ S(non)IntraSearchQ, it means the service of serving cell is not good. Then MR will perform the neighbour cell measurement. RAN4 shall also consider the MR neighbour cell measurement relaxation. In legacy, RAN2 defined the neighbour cell relaxation condition and RAN4 defined the relevant RRM requirements. In this case, RAN2 is discussing the related relaxed measurement condition as below:

RAN2 will further discuss the neighbor cell measurement relaxation criteria (if the UE is using LR to measure the serving cell), e.g., considering reuse Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’.

If the legacy condition can be reused, we deem that the legacy relaxed measurement requirements for neighbour cell shall be as baseline and RAN4 can discuss the suitable scaling factor.

- Proposal 10: RAN4 shall wait for RAN2 to define the final relaxed measurement conditions for neighbour cell measurement.

- Proposal 11: The legacy intra-/inter-frequency and inter-RAT neighbour cell measurement requirements can be the baseline and RAN4 can study the relaxed scaling factor.

If Srxlev > S(non)IntraSearchP or Squal > S(non)IntraSearchQ, it means the service of serving cell is better and UE is near the center of the current cell. Then MR will not perform neighbour cell measurement without configured high priority frequency.

Observation 10: When UE camps on MR+LR state, MR and LR shall be ON. MR performs serving cell measurement and neighbour cell measurement based on the service of current cell. If the service of serving cell is good, UE MR will not perform the neighbour cell measurement. On the contrary, if service of serving cell is bad, UE MR will perform neighbour cell measurement without high priority frequency.

- For Case#5 (legacy MR state):

In this case, UE stays in fully MR state, that is, only MR is ON and LR is OFF. From UE perspective, UE is far away from the center of the current cell and UE MR will perform the serving cell measurement based on SSB. There is no doubt that the measurement results SS-RSRP and SS-RSRQ will be smaller than the threshold ( Srxlev ≤ S(non)IntraSearchP or Squal ≤ S(non)IntraSearchQ) specified in TS38.304 and TS38.133. In this way, MR will not only measure the current serving cell but the neighbour cell. For neighbour cell measurement, UE shall perform intra-frequency measurement, inter-frequency measurement and inter-RAT measurement . Thus, the MR shall be always ON in case 1.

To my understanding, the legacy requirements shall be reused for case 1 since there is little different from legacy situation.

Observation 11: When UE camps on fully MR state, only MR is ON and LR is OFF. MR performs serving cell measurement and also neighbour cell measurement.

Proposal 12: RAN4 shall reuse the legacy cell re-selection requirements when UE stays in fully MR state.

2.2.2 Cases/States with high priority frequency

In 2.2..1, we just study the different cell re-selection requirements under different cases without high priority frequency, and we clarify the condition of neighbour cell measurement. When UE stays in center area of the current cell and the service of the cell is good enough, the UE will not perform neighbour cell measurement without configured high priority frequency. However, the UE shall search for inter-frequency and inter-RAT layers of high priority if high priority frequency information is provided by the serving cell based on TS 38.133:

If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ then the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7. If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ then the UE shall search for inter-RAT E-UTRAN layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.

- Observation 12: The different cases can be seen as following table with high priority frequency:

| RRM measurement case index | MR serving cell measurement | MR neighbouring cell measurement | LR measurement |
| --- | --- | --- | --- |
| Case#1 | OFF | OFF | ON |
| Case#2 | ON, with measurement relaxation | OFF | ON |
| Case#3 | ON, with measurement relaxation | ON, with measurement relaxation | ON |
| Case#4 | OFF | ON, with measurement relaxation | ON |
| Case#5 | ON | ON | OFF |

- For Case#1:

- In this case, the same situation in case 1 without configured high priority frequency.

- For Case#3:

If Srxlev ≤ S(non)IntraSearchP or Squal ≤ S(non)IntraSearchQ, it means the service of serving cell is not good enough. Then MR will perform the neighbour cell measurement which is similar to case 2 without configured high priority frequency.

If Srxlev > S(non)IntraSearchP or Squal > S(non)IntraSearchQ, it means the service of serving cell is good enough and UE is near the center of the current cell. Then MR will perform inter-frequency measurement with high priority frequency and inter-RAT measurement with high priority frequency if high priority frequency information is provided by the serving cell.

- For Case#4:

UE is in fully LR state and the MR shall be OFF, that is , UE is in the center area of the current cell and the service is good enough. In this case, serving cell has configured the high priority frequency information and UE shall search for high priority frequency layers and perform the neighbour cell measurement. Thus, MR shall be ON from OFF since UE search for the high priority frequency layer at Thigher_priority_search. But only LR performs serving cell measurement. As mentioned before, RAN2 is discussing the neighbour cell measurement relaxation condition and if the condition is the same as legacy, we deem that the legacy relaxed requirement can be as baseline and RAN4 shall only discuss the relaxed scaling factor.

- Observation 13: MR will perform inter-frequency measurement with high priority frequency and inter-RAT measurement with high priority frequency if high priority frequency information is provided by the serving cell even if the service of current cell is good enough.

- Proposal 13: RAN4 shall study the MR neighbour cell measurement relaxation when the high priority frequency is configured. If the legacy relaxed conditions are reused, the legacy relaxed measurement requirements can be the baseline and RAN4 shall study the relaxed scaling factor.

Core requirements to be specified for MR RRM relaxation

As indicated in the latest WID, RAN4 shall specify further RRM relaxation of UE MR for both serving and neighbor cell measurements. Legacy RRM relaxation at Idle/Inactive state have been specified since Rel-16 in the WID of power saving. Here we believe similar solution can be reused for MR RRM relaxation, i.e. introduce relaxation scaling factor.

Issue 2-3-1: MR RRM relaxation factor for serving cell/neighbour cell - Proposals - P1: Relaxation factors within the range from 8 to 16 as the starting point for the relaxation factor for the MR RRM relaxation (vivo) - P2-1: Equal or larger than 8 (CMCC) - P2-2: >=16 (Huawei) - P2: RAN4 should consider relaxation on higher priority frequency layer search requirements based on “conditions” defined by RAN2. (vivo) - P3: Neighbour cell evaluation can be discussed after further input from RAN2 (Samsung)

Based on the TR 38.869, there are two kinds of LP-WUS monitoring which are RRM with duty-cycle LP-WUS monitoring and RRM with continues LP-WUS monitoring. The power saving gain can be seen as the following table:

Table1 Power saving gain based on TR38.869

|  | RRM with duty-cycle LP-WUS | RRM with continues LP-WUS |
| --- | --- | --- |
| No MR RRM relaxed | average: -211% | average: -644% |
| MR relaxed < 8 times | average: 4% | average: 7% |
| 8 times<=MR relaxed<=16 times | average: 40% | average: 32% |
| MR relaxed > 16 times | average: 60% | average: 57% |
| MR offload RRM to LR | average: 83% | average: 74% |

For scenario 1 (No MR RRM relaxed), we find no power saving gain if no MR RRM relaxed. For scenario 3 (MR offload RRM to LR), we can infer that the propagation condition is the best and the MR will stop any RRM measurement and the power saving gain is the best during the whole table. For scenario 2, there are 3 cases can be seen, the different relaxation factors have the different power saving gain and the larger relaxation factors the larger power saving gain.

The above table shows us if the factor is less than 8 times the power saving gain is average 4%. However, if the relaxation factor is equal or larger than 8, the power saving gain has a sharp increase whatever with duty-cycle or continues LP-WUS. Thus, from my perspective, the factor within the range from 8 to 16 times can be the starting point and baseline since the power saving gain in that range is close to half the gain.

Observation 14: If the factor is less than 8 times the power saving gain is average 4%. However, if the relaxation factor is equal or larger than 8, the power saving gain has a sharp increase.

Proposal 14: Relaxation factors within the range from 8 to 16 as the starting point for the relaxation factor for the MR RRM relaxation.

LP-WUR at connected mode

We mainly focus on the measurement requirements in the above. For the connected mode, in the latest WID, we can see that no RAN4’s involvement is required:

- For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Check in RAN#105 for potential TU adjustment in RAN2 - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR

It is obviously the CONNECTED state is not in the scope of the WID.

Proposal 15: RAN4 shall not define the measurement requirements at CONNECTED state.

LP-WUS monitoring

In order to reduce the power consumption, the LP-WUR has been introduced. According to WI, “Specify procedure and configuration design for LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring” involves RAN4. When DRX is configured and there is no traffic transmission, the MR (main receiver) enters a sleep state, only waking up at corresponding moments (DRX ON) to perform paging monitoring. If the specific conditions for entering LR coverage are satisfied, LR allows the UE MR to enter a deep sleep state prior to monitoring the LP-WUS (Low Power Wake-Up Signal). When the UE receives SIB message, the UE initiates monitoring of LP-WUS, subsequently triggering the awakening of the UE MR to monitor PO (paging occasion), this process aims to further mitigate power consumption:

Figure 3 UE behavior upon detecting LP-WUS with paging indication in IDLE/INACTIVE state

RAN1 made the following working assumption regarding entry/exit conditions for LP-WUS monitoring:

Agreement LP-WUS occasions (LOs) are defined for LP-WUS monitoring. - Each LO has one or more LP-WUS monitoring occasions (MOs), where UE can monitors for LP-WUS transmission in each of the LP-WUS MOs. - Different LP-WUS MOs may correspond to different beams in multi-beam operation - It is not precluded that FFS whether or not each LO is defined as a time window that covers the corresponding LP-WUS MOs - FFS details - It is at least supported that a UE monitors LOs with a configured periodicity. Agreement It is supported that the UE monitors the legacy PO after receiving LP-WUS indicating wake-up. Working Assumption From RAN1 perspective, for the entry/exit conditions for LP-WUS monitoring in IDLE/inactive mode, - The UE may start LP-WUS monitoring if - the serving cell measurement performed by the MR is above entry threshold(s), if configured by the gNB, and/or - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied - If UE starts LP-WUS monitoring, it may stop the legacy PO monitoring before UE receives LP-WUS indicating wake-up - The UE monitors the legacy PO (and may monitor PEI) and may stop LP-WUS monitoring if - the serving cell measurement performed by the LR is below exit threshold(s), if configured by the gNB, and/or - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied - FFS the serving cell measurement metrics - The entry/exit thresholds can be configured separately for different types of LR - It is left to RAN2 discussion whether the threshold(s) are always configured by the gNB. - Note: This may be revisited based on the RAN2/RAN4 discussion.

According to the above working assumption, regarding when to monitor the LP-WUS, the downlink channel quality of the serving cell can be monitored and compared with the entry threshold to decide whether to enter the LP-WUS coverage or not. The conditions can be based on UE measurement, e.g. RSRP of signals.

If the entry threshold is satisfied, UE will enter the LP-WUS coverage and turn off MR for paging monitoring at the same time. If the SIB message has been received, UE would like to monitor the LP-WUS and wake up MR to monitor the legacy PO.

On the contrary, if the exit threshold is met, UE will exit the LR coverage and turn on the MR receiver for paging monitoring.

Based on the above analysis, it is crucial that UE shall not miss paging when entry/exit conditions for LP-WUS monitoring are met. RAN4 shall study and define the related requirements on timing from UE receiving SIB message to waking up MR to monitor legacy PO when the entry condition is met. It also can be seen that the time window LP-WUS occasions (LOs) are defined for LP-WUS monitoring, each LO has one or more LP-WUS occasions.

From my perspective, the following shall be considered so that the UE will not miss the paging:

Figure 4 Time delay for paging monitoring

MR wake-up time: UE detects the LP-WUS and decodes it, then LP-WUS wakes up MR to monitor the legacy PO.

When the entry condition for LP-WUS monitoring is met, the time delay shall contain MR wake-up time TMR_wake-up . This time will decide whether LP-WUS can timely wake up MR and ensure that it does not miss the paging monitoring.

On the contrary, UE will stop LP-WUS monitoring and turn on the MR to monitor legacy PO if MR turns off before when the exit condition is met. As we all know, the time delay for MR wake-up also exists during this process.  However, network seldom send the paging message to UE in this scenario, and the network does not page the UE in most cases since UE MR will stay deep sleep state and only LR is ON before UE exits LR coverage, there is even no paging. Thus, this time delay will not cause the bad service or communication delay.

Proposal 16: RAN4 shall clarify the timeline from decoding LP-WUS to MR wake-up in order to monitor legacy PO.

Conclusion

In this paper we provided our views on LPWUS.

Observation 1: Whether to study Case#2 or not shall consider the following reasons:

- MR measurement relaxation and power consumption

- The accuracy of LR measurement

Proposal 1: Case#2 is not necessary to consider.

Observation 2: MR and LR shall both perform serving cell measurement. Only MR performs neighbour cell measurement.

- Proposal 2: RAN4 shall specify the whole procedure based on serving cell measurement and specify the UE behaviour when it satisfies the entry/exit condition:

- When the entry condition is satisfied:

- Option 1: Fully MR →MR+LR →Fully LR

- Option 2: Fully MR →Fully LR

- When the exit condition is satisfied:

- Option 1: Fully LR →MR+LR→ Fully MR

- Option 2: Fully LR→ Fully MR

- Observation 3: When UE camps on fully LR state, MR is OFF and only LR is ON, it means that UE is in the center area of the current cell. UE LR performs serving cell measurement and there is no neighbour cell measurement without high priority frequency.

- Observation 4: In legacy, RAN1 introduced SMTC in order to reduce the complexity of UE measurements because of neighbour cell measurement.

Proposal 3: RAN4 shall define the serving cell measurement requirements based on LP-SS. However, we deem that the time window like SMTC may not be introduced.

Proposal 4: The legacy RRM requirements based on SSB can be the baseline when considering defining the concrete requirements based on LP-SS

Observation 5: In case#3, MR and LR are ON. The serving cell measurement is performed by LR and MR, MR measurement relaxation is aimed at reducing power consumption.

Observation 5: RAN4 only defines the relaxed measurement requirements for neighbour cell not for serving cell.

Proposal 6: RAN4 shall wait for RAN2 to define the relaxed measurement condition for serving cell measurement if it has.

Proposal 6: When UE stays in MR+LR state, RAN4 shall consider the MR serving cell measurement relaxation and also study the mobility performance to quantify the relaxation such as scaling factor for time period.

Observation 7: RAN2 deems that the entry condition for LP-WUS monitoring can be the only MR measurement result or MR measurement result and LR measurement result. However, the exit condition for LP-WUS monitoring can only b e the LR based measurement.

Observation 8: The above interpretation implies that the LR is ON or OFF is up to UE implementation. MR measurement shall be always be used. However, the accuracy of LR measurement is not confirmed, unless one offset for LR measurement is introduced which can reach the similar accuracy as MR based measurement.

Proposal 7: RAN4 shall not consider or study the combined measurement from two receivers for LP-WUS monitoring whatever the relationship between LR based threshold and MR based threshold is.

Observation 9:

- For the entry condition, there are two cases which from Case#5 to Case#3 and from Case#1 or Case#2 which is not confirmed to Case#3.

- For the exit condition, Case#3 shall enter into Case#5 or Case#1.

Proposal 8:

- If the MR based measurement and LR based measurement can be used and each measurement result shall satisfy each threshold, the UE can exit Case#3.

- If the MR based measurement and LR based measurement can be used and one of measurement results satisfy corresponding threshold, the UE can exit Case#3.

Proposal 9: From RAN4 requirement perspective, not consider combining the measurements across the two radios, i.e., MR and WUR.

- Proposal 10: RAN4 shall wait for RAN2 to define the final relaxed measurement conditions for neighbour cell measurement.

- Proposal 11: The legacy intra-/inter-frequency and inter-RAT neighbour cell measurement requirements can be the baseline and RAN4 can study the relaxed scaling factor.

Observation 10: When UE camps on MR+LR state, MR and LR shall be ON. MR performs serving cell measurement and neighbour cell measurement based on the service of current cell. If the service of serving cell is good, UE MR will not perform the neighbour cell measurement. On the contrary, if service of serving cell is bad, UE MR will perform neighbour cell measurement without high priority frequency.

Observation 11: When UE camps on fully MR state, only MR is ON and LR is OFF. MR performs serving cell measurement and also neighbour cell measurement.

Proposal 12: RAN4 shall reuse the legacy cell re-selection requirements when UE stays in fully MR state.

- Observation 12: The different cases can be seen as following table with high priority frequency:

| RRM measurement case index | MR serving cell measurement | MR neighbouring cell measurement | LR measurement |
| --- | --- | --- | --- |
| Case#1 | OFF | OFF | ON |
| Case#2 | ON, with measurement relaxation | OFF | ON |
| Case#3 | ON, with measurement relaxation | ON, with measurement relaxation | ON |
| Case#4 | OFF | ON, with measurement relaxation | ON |
| Case#5 | ON | ON | OFF |

- Observation 13: MR will perform inter-frequency measurement with high priority frequency and inter-RAT measurement with high priority frequency if high priority frequency information is provided by the serving cell even if the service of current cell is good enough.

- Proposal 13: RAN4 shall study the MR neighbour cell measurement relaxation when the high priority frequency is configured. If the legacy relaxed conditions are reused, the legacy relaxed measurement requirements can be the baseline and RAN4 shall study the relaxed scaling factor.

Observation 14: If the factor is less than 8 times the power saving gain is average 4%. However, if the relaxation factor is equal or larger than 8, the power saving gain has a sharp increase.

Proposal 14: Relaxation factors within the range from 8 to 16 as the starting point for the relaxation factor for the MR RRM relaxation.

Proposal 15: RAN4 shall not define the measurement requirements at CONNECTED state.

Proposal 16: RAN4 shall clarify the timeline from decoding LP-WUS to MR wake-up in order to monitor legacy PO.

References

[1] RP-221018, New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

[2] TR 38.869