# 3GPP TSG-RAN WG4 Meeting #112bis                                                       R4-2416370

# HeFei, China, 14th – 18rd October, 2024

Source:	ZTE Corporation, Sanechips

Title:	Discussion on RRM requirements for NTN enhancement

Agenda item:	6.8.3.4

Document for:	Approval

Introduction

At RAN 104# meeting, the revised WID on NR NTN evolution [1] has been approved, and several objectives in the WID have RAN4 RRM involvement. In the last meeting we provided our initial views on potential impacts on RRM requirements and in this meeting we will continue to study the concrete RRM requirements.

Discussion

The objectives of Rel-19 in NR NTN evolution are described in [1] including:

2.1 Less than 5MHz for NTN

- In this meeting, we will discuss the scenarios and express our views on potential impact to RRM requirements towards the above objective

  - 2.1 Less than 5MHz for NTN

Less than 5MHz for NTN - Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands - Consider the NR-NTN bands n255, n256, and n254 - Add less than 5MHz channel bandwidth - 3MHz with the RAN1 design in Rel-18 - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth - Leverage the work on less than 5MHz for TN - Note: no RAN1 impact is expected - Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands - Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Focus on power class 3 (PC3) for Tx RF requirements - Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary signaling to support the above objectives

- In R18, RAN4 just discussed less than 5MHz feature for TN, and the less than 5MHz feature will be applied to NTN scenario.

2.1.1 Baseline assumption

In legacy less than 5MHz in TN, RAN4 deems that the PDCCH transmission bandwidth parameter is 12PRB or 15PRB for 3MHz and 12PRB for SS/PBCH assumption. However, RAN4 reached the agreement for less than 5MHz in NTN as below:

Channel bandwidth assumption Agreement: - To align with the agreement in BDaT session. SSB/PBCH assumption Agreement - To align with the agreement in BDaT session. CORRSET Agreement - To align with the agreement in BDaT session.

The system parameters shall be defined in BDaT session and RRM session shall follow the agreements defined in BDaT session:

Issue 1-1-1: Channel bandwidth - Agreement - Only introduce 3MHz channel bandwidth with 15kHz SCS Issue 1-1-2: Operating bands for 3MHz channel bandwidth - Agreement - Applicable NTN bands: band n256, n255 and n254 Issue 1-1-3: Maximum transmission bandwidth configuration(NRB) - Agreement - Reuse 15 RBs for 3MHz channel bandwidth with 15kHz SCS for NR NTN.

For channel bandwidth, RAN4 RRM shall only study 3MHz channel bandwidth and the concrete SCS is 15kHz which has been agreed in the last meeting.

For SS/PBCH detection, it is 12PRBs without change which is similar to legacy.

For downlink transmission bandwidth parameter, it shall be only 15PRBs without 12PRBs. In legacy TN, the BW is 12PRBs for 3MHz case and band n100. In the above table, we find that the operating bands for 3MHz channel bandwidth in NTN is band n256, n255 and n254 without n100. Thus, the downlink transmission bandwidth shall be 15 PRBs only since 15PRB is not limited in NTN bands.

Observation 1: For RRM requirements, the operating bands of 3MHz channel bandwidth with 15kHz SCS in NTN is n255, n256 and n254.

Proposal 1: RAN4 RRM shall only introduce 3MHz channel bandwidth with 15kHz SCS for less than 5MHz in NTN.

Proposal 2: For SS/PBCH assumption, PBCH is punctured to 12PRBs which is same as legacy.

Proposal 3:  For downlink transmission bandwidth, reuse 15PRBs for 3MHz channel bandwidth with 15kHz SCS for less than 5MHz in NTN scenario.

2.1.2 Requirements for NTN less than 5MHz

In the last meeting, we had the agreements as below:

Agreement: An applicability requirement is needed to clarify which R17/R18 NTN requirement can be applied for less than 5MHz band without any change in R19. Agreement: RRM requirement for TN less than 5MHz shall be used as baseline to design NTN less than 5MHz requirement

First of all, the requirements for less than 5MHz in TN shall be baseline when we considering to define the requirements in NTN scenario. If there is no requirements change in R19, we shall add some clarifications.

Next, we shall study potential impacts based on the contents in the following table:

| RRM requirements | NTN requirements in TS38.133 v18.5.0 | Possible impacts if BW below 5MHz |
| --- | --- | --- |
| IDLE/inactive mode mobility | 4.2C Cell Re-selection for NR UE for Satellite Access | No RRM requirements impact |
| RRC connection mobility control | 6.2C RRC Connection Mobility Control for Satellite Access | - RRC Re-establishment - RRC connection release with redirection for satellite access |
| Handover | 6.1C Handover for SAN | - Time/location-based CHO delay - Satellite switch time - Handover delay(RACH-based and RACH-less) |
| UE Tx timing, MTTD/ MRTD, timer accuracy, TA accuracy | 7.1C, 7.2C, 7.3C | - Transmit timing (Te) - UE timer accuracy - Timing advance for satellite access |
| RLM | 8.1C Radio Link Monitoring for Satellite Access - 8.5C Link Recovery Procedures for Satellite Access | - Radio link monitoring (only SSB based) - Link recovery procedures for satellite access - Candidate beam detection (CBD) |
| Measurements |  | L3 measurement |

2.1.2.1 RLM/BFD/CBD

For RLM/BFD/CBD, we shall consider two points which are evaluation period requirements and hypothetical PDCCH transmission parameters.

2.1.2.1.1 PDCCH transmission bandwidth parameters

In legacy TN, RAN4 just discussed the PDCCH transmission parameters as below:

| Attribute | Value for BLER Configuration #0 |  |  |
| --- | --- | --- | --- |
| Channel BW | 3MHz |  | 5MHz |
| [DL Transmission BW] | 12 PRBs | 15 PRBs | 20 PRBs |

- We find that the BW is 12PRBs and 15PRBs for SSB based RLM in 3MHz case and also 20PRBs in 5MHz. 12PRBs and 15PRBs are enough to covering transmission of PSS/SSS which can effectively and continuously monitor the performance of radio link for RLM/BFD/CBD in legacy. However, only 15PRBs for 3MHz channel bandwidth with 15kHz SCS is enough as we mentioned before for less than 5MHz in NTN.

- Proposal 4: The PDCCH transmission bandwidth for RLM/BFD/CBD shall be 15PRBs for 3MHz.

2.1.2.1.2 Evaluation period

- The evaluation period for RLM/BFD/CBD has also been discussed in legacy：

RLM OOS evaluation period: Agreement: For SSB-based OOS in 3MHz, RAN4 will keep the existing SSB based RLM evaluation periods TEvaluate_out_SSB for CBW less than 5MHz RLM IS evaluation period: Agreement: for SSB-based IS in 3MHz, RAN4 will keep the existing SSB based RLM evaluation periods TEvaluate_in_SSB for CBW less than 5MHz Beam Failure Detection evaluation period: Agreement: for SSB-based BFD in 3MHz, RAN4 will keep the existing SSB based BFD evaluation periods TEvaluate_BFD_SSB for CBW less than 5MHz Candidate beam detection evaluation period: Agreement: for SSB-based CBD in 3MHz, RAN4 will apply the existing SSB based CBD evaluation period TEvaluate_CBD_SSB for CBW less than 5MHz.

- In legacy, RAN4 kept all the existing SSB related RLM/BFD/CBD evaluation period for CBW less than 5MHz.Thus, we  consider that the legacy RLM/BFD/CBD evaluation period for less than 5MHz in TN can be reused when defining the requirements for less than 5MHz in NTN scenario.

- Observation 2: In legacy, RAN4 kept all the existing SSB related RLM/BFD/CBD evaluation period for CBW less than 5MHz.

- Proposal 5: The legacy RLM/BFD/CBD evaluation period for less than 5MHz in TN can be reused when defining the related requirements for less than 5MHz in NTN scenario.

2.1.2.2 RRC Re-establishment/ RRC Connection release with redirection

- For this issue, the main requirements is the time for identifying the target NR cell in legacy TN and NTN which based on the SSB side conditions, the serving cell SSB Ês/Iot≥-8dB. However, we do not find the change of the SSB side conditions if the RRM requirement will be reused and even there is no RRM requirements for less than 5MHz in TN in spec []. Thus, we deem that for RRC Re-establishment/RRC connection Release with re-direction, there is no RRM impacts on 3MHz in NTN scenario.

- Observation 3: There is no RRM requirements for less than 5MHz in TN.

- Proposal 6: For RRC re-establishment and RRC connection release with redirection, there is no RRM impacts on  3MHz in NTN scenario.

2.1.2.3 Timing requirements

- In R17/R18 NTN timing related discussion, the UE timing including UE transmit timing, timing advance adjustment accuracy etc. The main concerns for NTN timing error (Te) is coming from time difference and Doppler shift instead of the channel bandwidth. Thus, we do not find any enhancement for timing requirements for less than 5MHz in NTN.

- Observation 4: The main concerns for NTN timing error (Te) is coming from time difference and Doppler shift instead of the channel bandwidth and we do not find any enhancement for timing requirements for less than 5MHz in NTN.

- Proposal 7: RAN4 shall reuse the existing NTN timing requirements for less than 5MHz in NTN.

2.1.2.5 Handover/Conditional handover/Satellite switch

In the last meeting, we reached the agreements as below:

Agreement: Define HO delay requirement for NTN less than 5MHz, and take extension of delay for TN less than 5MHz as baseline. For NTN specific HO requirement: - Time/location-based CHO delay requirement will be defined for NTN less than 5MHz. - Satellite switch time requirement will be defined for NTN less than 5MHz. - Further discuss the details of the requirements.

We will study the related delay requirements for handover.

2.1.2.5.1 Handover

When we decide to analyze the handover in NTN scenario, RACH-based and RACH-less shall be studied. The main difference is the time the UE starts transmission of the new PRACH or PUSCH. The main different component is TIU,

Tinterrupt = Tsearch + TIU + Tprocessing  + T∆ + Tmargin ms

In RACH-based handover, TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell.

In RACH-less handover, TIU is the interruption uncertainty in acquiring the first UL transmission resource.

For the other components, they have the same understanding without change. Now, we mainly study the punctured PBCH scenario, we shall study the each component which will be impacted by punctured PBCH. Based on the definition of each component, we find Tsearch and TΔ shall be studied without TIU since they depends on the SSB index reading. Thus, we deem that the enhancement shall be considered for these two parameters. We also find the legacy agreement for less than 5MHz in TN as below:

- Agreement: - T∆ = Trs for both known and unknown target cells operating with 20 PRB SSB BW. - T∆ = 3*Trs for both known and unknown target cells operating with 12 PRB SSB BW.

Since the PBCH has been partially punctured,the time for fine time tracking and acquiring full timing information of the target cell shall be extended which is three times as larger than the legacy TN in order to reduce the fine time tracking inaccuracy. We would like to provide the table as below:

|  | Legacy TN | Legacy 3MHz in TN | Legacy FR1-NTN |
| --- | --- | --- | --- |
| Tsearch | Intra-fre cell: Trs Inter-fre cell: 3*Trs | Intra-fre cell: Trs Inter-fre cell: 3*Trs | Intra-fre cell: Trs Inter-fre cell: 3*Trs |
| T∆ | Trs | 3*Trs | Trs |

Based on the above two tables and ahead analysis, although Tsearch and TΔ are both impacted, only TΔ was updated and Tsearch maintains the original value under the same condition ( Ês/Iot≥-2dB) . From my perspective, actually Tsearch contains TΔ is to search the target cell and perform the time domain synchronization. Thus, one of them shall be enhanced is fine for us. However, we shall reuse the requirements which defined in less than 5MHz in TN since we agreed TN less than 5MHz shall be used as baseline to design NTN less than 5MHz.

Agreements: RRM requirement for TN less than 5MHz shall be used as baseline to design NTN less than 5MHz requirement

Observation 5: The main difference between RACH-based and RACH-less handover is the time the UE starts transmission of the new PRACH or PUSCH. The main different component is TIU, and other components have the same understanding without change.

Observation 6: Tsearch and TΔ shall be studied without TIU since they depend on the SSB index reading.

Observation 7: Although Tsearch and TΔ are both impacted, only TΔ was updated and Tsearch maintains the original value under the same condition ( Ês/Iot≥-2dB) in legacy less than 5MHz.

Observation 8: RRM requirements for TN less than 5MHz shall be used as baseline to design NTN less than 5MHz.

Proposal 8: RACH-based handover and RACH-less handover shall have the same enhancement.

Proposal 9: Only TΔ shall be updated for NTN less than 5MHz and the concrete value is same as legacy TN less than 5MHz which is 3*Trs.

2.1.2.5.2 Conditional Handover

We also find there is the impact on conditional handover and RAN4 shall consider studying it. Conditional handover contains two parts: Conditional handover with L3 measurement in NTN and Conditional handover without L3 measurement in NTN. We will study them respectively.

Conditional handover delay with L3 measurement

UE shall start RRM measurement before the time or distance condition is met. For the conditional handover delay with L3 measurement, the main handover delay is:

DCHO = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution

And the definition of Tinterrupt is:

Tinterrupt = Tprocessing + TIU + T∆ + Tmargin ms

Each component has the different definition and we shall focus on the parameters which will be impacted by punctured PBCH. From my perspective, only Tmeasure and Tinterrupt will be impacted since they depend on PBCH detection. From the TS38.133, we know that the measurement time delay L3 filtering shall be less than Tidentify_inter_with_index or Tidentify_inter_without_index defined in clause nine which is related to TSSB time index and TPSS/SSS sync. For Tinterrupt, we have already analyzed in handover delay which depends on SSB index reading.

We find that the value of TΔis not changed and Tmeasure has been defined for TN less than 5MHz which refers to intra-frequency and inter-frequecny cell identification. Thus, we deem that conditional handover shall be enhanced and the Tmeasure shall be enhanced if RAN4 defines new L3 measurement requirements.

Observation 9: Tmeasure and Tinterrupt will be impacted since they depend on PBCH detection.

Observation 10: The value of TΔis not changed and Tmeasure has been defined for TN less than 5MHz which refers to L3 measurement.(Intra-frequency and Inter-frequecny cell identification)

Proposal 10: Tmeasure in conditional handover with L3 measurement shall be enhanced and the concrete values shall follow intra-/inter-frequency cell identification if RAN4 defines the new measurement requirements.

Proposal 11: TΔ shall maintain the original value and shall not be changed.

Conditional handover delay without L3 measurement criteria

UE is allowed to execute time-based and location-based conditional handover without RRM measurement once the time or distance condition is met. For the conditional handover delay without L3 measurement, the main handover delay is:

DCHO = TRRC + TEvent_DU + Tinterrupt + TCHO_execution

Compared to conditional handover with L3 measurement, the above formulation is lack of Tmeasure and also the definition of Tinterrupt is different as below:

Tinterrupt = Tprocessing + Tsearch + TIU + T∆ + Tmargin ms

Based on the ahead analysis, only Tinterrupt will be impacted including Tsearch and TΔ. And these two parameters are same as the handover we analyzed before. However, there is no change for Tsearch and TΔsince there is no conditional handover delay without L3 measurement requirements in legacy. From my perspective, TΔ shall also be changed to 3*Trs as legacy TN less than 5MHz in handover delay.

Observation 11: Only Tinterrupt will be impacted including Tsearch and TΔ.

Observation 12: There is no change for Tsearch and TΔsince there is no conditional handover delay without L3 measurement requirements in legacy.

Proposal 12: For NTN less than 5MHz, TΔ shall also be changed to 3*Trs as legacy TN less than 5MHz in handover delay.

2.1.2.5.3 Satellite switch

For satellite switch, this is NTN specific requirements and we have hard satellite switch and soft satellite switch.

The purpose of NR SAN Satellite switching with re-synchronization is to switch satellite in the same SSB frequency and same gNB without PCI changing. The requirements in this clause are applicable to SA NR SAN. The requirements in this clause are applicable to both hard and soft switch over in quasi-earth fixed scenario from NR SAN FR1 cell to NR SAN FR1 cell.

For hard satellite switch, when the UE switches from the current serving satellite to a new satellite, it first disconnects from the original satellite before establishing a connection with the new one.

For soft satellite switch, it allows the UE to maintain connections with multiple satellites simultaneously. During the handover process, the UE first establishes a communication link with the target satellite and only disconnects from the original satellite after ensuring the new connection is stable.

For different cases, the satellite switch delay is different.In this section, We will study both of them respectively.

Hard satellite switch delay

For hard satellite switch delay, the main delay can be seen as below:

Figure1  Hard satellite switch delay

The interruption time shall be less than Tinterrupt

Tinterrupt = Tsearch + TIU + Tprocessing  + T∆ + Tmargin ms

We know that the interruption time is the time between t-service and the time the UE starts transmission of the new PRACH. When PBCH is punctured, the Tsearch and TΔ may be impacted, and TΔ shall be extended to 3*Trs which is the same as the handover.

Observation 13: For hard satellite switch delay, Tsearch and TΔ may be impacted.

Proposal 13: TΔ shall be extended to 3*Trs which is the same as the handover.

Soft satellite switch delay

For soft satellite switch delay, the main delay can be seen as below:

Figure 2 Soft satellite switch delay

The soft satellite switch time shall be less than Tsoft_switch

Tsoft_switch = max(t-service-t-seviceStart, Tsearch + T∆ + Tmargin) + TIU + Tprocessing  ms

- We know that the soft satellite switch delay includes two parts: Sync to target satellite and the interruption. For the first part, it includes full or partial Tsearch + T∆ + Tmargin or t-service-t-serviceStart. The t-service-t-serviceStart is up to network configuration which is not impacted by punctured PBCH which is the service time window. For Tsearch + T∆ + Tmargin, if the delay of Tsearch + T∆ + Tmargin  is larger than t-service-t-serviceStart, the interruption will also contain partial Tsearch + T∆ + Tmargin  and the soft satellite switch delay will be much longer than the case which the delay of Tsearch + T∆ + Tmargin  is smaller than t-service-t-serviceStart. Thus, we deem that the soft satellite switch delay depends on the sync time. As the mentioned above, Tsearch + T∆ will be impacted by punctured PBCH, and TΔ  will be extended to 3*Trs.

- Observation 14:  The soft satellite switch delay includes two parts: Sync to target satellite and the interruption.

- Proposal 14: Tsearch + T∆ will be impacted by punctured PBCH, and TΔ  will be extended to 3*Trs.

2.1.2.6  L3 measurement

- For the measurement requirements for both intra-frequency and inter-frequency, the main factors to be impacted is the cell and SSB index detection since it depends on the PBCH detection directly. Hence, we shall also focus on this aspect also. Compared to legacy SSB with 20PRBs, the 12PRBs SSB can guarantee the transmission of PSS/SSS but part of PRBs of PBCH are punched. Thus, the new measurement requirements for less than 5MHz have been defined such as cell identification etc. We also can see the following agreements defined in less than 5MHz WI:

Agreement: - SSB index identification delay - Unknown intra-frequency cell (Es/Iot≥-6 dB): [3+4] samples - Unknown inter-frequency cell (Es/Iot≥-4 dB): [3+3] samples

- For unknown cell, we find that the measurement samples were increased based on 12PRBs SSB in order to  compensate the impact of puncturing for unknown cell searching, and the relevant requirements have been captured in the existing TS38.133 as below:

- Legacy TN time index detection requirements: Table 9.2.5.1-3: Time period for time index detection (FR1) DRX cycle; TSSB_time_index_intra. No DRX; max(120ms, ceil( 3 x Kp ) x SMTC period)Note 1 x CSSFintra. DRX cycle≤ 320ms; max(120ms, ceil (M2 Note 2 x 3 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; Ceil(3 x Kp) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1 NOTE 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured and UE supports measurementEnhancementCA-r17, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1. - Legacy TN less than 5MHz time index detection requirements: Table 9.2.5.1-23: Time period for time index detection for a UE operating on a target cell with 12 PRB SSB (Frequency range FR1) DRX cycle; TSSB_time_index_intra_less_than_5Mhz. No DRX; max(120ms, 7 x max(MGRP, SMTC period)) x CSSFintra_less_than_5Mhz. DRX cycle≤ 320ms; max(120ms, ceil(1.5 x 7) x max(MGRP, SMTC period,DRX cycle) x CSSFintra_less_than_5Mhz). DRX cycle>320ms; 7 x max(MGRP, DRX cycle) x CSSFintra_less_than_5Mhz. NOTE 1: FFS When highSpeedMeasInterFreq-r17. - Legacy NTN time index detection requirements: Table 9.2C.5.1-2: Time period for time index detection (FR1) DRX cycle; TSSB_time_index_intra. No DRX; max(120ms, ceil( 3 x Kp x Klayer1_measurement) x Kmulti_SMTC x SMTC period)Note 1 x CSSFintra_less_than_5Mhz. DRX cycle≤ 320ms; max(120ms, ceil (1.5 x 3 x Kp x Klayer1_measurement) x Kmulti_SMTC x max(SMTC period,DRX cycle)) x CSSFintra_less_than_5Mhz. DRX cycle>320ms; Ceil(3 x Kp x Klayer1_measurement) x Kmulti_SMTC x DRX cycle x CSSFintra_less_than_5Mhz. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified.

- Based on the table in above, we find that the time period for time index detection for NTN is different from legacy considering Klayer1_measurement (considering without measurement gap),Kgap (considering with measurement gap) , Kmulti_SMTC and CSSFintra_less_than_5Mhz. However, the number of samples is same. Hence, from my perspective, when considering defining the requirements for less than 5M in NTN scenario, RAN4 shall refer to time period defined in less than 5M with Klayer1_measurement , Kmulti_SMTC and Kgap.

Time period for time index detection for a UE operating on a target cell with 12 PRB SSB (Frequency range FR1) for intra-frequency without measurement gap

| DRX cycle | TSSB_time_index_intra_less_than_5Mhz |
| --- | --- |
| No DRX | max(120ms, 7 x max(MGRP, SMTC period))x Klayer1_measurement x Kmulti_SMTC x SMTC period)Note 1 x CSSFintra_less_than_5Mhz |
| DRX cycle≤ 320ms | max(120ms, ceil(1.5 x 7) x Klayer1_measurement x Kmulti_SMTC x max(MGRP, SMTC period,DRX cycle) x CSSFintra_less_than_5Mhz) |
| DRX cycle>320ms | 7 x max(MGRP, DRX cycle) x Kmulti_SMTC x CSSFintra_less_than_5Mhz |
| NOTE 1: FFS When highSpeedMeasInterFreq-r17 NOTE 2: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Time period for time index detection for a UE operating on a target cell with 12 PRB SSB (Frequency range FR1) (Frequency range FR1) for intra-frequency with measurement gap

| DRX cycle | TSSB_time_index_intra_less_than_5Mhz |
| --- | --- |
| No DRX | max(120ms, 7 x Kgap x Kmulti_SMTC x max(MGRP, SMTC period)) x CSSFintra_less_than_5Mhz |
| DRX cycle≤ 320ms | max(120ms, ceil(1.5 x 7) x Kgap x Kmulti_SMTC x max(MGRP, SMTC period,DRX cycle) x CSSFintra_less_than_5Mhz) |
| DRX cycle>320ms | 7 x Kgap x Kmulti_SMTC x max(MGRP, DRX cycle) x CSSFintra_less_than_5Mhz |
| Note 1: CSSFintra_less_than_5Mhz is 1 NOTE 2: FFS When highSpeedMeasInterFreq-r17 |  |

- We just provide the time period for time index detection for intra-frequency without/with measurement gap. The same situation can be studied for inter-frequency measurement. RAN4 shall refer to the legacy less than 5MHz requirements and legacy NTN requirements when considering defining the new requirements.

- Proposal 15: RAN4 shall define the related requirements for less than 5MHz in NTN scenario with considering Klayer1_measurement , Kmulti_SMTC, Kgap (intra-frequency with/without measurement gap) and CSSFintra_less_than_5Mhz :

Time period for time index detection for a UE operating on a target cell with 12 PRB SSB (Frequency range FR1) for intra-frequency without measurement gap

| DRX cycle | TSSB_time_index_intra_less_than_5Mhz |
| --- | --- |
| No DRX | max(120ms, 7 x max(MGRP, SMTC period))x Klayer1_measurement x Kmulti_SMTC x SMTC period)Note 1 x CSSFintra_less_than_5Mhz |
| DRX cycle≤ 320ms | max(120ms, ceil(1.5 x 7) x Klayer1_measurement x Kmulti_SMTC x max(MGRP, SMTC period,DRX cycle) x CSSFintra_less_than_5Mhz) |
| DRX cycle>320ms | 7 x max(MGRP, DRX cycle) x Kmulti_SMTC x CSSFintra_less_than_5Mhz |
| NOTE 1: FFS When highSpeedMeasInterFreq-r17 NOTE 2: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Time period for time index detection for a UE operating on a target cell with 12 PRB SSB (Frequency range FR1) (Frequency range FR1) for intra-frequency with measurement gap

| DRX cycle | TSSB_time_index_intra_less_than_5Mhz |
| --- | --- |
| No DRX | max(120ms, 7 x Kgap x Kmulti_SMTC x max(MGRP, SMTC period)) x CSSFintra_less_than_5Mhz |
| DRX cycle≤ 320ms | max(120ms, ceil(1.5 x 7) x Kgap x Kmulti_SMTC x max(MGRP, SMTC period,DRX cycle) x CSSFintra_less_than_5Mhz) |
| DRX cycle>320ms | 7 x Kgap x Kmulti_SMTC x max(MGRP, DRX cycle) x CSSFintra_less_than_5Mhz |
| Note 1: CSSFintra_less_than_5Mhz is 1 NOTE 2: FFS When highSpeedMeasInterFreq-r17 |  |

Conclusion

In this paper we provided our views on RRM requirements on R18 NR NTN enhancement.

- 4 References

- RP-240857, New WID: Enhanced requirements and test methodology for NR and IoT NTN.

- TS38.133