3GPP TSG RAN WG4 #112bis   						         			                    R4-2416371

Hefei, China, October 14th -18th, 2024

Source:	ZTE Corporation, Sanechips

Title:	Discussion on LS on common TA in a regenerative payload scenario

Agenda item:	6.26.4.2

Document for:	Approval

Introduction

              - In the incoming LS [1] from RAN2, the following questions and action are listed to clarify the value of the ta-Common in regenerative case:

# 1. Overall Description: In RAN2#127 meeting, the setting of common TA and Kmac for regenerative payload with full gNB on board was discussed, and a question related to the value setting was raised. Related to this, RAN2 kindly asks RAN1 and RAN4: Question: Whether in a regenerative payload scenario it would be a problem to stick to 0 as the minimum possible value for TA-Common or whether we should e.g. introduce negative values for ta-Common. Additionally, RAN2 understands in any case legacy UEs would have to rely on existing signaling and then for legacy UEs, minimum value of Common TA is equal to 0. # 2. Actions: # To RAN1 and RAN4: # RAN2 kindly requests RAN1 and RAN4 to provide feedback on above question.

              - In this contribution, the views and initial replies are elaborated.

Discussion

The LS specifies the scenario is the regenerative mode and study on its timing issues. First of all, we shall recall the timing requirements and then analyze the regenerative mode by comparison. In legacy, RAN4 has already defined the timing requirements for NR NTN as below:

The UE shall have capability to follow the frame timing change of the reference cell in connected state. The uplink frame transmission takes place NTA+NTA-offset+NTA,adjcommon+NTA,adjUE×Tc before the reception of the first detected path (in time) of the corresponding downlink frame from the reference cell. UE initial transmit timing accuracy and gradual timing adjustment requirements are defined in the following requirements.

The scenario for transparent payload can be seen as below:

Figure 1 Timing of transparent payload

We find the reference point is located at feeder link, however, RAN4 did not preclude the reference point is at satellite or gNB in legacy.

This timing requirement comprises four parts:

NTA: The value NTA of timing advance is indicated in Msg2 which is the timing adjustment be network.

NTA-offset: A UE can be provided a value NTA-offset of timing advance offset for a serving cell.

NTA,adjcommon: It is derived from the higher-layer parameters contained assistance information for the serving cell. It is the 			network-controlled common timing advanced value between reference point and satellite on feeder link.

NTA,adjUE: UE determines it by using the serving satellite position and its own position (GNSS capability) on service link. 			For the serving satellite position, it is calculated by UE based on higher-layer ephemeris parameters for a serving satellite.

The NTN UE has the GNSS capability which can obtain its own position and the SIB19 including ephemeris are configured to UE, then UE can combine with these information to calculate the distance between UE itself and the satellite, that is, this is up to UE implementation. When UE would like to access the network, these information can be sent to network via Msg1. Then, the network will send the Msg2 to UE and tell UE that the value of timing advance shall be adjusted. UE receives the timing advance adjustment via Msg2, it will transmit uplink signals to network with concrete timing advance value. However, the reference point is located at feeder link, the timing between the satellite and reference point shall also be compensated. The compensated value is configured by network which is the common TA as shown in figure 1. Based on the analysis above, we know that only UE specific TA is up to UE implementation and the rest of  parameters are configured by network.

Next, the LS specifies the scenario shall be the regenerative mode, the following figure can be seen:

Figure 2 Timing of regenerative mode

Compared to legacy transparent payload, the gNB is on the satellite and there is no feeder link compared to legacy transparent payload. The same thing is that the network will send the Msg2 to UE and tell UE that the value of timing advance shall be adjusted. UE receives the timing advance adjustment via Msg2, it will transmit uplink signals to network with concrete timing advance value. The main difference is the common TA equals 0 which is common understanding for us  based on the RAN2’s agreement, that is, network shall configure common TA=0 to UE if this component is not excluded.

RAN2 assumes that the typical setting for common TA and kmac would be zero in case of regenerative mode with full gNB on board. FFS whether we capture in the spec that common TA and Kmac shall/should be set to zero.

Thus, timing advance adjustment will depend on NTA and UE specific TA for regenerative mode. The accuracy of the UE specific TA compensation for service link, which is derived based on the satellite ephemeris and UE location, it should be ensured by the UE without any over-compensation or under-compensation to satisfy the timing requirements. Since UE with GNSS capability can be realized in TS38.171, the concrete GNSS values have already been listed and Te requirements are calculated refer to the values in TS38.171:

| Scenario Type | Weak satellite signal conditions | Ideal conditions | Satellites with rather different signal levels | Multi-path Scenario | Moving scenario with periodic update |
| --- | --- | --- | --- | --- | --- |
| Position Error | 100 m | 15 m | 100 m | 100 m | 50 m |

Also, we RAN4 have already agreed that the ephemeris information in SIB19 shall be updated with certain period which is default acknowledgement in order to guarantee accuracy of satellite position and Te requirements.

In the discussion on R17 NR NTN, the UE UL timing accuracy requirements can be formulated as: Te_NTN = Te_TN + Te_GNSS + Te_SAT , where - Te_TN: Legacy TN UE UL Timing Accuracy - Te_GNSS is the GNSS accuracy, Te_GNSS = 2* (GNSS positioning accuracy/c); - Te_SAT is the serving-satellite position estimation error, Te_SAT = 2* (serving-satellite positioning estimation accuracy /c)

Thus, we do not think the normal UE will experience the over-compensation situation since the factors of UE timing pre-compensation are all considered precisely and network can also help UE to adjust timing via Msg2. Therefore, there is no need to consider the negative values for common TA.

Observation 1: The GNSS capability and SIB19 information for UE timing pre-compensation are all considered precisely and network can also help UE to adjust timing via Msg2.

Proposal1: There is no need to consider the negative values for common TA.

Conclusion

In this paper we provided our views on LS from RAN2 on common TA in a regenerative mode.

Observation 1: The GNSS capability and SIB19 information for UE timing pre-compensation are all considered precisely and network can also help UE to adjust timing via Msg2.

Proposal 1: There is no need to consider the negative values for common TA. The concrete values for common TA shall be defined in RAN1.

- References

- TS38.171

- TS 38.133

- R2-2407623	LS on common TA in a regenerative payload scenario, CMCC